Source:	Orange

Title:	On FS_ULBC application scenarios

## Document for:	Agreement

## Agenda Item:	7.9

- Introduction

At SA#107, the “Study on Ultra Low Bitrate Speech Codec” (FS_ULBC) has been approved. In the present Tdoc we address the following study objective from the WID description [1]

- Document the application scenarios for ultra-low bit rate communication services taking into account the use cases and potential requirements documented in TR 22.887 related to IMS Voice Call Using GEO Access

The present Tdoc elaborates the extra scenario proposed in [2] taking into account some offline comments.

- Proposal

See Annex, including editorial updates and a discussion on the impact of including a ULBC codec in IMS voice services.

## References

- 3GPP SP-250378, SID on Ultra Low Bitrate Speech Codec, Source: China Mobile Com. Corporation, vivo, Fraunhofer IIS, Qualcomm Incorporated, Spreadtrum, Dolby Laboratories Inc., Xiaomi, Huawei

- 3GPP S4aA250025, On FS_ULBC application scenarios, Source: Orange

## Annex (pCR to 26.940)

* * * First Change * * * *

# 4	Application scenarios

[…]

## 4.3	Scenarios with IMS Voice Call with ULBC over other access types than GEO

### 4.3.0	General

By introducing a new codec (ULBC) for voice communication via Geostationary Earth Orbit (GEO) satellites as discussed in clause 4.2, IMS voice calls over other access may be impacted.

### 4.3.1 	Scenario Description

#### 4.3.1.1 	General

This scenario deals with the handling of ULBC codec in an IMS call between two UEs, UE1 and UE2, supporting ULBC but connected with no GEO access. The IMS voice call service is considered here between UEs using an access other than GEO satellite access (e.g., LTE, NR, WLAN).

#### 4.3.1.2 	Scenario 3: Both UEs support ULBC and connect via a 3GPP access other than GEO satellite access

Both UEs are on a terrestrial 3GPP mobile network (e.g., LTE, NR, WLAN), as outlined in Figure 4.3.1.2-1, where a sketch of the bi-directional voice data flow for this scenario is depicted for the LTE case.

Figure 4.3.1.2-1: Bi-directional voice data flow for scenario 3 for LTE

Option#1: derive high level requirements from SA4’s perspective

### 4.3.2	Derived high-level prerequisites

#### 4.3.2.1	General

The following general prerequisites for the ULBC apply based on application scenario 3.

Editor’s note: Some high-level prerequisites could be added such as: To serve application scenario 3, the ULBC codec is expected to meet the following high-level prerequisites:

-	Similar functionalities as existing 3GPP voice codecs (AMR, AMR-WB, EVS) including DTX, error concealment.

-	Implementable in real-time (encoding and decoding) on at least a selective set of smartphones (battery impact to be checked)

-	Audio quality at least on par with existing voice services

Considerations on whether existing audio quality tests (TS 26.131 and TS 26.132) may be considered.

Considerations on SDP handling of ULBC codec to be discussed

Option#2: Derive technical assumptions and open issues

### 4.3.2	Extracted technical assumptions

#### 4.3.2.1	General

The following general assumptions apply.

-	As the UE may be a “regular” smartphone, the ULBC needs to be implementable in real-time (encoding and decoding) on such devices and meet common complexity and power-consumption requirements.

-	The voice quality preferably meets the user expectation for deployed terrestrial voice services.

Editor’s Note: More details may be added. Considerations on whether existing audio quality tests (TS 26.131 and TS 26.132) may be considered.

#### 4.3.2.2	Assumptions for Scenario 3

The following assumptions apply for Scenario 3 described in clause 4.3.1.2.

-	The bit rate of ULBC needs to match operation on the An ultra-low bit rate codec fitting this transmission data rate for these links.

-	This scenario may operate in transcoding-free mode as ULBC may be negotiated used end-to-end.

Editor’s Note: More details may be added. Roaming and interworking scenarios may be also considered.

* * * End of Changes * * * *