3GPP TSG-SA WG4 Meeting #132						S4-251003

Japan, Fukuoka, 19 – 23 May 2025

Source:	HEAD acoustics GmbH

Title:	Analysis of nominal send loudness rating

Agenda item:	7.7

Document for:	DISCUSSION

# Introduction

For the loudness in send direction of IVAS-capable UEs, it is currently specified in 3GPP TS 26.261:

The nominal values of SLR to the POI shall be: SLR = 13 ± 3 dB

NOTE: The value of 13 dB is motivated by the corresponding SLR requirement for desktop hands-free and handheld hands-free in TS 26.131 [4].

Even though not (yet) supported by real IVAS-capable terminals, this requirement seems to be chosen in a reasonable way, as it provides level compatibility with traditional mono telephony. However, the calculation of Send Loudness Rating (SLR) is based on the electro-acoustical transfer function from an acoustic signal at MRP (in dBPa) to an analogue voltage (in dBV) at the POI, which originates from the times of analogue telephone networks.

To adapt measures like SLR also for digital telephone networks, existing analogue network levels were referenced to the digital world by the concept of an "intermediate level", measured in dBm0 and can be considered as a "gateway" between analogue and digital signals (see Recommendation ITU-T G.100.1 for details):

- for the analogue side, a signal power of 0 dBm0 corresponds to an electrical power of 1 mW. Assuming a nominal impedance of 600 Ω, this corresponds to an RMS amplitude of 0.775 V.

- for the digital side, an additionally defined "overload point" in dBm0 corresponds to -3.01 dBov (sine tone with maximum amplitude).

Historically, it was intended to have different overload points for each network and/or each codec (but actually, most of the time 3.17 dBm0 was used until today). The present document investigates if these "historical" level definitions are still compatible with and in general applicable to IVAS-capable UEs.

# Test setup

A general overview of an acoustic test setup for the evaluation of immersive UEs in User Capture mode according to TS 26.260/261 is illustrated in Figure 1. For determining the SLR of a UE, the electro-acoustic frequency response from MRP to POI after downmixing to mono (denoted as POImono) is measured in 1/3rd octave bands in the frequency range 100 Hz – 8 kHz and by applying the weights WS from Table A.2 Recommendation ITU-T P.79.

Figure 1: Test setup overview for SLR measurement

Since for IVAS UEs the signal captured at POI would be an immersive audio format, the external IVAS renderer configured for mono downmix is additionally utilized as part of the reference client/test equipment.

To obtain a SLR of 13 dB at the output of this processing chain, it is investigated in the following, which electrical/digital levels at which input/output block are necessary. Several assumptions and simplifications according to Figure 2 are made in the following:

- UE type: Handheld-hands-free UE in User Capture mode (see clause 5.4.2.4 in TS 26.260), focussing on a distant talker (not on single/complex sound sources)

- Talker source level at MRP: LASL, MRP = -1.7 dBPa

- The acoustic path from MRP to the input microphone(s) of the UE is assumed to be a simulated free-field propagation:

  - Spectral shaping is realized by using ISM1 and external IVAS renderer to generate an immersive audio format at the UEs input microphones.

  - Level loss is realized according to Recommendation ITU-T P.581:

    - 24 dB loss between MRP and HFRP at 0.5 m

    - +/-6dB per halved/ doubled distance

  - Default distance 30 cm (42 cm from HATS reference point)  LASL, UE-in = -21.3 dBPa

    - Larger distances may be considered as well (see resulting levels annotated in Figure 2)

- All components between UE input microphones and the IVAS encoder input are considered as a single “acoustic-to-digital immersive transducer” with a certain “encoder sensitivity”.

- The level of any IVAS-supported audio format is determined via ITU-R BS 1770-5l, as already used for level normalization in TS 26.260 (clause 5.5.1) and in the processing plan for IVAS selection and characterization phase (see IVAS-7a/8a).

- After encoding, decoding and mono downmix, the resulting signal is calibrated from dBov to dBV as follows:

  - dBov  dBm0: + (3.17 + 3.01) dB (see clause 8.8 Recommendation G.100.1)

  - dBm0  dBV: - 2.21 dB (see clause 8.3 Recommendation G.100.1)

- SLR is calculated between POImono (in dBV) and MRP (in dBPa) as described above.

Figure 2: Simplified signal path and intermediate levels for SLR measurement

# Backward estimate of SLR

With these simplifications, a "forward/backward pass" can be applied to derive the necessary input level at the IVAS encoder in the UE:

- A certain coded format used at the IVAS encoder is chosen.

- The loudness (in LKFS) according to ITU-R BS 1770 is calculated and scaled to an arbitrary start value (e.g., LEnc = -26 LKFS).

- The loudness (in LKFS) according to ITU-R BS 1770 is calculated again after applying IVAS encoding and decoding to same coded format, denoted as LEncDec(0).

- The loudness difference (in LKFS) between encoder input and decoder output is denoted as: d1 = LEnc(0) - LEncDec(0)

- After applying mono downmix, the signal is converted into dBV domain (see annotations in Figure 2) and the initial value SLR(0) is calculated as mentioned before.

- The required level at the output of the mono downmix is calculated as the difference between the initial and the target SLR value: d2 = (SLR(0) - 13 dB)

- The overall difference d = d2 + d1 is assumed to be the correction value of the coded format at the IVAS encoder input to obtain the target SLR = 13 dB, leading to an adjusted loudness (in LKFS) denoted as LEnc(1).

- Steps 3 to 8 are repeated (i = 1, 2, …) until d2 is less than 0.1 dB, leading to accordingly updated intermediate levels:

  - LEncDec(i)  LEncDec(i+1)

  - d1(i+1) = LEnc(i+1) - LEncDec(i+1)

  - SLR(i)  SLR(i+1)

  - d2(i+1) = (SLR(i+1) - 13 dB)

After SLR of 13 dB is obtained, the following additional levels are calculated for information:

- The active speech level LASL,mono according to ITU-T Recommendation P.56 in dBov is calculated at the mono downmix.

- The "encoder/overall electro-acoustic sensitivity" Sac2el (in LKFS/dBPa) is determined as the difference between the loudness (in LKFS) at the IVAS encoder of the UE and the acoustic overall level (in dBPa) determined by the simulated free-field propagation between MRP and UE input microphones.

# Example results

Example parameters

- Default distance MRP->UE-input: 30 cm

- Coded format set to 7.1.4 (simplifies loudness calculations according to ITU-R BS 1770)

- Bitrate of 512 kbit/s  d1 < 0.01 LKFS.

Results

Table 1: Resulting example levels to obtain SLR = 13 dB

| Variable | Value |
| --- | --- |
| LASL, MRP | -1.7 dBPa |
| LASL, UE-in | -21.3 dBPa |
| Sac2el | -3.60 LKFS/dBPa |
| LEnc | -24.95 LKFS |
| LEncDec | -24.95 LKFS |
| LASL,mono | -25.34 dBov |

# Conclusion

At least for the considered example shown in the present document, the test method currently specified in 3GPP TS 26.260 (as well as the current requirement of a target SLR = 13 dB) seems applicable and reasonable for testing IVAS-capable UEs. Loudness results for encoder input, decoder output and the active speech level after mono downmix are close to the "expected" optimum/target values of -26 dBov and -26 LKFS, respectively. For this reason, there seems to be no need to adapt the currently specified test method and requirements. However, the several optimum conditions and simplifications were assumed, like e.g.,

- acoustic path simulated only with free-field propagation and level loss (might be realistic at least for handheld hands-free)

- encoding/decoding is almost transparent (regarding level and frequency response) at the considered high bitrate

- loudness calculations according to Recommendation ITU-R BS 1770 for all IVAS coded formats other than multichannel & stereo rely on IVAS external renderer (with 7.1.4 output)

- approach not tested yet with different coded audio formats (work in progress - more results are available soon)

The mostly theoretically derived data should be confirmed and supported as soon as possible with measurement results from real IVAS-enabled devices.

The sensitivity parameter Sac2el might also be useful for the work item DaCAS.