Source:	Dolby Sweden AB, Ericsson LM, Fraunhofer IIS, NTT, Nokia, Orange, Philips International B.V., VoiceAge Corporation

Title:	Verification of IVAS BASOP Encoder

Document for:	Discussion and Agreement

Agenda Item:	7.5

# Introduction

The floating-point code of the IVAS codec was standardized in Rel-18 in TS 26.258 [1]. The task to convert the floating-point implementation to fixed-point (BASOP) code is being done by Ittiam, under a contract issued by ETSI. The verification of the IVAS BASOP Decoder and Renderer can be found in [2], Annex A. It was agreed that the Decoder and Renderer was completed to a level of 90%, and the remaining issues are to be addressed in a maintenance phase ending in July 2025. This document covers the verification of the IVAS BASOP encoder v2.1, delivered by Ittiam on March 7th, 2025. This version was found to be stable enough for starting verification on some of the operating points, while other parts required further work. For this reason, there are different versions used in the verification in this report. Unless stated otherwise, the versions used for the verification are these:

- Floating point reference:

  - Branch: ivas-float-update

  - Commit: f2eb9e0831ece72ec66a0c07e62d1f3302e2a910

  - Date: Thursday Mar 6 16:21:04 2025 +0000

- BASOP verification candidate

  - Tag: 20250307_Ittiam_Encoder_Release_Subjective_Testing

  - Branch: main

  - Commit: 08641917bc71f5cc1dac7f5b6db3fafe091de8d1

  - Date: Friday Mar 7 12:23:49 2025 +0000

# Objective measurement tools

The objective measurement tools are used for mono audio comparison. To assess the quality of the BASOP fixed-point (FX) bitstream to the floating-point (FL) bitstream, the bitstreams are decoded with the FL reference decoder and the output is compared, as shown in Figure 1.

Figure 1: Setup for comparison of BASOP encoder FX enc.

Same as in [2], a pair-wise comparison is done on each channel of the test and reference output audio samples, and the worst scoring channel is reported. The reference signal for the tool is always the floating-point encoded bitstream, decoded with the floating-point decoder. The objective measures used are:

- Maximum loudness difference (MLD)

- PQEvalAudio (PEAQ)

For MLD the maximum number among the channels is reported and for PEAQ the minimum value is shown.

## MLD

The MLD tool is the same as explained in [2], Annex A.

## PEAQ

The PEAQ Objective Difference Grade (ODG) was calculated using the PQevalAudio implementation in [3]. Note that the reference signal is the floating-point encoded bitstream, decoded with the floating-point decoder, as shown in Figure 1. For the multi-channel output signals, the lowest ODG score among the channels is reported. It is sometimes referred to as MIN_ODG, but also ODG is used. Note that some measurements were also done using a Delta-ODG, where the float reference and BASOP are compared to the input signal and the difference of BASOP minus float is computed.

## Test vector set

The main test vector sets used in the verification are the same as in [2], Annex A:

- Short test vector set (STV):

- Long test vector set (LTV)

- IVAS Selection test material

In addition, verifying companies have complemented with their own audio material to expand the test coverage.

# Comparison of EVS BASOP and EVS float

## Objective quality measures of EVS BASOP

Similar to the requirements for the IVAS BASOP decoder and renderer, the requirements in objective measures are set based on how EVS BASOP compares to EVS FLOAT [2]. Replicating the comparison for the EVS decoder, the objective measures were evaluated for the EVS encoder, comparing the EVS FLOAT to the EVS BASOP. Again, the official test vector set 26.444 was used, limited to the script Readme_EVS_enc.txt. Reusing the categories from Annex A, these are the measured operating points:

- Static operation – Fixed bit rate (123 test cases)

- Flexible operation – DTX, Bitrate switching (165 test cases)

A summary of the MLD values can be found in Figure 2, and the PEAQ ODG scores are found in Figure 3.

Figure 2: Histogram of MLD values for the EVS BASOP vs EVS FLOAT.

Figure 3: Histogram of MIN_ODG values for the EVS BASOP vs EVS FLOAT.

The requirements using objective measures for the IVAS BASOP conversion task was said it should be within the limit of MLD < [TBD], PEAQ (basic profile) < [0.1], where the brackets indicate the values are not agreed. The PEAQ limit is here interpreted as a maximum degradation (minimum PEAQ score across channels) of -0.1. During the development, the threshold of MLD < 10 was used as a rule-of-thumb, and the MIN_ODG was used as an indicator to filter out critical cases. A summary of the EVS BASOP vs EVS float using MLD < 10 and MIN_ODG > -0.1 is found in Table 1.

Table 1: EVS BASOP vs EVS FLOAT test cases below threshold MLD 10 and above MIN_ODG -0.1.

| Operation | MLD < 10 | Max MLD | MIN_ODG > -0.1 | Min MIN_ODG |
| --- | --- | --- | --- | --- |
| Static | 95% | 17.66 | 8% | -3.43 |
| Flexible | 94% | 17.67 | 1% | -3.50 |
| All | 94% | 17.67 | 4% | -3.50 |

## Complexity ratio for EVS BASOP

Using the ITU-T WMC tool v.1.8 [5], the floating-point implementation of the 3GPP EVS codec (TS 26.443, v.16.3.0), referred to as EVS FLOAT, has been instrumented for memory and complexity measurements. Similarly, the fixed-point implementation of the 3GPP EVS codec (TS 26.452, v.16.4.0), referred to as EVS BASOP, has been modified to allow memory instrumentation with the WMC tool. The complexity counting mechanism in EVS BASOP, implemented in the `count.h` and `count.c` files, has been replaced with `wmc_auto.h` and `wmc_auto.c` which gives consistent results with negligible differences around below 1% when compared to the results reported in the Performance Characterization report TR 26.952 [6].

The complexity of the encoder was measured using a 48kHz test signal comprising multiple short segments of clean speech, noisy speech, background noise, and generic audio, with a total duration of 38 seconds. Measurements covered 126 operating points across the EVS codec’s entire bitrate range, from 5.9 kbps (VBR) up to 128 kbps, and included all four supported bandwidths: NB, WB, SWB, and FB. Testing also included DTX operation, bitrate switching, and the 13.2 kbps channel-aware mode, but did not include error concealment or JBM among the tested operating points.

The EVS BASOP to FLOAT complexity ratio has been calculated for each tested operating point using the following formula:

Rk=CMPLXBASOPCMPLXFLOAT,         k=1,…,126

The ratios Rk were subsequently used to generate the histogram of complexity ratios, as illustrated in Figure 4 below:

Figure 4: Histogram of EVS BASOP to FLOAT complexity ratios

The average complexity ratio is 0.80, with a standard deviation of 0.13.

Memory consumption was evaluated using the same test set as for computational complexity. It is categorized into RAM and ROM usage. RAM is further divided into HEAP (dynamically allocated memory) and STACK (memory used by automatic variables). At each codec operating point, RAM usage was assessed by measuring both the dynamically allocated HEAP via `malloc()` and the maximum STACK usage. ROM consumption is split into PROM (program code) and TROM (global static variables and constants). Both PROM and TROM remain constant across all operating points.

Table 2 presents the memory consumption for the EVS encoder, with average MAX STACK and HEAP memory values calculated across all operating points.

Table 2: EVS BASOP vs FLOAT encoder memory consumption.

| Memory [kBytes] |  | EVS BASOP | EVS FLOAT | EVS BASOP-TO-FLOAT RATIO |
| --- | --- | --- | --- | --- |
| RAM | HEAP | 81644 | 153793 | 0.53 |
|  | STACK | 59498 | 78210 | 0.75 |
| ROM | PROM | 129420 | 174656 | 0.74 |
|  | TROM | 14936 | 16628 | 0.90 |

The last column of Table 2 indicates the target ratio that should be reached with the IVAS BASOP codec.

# Summary of requirements for IVAS BASOP

## Objective quality measures of IVAS BASOP

An automated test system evaluates the IVAS Encoder on a daily basis. The results from the following version can be found below:

- Commit SHA: 858df806

- Date: Thu May 8 11:29:37 2025

A histogram showing the distribution of the MLD scores is found in Figure 5. The number of test cases in the static operation is 507, while the flexible operation comprises 216 tests. The scores are based on the LTV tests, including nominal, +10 dB and -10 dB level scaling. The level has been restored before the comparison and calculation of the objective measures. The distribution of the minimum PEAQ ODG values is illustrated in Figure 6.

Figure 5: Histogram of MLD values for the IVAS Encoder.

Figure 6: Histogram of MIN_ODG values for the IVAS Encoder.

Comparing the IVAS BASOP Encoder objective scores with the rule-of-thumb for MLD and PEAQ ODG, we get the result in Table 3.

Table 3: IVAS BASOP Encoder test cases summary.

| Operation | MLD < 10 | Max MLD | MIN_ODG > -0.1 | Min MIN_ODG |
| --- | --- | --- | --- | --- |
| Static | 81% | 82.96 | 10% | -3.69 |
| Flexible | 47% | 115.12 | 0% | -3.75 |
| All | 71% | 115.12 | 7% | -3.75 |

## Complexity verification of IVAS Encoder

The complexity is assessed bi-daily via the 3GPP Forge repository by comparing the latest BASOP main branch against the current FLOAT reference branch (ivas-float-update). The specific versions used for the comparison were:

- BASOP main branch: 994b6859 (Wed May 14 10:57:31)

- FLOAT reference branch (ivas-float-update): 2e062fa2 (Tue May 13 11:29:38)

The test suite comprised 1377 encoder test cases across different formats. Figure 10 presents a histogram of the complexity ratio distribution between BASOP and FLOAT encoder implementations. The observed average complexity ratio is 1.17, with a standard deviation of 0.25. This result is approximately 1.46 times higher than the target complexity ratio of 0.80, as defined by the 3GPP EVS codec benchmark.

Figure 7: Histogram of encoder complexity ratios from the bi-daily WMOPS measurements

Figure 8 through Figure 15 below present the three test cases with the highest WMOPS values within each of the three bitrate ranges:

- bit rate <= 80 kbps

- 80 kbps < bit rate <= 192 kbps

- 192 kbps < bit rate <= 512 kbps

The upper bounds correspond to complexity levels 1, 2, and 3. Note that each level is defined only by a maximum threshold. For example, in the STEREO configuration, the highest complexity occurs at level 1 and remains unchanged for levels 2 and 3.

Figure 8: BASOP and FLOAT complexity for Stereo input

Figure 9: BASOP and FLOAT complexity for Stereo DMX to EVS operation

Figure 10: BASOP and FLOAT complexity for Multichannel input

Figure 11: BASOP and FLOAT complexity for ISM input

Figure 12: BASOP and FLOAT complexity for SBA input

Figure 13: BASOP and FLOAT complexity for MASA input

Figure 14: BASOP and FLOAT complexity for OSBA input

Figure 15: BASOP and FLOAT complexity for OMASA input

## Memory verification of IVAS Encoder

Memory consumption of the encoder is measured daily for both the IVAS FLOAT and IVAS BASOP code bases. It is divided into RAM and ROM usage. RAM consumption is further broken down into HEAP (dynamically allocated memory) and the STACK.

For ROM, memory consumption is separated into PROM (Program ROM) and TROM (global static variables and constants). Both, PROM and TROM consumption remain constant across all operating points

Table 4 presents the memory consumption for the IVAS encoder, with average STACK and HEAP memory values calculated across all operating points.

Table 4: IVAS BASOP vs FLOAT encoder memory consumption.

| Memory [kBytes] |  | IVAS BASOP | IVAS FLOAT | IVAS BASOP-TO-FLOAT RATIO | EVS BASOP-TO-FLOAT RATIO |
| --- | --- | --- | --- | --- | --- |
| RAM | HEAP | 300367 | 336398 | 0.89 | 0.53 |
|  | STACK | 189092 | 159675 | 1.16 | 0.75 |
| ROM | PROM | 373032 | 403612 | 0.92 | 0.74 |
|  | TROM | 49028 | 51332 | 0.96 | 0.90 |

In this report, only encoder modules are considered. Measurements were taken for 3,233 operating points. The average memory consumption for the encoder is presented in Figure 16.

Figure 16: IVAS BASOP to FLOAT Average Encoder Memory Consumption Ratios

The PROM memory ratio for the encoder is 0.92 and the TROM memory ratio for the encoder is 0.96. The average HEAP memory consumption for the encoder stands at 0.89, and the average MAX STACK memory consumption is 1.16.

A detailed breakdown of memory consumption for each input format is provided in Figure 17 to Figure 23.

.

Figure 17: IVAS BASOP to FLOAT Average STEREO Encoder Memory Consumption Ratios (sorted by bitrate and output format)

Figure 18: IVAS BASOP to FLOAT Average ISM+N Encoder Memory Consumption Ratios (sorted by bitrate and output format)

Figure 19: IVAS BASOP to FLOAT Average HOA3 Encoder Memory Consumption Ratios (sorted by bitrate and output format)

Figure 20: IVAS BASOP to FLOAT Average MASA Encoder Memory Consumption Ratios (sorted by bitrate and output format)

Figure 21: IVAS BASOP to FLOAT Average MC Encoder Memory Consumption Ratios (sorted by bitrate and output format)

Figure 22: IVAS BASOP to FLOAT Average OMASA Encoder Memory Consumption Ratios (sorted by bitrate and output format)

Figure 23: IVAS BASOP to FLOAT Average OSBA Encoder Memory Consumption Ratios (sorted by bitrate and output format)

# Observations on the source code

The BASOP code includes an excessive number of parameter and buffer precision conversions (Q-notation values) to match the varying precision requirements of different modules. Minimizing these intermediary conversions would enhance code readability and, more importantly, reduce the complexity overhead in the BASOP implementation.

Also, at certain places it is unclear, why a specific signal scaling was chosen. See e.g., https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/issues/1485.

Moreover, some parameters or buffers are defined twice and allocated simultaneously with just different data types, e.g. Word32 input_buff32[]; and Word16 input_buff[];, or Word32 new_swb_speech_buffer_fx[] and Word16 new_swb_speech_buffer_fx_16[]. A merge of these parameters and buffers would similarly enhance code readability and, more importantly, reduce the memory overhead in the BASOP implementation.

There are a number of leftovers from the conversion exercise in the BASOP code, e.g.

- defines IVAS_FLOAT_FIXED_TO_BE_DONE, IVAS_CODE_RANGE_CODER, IVAS_CODE_TCX_COM, IVAS_CODE_CNG_COM, etc. are deactivated, seem to be related to an unused code and should be thus removed.

- functions with float data type parameters are present, e.g. mean(), sign(), log2_f(), etc. These functions should be removed and float data type not used in the BASOP implementation.

- Similarly, parameters/buffers with float data type should be removed, e.g. extern const float h_fir[]; or const float tdm_lsfQ_PCh[M];.

- Similarly (u)intXX_t data types should be replaced by BASOP data types (U)WordXX, e.g. int16_t is extensively used in files tns_base.c, bitstream.c, or tools.c.

- Tens of source code files and functions do not contain the _fx suffix in their name thus should be renamed.

We further noticed a number of duplicated files which, however, concern the same functionality in the form file_name_fx() and file_name(). This leads to inconsistencies and a higher number of source code files in the BASOP implementation wrt. the FLP one: there are 178 source code files in the FLP implementation vs. 214 files in the BASOP implementation in the lib_com library, or 201 files in the FLP implementation vs. 208 files in the BASOP implementation in the lib_enc library. In addition, some code duplications were observed and reported in https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/issues/1450 (Code duplication in SBA encoder MD generation) or https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/issues/1446 (Bitstream handling for EVS and IVAS encoder). Common set of functions should be used like in the FLP implementation.

# Quality verification of delivered Encoder

## Stereo down-mix to EVS

### Objective evaluation

For various input signals, BASOP implementation has been compared with the float reference implementation by objective measures according to the following conditions.

- Reference Floating-point encoder: 
https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/tree/1522_ref_PortFlpMR1349?ref_type=heads

- BASOP encoder: Informal local version in #1522(basop) branch
https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/tree/1522_basop_PortFlpMR1349 ?ref_type=heads combined with the modifications in the branch for #1511
https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/tree/3gpp_issue_1511_fix?ref_type=heads

| Format | Bit rates | operation | Input | Max_band | Level |
| --- | --- | --- | --- | --- | --- |
| Stereo | 13.2 24.4 kbps | All normal mode | Speech, music, svt, ltv | WB, FB | nominal, +10 dB for some |

We have observed that there are only minor differences between float and BASOP for speech items according to preparatory investigations. Therefore, we selected mainly music items (five items rcxxxxxm.wav) from collections for selection phase. According to the objective evaluations, we still have noticed some items with big differences between BASOP and floating. Further checking is required.

### Subjective evaluation

MUSHRA listening test has been carried out ay NTT for picked-uped six music items which are suitable for listening and have bigger waveform differences than others under following conditions.

| Reference format | Bit rates | operation | Input | Max_band | Level | Number of listeners |
| --- | --- | --- | --- | --- | --- | --- |
| Stereo | 13.2 24.4 kbps | All normal mode | 6 music | FB | nominal | 5 |

Average scores and difference scores are shown as follows. Error bars for both are 95% confidential intervals.

According to the results, we see that float implementation has better tendency than BASOP at 13.2 kbps.

### Other observations

Void.

### Issues identified

Void.

## Stereo

### Stereo at 13.2 kbps and 16.4 kbps

#### Objective evaluation

VoiceAge conducted tests on various operating points using selection test material and internal databases. The tests utilized the fixed-point encoder version of April 24th (SHA a72a0f63e) and the floating point codec of April 23rd (SHA deee9b97). The goal was to obtain level-normalized Mean Loudness Deviation (MLD) for and differential Objective Difference Grade (ODG) for information. The different configurations tested and the resulting histograms are presented in the tables below:

| Format | Bit rates | DTX | FER | max_band | Level |
| --- | --- | --- | --- | --- | --- |
| Stereo | 13200, 16400 bps | On and Off | Clean channel | WB, FB | -10 dB, nominal, +10 dB |

| Max. MLD for stereo (13.2 and 16.4 kbps) |  |  |
| --- | --- | --- |
| Normal-level | High-level | Low-level |

| Difference of minimal ODG for stereo (13.2 and 16.4 kbps) |  |  |
| --- | --- | --- |
| Normal-level | High-level | Low-level |

#### Subjective evaluation

VoiceAge conducted four subjective tests for the stereo operation at bitrates of 13.2 kbps and 16.4 kbps. These tests utilized 16 samples coming from the selection material, the floating point encoder and decoder from April 23rd (SHA: deee9b97) and the fixed point encoder from the April 24th (SHA : a72a0f63). The subjective tests followed a ref-A-B comparison style, with 14 listeners comprising a mix of experts and university students. The table below shows the different test configurations and summarize the results:

| Format | Experiment name | Bit-rate (kbps) | Based on | Content | Additional parameter | Results |
| --- | --- | --- | --- | --- | --- | --- |
| Stereo | Exp-1 | 13.2 | P800-1 | Clean speech | DTX OFF | Fail |
|  | Exp-2 | 13.2 | P800-2 | Street and car noise | DTX ON | Pass |
|  | Exp-3 | 16.4 | P800-1 | Clean speech | DTX OFF, 5% FER | Pass |
|  | Exp-4 | 16.4 | P800-3 | Mix content | DTX ON | Fail |

The samples used in the test were selected based on the highest MLD and ensuring that eight samples per listening lab were chosen for each condition tested. The MLD distribution for all the listening test is shown below.

##### Results details for 13.2 kbps, clean speech and DTX off

Statistical summary:

| Number of votes | 224 |
| --- | --- |
| Average global | 0.152 |
| STD global | 0.748 |
| P(T<=t) one-tail | 0.0013 |
| Float and fixed are different |  |

##### Results details for 13.2 kbps, noisy speech and DTX on

Statistical summary:

| Number of votes | 224 |
| --- | --- |
| Average global | 0.036 |
| STD global | 0.836 |
| P(T<=t) one-tail | 0.2615 |
| Float and fixed are not different |  |

##### Results details for 16.4 kbps, clean speech, 5% FEC and DTX on

Statistical summary:

| Number of votes | 224 |
| --- | --- |
| Average global | 0.054 |
| STD global | 0.732 |
| P(T<=t) one-tail | 0.1371 |
| Float and fixed are not different |  |

##### Results details for 16.4 kbps, mix content and DTX on

Statistical summary:

| Number of votes | 224 |
| --- | --- |
| Average global | 0.147 |
| STD global | 0.746 |
| P(T<=t) one-tail | 0.0013 |
| Float and fixed are different |  |

#### Other observations

For the two instances where fixed-point encoder differs from the floating point, the samples were analysed to try to identify specific issues. Additionally, it was checked whether the issues identified in these samples persist in the latest version of the codec (SHA 1092cc74d, May 7th). Various types of artifacts were identified, including those related to core selection differences, BWE energy discrepancies, and noise presence during segments of active speech. Some problems were solved between the test and May 7th ,  however, the BWE-related issue persists.

#### Issues identified

The combination of code inspection, objective audio quality assessment and subjective listening tests identified the issues summarized in the table below. All issues have been reported on the BASOP repository [4].

| Issue # | Summary | SHA | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1545 | Different core decision, related to 1276 | a72a0f63 | Core coder, mode selection’ BWE | Major | Open |
| 1377 | Energy missing for FD stereo at 13.2kbps TCX core | 0fc0a5c6 | Stereo | Major | Open |
| 1337 | energy mismatch when switching from ACELP to TCX | ac454fc2 | Core coder, stereo | Major | Open |
| 1493 | BWE Energy missing between 16kHz and 20 kHz | 3f91f622 | BWE | Medium | Open |
| 1351 | Discontinuity on the first frame for stereo at 48 kbps | ca8df168 | Stereo | Medium | Open |

### Stereo at 24.4 kbps and 32 kbps

#### Objective evaluation

Stereo bandwidth and items test

An objective test was run with all input/output bandwidth configurations, with DTX on and off, using a set comprising 182 music items and 187 items containing speech and speech with background. This rendered 13284 test cases in total, and the MLD and PEAQ ODG scores were derived on the result. The processing included 3 sampling rates for both input and output, 16, 32 and 48 kHz, with DTX on/off. For this comparison, the BASOP encoder was compared to the float reference encoder, using the float reference decoder. The BASOP main version was 182c2a50 (Tue Apr 29 11:08:25) and the float reference ivas-float-update was deee9b97 (Wed Apr 23 06:31:22). The histogram of the MLD scores can be found in Figure 24 and the MIN_ODG scores are shown in Figure 25.

Figure 24: Histogram of MLD scores for Stereo bandwidth and items test.

Figure 25: Histogram of MIN_ODG scores for Stereo bandwidth and items test.

The worst scoring items from this evaluation were used in the subjective listening test below.

Selection tests (P.800-1, P.800-2, P.800-3, BS.1534-1a, BS.1534-1b)

The material from the IVAS selection tests was processed using the setup described in Figure 1. The BASOP main version was 291c48d3 (Thu Apr 17 09:45:10) and the float reference ivas-float-update was b63c033f (Thu Apr 3 13:16:24). The histogram of the MLD scores can be found in Figure 26 and the MIN_ODG scores are shown in Figure 27.

Figure 26: Histogram of MLD scores for selection test Stereo material.

Figure 27: Histogram of MIN_ODG scores for selection test Stereo material.

Notes from informal listening to the MLD outliers:

- No severe issues encountered.

- A drum beat was too loud, seemingly after a mode switch in the encoder. See Stereo encoder @ 13.2kbps, drum beat too loud (#1568) · Issue · sa4/audio/ivas-basop, rated Medium priority.

- The encoder mode selection impacts the decoder PLC method. See Stereo encoder @ 16.4 kbps with FEC decoding, different mode selection impacts FEC method (#1569) · Issue · sa4/audio/ivas-basop, rated Minor priority.

- Difference in stereo image in noisy speech. See Stereo encoder @ 32kbps, different mode decision moves speaker location (#1570) · Issue · sa4/audio/ivas-basop. This is likely due to a different stereo mode decision. The stereo mode decision is however tricky to align, since it is a binary decision and the precision is bound to have an impact. There are other observations of the BASOP giving a more stable stereo decision compared to float. Hence, this is set to Medium.

#### Subjective evaluation

The subjective quality was evaluated using a Ref-AB test, running stereo at 24.4 and 32 kbps.

- Method: Ref-AB

- Operating point: Stereo @ 24.4 kbps and 32 kbps, 48 kHz input and output, error free channel, DTX on/off.

- Material: 40 Mixed critical items (speech, music, mixed speech/music)

- Listeners: 9

The material was selected from the 13284 mixed speech and music items mentioned in in the “Stereo bandwidth and items test” by selecting the ones which had the worst objective scores. The objective measures used were MLD, SSNR and PEAQ. The test cases with 48 kHz input and 48 kHz output are included. For this comparison, the BASOP encoder was compared to the float reference encoder, using the float reference decoder. The positive scores indicate a preference for the BASOP version in Figure 28. There is no statistical difference between BASOP encoder and float reference encoder with a mean of -0.036 and standard deviation of 0.690.

Figure 28: Ref-AB Listening test results for Stereo@32 and 24.4 kbps.

#### Other observations

Void.

##### Front VAD Decision

The front VAD flag decisions were investigated for floating point and fixed-point code outputs for low bitrate (13.2kbps) Stereo DTX with 71 test items. Most of the items contain background noise of different versions (such as train station or office). Some test items contain both speech and background noise.

The percentage of frames with different front VAD flags (float vs fixed) for each test item is presented in Figure 29 obtained with commit 349ac688. The highest difference percentage is around almost 6% which is a car noise test item. An informal listening showed that the mismatch does not result in speech clipping. An issue is filed to highlight the differences and to report part of a train sound not being actively coded.

Figure 29: The percentage of frames with Front VAD mismatch between Floating Point and BASOP for 71 test items.

The analysis is repeated with a later version of the code: 994b6859.The results show an improvement in terms of mismatched frame percentage. Figure 30 displays the percentage of mismatched frames with later analysis.

Figure 30: The percentage of frames with Front VAD mismatch for 71 test items using a later version of the code.

#### Issues identified

Table 5 List of issues identified following the tests for Stereo at bitrates 24.4 and 32 kbps.

| Issue | Summary | Commit | Component | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1530 | Artefact in BWE during noisy stereo speech @ 24.4 kbps (#1530) · Issue · sa4/audio/ivas-basop | 182c2a50 | BWE | Major | Open |
| 1544 | Encoder Stereo Missing Energy in Higher Bands @32 kbps (#1544) · Issue · sa4/audio/ivas-basop | 182c2a50 | BWE | Major | Open |
| 1549 | Encoder Audible Artifact in Speech Stereo DTX @24.4 kbps High MLD (#1549) · Issue · sa4/audio/ivas-basop | 182c2a50 |  | Major | Open |
| 1487 | Basop Encoder: Stereo DTX Front VAD Flag Mismatch (#1487) · Issue · sa4/audio/ivas-basop | 1ccc3c58 | Front VAD | Medium | Open |
| 1293 | BASOP Encoder: Distortion at High Frequencies, Stereo (#1293) · Issue · sa4/audio/ivas-basop | 20cb1008 |  | Medium | Closed |
| 1419 | BER detect in Stereo Decoder at 13.2 kbps DTX (#1419) · Issue · sa4/audio/ivas-basop | c89d638e |  | Major | Closed |
| 1418 | Basop encoder Stereo at 24.4 kbps Artifact high MLD (#1418) · Issue · sa4/audio/ivas-basop | 339e804c |  | Medium | Closed |
| 1251 | Basop Encoder: Spectral holes between 2-4 kHz and spectral differences for Unified Stereo @24kbps (#1251) · Issue · sa4/audio/ivas-basop | 4c0dfa80 |  | Major | Closed |
| 1428 | Basop Encoder Artifact for Stereo 13.2 kps DTX Background Noise Segment (#1428) · Issue · sa4/audio/ivas-basop | c89d638e |  | Major | Open |
| 1407 | BASOP encoder: Distortion Stereo DTX 24.4 kbps, high MLD (#1407) · Issue · sa4/audio/ivas-basop | 339e804c |  | Medium | Closed |
| 1410 | Major Difference for Stereo input at noisy segments 13.2 kbps DTX 32kHz SWB (#1410) · Issue · sa4/audio/ivas-basop | 339e804c |  | Major | Open |
| 1427 | Basop Encoder Spectral Gaps in Stereo DTX 13.2 kbps Noisy Signal (#1427) · Issue · sa4/audio/ivas-basop | c89d638e |  | Medium | Open |
| 1568 | Stereo encoder @ 13.2kbps, drum beat too loud (#1568) · Issue · sa4/audio/ivas-basop | 291c48d3 | Unified stereo | Medium | Open |
| 1569 | Stereo encoder @ 16.4 kbps with FEC decoding, different mode selection impacts FEC method (#1569) · Issue · sa4/audio/ivas-basop | 291c48d3 | Core coder, mode selection | Minor | Open |
| 1570 | Stereo encoder @ 32kbps, different mode decision moves speaker location (#1570) · Issue · sa4/audio/ivas-basop | 291c48d3 | Unified stereo, mode selection | Medium | Open |

### Stereo from 48 kbps up to 256 kbps

#### Objective evaluation

For the objective test evaluation the selection test items were used and the testing was divided to two parts one for speech and noisy speech (test items used for experiments P.800-1 and P.800-2 of the selection testing [7]) and for mixed speech and music (test items selected for experiments P.800-3, BS1534-1a and BS1534-1a). Conditions tested are shown below:

| BASOP encoder SHA | 858df806 |
| --- | --- |
| Float decoder SHA | d3f01fcf |
| Bitrates | 48, 64, 80, 96, 128, 160, 192, 256 kbps |
| Sampling Rate | 16, 32, 48 kHz |
| Input Levels | -10, 0, +10 dB |
| DTX | On and off for speech and noisy speech – off for mixed and music |

The objective metrics of MLD, delta-ODG  and SSNR are shown below. The comparison between the two versions is done per channel and the worst-case value is taken. Output “EXT” means stereo output here.

#### Subjective evaluation

For the subjective test evaluation two Ref A-B tests were conducted one for 48 kbps FB with DTX on and one for 96 kbps FB with DTX off. For each test 12 items were selected, from a database consisting of a large set of critical items and the items used for the selection tests. For the selection of the items objective metrics like MLD and ODG were used and the worst cases for subjective testing were chosen.

For both test the SHA for the BASOP encoder is 149cfbaa and the float decoder a7a3d015. It should be noted that three input levels 0, -10, +10dB were assigned to the items in a circular manner, i.e. item0 –> 0dB, item1  -> -10dB, item3 -> +10dB and so forth.

Below are the test results for 48 kbps FB DTX on with 8 expert listeners:

From the test results it can be seen that on average the BASOP version performs on par with the float, with the overall score showing a tendency towards degradation. However, as can be seen, there are two outlying conditions with clear degradation:

- Item2 there was a prominent artifact for which issue #1536 was filed and is now fixed

- Item8 for which issue #1537 was filed and but underlying artefacts are not present anymore in latest version of the main

- Item 5 shows a tendency towards degradation. From comments given by multiple participants, this can be attributed to a difference in the generated comfort noise which at one spot in the item had an annoying modulation in the file encoded with the BASOP encoder. Further investigation of this issue is in progress.

For Item 6 where performance is better for BASOP than float, there seems to be a difference in the VAD decision.

The results of the second test for 96 kbps FB with 8 expert listeners are shown below:

From the listening test results again there is a tendency towards degradation with the overall mean score. There is a degradation for 4 items (however with large confidence intervals). Also, for a few items the score is positive for the BASOP code compared to float, which overall shows that there are audible differences for the two codebases.

#### Other observations

- Bandwise M/S decision. There seem to be an issue with the bandwise M/S decision that is the core of the MDCT stereo coding. This is reflected in both issues 1464 and 1308. As described in issue #1464 this comes from different estimations of the quantization gains and the cause of that is precision issues on the energy calculation. Latest observation with checking SHA version 1ccc3c58 shows an improvement for LTV but still there are some frames with high deviation as seen in the plot below where the max, min and median band-wise bit estimates over the ltv stereo file are shown:

- Precision issues for energy calculation and M/S representation. There seems to be precision issues in general for calculating signal energies. In MDCT stereo the power spectra and energy is calculated in quite a few places that are essential for the whole processing chain, that is for SNS, band-wise M/S decision (see above) and IGF to name the most prominent ones. It seems that one single common exponent/q is chosen for both MDCT and MDST spectra despite that they may have huge differences in magnitude. While calculating energies these dynamic differences double, leading to very low precision values and not representable values. Also for the representation of Mid/Side spectra it also might make sense to use separate exponent/q values because the magnitude of Mid and Side data is inherently very different. One potential solution would be to not use a common exponent/q.

- BWE- IGF issues. Very often in informal verification large differences  were observed for the BWE (IGF) region when comparing the spectrograms of the two signals. This is also depicted in issue #1312. This could be due to both of the previously mentioned issues, namely the M/S decision and the power spectrum calculation after the M/S processing.

#### Issues identified

| Issue | Summary | Commit | Component | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1537 | Plop artifacts in stereo speech item at 48 kbps with fx encoder | 338a38b9 | MDCT-stereo/ ITD compensation | Severe issues were resolved but there is a minor issue to be checked | open |
| 1464 | Energy calculation in quantization gain estimation for MDCT-Stereo stereo mode decision not working correctly | a584f6ee | MDCT -stereo/ MS Decision, may affect other tools | Major | open |
| 1312 | Spectral gaps starting from mid-frequency range for MDCT-stereo 48kbps with +10dB input signals | b4a8a48f | MDCT-stereo/ IGF Bandwidth extension | Major | open |
| 1308 | Large differences between BASOP and float for critical item in MDCT-stereo @96kbps . Differences in spectral envelope between float and fx output. | d818c148 | MDCT stereo/ M/S Decision | Critical (it should be noted that this | open |
| 1580 | High-frequency artifacts in 48kbps MDCT-Stereo encoding of frequency sweep from end of ltv48_STEREO.wav signal | c93084e6 | MDCT stereo/IGF | Critical | open |

## Multichannel 5.1

### Bitrates 13.2 to 32 kbps (McMASA)

#### Objective evaluation

The 5.1 input operation on bitrates 13.2 to 32 kbps was evaluated objectively using material from the selection experiments. Due to the existence of a known issue of downmixed input being too high in level for the IVAS BASOP core, a constant gain of -8 dB was applied to the input signals before processing them with the encoder. This gain was reversed in decoder outputs to provide more meaningful MLD scores.

The evaluation was performed with the fixed-point encoder commit with SHA ba1f52bd5 and the obtained bitstream was processed to selected outputs using the float reference decoder commit with SHA d3f01fcfb.

The following combinations of operation mode variations were used.

| Parameter |  |
| --- | --- |
| Output format | 5.1 |
| Bitrates | 13.2 – 32 |
| Bitrate switching | Within 13.2 – 32 |
| DTX | Off (not supported) |
| Bandwidth | WB, SWB, FB |
| Output FS | 48 |

As these parameter combinations would produce an excessive number of combinations to run, the all-pairs testing approach was applied to parameters bitrate and bandwidth. A total of 480 cases are presented below in the histogram figures. All combinations processed successfully.

As seen in the figure, the MLD scores are generally low but peak around 3-4. The highest values are over 40.

#### Subjective evaluation

No subjective evaluation was performed for 5.1 input on bitrates 13.2 – 32 kbps.

#### Other observations

See 7.1+4 section of McMASA other observations.

#### Issues identified

Void.

### Bitrates 48 to 80 kbps (ParamMC)

#### Objective evaluation

For the objective test evaluation the selection test items were used (test items used for experiments BS1534-2a and BS1534-2b of the selection testing [7] ). Conditions tested are shown below:

| BASOP encoder SHA | 182c2a50 |
| --- | --- |
| Float decoder SHA | d3f01fcf |
| Bitrates | 48, 64, 80 kbps |
| Sampling Rate | 16, 32, 48 kHz |
| Input Levels | -10, 0, +10 dB |
| DTX | Off (not supported) |

The objective metrics of MLD, delta-ODG and SSNR are shown below. The comparison between the two versions is done per channel and the worst-case value is taken.

#### Subjective evaluation

For the subjective evaluation a Ref A-B test was conducted for ParamMC and 5.1 input format with 64kbps FB. The items included in the test were selected from the same database as for the objective testing from the selection test items. The items with the worst MLD and ODG scores were selected. It should be noted that three input levels 0, -10, +10dB were assigned to the items in a circular manner, i.e. item0 –> 0dB, item1  -> -10dB, item3 -> +10dB and so forth. The SHA of the BASOP encoder version tested is 149cfbaa.

The results with 8 expert listeners are shown below.

The subjective evaluation results do not show any degradation of the perceptual quality.

#### Other observations

See 6.6.2.3

#### Issues identified

| Issue | Summary | Commit | Component | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1267 | High MLD for ParamMC 5.1 at 80 kbps with +10dB Input gain. For 48kHz input, there is an audible loudness difference of 0.4 LKFS. For the 16kHz input there is a high MLD of 34.94. Both input vectors indicate issues handling highly compressed inputs for BASOP. | 863ee6ff | ParamMC | Medium | Open |

### Bitrates 96 kbps to 512 kbps (MCT)

#### Objective evaluation

For the objective test evaluation the selection test items were used (test items used for experiments BS1534-2a and BS1534-2b of the selection testing [7]). Conditions tested are shown below:

| BASOP encoder SHA | 182c2a50 |
| --- | --- |
| Float decoder SHA | d3f01fcf |
| Bitrates | 96, 128, 160, 192, 256, 384, 512 kbps |
| Sampling Rate | 16, 32, 48 kHz |
| Input Levels | -10, 0, +10 dB |
| DTX | Off (not supported) |

The objective metrics of MLD, delta-ODG and SSNR are shown below. The comparison between the two versions is done per channel and the worst-case value is taken.

#### Subjective evaluation

Void

#### Other observations

#### Issues identified

| Issue | Summary | Commit | Component | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1275 | High MLD and quality degradations for MCT-MC at 96kbps. Situation seems much improved since issue was originally reported. | 863ee6ff | MCT | Medium | Open |
| 1445 | Deviations with MCT between BASOP and float for LTV 5.1 coded with 128kbps. Main issue seems to be resolved but deviations with channel pair selection based on energies may need further investigation. | e1b65cd1 | MCT | Major | Open |

## Multichannel 7.1

### Bitrates 13.2 to 32 kbps (McMASA)

#### Objective evaluation

The 7.1 input operation on bitrates 13.2 to 32 kbps was evaluated objectively using material derived with down-mixing from the selection experiments of 7.1+4. Due to the existence of a known issue of downmixed input being too high in level for the IVAS BASOP core, a constant gain of -10 dB was applied to the input signals before processing them with the BASOP encoder. This gain was reversed in decoder outputs to provide more meaningful MLD scores.

The evaluation was performed with the fixed-point encoder commit with SHA d61cd826a and the obtained bitstream was processed to selected outputs using the float reference decoder commit with SHA d3f01fcfb.

The following combinations of operation mode variations were used.

| Parameter |  |
| --- | --- |
| Output format | 7.1 |
| Bitrates | 13.2 – 32 |
| Bitrate switching | Within 13.2 – 32 |
| DTX | Off (not supported) |
| Bandwidth | WB, SWB, FB |
| Output FS | 48 |

As these parameter combinations would produce an excessive number of combinations to run, the all-pairs testing approach was applied to parameters bitrate and bandwidth. A total of 480 cases are presented below in the histogram figures. All combinations processed successfully.

As seen in the figure, the MLD scores are generally low but peak around 3 which is similar to 5.1. No significant outlier scores are present.

#### Subjective evaluation

No subjective evaluation was performed for 7.1 input on bitrates 13.2 – 32 kbps.

#### Other observations

See 7.1+4 section of McMASA other observations.

#### Issues identified

Void.

### Bitrates 48 to 96 kbps (ParamMC)

#### Objective evaluation

For the objective test evaluation the selection test items were used (test items used for experiments BS1534-3a and BS1534-3b of the selection testing [7]). Test items from selection testing experiments input format is 7.1+4 and was rendered to 7.1 Conditions tested are shown below:

| BASOP encoder SHA | 182c2a50 |
| --- | --- |
| Float decoder SHA | d3f01fcf |
| Bitrates | 48, 64, 80, 96 kbps |
| Sampling Rate | 16, 32, 48 kHz |
| Input Levels | -10, 0, +10 dB |
| DTX | Off (not supported) |

The objective metrics of MLD, delta-ODG and SSNR are shown below. The comparison between the two versions is done per channel and the worst-case value is taken.

#### Subjective evaluation

Void

#### Other observations

See 6.6.2.3

#### Issues identified

See 6.6.2.4

### Bitrates 128 to 512 kbps (MCT)

#### Objective evaluation

For the objective test evaluation the selection test items were used (test items used for experiments BS1534-3a and BS1534-3b of the selection testing [7]). Test items from selection testing experiments input format is 7.1+4 and was rendered to 7.1 Conditions tested are shown below:

| BASOP encoder SHA | 182c2a50 |
| --- | --- |
| Float decoder SHA | d3f01fcf |
| Bitrates | 128, 160, 192, 256, 384, 512 kbps |
| Sampling Rate | 16, 32, 48 kHz |
| Input Levels | -10, 0, +10 dB |
| DTX | Off (not supported) |

The objective metrics of MLD, delta-ODG and SSNR are shown below. The comparison between the two versions is done per channel and the worst-case value is taken.

#### Subjective evaluation

For the subjective evaluation of the MCT a ref A-B test was conducted testing the 7.1 input format  coded with 128kbps FB. The items were chosen from the outliers with the worst MLD scores from both the selection testing sample database and an FhG internal database of critical items. It should be noted that three input levels 0, -10, +10dB were assigned to the items in a circular manner, i.e. item0 –> 0dB, item1  -> -10dB, item3 -> +10dB and so forth. The BASOP encoder version for encoding the items is SHA 79e21b0c.

The results with 10 expert listeners are shown below:

The listening test results do not show a clear degradation for any of the tested items, however there is tendency towards degradation for some items that is reflected also in the overall grading.

#### Other observations

Void.

#### Issues identified

Void.

## Multichannel 5.1+2

### Bitrates 13.2 to 32 kbps (McMASA)

#### Objective evaluation

The 5.1+2 input operation on bitrates 13.2 to 32 kbps was evaluated objectively using material derived with down-mixing from the selection experiments of 7.1+4. Due to the existence of a known issue of downmixed input being too high in level for the IVAS BASOP core, a constant gain of -10 dB was applied to the input signals before processing them with the BASOP encoder. This gain was reversed in decoder outputs to provide more meaningful MLD scores.

The evaluation was performed with the fixed-point encoder commit with SHA d61cd826a and the obtained bitstream was processed to selected outputs using the float reference decoder commit with SHA d3f01fcfb.

The following combinations of operation mode variations were used.

| Parameter |  |
| --- | --- |
| Output format | 5.1+2 |
| Bitrates | 13.2 – 32 |
| Bitrate switching | Within 13.2 – 32 |
| DTX | Off (not supported) |
| Bandwidth | WB, SWB, FB |
| Output FS | 48 |

As these parameter combinations would produce an excessive number of combinations to run, the all-pairs testing approach was applied to parameters bitrate and bandwidth. A total of 480 cases are presented below in the histogram figures. All combinations processed successfully.

As seen in the figure, the MLD scores are generally low but peak around 3-4 which is similarly to other McMASA technology operations. A few cases are observed with MLD around 15.

#### Subjective evaluation

No subjective evaluation was performed for 5.1+2 input on bitrates 13.2 – 32 kbps.

#### Other observations

See 7.1+4 section of McMASA other observations.

#### Issues identified

Void.

### Bitrates 48 to 96 kbps (ParamMC)

#### Objective evaluation

For the objective test evaluation the selection test items were used (test items used for experiments BS1534-3a and BS1534-3b of the selection testing [7]). Test items from selection testing experiments input format is 7.1+4 and was rendered to 5.1+2 Conditions tested are shown below:

| BASOP encoder SHA | 182c2a50 |
| --- | --- |
| Float decoder SHA | d3f01fcf |
| Bitrates | 48, 64, 80, 96 kbps |
| Sampling Rate | 16, 32, 48 kHz |
| Input Levels | -10, 0, +10 dB |
| DTX | Off (not supported) |

The objective metrics of MLD, ODG and SSNR are shown below. The comparison between the two versions is done per channel and the worst-case value is taken.

#### Subjective evaluation

Void.

#### Void. Other observations

See 6.6.2.3.

#### Issues identified

### See 6.6.2.4Bitrates 128 to 512 kbps (MCT)

#### Objective evaluation

For the objective test evaluation the selection test items were used (test items used for experiments BS1534-3a and BS1534-3b of the selection testing [7]). Test items from selection testing experiments input format is 7.1+4 and was rendered to 5.1+2 Conditions tested are shown below:

| BASOP encoder SHA | 182c2a50 |
| --- | --- |
| Float decoder SHA | d3f01fcf |
| Bitrates | 128, 160, 192, 256, 384, 512 kbps |
| Sampling Rate | 16, 32, 48 kHz |
| Input Levels | -10, 0, +10 dB |
| DTX | Off (not supported) |

The objective metrics of MLD, delta-ODG and SSNR are shown below. The comparison between the two versions is done per channel and the worst-case value is taken.

#### Subjective evaluation

Void.

#### Other observations

Void.

#### Issues identified

Void.

## Multichannel 5.1+4

### Bitrates 13.2 to 80 kbps (McMASA)

#### Objective evaluation

The 5.1+4 input operation on bitrates 13.2 to 80 kbps was evaluated objectively using material derived with down-mixing from the selection experiments of 7.1+4. Due to the existence of a known issue of downmixed input being too high in level for the IVAS BASOP core, a constant gain of -10 dB was applied to the input signals before processing them with the BASOP encoder. This gain was reversed in decoder outputs to provide more meaningful MLD scores.

The evaluation was performed with the fixed-point encoder commit with SHA 863ee6ff2 and the obtained bitstream was processed to selected outputs using the float reference decoder commit with SHA d3f01fcfb.

The following combinations of operation mode variations were used.

| Parameter |  |
| --- | --- |
| Output format | 5.1+4 |
| Bitrates | 13.2 – 80 |
| Bitrate switching | Within 13.2 – 80 |
| DTX | Off (not supported) |
| Bandwidth | WB, SWB, FB |
| Output FS | 48 |

As these parameter combinations would produce an excessive number of combinations to run, the all-pairs testing approach was applied to parameters bitrate and bandwidth. A total of 960 cases are presented below in the histogram figures. All combinations processed successfully.

As seen in the figures, the 5.1+4 operation seems to include more high MLD scores than 5.1, 7.1, and 5.1+4 operation points. A further inspection shows that almost all of the high MLD cases are observed on bitrate switching within the McMASA bitrates.

#### Subjective evaluation

No subjective evaluation was performed for 5.1+4 input on bitrates 13.2 – 80 kbps.

#### Other observations

See 7.1+4 section of McMASA other observations.

#### Issues identified

Void.

### Bitrates 96 to 128 kbps (ParamMC)

#### Objective evaluation

For the objective test evaluation the selection test items were used (test items used for experiments BS1534-3a and BS1534-3b of the selection testing [7]). Test items from selection testing experiments input format is 7.1+4 and was rendered to 5.1+4 Conditions tested are shown below:

| BASOP encoder SHA | 182c2a50 |
| --- | --- |
| Float decoder SHA | d3f01fcf |
| Bitrates | 96, 128 kbps |
| Sampling Rate | 16, 32, 48 kHz |
| Input Levels | -10, 0, +10 dB |
| DTX | Off (not supported) |

The objective metrics of MLD, delta-ODG and SSNR are shown below. The comparison between the two versions is done per channel and the worst-case value is taken.

#### Subjective evaluation

Void.

#### Other observations

There is an issue with loudness for 16 kHz sampling rate and WB. For a large number of samples there is a clear loudness difference (in the example of issue 1583 in 6.6.2.4 of 2.84 LUFS). This happens only for this sampling rate and regardless of the input loudness but mainly occurs for +10dB and 0 dB cases. Additionally, this is present for all MC input formats and are the cause of the very high MLD scores that can be seen in the respective plots (e.g. in 6.6.2.1)

#### Issues identified

| Issue | Summary | Commit | Component | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1583 | Very high MLD for ParamMC 5.1+4 at 96kbps 16kHz input | 182c2a50 | ParamMC | major | Open |

### Bitrates 160 to 512 kbps (MCT)

#### Objective evaluation

For the objective test evaluation the selection test items were used (test items used for experiments BS1534-3a and BS1534-3b of the selection testing [7]). Test items from selection testing experiments input format is 7.1+4 and was rendered to 5.1+4 Conditions tested are shown below:

| BASOP encoder SHA | 182c2a50 |
| --- | --- |
| Float decoder SHA | d3f01fcf |
| Bitrates | 160, 192, 256, 384, 512 kbps |
| Sampling Rate | 16, 32, 48 kHz |
| Input Levels | -10, 0, +10 dB |
| DTX | Off (not supported) |

The objective metrics of MLD, ODG and SSNR are shown below. The comparison between the two versions is done per channel and the worst-case value is taken.

#### Subjective evaluation

Void.

#### Other observations

Void.

#### Issues identified

Void.

## Multichannel 7.1+4

### Bitrates 13.2 to 96 kbps (McMASA)

#### Objective evaluation

The 7.1+4 input operation on bitrates 13.2 to 96 kbps was evaluated objectively using material from the selection experiments. Due to the existence of a known issue of downmixed input being too high in level for the IVAS BASOP core, a constant gain of -11 dB was applied to the input signals before processing them with the encoder. This gain was reversed in decoder outputs to provide more meaningful MLD scores.

The evaluation was performed with the fixed-point encoder commit with SHA ba1f52bd5 and the obtained bitstream was processed to selected outputs using the float reference decoder commit with SHA d3f01fcfb.

The following combinations of operation mode variations were used.

| Parameter |  |
| --- | --- |
| Output format | 7.1+4, Binaural |
| Bitrates | 13.2 – 96 |
| Bitrate switching | Within 13.2 – 96 |
| DTX | Off (not supported) |
| Bandwidth | WB, SWB, FB |
| Output FS | 48 |

As these parameter combinations would produce an excessive number of combinations to run, the all-pairs testing approach was applied to parameters bitrate and bandwidth. A total of 1056 cases are presented below in the histogram figures. All combinations processed successfully.

As seen in the figures, the MLD values are quite low but the peak is around 2-3 MLD and quite broad with 7.1+4 output. There are also several high MLD values encountered with 7.1+4 output combined with bitrate switching.

#### Subjective evaluation

- BASOP main: cf34e040c (2025-04-17)

- Float reference ivas-float-update: 53594392 (2025-04-03)

DCR listening test was conducted with naïve listeners. Binaural rendering was used for all conditions. There were 17 naïve listeners. Sennheiser HD650 headphones were used for binaural listening. Test material was generic multichannel in 7.1.4 channel format. Material consisted of movie trailers, music sequences both pop/rock and classical and some speech items with sound effects. There was a total of 16 different test sequences of about 8 seconds long.

The green color shows results encoded and decoded with float IVAS codec. Blue color shows results encoded with fixed point (BASOP) and decoded with floating point IVAS codec. In addition to a single bitrate per condition, bit-rate switching was tested. Bitrate switching to a new bitrate occurred randomly within the notified range. The rate of change was also randomized and was done every 400ms to 800ms. i.e. within each listened sequence of 8 seconds there were between 10 and 20 bitrate changes. In addition, frame erasures were added to some conditions with 5% frame error rate.

From the results no significant subjective degradations can be seen with any of the tested operation points. Additionally, the largest degradation (24.4k FER5) was additionally listened by the Nokia experts and no significant issue were noticed with this condition.

#### Other observations

In general, the McMASA technology bitrates of MC format work relatively well with the BASOP encoder. However, bitrate switching operation points provide most differences compared to the float encoder. Another possible identified source of difference is the spatial metadata analysis and the derived MASA metadata encoding. The BASOP repository issue 1200 in https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/issues/1200 tracks the need for this works. As differences in metadata change the bit use of the metadata, the bit use of the core coding for the transport channels also changes.

#### Issues identified

| Issue # | Summary | SHA | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1584 | Accuracy of spatial metadata in ivas_mcmasa_enc_fx compared to the floating point equivalent | 994b6859 | Multichannel, McMASA | Critical | Open |

### Bitrates 128 kbps (ParamMC)

#### Objective evaluation

For the objective test evaluation the selection test items were used (test items used for experiments BS1534-3a and BS1534-3b of the selection testing [7]). Conditions tested are shown below:

| BASOP encoder SHA | 182c2a50 |
| --- | --- |
| Float decoder SHA | d3f01fcf |
| Bitrates | 128 kbps |
| Sampling Rate | 16, 32, 48 kHz |
| Input Levels | -10, 0, +10 dB |
| DTX | Off (not supported) |

The objective metrics of MLD, delta-ODG and SSNR are shown below. The comparison between the two versions is done per channel and the worst-case value is taken.

#### Subjective evaluation

For the subjective testing 7.1+4 input format was tested with 128kbps FB with the Ref A-B test methodology. The items were chosen from the outliers with the worst MLD scores from the samples of the selection testing database that were also used for the objective evaluation. It should be noted that three input levels 0, -10, +10dB were assigned to the items in a circular manner, i.e. item0 –> 0dB, item1  -> -10dB, item3 -> +10dB and so forth. The BASOP encoder version is SHA 149cfbaa.

The results with 8 expert listeners are shown below.

The subjective evaluation results show no significant degradation for any item, while the overall score shows that the two versions are perceptually on par. There is a tendency towards degradation for Item 1 that should be further checked.

#### Other observations

See 6.6.2.3

#### Issues identified

See 6.6.2.4

### Bitrates 160 kbps (ParamUpmix)

#### Objective evaluation

Objective evaluation was carried out on 64 IVAS selection vectors (speech, music, mixed content) using the following software versions:

Floating point: 1bbe1956

Fixed point: 858df806

The following parameter combinations were tested exhaustively, resulting in 200 test cases.

| Parameter | Values |
| --- | --- |
| Input Format | 7.1.4 |
| Bitrate | 160Kbps |
| Sampling Rate (kHz) | 16, 32, 48 |
| Input signal level (LKFS) | -36 (-10dB), -26 (nominal), -16 (+10dB) |

No crashes were observed. The following figures show MLD and ODG histograms for relevant test case subsets.

Figure 31: MC ParamUpmix MLD and ODG histograms, all tested conditions.

Figure 32: MC ParamUpmix MLD and ODG histograms, -36 LKFS input loudness (-10dB from nominal).

Figure 33: MC ParamUpmix MLD and ODG histograms, -26 LKFS input loudness (nominal).

Figure 34: MC ParamUpmix MLD and ODG histograms, -16 LKFS input loudness (+10dB from nominal).

Figure 35: MC ParamUpmix MLD and ODG histograms, 16kHz sampling rate.

Figure 36: MC ParamUpmix MLD and ODG histograms, 32kHz sampling rate.

Figure 37: MC ParamUpmix MLD and ODG histograms, 48kHz sampling rate.

MLD results show good overall behavior of the fixed point code, but the increased MLD values at -16 LKFS and at 48kHz show, respectively, that potential issues with saturation (scaling) and bandwidth extension may be present, warranting further investigation.

#### Subjective evaluation

Subjective evaluation was carried out via a MUSHRA test using 9 audio vectors from the IVAS selection material (speech, music, mixed content). The sampling rate was 48kHz and the output format was binaural. Inputs were normalized to -26LKFS loudness. The results from 8 expert listeners suggest equivalence in audio quality between the fixed-point and floating-point encoder.

Figure 38: MC ParamUpmix MUSHRA test results (8 listeners)

#### Issues identified

The following two issues remain open as of May 13, 2025. The oldest issue is from February 2025. One is an audio quality issue and the other is a numerical underflow. An additional 5 issues have been already addressed and are not reported herein.

| Issue | Description | Date Created | Severity | Status |
| --- | --- | --- | --- | --- |
| #1316 | Energy leak into LFE channel, 48kHz, -16LKFS | Feb 24, 2025 | Major | Open |
| #1375 | Underflow in quantize_pars_fx() function. | Mar. 11, 2025 | Major | Open |

### Bitrates 192 - 512 kbps (MCT)

#### Objective evaluation

For the objective test evaluation the selection test items were used (test items used for experiments BS1534-3a and BS1534-3b of the selection testing [7]). Conditions tested are shown below:

| BASOP encoder SHA | 182c2a50 |
| --- | --- |
| Float decoder SHA | d3f01fcf |
| Bitrates | 192, 256, 384, 512 kbps |
| Sampling Rate | 16, 32, 48 kHz |
| Input Levels | -10, 0, +10 dB |
| DTX | Off (not supported) |

The objective metrics of MLD, delta-ODG and SSNR are shown below. The comparison between the two versions is done per channel and the worst-case value is taken.

#### Subjective evaluation

Void.

#### Other observations

Void.

#### Issues identified

Void.

## ISM

### ISM from 13.2 kbps up 48 kbps

#### Objective evaluation

VoiceAge conducted tests on various operating points using selection test material and internal databases. The tests utilized the fixed-point encoder version of April 24th (SHA a72a0f63e) and the floating-point codec of April 23rd (SHA deee9b97). The goal was to obtain level-normalized Mean Loudness Deviation (MLD) for and differential Objective Difference Grade (ODG) for information. The different configurations tested, and the resulting histograms are presented in the tables below:

| Format | Bit rates | DTX | FER | max_band | Level |
| --- | --- | --- | --- | --- | --- |
| ISM 1 | 13.2, 16.4, 24.4, 32 and 48 kbps | On and Off | Clean channel | WB, SWB, FB | -10 dB, nominal, +10 dB |
| ISM 2 | 24.4 and 32 kbps | On and Off | Clean channel | WB, SWB, FB | -10 dB, nominal, +10 dB |
| ISM 3 | 48 kbps | On and Off | Clean channel | WB, FB | -10 dB, nominal, +10 dB |
| ISM 4 | 48 kbps | On and Off | Clean channel | WB, FB | -10 dB, nominal, +10 dB |

| MLD for ISM 1 (13.2, 16.4, 24.4, 32 and 48 kbps) |  |  |
| --- | --- | --- |
| Normal-level | High-level | Low-level |
| Max. MLD for ISM 2 (24.4 and 32 kbps) |  |  |
| Normal-level | High-level | Low-level |
| Max. MLD for ISM 3 (48 kbps) |  |  |
| Normal-level | High-level | Low-level |
| Max. MLD for ISM 4 (48kbps) |  |  |
| Normal-level | High-level | Low-level |

| Difference of minimal ODG for ISM 1 (13.2, 16.4, 24.4, 32 and 48 kbps) |  |  |
| --- | --- | --- |
| Normal-level | High-level | Low-level |
| Difference of minimal ODG for ISM 2 (24.4 and 32 kbps) |  |  |
| Normal-level | High-level | Low-level |
| Difference of minimal ODG for ISM 3 (48 kbps) |  |  |
| Normal-level | High-level | Low-level |
| Difference of minimal ODG for ISM 4 (48 kbps) |  |  |
| Normal-level | High-level | Low-level |

#### Subjective evaluation

VoiceAge conducted four subjective tests for the ISM operation at bitrates from 13.2 kbps to 48 kbps. These tests utilized 16 samples coming from the selection material, the floating point encoder and decoder from April 23rd (SHA: deee9b97) and the fixed point encoder from the April 24th (SHA : a72a0f63). The subjective tests followed a ref-A-B comparison style, with 14 or 15 listeners comprising a mix of experts and university students. The table below shows the different test configurations and summarize the results:

| Format | Experiment name | Bit-rate (kbps) | Based on | Content | Additional parameter | Results |
| --- | --- | --- | --- | --- | --- | --- |
| ISM-1, Binaural out | Exp-5 | 13.2 | P800-6 | Clean speech | DTX ON, 5% FER | Pass |
| ISM-2, Binaural out | Exp-6 | 16.4 | P800-7 | Clean speech |  | Pass |
|  | Exp-7 | 32.0 |  | Clean speech | 5% FER | Pass |
|  | Exp-8 | 48.0 |  | Clean speech |  | Pass |
| ISM-4, Binaural out | Exp-9 | 48.0 | BS1534-6b | Clean speech + music |  | Fail |

The samples used in the test were selected based on the highest MLD and ensuring that eight samples per listening lab were chosen for each condition tested. The MLD distribution for all the listening test is shown below.

##### Results details for ISM-1, 13.2  kbps, clean speech, 5% FEC and DTX on

Statistical summary:

| Number of votes | 224 |
| --- | --- |
| Average global | 0.080 |
| STD global | 0.885 |
| P(T<=t) one-tail | 0.0877 |
| Float and fixed are not different |  |

##### Results details for ISM-2, 16.4 kbps, clean speech and DTX off

Statistical summary:

| Number of votes | 224 |
| --- | --- |
| Average global | 0.049 |
| STD global | 0.554 |
| P(T<=t) one-tail | 0.0930 |
| Float and fixed are not different |  |

##### Results details for ISM-2, 32.0 kbps, clean speech, 5% FEC and DTX off

Statistical summary:

| Number of votes | 240 |
| --- | --- |
| Average global | 0.050 |
| STD global | 0.719 |
| P(T<=t) one-tail | 0.1411 |
| Float and fixed are not different |  |

##### Results details for ISM-2, 48.0 kbps, clean speech and DTX off

Statistical summary:

| Number of votes | 240 |
| --- | --- |
| Average global | 0.000 |
| STD global | 0.501 |
| P(T<=t) one-tail | 0.5 |
| Float and fixed are not different |  |

##### Results details for ISM-4, 48.0 kbps, clean speech with music background

Statistical summary:

| Number of votes | 240 |
| --- | --- |
| Average global | 0.104 |
| STD global | 0.772 |
| P(T<=t) one-tail | 0.0188 |
| Float and fixed are different |  |

#### Other observations

For four out of the five tests, both the floating-point and fixed-point encoders were considered equivalent. However, in the 48 kbps ISM-4 test, the fixed-point encoder was found to be different. Some samples were analysed to identify specific issues, revealing that energy was often missing in the higher part of the spectrum, leading to a duller subjective quality. Additionally, it was checked whether these energy issues persisted or not in the latest version of the codec (SHA 1092cc74d, May 7th). The energy-related issues appear to have been corrected in the May 7th encoder.

#### Issues identified

| Issue # | Summary | SHA | Component(s) | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1094 | Very different coding of an attack at 48 kbps for ISM-1 | 45f78445 | Core coder, ISM, BWE | Major | Open |
| 1091 | Difference in BWE at 13.2 kbps for ISM-1 | 6362e224 | Core coder, ISM, BWE | Medium | Open |

### ISM3 and ISM4 from 24.4 and 32 kbps

#### Objective evaluation

For the objective test evaluation the selection test items were used (test items used for experiments BS1534-6a and BS1534-6b of the selection testing [7]). Conditions tested are shown below:

| BASOP encoder SHA | 182c2a50 |
| --- | --- |
| Float decoder SHA | d3f01fcf |
| Bitrates | 24.4, 32 kbps |
| Sampling Rate | 16, 32, 48 kHz |
| Input Levels | -10, 0, +10 dB |
| DTX | On and off |

The objective metrics of MLD, delta-ODG and SSNR are shown below. The comparison between the two versions is done per channel and the worst-case value is taken.

#### Subjective evaluation

For the subjective test evaluation a Ref A-B test was conducted for ISM3 and ISM4 with 32kbps  FB. The items were selected from the outliers of the objective test evaluation, namely from the items with the worst scores in MLD and ODG. It should be noted that three input levels 0, -10, +10dB were assigned to the items in a circular manner, i.e. item0 –> 0dB, item1  -> -10dB, item3 -> +10dB and so forth. The BASOP encoder version used is SHA 149cfbaa.

The results with 9 expert listeners are shown below:

From the subjective evaluation results there is no significant degradation observed for any of the tested items and the overall score indicates that the perceptual quality of the two versions is on par.

#### Other observations

#### Issues identified

| Issue | Summary | Commit | Component | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1510 | ParamISM Enc, 24.4 kbit/s: Energy missing > 6.4 kHz | 99e792d1 | ParamISM encoder | Medium | Open |
| 1206 | high ODG value ( -1.8) and audible "scratching" artifacts | 12778592 | ParamISM encoder | Medium | Open |
| 1201 | high MLD values ( approx 21 ), audible artifacts in different places, fx has additions strong "scratching" artifacts, "kling" sound is really muffled | 12778592 | ParamISM encoder | Medium | Open |

### ISM from 64 kbps up to 512 kbps

#### Objective evaluation

Bandwidth and bitrate test

An objective evaluation was done on ISM1 without input metadata, covering

- 377 test files,

- all 9 supported bit rates: 13200, 16400, 24400, 32000, 48000, 64000, 80000, 96000,128000,  and

- all 9 input/output bandwidth combinations: 16, 32, 48,

- DTX on and off,

rendering 377*9*9*2 = 61074 test cases. The BASOP main version was 5156ee35 (Wed May 7 03:59:26) and the float reference ivas-float-update was d3f01fcf (Thu Apr 24 11:42:19). The MLD and MIN_ODG scores can be found in Figure 39.

Figure 39: Histograms of MLD and MIN_ODG values for Bitrate and bandwidth test with ISM1.

Informal listening notes from the highest MLD items:

- No severe issues encountered.

- Occasionally differing core selection. An earlier version had more severe problems, see e.g. ISM1 core coder selection issue @ 13.2 kbps (#1276) · Issue · sa4/audio/ivas-basop. However, the latest version that was tested was improved. There are still classification differences, but sometimes BASOP is better and sometimes float is better.

- Detecting transients and activating ACELP for lower bit rates differs between BASOP and float. In some cases float is better, in other cases BASOP is preferred. See ISM1, core classifier (#1576) · Issue · sa4/audio/ivas-basop.

- Different DTX activity patterns for some signals, e.g. applause. Both BASOP and float use DTX, which degrades the signal, but it is difficult to classify this type of signal. Should it be labeled as a background or foreground signal?

Selection test processing for ISM tests (P800-6, P800-7, BS1534-6a and BS1534-6b)

The tests from the IVAS selection tests were run, and the MLD and PEAQ ODG scores were derived on the result. For this comparison, the BASOP encoder was compared to the float reference encoder, using the float reference decoder as shown in Figure 1. The BASOP main version was 291c48d3 (Thu Apr 17 09:45:10) and the float reference ivas-float-update was b63c033f (Thu Apr 3 13:16:24). The histogram of the MLD scores can be found in Figure 40 and the MIN_ODG scores are shown in Figure 41. The MLD scores are not that high, with 99.7% of the test cases below MLD 10. Notes from the informal listening of the highest MLD items:

- Speech distortion is observed for two ISM 1 items with 5% FER at low bitrates (13.2, 16.4, 24.4 and 32 kbps). Same issue was repeated for the same item at different bitrates. An issue is filed: BASOP Encoder Distortion for ISM 1 FER at 32kbps, EXT output, high MLD (#1534) · Issue · sa4/audio/ivas-basop

- Spectral differences are observed for bell ringing, coffee pouring, carrot chopping and water flowing sound items. An issue is filed for one of them: Encoder Spectral Differences for ISM 3 item with a bell ringing sound at 24.4 kbps, High MLD (#1538) · Issue · sa4/audio/ivas-basop.

- High band energy differences are observed for some of the speech items for frequencies greater than 16 kHz.

Figure 40: Histogram of MLD scores for selection test ISM material.

Figure 41: Histogram of MIN_ODG scores for selection test ISM material.

#### Subjective evaluation

A Ref-AB test was set up, comprising all 28 ISM4 items from the IVAS selection test experiment BS1534-6b.

- Method: Ref-AB

- Operating point: ISM4@64kbps, 48 kHz input and output, error free channel

- Material: 28 Mixed items (mixed speech/music, sound effects) with relevant metadata

- Listeners: 9

The result can be found in Figure 42, where no statistical difference was found between BASOP and float. The overall tendency is in favour of BASOP.

Figure 42: Ref-AB Listening test results for ISM4@64kbps.

#### Other observations

Void.

#### Issues identified

Table 6 List of issues identified following the tests for ISM.

| Issue | Summary | Commit | Component | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| #1276 | ISM1 core coder selection issue @ 13.2 kbps | 6432cc34 | Core classifier | Critical | Resolved |
| 1538 | Encoder Spectral Differences for ISM 3 item with a bell ringing sound at 24.4 kbps, High MLD (#1538) · Issue · sa4/audio/ivas-basop | 182c2a50 |  | Major | Open |
| 1534 | BASOP Encoder Distortion for ISM 1 FER at 32kbps, EXT output, high MLD (#1534) · Issue · sa4/audio/ivas-basop | 182c2a50 |  |  | Open |
| #1576 | ISM1, core classifier (#1576) · Issue · sa4/audio/ivas-basop | 5156ee35 | Core classifier | Minor | Open |

## SBA

### Objective evaluation

Objective evaluation was carried out on 184 IVAS selection vectors (speech, music, mixed content) using the following software versions:

Floating point: 1bbe1956

Fixed point: 858df806

The following parameter combinations were tested exhaustively, resulting in 16695 test cases. The output format was kept the same as the input format. Note that DTX operation is only supported by IVAS SBA mode at bitrates <= 80kbps.

| Parameter | Values |
| --- | --- |
| Input Format | FOA, HOA2, HOA3 |
| Bitrate | All IVAS bitrates |
| Sampling Rate (kHz) | 16, 32, 48 |
| Input signal level (LKFS) | -36 (-10dB), -26 (nominal), -16 (+10dB) |
| DTX | disabled, enabled |

No crashes were observed. The following figures show MLD and ODG histograms for relevant test case subsets.

Figure 43: SBA MLD and ODG histograms, all tested conditions.

Figure 44: SBA MLD and ODG histograms, DTX disabled.

Figure 45: SBA MLD and ODG histograms, DTX enabled.

Figure 46: SBA MLD and ODG histograms, -36 LKFS input loudness (-10dB from nominal).

Figure 47: SBA MLD and ODG histograms, -26 LKFS input loudness (nominal).

Figure 48: SBA MLD and ODG histograms, -16 LKFS input loudness (+10dB from nominal).

Figure 49: SBA MLD and ODG histograms, 1 transport channel (TC) (bitrate <= 32kbps)

Figure 50: SBA MLD and ODG histograms, 2 transport channels (TC) (48kbps <= bitrate <= 80kbps)

Figure 51: SBA MLD and ODG histograms, 3 transport channels (TC) (96kbps <= bitrate <= 192kbps)

Figure 52: SBA MLD and ODG histograms, 4 transport channels (TC) (256kbps <= bitrate <= 512kbps)

Figure 53: SBA MLD and ODG histograms, 16kHz sampling rate

Figure 54: SBA MLD and ODG histograms, 32kHz sampling rate

Figure 55: SBA MLD and ODG histograms, 48kHz sampling rate

MLD results suggest very good overall behavior of the fixed-point encoder, but some quality issues remain especially in the lower bit rates (1 TC). This is also reflected in the open issues reported in section 6.9.3. In addition, a small increase in MLD statistics for -36LKFS may suggest a slightly higher noise floor in the fixed-point encoder. This is worth further investigation.

### Subjective evaluation

Subjective evaluation of the SBA input format was carried out via MUSHRA tests. Seven MUSHRA test were carried out covering a sampling of high, medium and low bitrates. Two of these tests were performed with DTX operation. The number of listeners varied per test from 7 to 9 experienced listeners. Each test contained four conditions under test (CuTs) consisting of a fixed-point and a floating-point encoder condition for each of two bitrates. Bitrates were tested in pairs to reduce the overall number of listening tests. The output format in each test was binaural and the input format was HOA3, full band (FB), at 48kHz sampling rate and normalized to -26LKFS loudness. Audio was decoded and rendered using the floating-point IVAS decoder and renderer in all conditions. The audio content used for tests SBA01 - SBA06 was primarily sourced from the IVAS selection tests and encompassed a variety of speech and music items. For test SBA07, content was selected explicitly from material that is known to expose unresolved quality issues as of the launch date of the test.

The testing was performed between March and May 2025, mostly because the March release of the fixed point encoder was found to contain obvious audio quality issues that prevented subjective testing at certain bitrates and/or DTX operation. The table below provides relevant details about each test and includes the fixed-point and floating-point software versions used in each test. The tests are listed chronologically.

| Test Name | Bit rates (Kbps) | DTX | Fixed Point Commit | Floating Point Commit |
| --- | --- | --- | --- | --- |
| SBA1 | 256, 512 | Off | c602cc48 (1 Mar. 2025) | 73d5e7cd (28 Feb. 2025) |
| SBA2 | 48, 80 | Off | c602cc48 (1 Mar. 2025) | 73d5e7cd (28 Feb. 2025) |
| SBA3 | 13.2, 24.4 | Off | 08641917 (7 Mar. 2025) | e15ad0ad (3 Mar. 2025) |
| SBA4 | 96, 160 | Off | 816c984c (3 Apr. 2025) | 53594392 (28 Mar. 2025) |
| SBA5 | 48, 80 | On | 56ce55f8 (29 Apr. 2025) | d3f01fcfb (24 Apr. 2025) |
| SBA6 | 32, 16.4 | On | 56ce55f8 (29 Apr. 2025) | d3f01fcfb (24 Apr. 2025) |
| SBA7 | 13.2, 24.4 | Off | 56ce55f8 (29 Apr. 2025) | d3f01fcfb (24 Apr. 2025) |

MUSHRA test results are shown on the figures below. The bars indicate 95% confidence intervals.

Figure 56: SBA1 MUSHRA test results (512 and 256 kbps, 8 listeners).

Figure 57: SBA2 MUSHRA test results (80 and 48 kbps, 8 listeners).

Figure 58: SBA3 MUSHRA test results (24.4 and 13.2 kbps, 8 listeners).

Figure 59: SBA4 MUSHRA test results (160 and 96 kbps, 7 listeners).

Figure 60: SBA5 MUSHRA test results (80 and 48 kbps, DTX enabled, 9 listeners).

Figure 61: SBA6 MUSHRA test results (32 and 16.4 kbps, DTX enabled, 8 listeners).

Figure 62: SBA7 MUSHRA test results (32 and 13.2 kbps, 8 listeners).

The results of test SBA1 through SBA6 generally show equivalence between fixed-point and floating-point quality. For certain items, downward quality trends were identified in the results and these items were then given to expert listeners for further assessment. This led to new issues which were reported in 3GPP Forge. The overall status of SBA issues as of the writing of this report is discussed in section 6.9.3. Test SBA7 suggests some degradation on item v4, which is related to the open issues for 13,2kbps reported in section 6.9.3.

### Issues identified

The following 19 issues remain open as of May 13, 2025. Out of these, 5 are potentially fixed and awaiting review. The earliest open issue is from December 2024. Most issues consist of audible differences related to low bitrate (1 TC) operation. An additional 35 issues have been reported and closed; they are not listed herein.

(Note that OSBA open issues related to SBA are listed separately in section 6.11.)

| Issue | Description | Date Created | Component | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| #1084 | Level drop for SBA@24.4 kbps | Dec 11, 2024 | Low bitrate | Critical | Open (In Review) |
| #1085 | Major waveform discrepancy across all channels | Dec 11, 2024 | Low bitrate | Critical | Open |
| #1115 | High MLD between signals decoded from 13.2kbps | Dec 16, 2024 | Low bitrate | Major | Open |
| #1116 | High MLD between signals decoded from 24.4kbps | Dec 16, 2024 | Low bitrate | Major | Open |
| #1122 | MLD in the range of 5 to 12 between signals decoded from 512kbps bitstreams at input level -16LKFS | Dec 16, 2024 | SBA | Medium | Open |
| #1123 | MLD in the range of 5 to 12 between signals decoded from 256kbps bitstreams at input level -16LKFS | Dec 16, 2024 | SBA | Major | Open |
| #1180 | 5 to 15 dB spectral differences in top and bottom end of spectra, 16.4kbps HOA3 | Jan 15, 2025 | Low bitrate | Major | Open |
| #1181 | Major MLD regression for encoding HOA1, -16LKFS, 16kHz signals at 13.2kbps | Jan 15, 2025 | Low bitrate | Major | Open |
| #1196 | Major wave form discrepancy: 13.2kbps, 36LKFS, 16kHz | Jan 17, 2025 | Low bitrate | Critical | Open |
| #1249 | Spectral differences observed: 48kHz, HOA3 speech input at 16LKFS and 13200 bps | Jan 31, 2025 | Low bitrate | Major | Open |
| #1303 | Obvious spectral discrepencies at 13.2kbps with -16LKFS FOA input at 48kHz | Feb 17, 2025 | Low bitrate | Critical | Open |
| #1317 | Spectral distortion, 13.2kbps, FOA input at -16LKFS | Feb 24, 2025 | Low bitrate | Critical | Open (In Review) |
| #1412 | Large difference SBA with LTV at 48 kbps | Mar 18, 2025 | SBA | Medium | Open |
| #1417 | Audible artifact at 13.2 kbps, 48KHz | Mar 19, 2025 | Low bitrate | Critical | Open (In Review) |
| #1424 | Audible differences in comfort noise at SBA 80, 48, 32, 16.4 kbps | Mar 20, 2025 | DTX | Critical | Open (In Review) |
| #1450 | Code duplication in SBA encoder MD generation | Mar 31, 2025 | Code quality | Medium | Open |
| #1461 | Issue with VAD sensitivity in SBA DTX mode at 48 and 80 kbps | Apr 4, 2025 | DTX | Critical | Open (In Review) |
| #1476 | Degradation in channel check vector at 13.2, 16.4, 24.4kbps SBA | Apr 11, 2025 | Low bitrate | Major | Open |
| #1513 | Very obvious audible artifacts at 13.2 kbps | Apr 24, 2025 | Low bitrate | Critical | Open |

## MASA

### Objective evaluation

Objective evaluation of the MASA format was performed on the fixed-point encoder commit with SHA ba1f52bd5 and the obtained bitstream was processed to selected outputs using the float reference decoder commit with SHA d3f01fcfb. Evaluation was performed for two separate sets of input. The first set contains input vectors taken from the selection experiments. The second set contains the long test vectors used in the automated testing of the BASOP conversion, namely ltv48_MASA1TC.wav/.met and ltv48_MASA2TC.wav/.met available in https://forge.3gpp.org/rep/ivas-codec-pc/ivas-pc-testfiles. These long test vectors have been designed specifically to test MASA codec as widely as possible.

The following combinations of operation mode variations were used.

| Parameter | First set (selection experiments) | Second set (LTV) |
| --- | --- | --- |
| Output format | EXT, Binaural | EXT, Binaural |
| Bitrates | 13.2 – 512 (all IVAS bitrates) | 13.2 – 512 (all IVAS bitrates) |
| Bitrate switching | All bitrates | All bitrates |
| DTX | On/off | On/off |
| Bandwidth | WB, SWB, FB | WB, SWB, FB |
| Input FS | 16, 32, 48 | 16, 32, 48 |
| Output FS | 48 | 48 |
| Transport channels | 2 TC | 1 TC and 2 TC |
| Number of directions | 1dir and 2dir | Switching 1dir/2dir in one file |

As these parameter combinations would produce an excessive number of combinations to run, the all-pairs testing approach was applied to parameters bitrate, DTX, bandwidth and input fs. A total of 2856 cases for first set and 306 results for the second set are presented below in the histogram figures. All combinations processed successfully.

The above figures contain all the operation point combinations pooled in. As seen in the figures, a significant portion of the tested operation points are with low MLD scores. However, some scores are also higher while otherwise reasonable in detailed inspection. What is not seen in the histograms is that there are a few bitrate switching cases with very high MLD values (highest with 120). Detailed inspections show that there are issues in the float reference signal including periodic silence and other artifacts while the BASOP version does not contain these artefacts. These issues have been fixed in the later floating point codec development.

The EXT output MLD scores contain only the effect of transport channels and the binaural output also includes the effect of metadata parameters in practical use. As is evident, the histograms between EXT and BIN are similar.

Metadata coding was also inspected and there are differences observed from 0% to 50% of frames of a single file. Some differences are expected as metadata encoding decisions may differ due to minor numerical differences while producing perceptually equivalent quality bitstream.

### Subjective evaluation

- BASOP main: faf1 f546 (2025-03-27)

- Float reference ivas-float-update: 5359 4392 (2025-03-27)

Two DCR listening tests were conducted with naïve listeners for MASA 1TC and MASA 2TC. Binaural rendering was used for all conditions. There were 18 and 20 naïve listeners, respectively. Sennheiser HD650 headphones were used for binaural listening. No head-tracking was used. Test material was speech material half with and half without background noise. One of the clean speech categories contained over-talking. There was a total of 16 different test sequences of about 8 seconds.

The green color shows results encoded and decoded with float IVAS codec. Blue color shows results encoded with fixed point (BASOP) and decoded with floating point IVAS codec (ivas-float-update). In addition to a single bitrate per condition, also bit-rate switching was tested. Bitrate switching to a new bitrate occurred randomly within the notified range. The rate of change was also randomized and was done every 400ms to 800ms. i.e. within each listened sequence of 8 seconds there were between 10 and 20 bitrate changes. In addition, frame erasures were added to some conditions with 5% frame error rate. MASA tests also included conditions with DTX enabled.

One condition had all: bitrate switching, DTX and FER5 enabled.

One issue can be seen in the MASA 1TC results. MASA 1TC 48k DTX (BASOP encoder) is significantly worse than the reference condition (ivas-float-update). Expert verification was done on this condition, and indeed it sounded much worse, during active speech there was significant distortion audible. Since the tested version was from late March a newer version was pulled from the BASOP repo

•	BASOP main: 687c d90d (2025-04-28)

•	Float reference ivas-float-update: d3f0 1fcf (2025-04-24)

With this version the whole listening test was reprocessed and the problematic condition was listened by experts. The issue was fixed, since now the condition sounded the same as the float-reference. The issue was fixed while we were running the listening test.

Second listening test was conducted with MASA 2TC. This test had no statistically significant issues. Some concerning conditions (13.2k DTX and bitrate switching13.2k-48k with DTX) were listened with experts, but no issues were heard.

### Other observations

In general, most serious of the MASA operation mode differences seem to be explained by differences in the stereo core. In addition, it has been observed that the BASOP MASA encoder does introduce decision differences which create large metadata structure differences. This is seen in a rather significant number of differing metadata frames while the perceptual effect of the difference can be less significant.

### Issues identified

| Issue | Description | Date Created | Component | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| #1498 | BASOP encoder has a large difference signal in noise signal using MASA selection items with DTX | April 17, 2025 | MASA, Core, DTX | Medium | Open |

## OSBA

### Low bitrates 13.2 to 128 kbps

#### Objective evaluation

Objective evaluation was carried out on 32 vectors (speech, music, mixed content). OSBA test items were constructed by combining ISM and SBA material from the IVAS selection tests. The following software versions were used:

Floating point: 1bbe1956

Fixed point: 858df806

The following parameter combinations were tested exhaustively, resulting in 810 test cases. The output format was FOA.

| Parameter | Values |
| --- | --- |
| Input Format | ISM4+HOA3 |
| Bitrate (kbps) | 13.2, 16.4, 24.4, 32, 48, 64, 80, 96, 128 |
| Sampling Rate (kHz) | 16, 32, 48 |
| Input signal level (LKFS) | -36 (-10dB), -26 (nominal), -16 (+10dB) |
| DTX | disabled, enabled |

No crashes were observed. The following figures show MLD and ODG histograms for relevant test case subsets.

Figure 63: OSBA MLD and ODG histograms, all tested conditions.

Figure 64: OSBA MLD and ODG histograms, -36LKFS loudness (-10dB from nominal).

Figure 65: OSBA MLD and ODG histograms, -26LKFS loudness nominal).

Figure 66: OSBA MLD and ODG histograms, -16LKFS loudness (+10dB from nominal).

Figure 67: OSBA MLD and ODG histograms, 1 transport channel (TC).

Figure 68: OSBA MLD and ODG histograms, 2 transport channels (TC).

Figure 69: OSBA MLD and ODG histograms, 3 transport channels (TC).

Figure 70: OSBA MLD and ODG histograms, 16kHz sampling rate

Figure 71: OSBA MLD and ODG histograms, 32kHz sampling rate

Figure 72: OSBA MLD and ODG histograms, 48kHz sampling rate

MLD results suggest very good overall behaviour of the fixed point encoder, but the elevated MLD numbers for 1TC operation evidence the related low-bitrate SBA and OSBA issues reported in sections 6.9.3 and 6.11.1.3, respectively.

#### Subjective evaluation

At bitrates up to 128 kbps, OSBA operation consists of a pre-render to SBA, followed by an SBA encode step. Because of this, objective evaluation of OSBA at these bitrates was deemed sufficient and no subjective evaluation has been carried out explicitly. The reader is referred to the results of subjective SBA evaluation at the same bitrates, shown in section 6.9.2.

#### Issues identified

The following 4 issues remain open as of May 13, 2025. The earliest open issue is from December 2024. Most issues consist of audible differences related to low bitrate (1 TC) operation.

| Issue | Description | Date Created | Component | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| #1138 | High MLD between signals decoded from low bitrate fixed- and floating-point encoded OSBA inputs | Dec 19, 2024 | Low bitrate | Major | Open |
| #1287 | Major spectral difference between signal decoded from fixed- and floating-point encoded bitstreams at 13.2kbps for OSBA inputs | Feb 12, 2025 | Low bitrate | Critical | Open |
| #1305 | Audible artifact in signal decoded from fixed-point encoded OSBA low bitrate bitstreams with -16LKFS input sampled at 16/32 kHz | Feb 17, 2025 | Low bitrate | Critical | Open |
| #1442 | BASOP encoder: audible differences in noise signal in OSBA at 48 kbps | Mar 26, 2025 | SBA | Medium | Open |

### High bitrates 160 to 512 kbps

#### HOA3 with ISM1, ISM2, ISM3 and ISM4

##### Objective evaluation

For the OSBA objective evaluation, test items of combining samples from FhG internal ISM1-4 database with samples from an HOA3 database were used. Conditions tested are shown below:

| BASOP encoder SHA | d32077e8 |
| --- | --- |
| Float decoder SHA | d3f01fcf |
| Bitrates | 160, 192, 256, 384, 512 kbps |
| Sampling Rate | 16, 32, 48 kHz |
| Input Levels | -10, 0, +10 dB |
| DTX | Off (not supported) |

The objective metrics of MLD, delta-ODG and SSNR are shown below. The comparison between the two versions is done per channel and the worst-case value is taken.

It should be noted that the very high MLD values seem to come from the bug reported in issue 1254 (see 6.11.2.1.4 )

##### Subjective evaluation

For the subjective evaluation a Ref A-B test was conducted testing OSBA with HOA3 and ISM4 coded with 256kbps FB. The items were a critical subset of those in the objective evaluation  It should be noted that three input levels 0, -10, +10dB were assigned to the items in a circular manner, i.e. item0 –> 0dB, item1  -> -10dB, item3 -> +10dB and so forth. The BASOP encoder version that was tested is 641c78fa .

The results with 8 expert listeners are shown below.

From the listening test result there is a clear degradation for Item 2 . After investigation it was found that it is linked with the issue reported in 1454. Although there is a tendency towards for degradation for some items, there is also positive grading compared to float, averaging out the overall score as being on par. However, this may also be an indication that there are significant differences between the two versions.

##### Other observations

There seems to be an issue in some cases with having different bitrate distribution for MCT that is used for the core coder for both ISMs and SBA that is then affecting the estimation of the DirAC parameters and therefore the spatial image. This was revealed also with Item 2 of the subjective listening test and reported in issue 1454.

##### Issues identified

| Issue | Summary | Commit | Component | Severity | Status |
| --- | --- | --- | --- | --- | --- |
| 1454 | For the 11 lower CLDFB bands, the DirAC parameters are estimated in the decoder. With the LTVs one can see very large differences in these parameters. These occur when the same signal is encoded with the BASOP and float encoders, these two bitstreams are decoded with the float decoder, and the DirAC parameters are written to a file in the float decoder. This is a secondary problem that comes from differences in the MCT channels and can be revisited once some progress has been made there. | d3fbdc6b | MCT | Medium | Open |
| 1442 | Noise sounds audibly different in the OSBA LTV with 1 ISM and FOA. The spectrum of the difference signal shows that high frequencies are most strongly affected. | a43b5c15 | SBA encoder | Medium | Open |
| 1435 | At some locations in the LTV for 2 ISM 3OA OSBA, there are large differences in the W signal. | d74de338 | MCT encoder | Medium | Open |
| 1254 | The output signal exhibits strong differences between the two encoders. BASOP output is completely broken in this case. I suspect that there is a connection with the very high signal levels and the strong correlation between the channels in this test item. There could be a connection with the downmix scaling issue. These items were made to test the worst case signal amplitude. | f5041511 | OSBA encoder | Critical | Open |
| 1026 | For OSBA at 512 kbit/s, there are differences in the low-band of the SBA components: Several SBA components (e.g. 6, 8, 11, 13, 15 corresponding to audio channels 10, 12, 15, 17, 19) are almost completely zero. In the floating-point version, there is basically no content below 10 kHz, whereas in BASOP there are some leftovers. | 916e6860 | OSBA encoder | Medium | Open |

#### HOA2 with ISM1, ISM2, ISM3 and ISM4

##### Objective evaluation

For the objective evaluation the same items for testing as described in 6.11.2.1.1 were used by converting the SBA part to HOA2. Conditions tested are shown below:

| BASOP encoder SHA | d32077e8 |
| --- | --- |
| Float decoder SHA | d3f01fcf |
| Bitrates | 160, 192, 256, 384, 512 kbps |
| Sampling Rate | 16, 32, 48 kHz |
| Input Levels | -10, 0, +10 dB |
| DTX | Off (not supported) |

The objective metrics of MLD, ODG and SSNR are shown below. The comparison between the two versions is done per channel and the worst-case value is taken.

##### Subjective evaluation

Void.

##### Other observations

See 6.11.2.1.3

##### Issues identified

See 6.11.2.2.4

#### FOA with ISM1, ISM2, ISM3 and ISM4

##### Objective evaluation

For the objective evaluation the same items for testing as described in 6.11.2.1.1 were used by converting the SBA part to FOA. Conditions tested are shown below:

| BASOP encoder SHA | d32077e8 |
| --- | --- |
| Float decoder SHA | d3f01fcf |
| Bitrates | 160, 192, 256, 384, 512 kbps |
| Sampling Rate | 16, 32, 48 kHz |
| Input Levels | -10, 0, +10 dB |
| DTX | Off (not supported) |

The objective metrics of MLD, ODG and SSNR are shown below. The comparison between the two versions is done per channel and the worst-case value is taken.

##### Subjective evaluation

Void.

##### Other observations

See 6.11.2.1.3

##### Issues identified

See 6.11.2.1.4

## OMASA

### Objective evaluation

An objective evaluation of OMASA was performed by Nokia to evaluate the performance of OMASA using the long test vectors used in the automated testing of the BASOP conversion, namely ltv48_OMASA*.wav/.met variants available in https://forge.3gpp.org/rep/ivas-codec-pc/ivas-pc-testfiles. These inputs contain 1-4 ISMs and 1-2 transport channels for MASA. The input has been specifically designed to test different paths of the OMASA codec. The evaluation was performed with the fixed-point encoder commit with SHA ba1f52bd5 and the obtained bitstream was processed to selected outputs using the float reference decoder commit with SHA d3f01fcfb.

The following combinations of operation mode variations were used.

| Parameter |  |
| --- | --- |
| Output format | Binaural |
| Bitrates | 13.2 – 512 (all IVAS bitrates) |
| Bitrate switching | All bitrates |
| DTX | Off (not supported) |
| Bandwidth | WB, SWB, FB |
| Input FS | 16, 32, 48 |
| Output FS | 48 |
| Transport channels | 1 TC and 2 TC |
| Number of directions | Switching 1dir/2dir in one file |
| Number of ISM | 1 – 4 |

As these parameter combinations would produce an excessive number of combinations to run, the all-pairs testing approach was applied to parameters bitrate, bandwidth, and input FS. A total of 369 cases are presented below in the histogram figures. All combinations processed successfully.

As seen from the figure, the overall MLD score is higher than, e.g., the one observed with MASA.

### Subjective evaluation

- BASOP main: cf34 e040 (2025-04-17)

- Float reference ivas-float-update: b63c 033f (2025-04-03)

DCR listening test was conducted with naïve listeners. Binaural rendering was used for all conditions. There were 17 naïve listeners. Sennheiser HD650 headphones were used for binaural listening. No head-tracking. Test material was generic OMASA material. Material consisted mainly of talkers in various background conditions (13), as well few musical objects with background (3). There was a total of 16 different test sequences of about 8 seconds. OMASA samples were in categories with 4 samples each with 1-4 ISMs respectively. This means that all OMASA configurations (MASA + 1 ISM, MASA + 2 ISM, MASA + 3 ISM, MASA + 4 ISM) were tested for each bitrate.

The green color shows results encoded and decoded with float IVAS codec. Blue color shows results encoded with fixed point and decoded with floating point IVAS codec. In addition to a single bitrate per condition, also bit-rate switching was tested. Bitrate switching to a new bitrate occurred randomly within the notified range. The rate of change was also randomized and was done every 400ms to 800ms. i.e. within each listened sequence of 8 seconds there were between 10 and 20 bitrate changes. In addition, frame erasures were added to some conditions with 5% frame error rate.

From the results no statistically significant subjective degradations can be seen with any of the tested operation points. Additionally, the largest degradations (13.2k, 32k, 80k all with FER5) were listened by the Nokia experts and no significant issues were noticed with these conditions.

### Other observations

The OMASA spatial metadata analysis may also contain differences between BASOP and float as shown in the BASOP issue 1200 in https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/issues/1200.

### Issues identified

Void.

# Summary of identified issues

This is a list of the main problematic areas to be addressed.

| Issue category | Description |
| --- | --- |
| Complexity and memory | The IVAS BASOP encoder is, on average, 1.46 times more complex than the IVAS FLOAT encoder, whereas the target ratio, based on the EVS encoder, is 0.79. In the worst-case scenario, the IVAS BASOP encoder is 2.47 times more complex than the IVAS FLOAT codec. The corresponding worst-case number for EVS BASOP vs EVS FLOAT is 1.37. The average encoder HEAP memory consumption of the IVAS BASOP codec is 0.89 times higher compared to the IVAS FLOAT codec, while the target value derived from the EVS codec is 0.53. Similarly, the average encoder STACK memory consumption of the IVAS BASOP codec is 1.16 times higher than that of the IVAS FLOAT codec, with the target value from the EVS codec being 0.75. The PROM and TROM consumption of the IVAS BASOP codec are in line with the IVAS FLOAT codec with average ratios of 0.92, resp. 0.96. The target values derived from the EVS BASOP and FLOAT codecs are 0.74 and 0.90, respectively. |
| SBA and OSBA low bitrates (1TC) | Elevated MLD scores and several open issues related to audible quality degradations. |
| MC ParamUpmix (7.1.4 at 160kbps) | Energy leakage to LFE channel. |  |
| DTX | The DTX activity pattern differs between BASOP in relation to float. In some cases CNG is triggered in the wrong segments, leading to degradations. |
| Energy calculation | There seems to be precision issues in general for calculating signal energies. In MDCT stereo the power spectra and energy is calculated in quite a few places that are essential for the whole processing chain, that is for SNS, band-wise M/S decision (see above) and IGF to name the most prominent ones. It seems that one single common exponent/q is chosen for both MDCT and MDST spectra despite that they may have large differences in magnitude. While calculating energies these dynamic differences double, leading to very low precision values and not representable values. One potential solution would be to not use a common exponent/q. Also for the representation of Mid/Side spectra it also might make sense to use separate exponent/q values because the magnitude of Mid and Side data is inherently very different. |
| Stereo MS decision | There seem to be an issue with the bandwise M/S decision that is the core of the MDCT stereo coding. This is reflected in both issues #1464 and #1308. As described in issue #1464 this comes from different estimations of the quantization gains and the cause of that is precision issues on the energy calculation. Latest observation with checking SHA version 1ccc3c58 shows an improvement for LTV but still there are some frames with high deviation |
| Bandwidth extension | Various open issues pointing towards problems with bandwidth extension, for example, as reported in issues #1091, #1475, #1493, #1510, or #1530. |
| S/M classifier | Precision issues in the speech/music classifier impacts the core coder decision which was observed to degrade several signals. |
| Various discontinuities in signal, especially but not limited to 16 kHz sampling rate | Various discontinuities are present in the encoded signal, especially but not limited to 16 kHz sampling rate of the input signal. These issues have for example, been reported in issues #1388, #1392, and #1457. |
| Loudness issue with ParamMC for all MC input modes for 16kHz sampling rate | There is an issue with loudness for 16 kHz sampling rate and WB. For a large number of samples there is a clear loudness difference (in the example of issue 1583 in 6.6.2.4 of 2.84 LUFS). This happens only for this sampling rate and regardless of the input loudness but mainly occurs for +10dB and 0 dB cases. Additionally, this is present for all MC input formats and is the cause of the very high MLD scores that can be seen in the respective plots (e.g. in 6.6.2.1) |

# Overview of open issues at 3GPP Forge

The issue tracker at 3GPP Forge [4] is continuously being updated, with problems being reported and resolved. This is a snapshot reflecting the current status on May 16, 2025. Some issues are resolved fast while a few other issues have been open for a longer time, up to 7-11 months. The summary below excludes issues that are not relevant for the BASOP conversion and have any of the labels:

- Type:FloatUpdatePorting

- Type:FloatReferenceIssue

- Maintenance:CI

Table 7: Open issues per module

| Category |  | Number of issues |  |
| --- | --- | --- | --- |
| Assigned to Ittiam | 83 | Encoder | 51 |
|  |  | Decoder/Renderer | 31 |
|  |  | Other | 1 |
| Assigned to PC, or unassigned | 71 | Encoder | 31 |
|  |  | Decoder/Renderer | 25 |
|  |  | Other | 15 |

Table 8: Open issues with prioritization labels

| Category |  | Priority label |  |
| --- | --- | --- | --- |
| Assigned to Ittiam | 83 | Blocker | - |
|  |  | Critical | 9 |
|  |  | Major | 35 |
|  |  | Medium | 29 |
|  |  | Minor | 4 |
|  |  | No priority | 6 |
| Assigned to PC, or unassigned | 71 | Blocker | - |
|  |  | Critical | 10 |
|  |  | Major | 26 |
|  |  | Medium | 14 |
|  |  | Minor | 7 |
|  |  | No priority | 14 |

# Summary

A verification of the delivered IVAS BASOP Encoder has been carried out, covering all supported formats. As shown in the objective scores in 4.1, the percentage of IVAS test cases below MLD 10 is 81%, compared to 94% of EVS test cases. The threshold of MIN_ODG > -0.1 was in practice not used, since EVS has only 4% of the test cases above this limit. The corresponding figure for the IVAS BASOP Encoder is 7%. The computational complexity is on average 1.45 times the target ratio of EVS BASOP vs EVS FLOAT. A summary of the listening test results can be found in Table 9.

Summary: Out of 79 tests, there are 73 passes and 6 failures.

- Note 1: Based on different versions of the BASOP encoder.

- Note 2: Pass/fail derived on average results.

Table 9: Summary of subjective listening test results.

| Test | Clause | Operating point | Test method | Result |
| --- | --- | --- | --- | --- |
| StereoDmxEVS | 6.1.2 | StereoDmxEVS@13.2 kbps, critical music | MUSHRA | Fail |
|  |  | StereoDmxEVS@24.4 kbps, critical music | MUSHRA | Pass |
| Stereo 13.2, 16.4 | 6.2.1.2 | Stereo@13.2 kbps clean speech | Ref-AB | Fail |
|  |  | Stereo@13.2 kbps street & noise | Ref-AB | Pass |
|  |  | Stereo@16.4 kbps clean speech | Ref-AB | Pass |
|  |  | Stereo@16.4 kbps mixed content | Ref-AB | Fail |
| Stereo 24.4, 32 | 6.2.2.2 | Stereo@24.4 and 32 kbps, mixed content | Ref-AB | Pass |
| Stereo 48 - 256 | 6.2.3.2 | Stereo@48 kbps, critical items | Ref-AB | Pass |
|  |  | Stereo@96 kbps, critical items | Ref-AB | Pass |
| ParamMC 5.1 | 6.3.2.2 | ParamMC@64 kbps, critical items | Ref-AB | Pass |
| MCT 7.1 | 6.4.3.2 | MC@128 kbps, critical items | Ref-AB | Pass |
| MC 7.1+4 McMASA | 6.7.1.2 | MC@16.4 kbps, generic content | DCR | Pass |
|  |  | MC@32 kbps, generic content | DCR | Pass |
|  |  | MC@48 kbps, generic content | DCR | Pass |
|  |  | MC@64 kbps, generic content | DCR | Pass |
|  |  | MC@96 kbps, generic content | DCR | Pass |
|  |  | MC@24.4 kbps, generic content, 5% FER | DCR | Pass |
|  |  | MC@13.2 – 96 kbps, generic content | DCR | Pass |
|  |  | MC@32 – 96 kbps, generic content, 5% FER | DCR | Pass |
| ParamMC 7.1+4 | 6.7.2.2 | MC@128 kbps, critical items | Ref-AB | Pass |
| ParamUpMix 7.1+4 | 6.7.3.2 | MC@160 kbps, speech, music & mixed | MUSHRA | Pass |
| ISM 13.2 - 48 | 6.8.1.2 | ISM1@13.2 kbps, clean speech | Ref-AB | Pass |
|  |  | ISM2@16.4 kbps, clean speech | Ref-AB | Pass |
|  |  | ISM2@32 kbps, clean speech | Ref-AB | Pass |
|  |  | ISM2@48 kbps, clean speech | Ref-AB | Pass |
|  |  | ISM4@48 kbps, clean speech + music | Ref-AB | Fail |
| ISM3-4 32 kbps | 6.8.2.2 | ISM3-4@32 kbps, critical items | Ref-AB | Pass |
| ISM4 64 kbps | 6.8.3.2 | ISM4@64 kbps, critical items | Ref-AB | Pass |
| SBA | 6.9.2 | SBA@256 kbps | MUSHRA | Pass |
|  |  | SBA@512 kbps | MUSHRA | Pass |
|  |  | SBA@48 kbps | MUSHRA | Pass |
|  |  | SBA@80 kbps | MUSHRA | Pass |
|  |  | SBA@13.2 kbps | MUSHRA | Pass |
|  |  | SBA@24.4 kbps | MUSHRA | Pass |
|  |  | SBA@96 kbps | MUSHRA | Pass |
|  |  | SBA@160 kbps | MUSHRA | Pass |
|  |  | SBA@48 kbps | MUSHRA | Pass |
|  |  | SBA@80 kbps | MUSHRA | Pass |
|  |  | SBA@32 kbps | MUSHRA | Pass |
|  |  | SBA@16.4 kbps | MUSHRA | Pass |
|  |  | SBA@13.2 kbps | MUSHRA | Pass |
|  |  | SBA@24.4 kbps | MUSHRA | Pass |
| MASA 1TC | 6.10.2 | MASA@32 kbps speech clean+bg | DCR | Pass |
|  |  | MASA@64 kbps speech clean+bg | DCR | Pass |
|  |  | MASA@128 kbps speech clean+bg | DCR | Pass |
|  |  | MASA@13.2 kbps speech clean+bg, DTX | DCR | Pass |
|  |  | MASA@48 kbps speech clean+bg, DTX | DCR | Fail |
|  |  | MASA@24.4 kbps speech clean+bg, 5% FER | DCR | Pass |
|  |  | MASA@80 kbps speech clean+bg, 5% FER | DCR | Pass |
|  |  | MASA@16.4 kbps speech clean+bg, DTX, 5% FER | DCR | Pass |
|  |  | MASA@32 kbps speech clean+bg, DTX, 5% FER | DCR | Pass |
|  |  | MASA@32-512 kbps speech clean+bg | DCR | Pass |
|  |  | MASA@13.2-48 kbps speech clean+bg, DTX | DCR | Pass |
|  |  | MASA@16.4-96 kbps speech clean+bg, DTX, 5% FER | DCR | Pass |
| MASA 2TC | 6.10.2 | MASA@32 kbps speech clean+bg | DCR | Pass |
|  |  | MASA@64 kbps speech clean+bg | DCR | Pass |
|  |  | MASA@128 kbps speech clean+bg | DCR | Pass |
|  |  | MASA@13.2 kbps speech clean+bg, DTX | DCR | Pass |
|  |  | MASA@48 kbps speech clean+bg, DTX | DCR | Fail |
|  |  | MASA@24.4 kbps speech clean+bg, 5% FER | DCR | Pass |
|  |  | MASA@80 kbps speech clean+bg, 5% FER | DCR | Pass |
|  |  | MASA@16.4 kbps speech clean+bg, DTX, 5% FER | DCR | Pass |
|  |  | MASA@32 kbps speech clean+bg, DTX, 5% FER | DCR | Pass |
|  |  | MASA@32-512 kbps speech clean+bg | DCR | Pass |
|  |  | MASA@13.2-48 kbps speech clean+bg, DTX | DCR | Pass |
|  |  | MASA@16.4-96 kbps speech clean+bg, DTX, 5% FER | DCR | Pass |
| OSBA | 6.11.2.1.2 | OSBA@256 kbps, 4 ISM HOA3, critical items | Ref-AB | Pass |
| OMASA | 6.12.2 | OMASA@24.4 kbps, generic items | DCR | Pass |
|  |  | OMASA@32 kbps, generic items | DCR | Pass |
|  |  | OMASA@48 kbps, generic items | DCR | Pass |
|  |  | OMASA@64 kbps, generic items | DCR | Pass |
|  |  | OMASA@128 kbps, generic items | DCR | Pass |
|  |  | OMASA@13.2 kbps, generic items, 5% FER | DCR | Pass |
|  |  | OMASA@32 kbps, generic items, 5% FER | DCR | Pass |
|  |  | OMASA@80 kbps, generic items, 5% FER | DCR | Pass |
|  |  | OMASA@96 kbps, generic items, 5% FER | DCR | Pass |
|  |  | OMASA@32-512 kbps, generic items | DCR | Pass |
|  |  | OMASA@13.2-512 kbps, generic items, 5% FER | DCR | Pass |
|  |  | OMASA@16.4-96 kbps, generic items, 5% FER | DCR | Pass |

# References

- TR 26.258 Codec for Immersive Voice and Audio Services (IVAS); C code (floating-point)

- IVAS-10 IVAS BASOP Verification (IVAS-10), https://www.3gpp.org/ftp/tsg_sa/WG4_CODEC/TSGS4_132_Fukuoka/Docs/S4-251012.zip

- AFsp Package, Audio File Utility Programs, https://www.mmsp.ece.mcgill.ca/Documents/Downloads/AFsp/AFsp-v9r0.tar.gz

- IVAS BASOP repository on 3GPP Forge, https://forge.3gpp.org/rep/sa4/audio/ivas-basop

- ITU-T WMC Tool, https://github.com/openitu/STL/tree/dev/src/wmc_tool

- TR 26.952, Codec for Enhanced Voice Services (EVS); Performance characterization, https://www.etsi.org/deliver/etsi_tr/126900_126999/126952/12.05.00_60/tr_126952v120500p.pdf

- S4aA230086-IVAS Permanent Document IVAS-8a:Test Plan for Selection Phase v.1.1.0

It should be noted that the ODG scores plots represent the difference of ODG using the original input as reference vs the float coded output minus ODG using original input as reference vs BASOP encoded float decoded output