3GPP TSG-SA4 Meeting #132                                                                             S4-251014

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.822 | CR | 0003 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Communication overhead of measurement-based pre-compensation for PDU Set size correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Innovation Center Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The range of how often a message exchange is needed for the measurement-based pre-compensation for PDU Set size correction is too coarse. There is formatting issue in the caption of Figure 6.4.2-4.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1st change: added 2 references. 2nd change: corrected missing break line in the caption of Figure 6.4.2-4. 3rd change: added in clause 6.4.2 improved estimates on the need for update (the frequency of which determines the communicaiton overhead) for the measurement-based pre-compensation for PDU Set size correction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading informaiton on the communicaiton overhead for the measurement-based pre-compensation for PDU Set size correction. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: showed track changes that were missing in the original S4-250779. |  |  |  |  |  |  |  |  |

Add the following references, and revise clause 6.4.2.

1st Change

[62]	IETF RFC 2453: "RIP Version 2."

[63]	IETF RFC 4271: "A Border Gateway Protocol 4 (BGP-4)."

2nd Change

Figure 6.4.2-4: PSSize error for the cases of without correction (blue), with correction assuming NAT46 only (green), and with the proposed measurement-based correction with per-frame feedback(red) for Scenario 1.

Scenario 2 (MTU=1300 bytes):

This is to show the effect of MTU size. The MTU size is set to 1300 bytes. A typical packet of 1400 bytes is fragmented into 2 packets. The errors in the PSSize are shown in Figure 6.4.2-5. The mean absolute error in the PSSize are 963.3 bytes, 630.0 bytes and 24.7 bytes for the case of no PSSize correction, NAT46 only correction and measurement-based correction, respectively.

3rd Change

Observation 5: The proposed pre-compensation based PSSize correction method effectively reduces the PSSize error.

The frequency at which the UE sends the feedback does not exceed the frequency of routing changes. For major routing protocols, the routing changes at a time scale much larger than the video frame period (which, for example, is 16.7ms for 60 frames per second). For example, for Routing Infomration Protocol (RIP), the route is updated every 30 seconds [62]. The minimum amount of time between two route updates is 30 seconds for External BGP (EBGP), and 5 seonds for Internal BGP (IBGP) [63].

Observation 6: The proposed pre-compensation based PSSize correction method incurs a feedback message at a frequency less than once per 5 seconds.

For the ‘NAT46/64 only correction’ method to work, the sender needs to be aware of the presence of NAT46/64. The awareness may be obtained by feedback from the receiver on the type of the IP address type seen by the receiver. The feedback may be one time during a session, and can be done by SDP signaling, Simple WebRTC Application Protocol (SWAP) in TS 26.113 [58], or RTCP.

TS 26.522 [2] provides guidelines for preventing IP fragmentation, either through path MTU discovery or by assuming a conservative MTU size at the sender in generating IP packets. Path MTU discovery needs support from the routers on the end-to-end path and incurs communication overhead, and a conservative MTU size may lead to unnecessarily small IP packet sizes which come with a higher packet header cost (i.e., the ratio of the size of packet headers to the size of the media). We consider the guidelines as a solution for IP fragmentation prevention.

We compare the three solutions. The first criteria is whether the solution is generic, i.e., whether the solution can tackle multiple and even unknown causes to the error in the PSSize. The criteria ‘need support from the network?’ means whether the network needs to be configured (e.g., configured to support path MTU discovery) to enable a solution.

Table 6.4.2-1: Pros and Cons of the three solutions

| Solution | Generic | Accuracy | Need support from the network? | Communication overhead? | Need spec change? |
| --- | --- | --- | --- | --- | --- |
| NAT46/64 only correction | No | Low | No | Low | Yes |
| IP fragmentation prevention guidelines | No | Low | Yes if use path MTU discovery; no if use conservative MTU size. | Moderate if use path MTU discovery; none if use conservative MTU size. | No |
| Measurement-based correction | Yes | High | No | From low (one-time feedback) to at most one message per a few seconds | Yes |