# 3GPP TSG-SA WG4 Meeting #132																S4-250996

# Japan, Fukuoka, 19 – 23 May 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR | 0021 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5G_RTP_Ph2] RTP Header Extension for Expedited Transfer Indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Stage 2 definition of Expedited Transfer Indication (ETI) in TS 23.501 expects packets at N6 to be marked with ETI when expedited data transfer is enabled. For RTP traffic over the service data flow, the ETI marking is supposed to be part of an RTP header extension captured in TS 26.522. However, such ETI signalling in an RTP header extension is currently missing from TS 26.522. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new RTP HE for ETI |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Stage 3 specification for ETI signalling over user plane. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4.x (new), Annex D.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 26.113 CR 0008r1 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AP	Aggregation Packet

AVC	Advanced Video Coding

BLA	Broken Link Access

CRA	Clean Random Access

DoF	Degrees of Freedom

ETI	Expedited Transfer Indication

FU	Fragmentation Unit

HE	(RTP) Header Extension

HEVC	High Efficiency Video Coding

IDR	Instantaneous Decoder Refresh

IRAP	Intra Random Access Picture

NAL	Network Abstraction Layer

NRI	nal_ref_idc

NTP	Network Time Protocol

OS	Operating System

PACI	Payload Content Information

PPS	Picture Parameter Set

PSI	PDU Set Importance

PTP	Precision Time Protocol

RADL	Random Access Decodable Leading

RASL 	Random Access Skipped Leading

RTCP	RTP Control Protocol

RTCP XR	RTCP eXtended Report

SPS	Sequence Parameter Set

SRS	Split Rendering Server

SRTP	Secure RTP

TID	Temporal Identifier

UPF	User Plane Function

UDP	User Datagram Protocol

VCL	Video Coding Layer

VPS	Video Parameter Set

XR	eXtended Reality

# * * * * Next change * * * *

## 4.x	RTP header extension for expedited transfer indication

### 4.x.1	Description

The RTP HE for ETI marking is defined in this clause. ETI marking can be performed by an RTP sender, such as an Application Server or a sender UE, that sends media to an RTP receiver, such as a UE.

The XR traffic characteristics can be dynamic and vary greatly based on user interactions. The associated media payloads (e.g., video key frames, avatar representations) can, at times, have larger sizes challenging for networks to handle with short delays. It can be therefore desirable for an RTP sender to dynamically request access to expedited data transfer for a flow when media payloads are large.

NOTE 1: 	An expedited transfer indication is a dynamically changing traffic characteristic, and the expedited data transfer is applicable to both downlink and uplink in the 5G System only when the 5G NR modem at the RTP receiver supports Reflective QoS, as defined in clause 5.37.10.3 of [12].

Endpoints that support the RTP HE for ETI marking shall support marking shall support both RTP HE formats (i.e., the one-byte and the two-byte formats) according to RFC 8285 [11].

If the RTP HE for ETI marking is the only RTP HE used, the endpoints shall use the one-byte header format. If other 2-byte RTP HE elements are used in any other RTP stream, then the two-byte header shall be used, unless the "a=extmap-allow-mixed" is successfully negotiated through SDP offer/answer, as described by RFC 8285 [11].

NOTE 2:	The headers are not shown with padding as this depends on other prospective extension elements in use, as per RFC 8285 [11] alignment specifications.

The IANA registration information for the RTP HE for ETI marking is provided in Annex D.X.

### 4.x.2	One-byte RTP Header Extension Format

The one-byte RTP HE for ETI marking is defined as follows:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID   | len   |      R      |B|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

### 4.x.3	Two-byte RTP Header Extension Format

The two-byte RTP HE for ETI marking is defined as follows:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|        0x100          |appbits|           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      ID       |      len      |      R      |B|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

### 4.x.4	Semantics

The semantics of the RTP HE for ETI marking fields are defined as follows:

-	Reserved [R] (7 bits): This field is reserved for future use. It shall be set to 0 by the RTP sender and shall be ignored by the RTP receiver.

-	Expedited Transfer Indication [B] (1 bit): This field indicates the RTP sender preference for expedited data transfer for the current PDU. It shall be set to 1 to indicate RTP sender preference to enable expedited data transfer. Otherwise, it shall be set to 0.

### 4.x.5	SDP Signalling

An RTP sender capable of sending RTP HE for ETI Set marking shall use the SDP extmap attribute for RTP HE for ETI marking in the media description of the RTP stream carrying the RTP HE for ETI marking. An RTP receiver that does not support RTP HE for ETI marking can ignore that RTP HE when included. The signaling of the ETI marking RTP HE shall follow the SDP signaling design and the syntax and semantics of the "extmap" attribute as outlined in RFC 8285 [11]. The URN for the RTP HE for ETI marking shall be set to "urn:3gpp:expedited-transfer-indication-marking:rel-19".

The ABNF syntax for the extmap attribute for the signaling of RTP HE for ETI marking is defined as follows, extending the ABNF in RFC 8285 [11]:

extensionname = "urn:3gpp:expedited-transfer-indication -marking:rel-19"

extensionattributes = [format]

format = "short" / "long"

The extension attributes have the following semantics:

-	format: indicates if the RTP HE for ETI marking uses the one-byte (short) or the two-byte (long) format

NOTE:	Regardless if this extension attribute is present or not, the use of short or long format is determined as described by section 4.1.2 of RFC 8285 [11], i.e., based on what format other RTP HEs use in the same RTP session, unless both endpoints announced support for handling mixed format with "a=extmap-allow-mixed" as described by section 6 of RFC 8285 [11].

Below is an example:

a=extmap:7 urn:3gpp:expedited-transfer-indication:rel-19 short

### 4.x.6	Guidelines for ETI Marking

When an RTP sender uses expedited data transfer in an RTP session, the RTP sender should mark all the RTP PDUs with the RTP HE for ETI marking according to semantics defined in Clause 4.x.4. Specifically, if expedited data transfer is desired for RTP PDUs of a media payload (e.g., a video frame), then the RTP sender shall mark the RTP PDUs with RTP HE for ETI marking with the Expedited Transfer Indication bit set to value “1”.

NOTE 1:	It is up to an RTP sender implementation to decide when an expedited data transfer is desired or not for an RTP media payload and its associated RTP PDUs.

NOTE 2: 	Non-RTP PDUs (e.g., RTCP or STUN PDUs) cannot be marked with RTP HE for ETI marking and thus are not expedited.

NOTE 3:	When expedited data transfer is used, any PDUs that are not marked by the RTP sender with RTP HE for ETI marking (including RTP, RTCP or other PDUs) can be handled by the 5G System according to a default QoS treatment.

# * * * * Next change * * * *

# D.X	urn:3gpp:expedited-transfer-indication:rel-19

The desired extension naming URI:

urn:3gpp:expedited-transfer-indication:rel-19

A formal reference to the publicly available specification:

TS 26.522

A short phrase describing the function of the extension:

Expedited transfer indication marking signalling for expedited data transfer, see clause 4.x

Contact information for the organization or person making the registration:

3GPP Specifications Manager

3gppContact@etsi.org

+33 (0)492944200

# * * * * End of changes * * * *