3GPP TSG-SA WG4 Meeting #132							S4-251085

Japan, Fukuoka, 19 – 23 May 2025

# Title:	Reply LS on IMS DC Multiplexing

# Response to:	S2-2502522 (S4-250769)

# Release:	Rel-19

# Work Item:	NG_RTC_Ph2

Source:	SA WG4

To:	SA WG2

CC:	-

Contact person:	Bo Burman

bo (dot) burman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: -

# 1	Overall description

SA4 thanks SA2 for their liaison on IMS DC Multiplexing.

SA2 Question:

The 3GPP SA2 R19 NG_RTC_Ph2 includes KI#7 supporting multiplexing multiple DC applications over a single SCTP connection. In the clause AC.7.10 of TS 23.228, SA2 has defined the following multiplexing scenarios:

-	Multiplexing local bootstrap data channel and remote bootstrap data channel between the UE and its home IMS network using a single SDP m line media description if both support data channel multiplexing capability;

-	Multiplexing different application data channels for applications with compatible QoS requirements between the UE and its IMS network using a single SDP m line media description if both support multiplexing capability;

-	Multiplexing different application data channels for applications with compatible QoS requirements between the originating IMS network and terminating IMS network using a single SDP m line media description if both support DC multiplexing capability.

SA4 Answer:

SA4 would like to inform that TS 26.114 v19.0.0 supports all three of the scenarios described above. The first scenario is supported by this statement in clause 6.2.10.2:

A DCMTSI client that supports bootstrap data channel multiplexing may include bootstrap data channels for both local and remote bootstrap data channel content sources (see Table 6.2.10.1-2) in a single SDP m= line media description in the SDP offer.

The second scenario is supported by this statement in clause 6.2.10.2:

A DCMTSI client that supports application data channel multiplexing may include multiple "a=3gpp-req-app" attributes for different data channel applications with compatible QoS requirements in a single SDP m= line media description in the SDP offer.

The third scenario is supported by the same statement as for the second scenario above, taken in combination with the "a=3gpp-req-app" SDP attribute’s previously existing "adc-stream-id-endpoint" parameter as part of the "app-dc-info" parameter, which allows to differentiate what the SDP offerer intends the remote, answering endpoint to be ("Server" or "UE").

Further text in TS 26.114 v19.0.0, not quoted here, details SDP offer/answer aspects of the above, e.g., how an answering client that does not support multiplexing rejects its usage.

# 2	Actions

To SA WG2

ACTION: 	SA4 kindly asks SA2 to take the above provided answer into consideration.

# 3	Dates of next SA WG4 meetings

SA4#133-e:	21 - 25 July, 2025		Electronic meeting

SA4#134:	17 - 21 November, 2025	Dallas, U.S.

SA4#135:	9 - 13 February, 2026	TBD, India