3GPP TSG SA WG4 Meeting 132	S4-250797

# Fukuoka, JP, 19 - 23 May 2025

Source:	Qualcomm Sweden, Tencent

Title:	Pseudo-CR on [VOPS] Updated Terminology

Spec:	3GPP TS26.265v1.1.0

Agenda item:	9.5

Document for:	Decision

1. Introduction

Discussion terminology

2. Reason for Change

Missing

3. Conclusions

Do it

4. Proposal

It is proposed to agree the following changes to 3GPP TS26.265v1.1.0 at SA4-132.

5. Revision

Online edits addressed the updates in clauses 1-4.

To be safe, the integrated version is attached.

* * * First Change * * * *

# Introduction

The present document defines service-independent video operation points and capabilities.   These may be referenced in 3GPP service specifications or in third-party services.

# 1	Scope

Video codecs, encoders, and decoders are core components of 3GPP services. At the same time, video encoders and decoders, residing on 3GPP User Equipment (UE) and defined in 3GPP specifications, also provide interoperability points for third-party services. Video capabilities are predominantly independent of the service in use. This specification addresses the definition of video capabilities and Operation points such that 3GPP service specifications as well as third-party service providers can refer to the interoperability points defined in this specification.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor’s Note: References need to be updated to latest versions and to include ISO/IEC for dual publications.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[bt709]	Recommendation ITU-R BT.709-6 (06/2015): "Parameter values for the HDTV standards for production and international programme exchange"

[bt2100]	Recommendation ITU-R BT.2100-2 (07/2018): "Image parameter values for high dynamic range television for use in production and international programme exchange"

[h264]	Recommendation ITU-T H.264 (08/2021): "Advanced video coding for generic audiovisual services".

[h265]	Recommendation ITU-T H.265 (09/2023): "High efficiency video coding".

[h273]	Recommendation ITU-T H.273 (09/2023): "Coding-independent code points for video signal type identification".

[h274]	Recommendation ITU-T H.274 (09/2023): "Versatile supplemental enhancement information messages for coded video bitstreams".

[CMAF]	ISO/IEC 23000-19: "Information Technology Multimedia Application Format (MPEG-A) – Part 19: Common Media Application Format (CMAF) for segmented media".

[CENC]	ISO/IEC 23001-7: "MPEG systems technologies - Part 7: Common encryption in ISO base media file format files".

[DPC]	CTA-5003-B: "Web Application Video Ecosystem (WAVE): Device Playback Capabilities Specification", available at https://shop.cta.tech/products/web-application-video-ecosystem-device-playback-capabilities-cta-5003-b .

[6381]	IETF RFC 6381: The 'Codecs' and 'Profiles' Parameters for "Bucket" Media Types.

[MSE]	3GPP TR 26.857, "5G Media Service Enablers"

[3dtv]	A. Quested and B. Zegel, "3D-TV production standards - first report of the ITU-R Rapporteurs", EBU Technical Review, 2011 Q2, https://tech.ebu.ch/publications/trev_2011-Q2_3dtv_quested

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Access Unit: Smallest individually accessible portion of data within a Bitstream to which unique timing information can be attributed.

Bitstream: A sequence of bits that forms the representation of any coded pictures and their associated data. This sequence of bits is formed by one or more coded video sequences (CVSs).

Coded Video Sequence: A sequence of access units that consists of a series of coded frames and any associated metadata (required for decoder and rendering initialization and operations) and conforms to a specific video encoding format and aligns with a certain Operation Point, as defined in this document The first access unit of a CVS is a random access point.

Chroma: a sample array or single sample representing one of the two colour difference signals related to the primary colours, represented by the symbols Cb and Cr.

Coded Video Layer: A sequence of coded pictures within a Coded Video Sequence that can be identified by an unique identifier within the CVS, referred to as layer ID, and that represents one or more video signal components.

Hero Eye: The default eye in a stereo (stereoscopic) video pair, often determined by tags set by the cameras used to capture the video.

Luma: a sample array or single sample representing the monochrome signal related to the primary colours (denoted with the symbol Y),

Operation Point: A collection of discrete combinations of different video representation formats, including spatial and temporal resolutions, colour mapping, transfer functions, and the encoding format.

Random Access Point: see below (add)

Receiver: A device capable of decoding and rendering any bitstream that is conforming to a certain Operation Point.

Representation Format:

Video Layer sub-bitstream: The sub-bitstream generated by extracting one or more CVLs from a source bitstream.

Video Signal: to be added

Video Signal Component: to be added

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AVC	Advanced Video Coding

CENC	Common ENCryption

CMAF	Common Media Application Format

CVL	Coded Video Layer

CLVS	Coded layer-wise video sequence

CVS		Coded Video Sequence

DPC	Device Playback Capabilities

FFS	For Further Study

HDR	High Dynamic Range

HDTV	High-Definition TeleVision

HEVC	High Efficiency Video Coding

HLG	Hybrid Log-Gamma

MSE	Media Source Extension

MSE	Media Source Extension

MV-HEVC	MultiView extensions of HEVC

NAL	Network Abstraction Layer

RAP	Random access point

SDR	Standard Dynamic Range

UHD	Ultra-High Definition

VCL	Video Coding Layer

WCG	Wide Colour Gamut

# 4	Context and Definitions

## 4.1	Motivation

Video codecs, encoders, and decoders are core components of 3GPP services. At the same time, video encoders and decoders, residing on 3GPP User Equipment (UE) and defined in 3GPP specifications, also provide interoperability points for third-party services. Video capabilities are predominantly independent of the service in use. This specification addresses the definition of video capabilities and Operation points such that 3GPP service specifications as well as third-party service providers can refer to the interoperability points defined in this specification.

The present specification makes use some of the concepts recommended in TR 26.857 [2], i.e. the concept of Media Service Enablers.

## 4.2	Reference architectures and definitions

In order to define the normative aspects of this specification, reference architectures are defined. The core architecture is provided in Figure 4.2-1. The workflow addresses the generation of a video bitstream from a video signal using a video encoder as well as the decoding of a video bitstream by a video decoder and providing the resulting decoded video signal as well as associated metadata to a rendering and display process. The video signal follows a representation format. The video signal can be composed of one or more video signal components, for example a video signal can include multiple views. The representation format defines the signal components and each of its properties.

The video encoder as well as the video decoder may be configured to certain operations indicated by APIs in Figure 4.2-1. These APIs are not normatively specified but serve as an example reference to configure encoders and decoders as documented in Annex [A].

Figure 4.2-1 Reference architecture for video Operation points and capabilities

Video encoders produce a sequence of Coded Video Sequences (CVSs). A CVS is a sequence of access units that consists of a series of coded frames and any associated metadata (required for decoder and rendering initialization and operations). The first access unit of a CVS is a random access point (RAP).

An intra random access coded frame, together with the associated metadata, forms a Random Access Point (RAP) that permits to initialize decoding of the CVS.

The sequence of CVSs is referred to as Bitstream. In the context of this specification, Bitstreams conform to a specific video coding format and a specific representation format. The combination of video coding format and a specific representation format is referred to as Operation Point.

Receivers conforming to an Operation Point can decode the bitstream and render the included video signal together with the provided metadata.

In the decoding process, the decoder is provided with access units which correspond to pieces of the Bitstream that can be processed by the decoder to regenerate decoded video frames.

In an extension to the Figure 4.2-1 presented in Figure 4.2-2, a video signal 1 may include another video signal 2 (for example a lower resolution, a hero eye signal), and the video encoder may generate a Video Bitstream such that:

-	A receiver conforming to operation point 2 is able to decode the entire video bitstream and supports rendering of the included video signal 2.

-	In addition, a receiver conforming to operation point 1 is able to extract the relevant data and access units of the the entire video bitstream to decode video signal 1.

Figure 4.2-2 Extended Reference architecture for video Operation points and capabilities with multi-layer Bitstream.

Figure 4.2-2 provides an overview of the data model and the definitions in this specification.

Editor’s Note: This figure is for illustrative purposes, informative and may be moved to an Annex.

Figure 4.2-2 Informative Data model for illustration purposes

In this case, configuration information is coded into metadata, that can be provided to the decoder to initialize the decoding of the CSVs included in the Bitstream.

Editor’s Note: The following text needs to be updated. If not it will be removed.

[

Operation

Based on this introduction, the following terms are defined:

-	Operation Point: A collection of different possible video formats including spatial and temporal resolutions, colour mapping, transfer functions, etc. and a video encoding format.

-	Bitstream: A compressed media representation presented as a sequence of bits

-	that forms the representation of any coded pictures and associated metadata data,

-	this sequence of bits is formed by one or more CVSs and each CVS has identical metadata

-	the sequence of bits conforms to a particular video coding specification/format and one or more Operation Points.

-	comprised by access units that serve as units to be provided to decoders for regenerating frames.

-	Receiver: A device that can ingest and decode any Bitstream that is conforming to a particular video coding specification and Operation Point and optionally render it.

]

Operation

Operation

Operation

Operation

## 4.3	Capability Specification

This specification defines the following capabilities:

-	Video Decoding capability: The capability to decode any video bitstream that conforms to an Operation point and provides a conforming output video signal and possibly associated metadata.

-	Video Encoding capability: The capability to encode any video signal included in the Operation point to a bitstream that is decodable by decoder that conforms to the same Operation point.

While not explicitly stated in the capabilities, it is a requirement for decoders and receivers to process the data in real-time. For encoder, real-time encoding is typically also a requirement.

## 4.4	Video representation formats

### 4.4.1	Overview

This clause defines video representation formats in the context of media delivery in 3GPP. Video Representation Formats are parameters assigned to Video Signals for typical 3GPP service constraints.

In order to define video signals, a set of video signal parameters are defined in clause 4.4.2, with general  restrictions on what is defined in 3GPP media delivery.

Based on the defined video signal parameters, clause 4.4.3 defines several 3GPP video representation formats providing a subset of well-defined representation formats.

### 4.4.2	Video signal parameters

Video signals considered in this specification are represented by a sequence of pictures, where a picture can represent either an array of luma samples in a monochrome format or an array of luma samples and two corresponding arrays of chroma samples in a 4:2:0, 4:2:2, or 4:4:4 colour format. Only progressive signals are considered. A component refers to an array or single sample from one of the three arrays (luma and two chroma) that compose a picture. The Luma component represents a sample array or single sample representing the monochrome signal related to the primary colours (denoted with the symbol Y), and a chroma component represents a sample array or single sample representing one of the two colour difference signals related to the primary colours, represented by the symbols Cb and Cr.

Video signals are typically described by a set of parameters that are required for the proper rendering of the decoded signal. Table 4.4.2-1 documents typical video signal parameters and provides a definition and/or reference.

Table 4.4.2-1	Video Signal Parameters

| Parameter | Definition | 3GPP restrictions | Service or Application restrictions |
| --- | --- | --- | --- |
| Spatial Resolution width | The number of active samples per line for the luma component. Example values are 1280 or 1920 for HD, and 3840 for UHD. NOTE: The width does not restrict the encoding resolution to fixed values. Cropping parameters can be indicated that prescribe decoders the need to remove spatial video samples in a partially filled coding block that are not intended for presentation. | No restrictions | Restrictions possible |
| Spatial Resolution height | The number of active lines per picture for the luma component. Example values are 720 or 1080 for HD, and 2160 for UHD. NOTE: The height does not restrict the encoding resolution to fixed values. Cropping parameters can be indicated that prescribe decoders the need to remove spatial video samples in a partially filled coding block that are not intended for presentation. | No restrictions | Restrictions possible |
| Scan Type | Indicates the source scan type of the pictures as defined in clause 7.3 of Rec. ITU-T H.273. | Progressive |  |
| Chroma format indicator | Indicates whether the picture has only a luma component or that the picture has three colour components that consist of a luma component and two associated chroma components, such that the width and height of each chroma component are the width and height of the luma component divided by a factor defined by the chroma format as defined in Rec. ITU-T H.274, clause 7.3. | 4:2:0 |  |
| Bit depth | Indicates the bit depth for the samples of the luma component and the samples of the two associated chroma components. Note that in general, the bit depth of the luma component and of the two associated chroma components may differ. Typical values are 8 or 10 bits. | 8 or 10 bits Luma and chroma components shall use the same bit-depth |  |
| Colour primaries | Indicates the chromaticity coordinates of the source colour primaries as specified in clause 8.1 of Rec. ITU-T H.273. Typical values are 1 to refer to Rec. ITU-R BT.709-6 [bt709] or 9 to refer to Rec. ITU-R BT.2020-2 and Rec. ITU-R BT.2100-2. | BT.709 or BT.2020/BT.2100 |  |
| Transfer Characteristics | Either indicates the reference opto-electronic transfer characteristic function of the source picture as a function of a source input linear optical intensity input or indicates the inverse of the reference electro-optical transfer characteristic function as a function of an output linear optical intensity as defined in clause 8.2 of Rec. ITU-T H.273. Typical values are 1 to refer to Rec. ITU-R BT.709-6, 14 to refer to Rec. ITU-R BT.2020-2 (10 bit), 16 to refer to the Rec. ITU-R BT.2100-2 perceptual quantization (PQ) system, or 18 to refer to the Rec. ITU-R BT.2100-2 hybrid log-gamma (HLG) system | BT.709, BT.2020 SDR, BT.2100 PQ, or BT.2100 HLG |  |
| Matrix Coefficients | Describes the matrix coefficients used in deriving the luma and chroma signals from the green, blue, and red primaries. A video full range flag may be supplied with this parameter specifying the scaling and offset values applied in association with the Matrix coefficients. For detailed definition refer to clause 8.2 of Rec. ITU-T H.273. Typical values are 1 to refer to the non constant luminance YCbCr representation in Rec. ITU-R BT.709-6 or 9 to refer to the non constant luminance YCbCr representations in Rec. ITU-R BT.2020-2 and Rec. ITU-R BT.2100-2. | YCbCr BT.709, YCbCr BT.2020, or YCbCr BT.2100 |  |
| Frame rate | Typical values, using frames per second, are: 120, 120/1.001, 100, 60, 60/1.001, 50, 30, 30/1.001, 25, 24, 24/1.001 | No restrictions | services may only permit a restricted subset |
| Frame packing | Indicates a frame packing arrangement, if present, as defined in clause 8.4 of Rec. ITU-T H.273. | No, SbS, TaB | Some applications may use frame packing. |
| Projection | Indicates a projection, if present, as defined in Rec. ITU-T H.274, clause 7.3, and typically refers to packing arrangements in clause 8.6 of Rec. ITU-T H.274. | No projection. | Some applications may use projections. |
| Sample aspect ratio | Indicates width-to-height aspect ratio of the luma samples of the associated pictures as defined in clause 7.3 of Rec. ITU-T H.273. Typical value is 1 | No specific restrictions, but 1 is expected. |  |
| Chroma sample location type | Specifies the location of the chroma samples relative to the luma samples for frames as defined in Rec. ITU-T H.273, clause 8.7. Typical values are 0 (chroma samples are horizontally co-sited with and vertically centered between the first luma sample at the top-left corner and the first two luma samples at the top-left corner, respectively) or 2 (chroma samples are co-sited with the luma sample at the top-left corner). Note that a value of 1 is common for still images. | No specific restrictions, but 0 is expected if not present. For HDR the value is typically set to 2. |  |
| Range | Specifies how luma and chroma samples are represented in digital video as defined in Rec. ITU-T H.273, clause 8.3 using the parameter VideoFullRangeFlag. For video applications only the value set to 0 is used, i.e. the video range or restricted range is applied where the luma values range from 16 to 235 in an 8-bit system, and chroma values range from 16 to 240. For 10-bit systems, the values are multiplied by 4. Note that for still images full range (value set to 1) is commonly used. | No specific restrictions, but 0 is expected if not present. |  |

Certain video experiences are concurrently displaying video signals composed of multiple components. In this case, the video representation format describes each video signal component individually with the parameters defined in Table 4.4.2-1. Additionally, the components of the same video signal are typically jointly described and constrained for properly rendering the video representation.

The video signals made of multiple components can be delivered in either of the following forms:

- As a single encoded video signal using frame packing as defined in Table 4.4.2-1.

- As multiple encoded video signals

Table 4.4.2-2 lists the multi-component video signal parameters.

Table 4.4.2-2	Multi-component Video Signal Parameters

| Parameter | Definition | 3GPP restrictions | Service or Application restrictions |
| --- | --- | --- | --- |
| Stereoscopic Video | Visual media may be stereoscopic, in which case the video signal is composed of two signal components: a view is available to be presented to the left eye and another view is available to be presented simultaneously to the right eye. The presentation of both the left and right views allows for an effect known as stereopsis, which can be defined as "the perception of depth produced by the reception in the brain of visual stimuli from both eyes in combination; binocular vision." For signal representations, [3dtv] recommends that the Left and Right eyes comply to regular image formats such as Rec. ITU-R BT.709 and any necessary 3D-specific metadata is incorporated with the data. Hence, for stereoscopic video, two synchronized video signals are available, each with identical format parameters (such as the ones defined in this table). NOTE: When distributing the signal, some systems may use different resolutions for one of the views. Additional metadata that may be added with stereoscopic video: - "Hero eye" is the default eye in a stereo (stereoscopic) video pair, often determined by tags set by the cameras used to capture the video. If so signaled, this indicates that the other stereo eye view is derived from the specified stereo eye and may be useful when choosing which eye to use in a monoscopic viewing environment. There is no requirement that either of the two eyes (or views) is tagged as the hero eye, in which case no hero eye tagging may be present. |  |  |

### 4.4.3	3GPP Video Representation Formats

#### 4.4.3.1	Introduction

While a variety of formats may be used based on the video signal parameters defined in clause 4.4.2, for consistent programs and signals, several 3GPP video representation formats are defined by a set of restrictions using the video signal parameters in clause 4.4.2. These signals are typically used to develop interoperability points for TV and movie content distribution, but also have application for user-generated content.

The present clause describes the signal characteristics of the following 3GPP video formats:

-	3GPP High Definition (HD): is meant to address the distribution of conventional 2D video services including HDTV and other conventional 2D formats.

-	3GPP High Dynamic Range (HDR): enables the distribution of 2D video up to 4K, e.g., for Ultra HD TV, and adds the support of high dynamic range capability on top of the 3GPP HD format.

-	3GPP Stereoscopic: is a format suitable for the video consumption of devices creating a depth perception using 2 images, one for each eye.

#### 4.4.3.2	High-Definition

The 3GPP High-Definition (HD) format is defined based on Rec. ITU-R BT-709-6 [bt709]. 3GPP HD formats shall conform to Rec. ITU-R BT-709-6 [bt709] with the following restrictions and extensions:

-	Only the following formats are included 24/P, 25/P, 30/P, 50/P and 60/P. Interlace and progressive segmented frame signals are excluded.

-	Only the Non-Constant Luminance YCbCr signal format is included.

-	Other aspect ratios than 16:9 may be considered to address different screen sizes and orientations.

The definition of the 3GPP HD format based on the parameters defined in Table 4.4.2-1 is provided in Table 4.4.3.2-1.

Table 4.4.3.2-1	Video Signal Parameters for 3GPP HD format

| Parameter | Settings |
| --- | --- |
| Picture aspect ratio | 16:9 should be used as it is the only format defined in ITU-R BT-709-6 [bt709]. However, to support different applications with different screen sizes and orientations, other picture aspect ratios may be used including 9:16 and 1:1. NOTE 1: The display orientation of the pictures in the video signal, for example portrait or landscape mode is implicit to the picture aspect ratio, but may be explicitly signalled. NOTE 2: The aspect ratio of the encoded pictures may be different from the picture aspect ratio of the video signal. |
| Spatial Resolution width x height | 1920 × 1080 should be used as it is the only format defined in ITU-R BT-709-6 [bt709]. However, to support different applications, other spatial resolutions may be used to address different aspect ratios, for example 1080 x 1920, 1024 x 1024, 1440 x 1440. NOTE 1: Down-sampled resolutions may be created for distribution, for example in case of adaptive streaming. NOTE 2: To accommodate the block coding structure of a given specification, quite often the encoded signal may be padded. In such cases, normative cropping is typically applied to remove spatial samples that are not intended to be presented. NOTE 3: The width and the height of the encoded pictures may be different from the width and the height of the pictures in the video signal. |
| Scan Type | The source scan type of the pictures as defined in clause 7.3 of Rec. ITU-T H.273 shall be progressive. |
| Chroma format indicator | The chroma format indicator shall be 4:2:0. |
| Bit depth | The values shall be either 8 or 10 bit. The bit depth shall be the same for all samples. |
| Colour primaries | Only the value 1, as defined in clause 8.2 of Rec. ITU-T H.273, is permitted. |
| Transfer Characteristics | Only the value 1, as defined in clause 8.2 of Rec. ITU-T H.273 is permitted. |
| Matrix Coefficients | Only the value 1, as defined in clause 8.2 of Rec. ITU-T H.273, is permitted. |
| Frame rates | The permitted values are 60, 60/1.001, 50, 30, 30/1.001, 25, 24, 24/1.001 fps. |
| Frame packing | No frame packing shall be applied. |
| Projection | No projection shall be used. |
| Sample aspect ratio | The pixel aspect ratio shall be 1 (square pixel), i.e. only the value 1 as defined in clause 7.3 of Rec. ITU-T H.273 is permitted. |
| Chroma sample location type | The location of the chroma samples relative to the luma samples for progressive frames as defined in Rec. ITU-T H.273, clause 8.7, shall be set to 0 (chroma samples are horizontally co-sited with and vertically centered between the first luma sample at the top-left corner and the first two luma samples at the top-left corner, respectively). |
| Range | The restricted video range shall be used. |

#### 4.4.3.3	High Dynamic Range

The 3GPP High Dynamic Range (HDR) format is defined based on Rec. ITU-R BT-2100-2 [bt2100]. 3GPP HDR TV formats shall conform to ITU-R BT-2100-2 [bt2100] with the following restrictions and extensions:

-	Only 4:2:0 colour subsampling is  used

-	Only the Non-Constant Luminance YCbCr signal format is used

-	Only 10-bit representations are used

-	Other aspect ratios than 16:9 may be used in order to address different screen sizes and orientations.

The definition of the 3GPP HDR format based on the parameters defined in Table 4.4.2-1 is provided in Table 4.4.3.3-1.

Table 4.4.3.3-1	Video Signal Parameters for 3GPP HDR format

| Parameter | Settings |
| --- | --- |
| Picture aspect ratio | 16:9 should be used as it is the only format defined in ITU-R BT-2100-2 [bt2100]. However, to support different applications with different screen sizes and orientations, other picture aspect ratios may be used including 9:16 and 1:1. NOTE 1: The display orientation of the pictures in the video signal, for example portrait or landscape mode is implicit to the picture aspect ratio, but may be explicitly signalled. NOTE 2: The aspect ratio of the encoded pictures may be different from the picture aspect ratio of the video signal. |
| Spatial Resolution width x height | 7680 × 4320, 3840 × 2160, 1920 × 1080 are the only formats supported in ITU-R BT-2100-2 [bt2100] and should therefore be used. Other spatial resolutions may be used to address different aspect ratios, for example 1080 x 1920, 1024 x 1024, 1440 x 1440. NOTE 1: Down-sampled resolutions may be created for distribution, for example in case of adaptive streaming. NOTE 2: To accommodate the block coding structure of a given specification, quite often the encoded signal may be padded. In such cases, normative cropping is typically applied to remove spatial samples that are not intended to be presented. NOTE 3: The width and the height of the encoded pictures may be different from the with and the height of the pictures in the video signal. |
| Scan Type | the source scan type of the pictures as defined in clause 7.3 of Rec. ITU-T H.273 is progressive |
| Chroma format indicator | The chroma format indicator shall be 4:2:0. |
| Bit depth | The permitted value shall be 10 bit. |
| Colour primaries | Only the value 9 as defined in clause 8.2 of Rec. ITU-T H.273 is permitted. |
| Transfer Characteristics | Only the values 14 (for SDR with WCG), 16 (for PQ) and 18 (for HLG) as defined in clause 8.2 of Rec. ITU-T H.273 are permitted. |
| Matrix Coefficients | Only the value 9 as defined in clause 8.2 of Rec. ITU-T H.273 is permitted. |
| Frame rates | The permitted values are 120, 120/1.001,100, 60, 60/1.001, 50, 30, 30/1.001, 25, 24, 24/1.001 fps. |
| Frame packing | No frame packing shall be applied. |
| Projection | No projection shall be used. |
| Sample aspect ratio | The pixel aspect ratio is 1 (square pixel), i.e. only the value 1 as defined in clause 7.3 of Rec. ITU-T H.273 is permitted. |
| Chroma sample location type | the location of chroma samples relative to the luma samples for progressive frames as defined in Rec. ITU-T H.273, clause 8.7 shall be set to 2 (chroma samples are co-sited with the luma samples at the top-left corner). |
| Range | The restricted video range shall be used. |

#### 4.4.3.4	Stereoscopic format

The 3GPP Stereoscopic format uses a two-component video signal, one component for the left eye and another component for the right eye as defined in Table 4.4.3-1. The components for each eye follow the specifications of the 3GPP HDR format, but there are some restrictions and extensions, namely:

-	Only 4:2:0 colour subsampling is used.

-	Frame rates include high frame rate for movies, namely 48 fps.

-	the spatial resolution for each component is restricted to a maximum value of 4K (3840 × 2160).

-	Only the Non-Constant Luminance YCbCr signal format is used.

-	Square picture aspect ratios are supported for different screen sizes.

The definition of the 3GPP Stereoscopic format based on the parameters defined in Table 4.4.2-1 is provided in Table 4.4.3.4-1.

Table 4.4.3.4-1	Video Signal Parameters for 3GPP Stereoscopic format

| Parameter |  |  | Settings |  |  |
| --- | --- | --- | --- | --- | --- |
| Stereoscopic Video |  |  | A video signal for the Left and for the Right Eye shall be provided whereby the signals shall have identical parameters as above and the frames are timely synchronized. The signal may be provided as two individual signals for each eye, or in a frame-packed version. |  |  |
|  | Picture aspect ratio |  |  | Shall be set to 16:9, 1:1. |  |  |
|  | Spatial Resolution width x height |  |  | Should be set to 3840 × 2160, 1920 × 1080, 2048 × 2048, 1024 × 1024. However, other resolutions are permitted. NOTE 1: Down-sampled resolutions may be created for distribution, for example in case of adaptive streaming. NOTE 2: To accommodate the block coding structure of a given specification, quite often the encoded signal may be padded. In such cases, normative cropping is typically applied to remove spatial samples that are not intended to be presented. |  |  |
|  | Scan Type |  |  | The source scan type of the pictures as defined in clause 7.3 of Rec. ITU-T H.273 shall be progressive |  |  |
|  | Chroma format indicator |  |  | The chroma format indicator shall 4:2:0. |  |  |
|  | Bit depth |  |  | The permitted values are 8 or 10 bit. 8 bit is only permitted for SDR. |  |  |
|  | Colour primaries Transfer Characteristics Matrix Coefficients |  |  | Only the following value combinations are permitted: (1, 1, 1), (9, 14, 9), (9, 16, 9), and (9, 18, 9) for SDR HD, SDR UHD, HDR PQ, and HDR HLG, respectively. |  |  |
|  | Frame rates |  |  | The permitted values are 60, 60/1.001, 48, 48/1.001, 50, 30, 30/1.001, 25, 24, 24/1.001 fps. |  |  |
|  | Frame packing |  |  | The permitted values are no frame packing, side-by-side, top-and-bottom. |  |  |
|  | Projection |  |  | No projection is used. |  |  |
|  | Sample aspect ratio |  |  | The pixel aspect ratio shall be 1 (square pixel), i.e. only the value 1 as defined in clause 7.3 of Rec. ITU-T H.273 is permitted. |  |  |
|  | Chroma sample location type |  |  | For SDR HD, the location of chroma samples relative to the luma samples for progressive frames as defined in Rec. ITU-T H.273, clause 8.7 shall be set to 0. For SDR UHD, HDR PQ, and HDR HLG, the location of chroma samples relative to the luma samples for progressive frames as defined in Rec. ITU-T H.273, clause 8.7, shall be set to 2. |  |  |
|  | Range |  |  | The restricted video range shall be used. |  |  |

## 4.5	Common Bitstream Constraints

### 4.5.1	General

This clause defines common definitions for bitstreams that are used in capability definitions in the remainder of this document.

### 4.5.2	AVC Bitstreams

For an AVC/ITU-T H.264 [h264] bitstream, motion-vector constraints are defined that the bitstream does neither include horizontal motion vector component values that exceed the range from −2048 to 2047, inclusive, nor does have vertical motion vector component values that exceed the range from −512 to 511, inclusive, in units of ¼ luma sample displacement.

NOTE: 	This constraint should be indicated by using values of log2_max_mv_length_horizontal less than or equal to 11 and values of log2_max_mv_length_vertical less than or equal to 9.

For an AVC/ITU-T H.264 [h264] bitstream, rate constraints are defined that the for the bitstream,

-	the maximum VCL Bit Rate is constrained to be 120 Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively; and

-	the bitstream does not contain more than 16 slices per picture.

### 4.5.3	HEVC Bitstreams

The following definitions are provided for HEVC/ITU-T H.265 [h265] bitstreams.

For an HEVC/ITU-T H.265 [h265] bitstream, progressive constraints are defined that the following flags in the active Sequence Parameter Set (SPS):

-	general_progressive_source_flag shall be set to 1,

-	general interlaced_source_flag shall be set to 0,

-	general_non_packed_constraint_flag shall be set to 1, and

-	general_frame_only_constraint_flag shall be set to 1.

For an HEVC/ITU-T H.265 [h265] bitstream, VUI constraints are defined:

-	Video Parameter Sets (VPS) NAL units as defined in Recommendation ITU-T H.265 / ISO/IEC 23008-2 [h265] may be present, but the Bitstream shall be valid if the Receiver ignores the VPS.

-	The Video Usability Information (VUI) is present in the active Sequence Parameter Set, i.e. the vui_parameters_present_flag shall be set to 1.

-	In the VUI,

-	the aspect ratio information is present, i.e. the aspect_ratio_info_present_flag value shall be set to 1,

-	the colour parameter information is present, i.e.  video_signal_type_present_flag value shall be set to 1 and the colour_description_present_flag value shall be set to 1.

-	only video range signals are used, i.e. the video_full_range_flag shall be set to 0,

-	no overscan signalling is present, i.e. the overscan_info_present_flag shall be set to 0,

-	the chroma location shall be signalled, i.e. chroma_loc_info_present_flag shall be set to 1,

-	the timing information may be present. If the timing information is present, i.e. the value of vui_timing_info_present_flag is set to 1, then the values of vui_num_units_in_tick and vui_time_scale shall be set according to the frame rates allowed for each operation point. The timing information present in the video Bitstream should be consistent with the timing information signalled at the system level. The frame rate shall not change between two RAPs. fixed_frame_rate_flag value, if present, shall be set to 1.

[For an HEVC/ITU-T H.265 [h265] bitstream, frame-packing constraints are defined:

-	the following flags in the active Sequence Parameter Set (SPS):

-	general_progressive_source_flag shall be set to 1,

-	general interlaced_source_flag shall be set to 0,

-	general_non_packed_constraint_flag shall be set to 0, and

-	general_frame_only_constraint_flag shall be set to 1.

-	The frame packing arrangement SEI message shall be present with the following characteristics:

-	The value of frame_packing_arrangement_type shall be set to either the value of 3 for the side-by-side packing arrangement, or the value of 4 for the top-bottom/over-under packing arrangement.

-	The value of quincunx_sampling_flag shall be set to 0.

-	The value of content_interpretation_type shall be set to either 1 or 2.

-	The value of spatial_flipping_flag shall be set to 0.

-	The value of frame0_flipped_flag shall be set to 0.

-	The value of field_views_flag shall be set to 0.

-	The value of current_frame_is_frame0_flag shall be set to 0.

-	The values of frame0_grid_position_x, frame0_grid_position_y, frame1_grid_position_x, and frame1_grid_position_y, shall remain the same throughout the bitstream.

-	The value of upsampled_aspect_ratio_flag shall be set to 0, indicating the presence of full resolution frame packed video and the aspect_ratio_idc shall be set to 1.

- 	All parameters shall remain the same for the entire bitstream.

## 4.6	Reference API parameters

### 4.6.1	Introduction

When media is played back, the decoder and the playback pipeline need to be initialized. For this purpose, certain parameters are required. In CTA-5003 [DPC], a media playback model is described that is aligned with HTML 5.1 and the <video> element, as well as the Media Source Extensions.

### 4.6.2	Video Decoder API Parameters

Video decoders are typically accessed by API parameters. The parameters are used for the following purposes:

-	to identify the capability of the device in order to check whether the signal can be played back

-	to initialize the decoding and playback platform to allocate the resources for decoding and rendering

Table 4.6.2-1 provide relevant parameters for Video Decoder APIs.

Table 4.6.2-1 Video Decoder API Parameters

| Parameter | Restrictions | Status |
| --- | --- | --- |
| media type | Specifies the media type of the component, in this case video. | required |
| codecs | Specifies through a well-defined string the codec parameters which the encoded video signal is compliant to. | required |
| video format parameters | Specifies additional video format parameters as defined in Table 4.4.2.1 to describe the signal and to initialize the encoder. | optional |

Editor’s Note: The capability of such API for decoding and playback of multilayer content, e.g. for stereoscopic content needs to be documented.

### 4.6.3	Video Encoder API Parameters

Video encoder API parameters are for further study.

### 4.6.4	Player API Parameters

Media players are typically configurable via API parameter. The main purpose of the API are:

-	For video components, to create one or more display windows to display the decoded video signal

-	To bind a media source, possibly remote, to the one or more created display windows.

Table 4.6.2-2 Display Window Object Parameters

| Parameter | Restrictions | Status |
| --- | --- | --- |
| width | Specifies the width of a video player window, in pixels | required |
| height | Specifies the width of a video player window, in pixels. | required |
| video format parameters | Specifies additional video format parameters as defined in Table 4.4.2-1 to describe the signal. | optional |

Editor’s Note: The relationship between the width and height in the above table and the spatial resolution of the video signal needs be to be clarified.

* * * End of Changes * * * *