3GPP TSG SA WG4 Meeting 132	S4-250798

# Fukuoka, JP, 19 - 23 May 2025

Source:	Qualcomm Sweden, Tencent

Title:	Pseudo-CR on [VOPS] AVC Operation Points

Spec:	3GPP TS26.265v1.1.0

Agenda item:	9.5

Document for:	Decision

1. Introduction

Discussion AVC

2. Reason for Change

Missing

3. Conclusions

Do it

4. Proposal

It is proposed to agree the following changes to 3GPP TS26.265v1.1.0 at SA4-132.

* * * First Change * * * *

## 4.5	Common Bitstream Constraints

### 4.5.1	General

This clause defines common definitions for bitstreams that are used in capability definitions in the remainder of this document.

### 4.5.2	AVC Bitstreams

The following definitions are provided for AVC/ITU-T H.264 [h264] bitstreams.

For an AVC/ITU-T H.264 [h265] bitstream, motion-vector constraints are defined that the bitstream does neither include horizontal motion vector component values that exceed the range from −2048 to 2047, inclusive, nor does have vertical motion vector component values that exceed the range from −512 to 511, inclusive, in units of ¼ luma sample displacement.

NOTE: 	This constraint should be indicated by using values of log2_max_mv_length_horizontal less than or equal to 11 and values of log2_max_mv_length_vertical less than or equal to 9.

For an AVC/ITU-T H.264 [h265] bitstream, rate constraints are defined that the for the bitstream,

-	the maximum VCL Bit Rate is constrained to be 120 Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively; and

-	the bitstream does not contain more than 16 slices per picture.

* * * Next Change * * * *

### 5.3.1	AVC Decoding Capabilities

### 5.3.1	AVC Decoding Capabilities

The following decoding capabilities are defined:

-	AVC-FullHD-Dec: the capability to decode AVC/ITU-T H.264 Progressive High Profile Level 4.0 [h264] bitstreams.

-	AVC-UHD-Dec: the capability to decode AVC/ITU-T H.264 Progressive High Profile Level 5.1 [h264] bitstreams with rate constraints as defined in clause 4.5.2.

-	AVC-8K-Dec: the capability to decode AVC/ITU-T H.264 Progressive High Profile Level 6.1 [h264] bitstreams with motion-vector constraints and rate constraints as defined in clause 4.5.2.

* * * Next Change * * * *

### 6.2.2	3GPP AVC HD Operation Point

#### 6.2.2.1	Introduction

The AVC HD Operation Point permits consistent distribution of HD-based video using AVC. The remainder of this clause 6.2.2 defines the Bitstream and Receiver requirements for the 3GPP-AVC-HD receiver.

### 6.2.2	3GPP AVC HD Operation Point

#### 6.2.2.1	Introduction

The AVC HD Operation Point permits consistent distribution of HD-based video using AVC. The remainder of this clause 6.3.2 defines the Bitstream and Receiver requirements for the 3GPP-AVC-HD receiver.

#### 6.2.2.2	Bitstream Requirements

A 3GPP-AVC-HD Bitstream shall conform to the following requirements

-	the Bitstream shall conform to AVC/ITU-T H.264 High Progressive Profile, Level 4.0 [h264] bitstreams with rate constraints as defined in clause 4.5.2.

-	the Representation Format included in the Bitstream shall conform to the 3GPP-HD Representation format as defined in clause 4.4.3.2.

-	the Bitstream shall be decodable by a decoder with AVC-FullHD-Dec decoding capabilities.

#### 6.2.2.3	Receiver Requirements

Receivers conforming to the Operation Point 3GPP-AVC-HD shall support decoding and rendering Bitstreams with the restrictions defined in clause 6.2.2.2.

NOTE 1:	Rendering includes adherence to the parameters signalled in the bitstream to characterize the distributed Representation format.

* * * End of Changes * * * *