3GPP TSG-SA WG4 Meeting #132	S4-250880

# Fukuoka, JP, 19 – 23 May 2025

Source:	China Mobile Com. Corporation

Title:	[FS_Beyond2D] Scenario 1: Detailed Test Conditions

Agenda item:	9.7

Document for:	Agreement

1. Introduction

This proposal provides updates on encoding and decoding constraints, and detailed test condition for Scenario 1: UE-to-UE Stereoscopic Video Live Streaming.

2. Proposal

It is proposed to agree the following changes to the 3GPP draft TR 26.956 V0.4.0

* * * First Change * * *

[LS-13]	HTM Codec Software, version 16.3, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/branches/HTM-16.3-fixes/cfg/MV-HEVC/

* * * Second Change * * *

7.2.4	Encoding and Decoding Constraints

Table 7.2.4-1 provides an overview of encoding and decoding constraints for UE-to-UE Stereoscopic Video Live Streaming scenario using H.265/HEVC and MV-HEVC. This information supports the definition of detailed anchor conditions.

Table 7.2.4-1 Encoding and Decoding Constraints

| Encoding and Decoding Constraints | H.265/HEVC | MV-HEVC |
| --- | --- | --- |
| Relevant Codec and Codec Profile/Levels | H.265/HEVC Main 10 Profile Level 4.1, 5.1 | Multiview Main or Multiview Main10 profile Level 4, 5.1 and higher |
| Random access frequency | 1 second | 1 second |
| Bit rates and quality configuration | Fixed QP: [17, 22, 27, 32, 37] CBR Half Width/Height: 5-8Mbps Full Width/Height: 8-16Mbps Capped-VBR | Fixed QP: [17, 22, 27, 32, 37] |
| Bit rate parameters (CBR, VBR, CAE, HRD parameters) | Covering a range of relevant bitrates and qualities | Covering a range of relevant bitrates and qualities |
| Latency requirements and specific encoding settings | Low latency requirements | Low latency requirements |
| Encoding complexity context | Real-time encoding, Cloud-based encoding | Real-time encoding, Cloud-based encoding |
| Required decoding capabilities | H.265/HEVC Main 10 Profile Level 4.1, 5.1 | Multiview Main or Multiview Main10 profile Level 4, 5.1 and higher |

* * * Third Change (All New) * * *

7.2.8	Test Condition

7.2.8.1	Test model and configuration files

The encoder configuration settings for both encodings are are outlined below:

-	Inter-view coding structure

-	2 view case: left-right (in coding order)

-	I-P inter-view prediction MV-HEVC

-	Temporal prediction structure: GOP 8, intra every 24 frames (random access at ~1sec)

-	Full resolution texture coding

-	Codec software: HTM v16.3 for Simulcast HEVC and MV-HEVC

The following configuration files are provided in [LS-13]:

-	/HTM-16.3-fixed/cfg/MV-HEVC/baseCfg_2view.cfg: Used to configure input/output filenames and encoder parameters (I-frame interval, number of B-frames, etc.)

-	/HTM-16.3-fixed/cfg/MV-HEVC/qpCfg_QP25.cfg: : Used to configure the encoding QP

-	/HTM-16.3-fixed/cfg/MV-HEVC/seqCfg_Shark.cfg: Contains the source sequence parameters (resolution, frame count, frame rate, etc.)

For each selected test sequence,  configuration files containing information needed for HTM-16.3 configuration will be provided.

7.3.9.2	Rate points and test conditions

Fixed QP was used to evaluate and compare the performance of MV-HEVC and Simulcast HEVC is compared and evaluated in terms of PSNR (dB) and bitrate (kbps) over the set of QP values [17, 22, 27, 32, 37].

7.3.9.3	Profiles

MV-HEVC Main Profile is used.

7.3.9.4	Bitstream Generation

The HTM v16.3 is used to encode and decode test sequences as described in clause 7.2.8.1.

Below are examples of command lines for encoding and decoding test sequence:

-	Before compilation, navigate to source/Lib/TLibCommon/TypeDef.h and modify the following parameter to configure the software as an MV-HEVC encoder:

/** \file     TypeDef.h

\brief    Define macros, basic types, new types and enumerations

*/

(…)

/* HEVC_EXT might be defined by compiler/makefile options.

Linux makefiles support the following settings:

make             -> HEVC_EXT not defined

make HEVC_EXT=0  -> NH_MV=0 H_3D=0   --> plain HM

make HEVC_EXT=1  -> NH_MV=1 H_3D=0   --> MV only

make HEVC_EXT=2  -> NH_MV=1 H_3D=1   --> full 3D

*/

#ifndef HEVC_EXT

#define HEVC_EXT                    1

#endif

(…)

-	to encode a test sequence:

./TAppEncoder.exe -c baseCfg_2view.cfg -c seqCfg_Shark.cfg -c qpCfg_QP25.cfg

-	to decode a test sequence:

./TAppDecoder.exe -b stream.bit -o shark_qp25.yuv -w 1

* * * End of Changes * * *