3GPP TSG-SA WG4 Meeting #132	S4-250813

Fukuoka, Japan, 19 – 23 May 2025

Source:	Orange, Interdigital

Title:	Discussion paper on potential planning and objectives for FS_MediaEnergyGREEN phase 2

Agenda item:	17.1

Document for:	Discussion

# 1.	Introduction

During SA4#131 meeting in Geneva, TR 26.942 (Study on Media Energy Consumption Exposure and Evaluation Framework) has been completed with the following recommendations:

1.	Wait for the full definition of the Energy Information Function (EIF) and the completion of TR 22.883.

2.		Then, update the present document with:

a. More details on the interaction with the Energy Information Function (EIF).

b. New Key Issues addressing questions not yet addressed.

c. New Key Issues addressing new relevant use cases and requirements defined in TR 22.883.

3.		Then, initiate normative work including:

a.	Normative work already proposed for addressing Key Issue #1 (Energy-related information exposure) in clause 8.2.

b.	   Potential other work identified in the second phase of the study.

The goal of this document is to provide updates on the full definition of the Energy Information Function (EIF) and the completion of TR 22.883, progress on potential objectives of the phase 2 of the study and discuss on the potential planning of this phase 2.

# 2.	Updates on related work in other 3GPP WGs

## 2.1 	SA1#109

Progress:

Study on Energy Efficiency as Service Criteria Phase 2 (FS_EnergyServ_Ph2) has been completed during SA1#109 (Feb. 25). TR 22.883 (Feasibility Study on Energy Efficiency as service criteria Phase 2) v2.0.0 has been published, providing stage 1 use cases and potential requirements on the following aspects regarding enhancements on energy as service criteria:

- Information exposure of energy-related characteristics of the network for the communication service (i.e. energy consumption, energy supply mix, carbon footprint, energy capacity and availability conditions) to authorized users or authorized 3rd parties.

- Potential dynamic adjustments of the delivered communication service from 5G system perspective (including service performance adjustments) resulting from the changes of energy-related characteristics of this service. Dynamic adjustments can be based on criteria such as network decision, user preference or agreement between authorized 3rd parties and network.

- Other aspects including security, charging and privacy for the scenarios above.

New WID on Stage 1 for Energy Efficiency as Service Criteria (EnergyServ_Ph2-REQ) and CR to TS 22.261 (Service requirements for the 5G system) to incorporate conclusions and recommendations of TR 22.883 have been agreed. The WI is 90% completed, full completion is planned in 06/2025.

Impacts on TR 26.942:

One use case in TR 22.883 on notifying UEs about network energy-related characteristics and several requirements in TS 22.261 have not been taken into consideration in TR 26.942 v2.0.0.

Here is the list of new requirements:

- In sub-clause 6.15a.2, normative requirements corresponding to the consolidated potential requirements of TR22.883 in clauses 6.1.2/3/4 are added:

  - Subject to operator’s policy, regulatory requirements and user consent, the 5G network shall enable the operator to provide means to degrade service performance (e.g. QoS, bitrate) to meet energy rationing constraints.

  - Subject to operator’s policy, regulatory requirements and user consent, the 5G network shall support subscription policies that include alternative (i.e. degraded) service performance (e.g. QoS parameters, maximum bitrate) of services with QoS criteria for energy saving reasons.

  - Subject to operator’s policy, the 5G network shall be able to support a means to target per UE energy saving actions, based on subscription policies.

  - Subject to operators’ policy, regulatory requirements, 5G network shall provide mechanisms to adjust communication service (e.g. user plane path, suitable Service Hosting Environment, defer background traffic delivery) considering the change of energy supply mix of the network as one of the factors.

  - Subject to operator’s policy, regulatory requirements and user consent, the 5G network shall enable to control the access of UEs to the network (e.g. block traffic, disable specific application etc) based on the energy-related characteristics of the network.

  - Subject to operator’s policy and regulatory requirements, the 5G network shall be able to trigger charging events corresponding to an impacted UE when degrading performance of services with QoS criteria (e.g. to a lower bitrate) in order to achieve energy saving.

  - Subject to user consent, operator policy and regulatory requirements, the 5G network shall be able to assist an authorized 3rd party to identify a set of target UEs for whom to adjust the provided application service, considering criteria such as the current and future (e.g. predicted) energy-related characteristics of their serving network.

- In sub-clause 6.15a.5, normative requirements corresponding to the consolidated potential requirements of TR22.883 in clauses 6.1.1 are added:

  - Subject to operator’s policy, regulatory requirements and user consent, the 5G network shall be able to collect, and expose to authorized 3rd parties, the carbon equivalent emissions resulting from the use of the communication service, related to one or more specific UEs of home network subscribers (e.g. fleet of vehicles, IoT devices, company phones etc), over a specific time period (e.g. month etc).

  - Subject to operator’s policy and user consent, the 5G network shall be able to expose to an authorized 3rd party information about network energy consumption and carbon equivalent emissions for a specific service data flow.

  - Subject to operator’s policy, regulatory requirements and user consent, the 5G system shall be able to expose to an authorized 3rd party the current and expected energy-related characteristics of the network resources used in serving a UE.

  - Subject to operator’s policy, regulatory requirements and user consent, the 5G system shall be able to provide UEs with information related to the current energy-related characteristics of their serving network.

In TR 26.942, some potential solutions already took into account the use case on notifying UEs about network energy-related characteristics, including solution #5 in clause 7.6 for which normative work was already recommended. But this solution only addresses energy-related information exposure without any means for service adjustment by the operator or authorized 3rd parties. Those new requirements also provide some details on network energy-related information to be shared like network energy consumption and carbon equivalent emissions for a specific service data flow, a UE or specific UEs.

## 2.2 	SA2#167

Progress:

During SA2#167 (Feb. 25), Energy Efficiency and Energy Saving (EnergySys) WID has been revised to remove the objective to specify the 5GS enhancements to support network energy saving and efficiency including:

- Enhancements of NF discovery and (re-)selection based on energy related information.

- New energy related information and/or existing NF profile parameters in the NF Profile to allow an operator to influence NF discovery and selection based on their energy strategy.

- Enhancement on NF discovery and (re-)selection to consider the energy related information from the NF profiles and/or discovery request from the NF consumer.

- Enhancements on existing operations and procedures for energy saving and energy efficiency: The optional UP path adjustment for a PDU session(s) will be specified.

Many new CRs have been agreed on TS 23.501 (System architecture for the 5G System), TS 23.502 (Procedures for the 5G System) and TS 23.503 (Policy and charging control framework for the 5G System) for support of Energy Efficiency and Energy Saving. The Work Item EnergySys is now completed.

A new SID on Energy Efficiency and Energy Saving Phase2 (FS_EnergySys_Ph2 - SP-250417) has been submitted but noted during SA2 meeting. The latest revision has been submitted to SA Plenary as company contribution and approved. The plan is to start the study at the SA2#168 (04/25) and complete it in 12/25. The Technical Report of the study will be TR 23.700-67. The goal is to address the new requirements from the SA1 EnergyServ Phase 2 and continue the enhancements from the previous release in the system perspective for further improvement on the network energy efficiency and energy saving. Three Working Tasks have been identified:

- WT#1: Whether and how to enhance the collection, determination, and exposure of energy related information in the 5GS.

- WT#2: Whether and how to support service adjustments for the UE to enable energy saving. This may include:

  - Whether and how to support taking the energy attributes (e.g., renewable energy information) into account to determine UE traffic handling (e.g., QoS adjustment)

  - Whether and how to support policy control based on interaction with EIF

  - Identify charging impacts caused by service adjustment (in cooperation with SA5)

- WT#3: Whether and how to support NF (re)selection and/or UP path adjustment based on energy related information.

Impacts on TR 26.942:

With EnergySys WI, SA2 has already defined the basic elements for supporting the collection and calculation of energy saving and efficiency and the exposure of network energy related information based on the new Energy Information Function (EIF). SA2 also specified the enhancements for subscription to enable network energy savings as service criteria.

The mapping with 5G architecture used for media has been already studied during phase 1 but as the work was not fully completed on SA2 side before the completion of the current version of the TR, an update of the relevant potential solutions and the description of the normative work associated is needed.

A follow up of SA2 study will also be needed to be sure to not address the same issue with different solutions.

## 2.3	SA5

Progress:

The Work Item on Energy efficiency and energy saving aspects of 5G networks and services (Energy_OAM_Ph3 – SP-241940) has progressed to 20%. The goal of this WI is to address use cases with potential solutions recommended to be pursued in normative work in TR 28.880 (Study on energy efficiency and energy saving aspects of 5G networks and services – Rel 19). 9 Work Tasks have been identified:

- WT-1 Energy consumption of a NF realized by VM or OS container based VNF/VNFC

  - WT-1.1: describe a new estimation method, alternative to the existing one, to estimate the Energy Consumption (EC) of VM-based VNF/VNFCs, and define corresponding KPI(s);

  - WT-1.2: extend WT-1.1 method to estimate the Energy Consumption (EC) of containerized VNF/VNFCs, and define corresponding KPI(s).

- WT-2 Energy supply, carbon emission and renewable energy information elements

  - WT-2.1: specify energy supply, carbon emission and renewable energy information relative to gNBs and 5G NFs.

- WT-3 Multi-dimensional network energy efficiency metrics

  - WT-3.1: Pursue investigations on multi-dimensional EE metrics as per the recommendation in clause 6.6 of TR 28.880.

- WT-4 Renewable energy-based LBO

  - WT-4.1: Describe the use case, requirements and procedure.

  - WT-4.2: Extend the NRM to support renewable energy-based LBO.

- WT-5 Renewable energy enabling 5GC NF re-selection

  - WT-5.1: Describe the use case, requirements and procedure.

  - WT-5.2: Extend the NRM to support renewable energy enabling 5GC NF re-selection.

- WT-6 Provisioning of network slices depending on the energy source of the operator site

  - WT-6.1: Describe the use case, requirements and procedure.

  - WT-6.2: Extend the NRM to support provisioning of network slices depending on the energy source of the operator site.

- WT-7 Handling of power shortages

  - WT-7.1: Specify information elements to enable the operator to associate power shortage, outage information and available energy information for the managed functions

  - WT-7.2: Enhance the MDA assisted Energy Saving use case for handling of power shortages

- WT-8 Cell proximity-based energy saving

  - WT-8.1: Describe the use case, requirements and procedure.

  - WT-8.2: Extend the NRM to support cell deactivation in scenarios where candidate cells have a many-many relationship with capacity booster cells.

- WT-9 Consideration of input from other 3GPP WGs

The goal is to update associated TS (28.310, 28.622, 28.623, 28.541, 28.554 and 28.104) by Sep 2025

Impacts on TR 26.942:

SA5 work is not directly used in TR 26.942. This is used via SA2 architecture, and details on network KPIs like Energy Consumption (EC) or Energy Efficiency (EE) KPIs have not been details in the phase 1 of the study. No modification is expected to take into consideration SA5 progress, but this could be taken into consideration if some new key issues are using network KPIs or network Energy Saving solutions.

## 2.4 	CT3/CT4

Progress:

During CT3#139 (Feb. 25), a new Rel-19 WID on CT aspects of energy efficiency and energy saving. The objective of this work is to specify the CT aspects of Energy Efficiency and Energy Saving in order to implement the stage 2 normative work. The following aspects of work are expected to be covered:

- CT3:

  - Define protocols aspects for the EIF (Energy Information Function) services.

  - Potential update on PCF to support the acquisition of the required energy related information for policy control.

  - Potential update on NEF to support energy information exposure.

  - Potential update on NFs (e.g. SMF) to support energy related information collection.

  - Potential extension of UDR application data to support energy indicator.

- CT4:

  - For network energy related information exposure

  - Updates on the Namf_Communication Service for the AMF to transfer the energy saving indication.

  - Updates to Nnrf_NFManagement Service and Nnrf_NFDiscovery Service to support selection and discovery of EIF.

  - Update to the Nudm_SDM services to support the energy saving subscription information.

Completion is planned in 09/25), including a new TS 29.xxx (5G System; Energy Information Function Services; Stage 3).

Impacts on TR 26.942:

No direct impact on TR 26.942 as the study is currently high-level solutions but work done in this Work Item, could be useful for the phase 2 of the study and following normative work.

# 3.	Potential new key issues

The current version of TR 26.942 includes 3 key issues:

- Energy-related Information exposure

- Energy-related monitoring and measurement

- Evaluation framework

Among those 3 key issues, normative work has only identified on the key issue about the energy-related information exposure. No consensus has been reached or more work is needed on the two others.

In addition of those 3 key issues, new ones have already been identified:

- Energy-related configuration by the Application Service Provider: How can the Application Service Provider configure energy-related information collection and exposure?

- 5GMS AS Energy management: How might the 5GMS AS (and equivalent functions of the RTC System) react to energy-related information shared by the network via the Energy Information AF instantiated in the 5GMS AF and/or the Energy Information Collector?

- 5GMS Client Energy management: How might the 5GMS Client (and equivalent functions of the RTC System) react to energy-related information shared by the network?

and other ones should be required to consider the new requirements in TS 22.261 and the new architecture updates from SA2.

Solutions to those new key issues will have to be studied and improvement of solutions proposed for existing key issues need to be explored.

# 4.	Proposal

According to the information above and the conclusions of the study on FS_MediaEnergyGREEN, it is proposed to work on a new SID for a phase 2 of this study. The following text is proposed as basis for further work for the objectives of this SID:

This study aims to take into consideration updates from other 3GPP working groups on efficient energy use and energy saving for media services impacting TR 26.942, in particular new requirements in TS 22.261 and architecture updates in TS 23.501. This phase 2 will also enhance the phase 1 addressing new key issues identified in phase 1 and develop the proposed solution of existing key issues.

The main objectives of this study include:

- Address, in the context of media delivery, the new Release 20 requirements in TS 22.261 on:

  - Energy-related information as a service criterion as required by clause 6.15a.2, including:

    - The degradation of media delivery performance (e.g. adjustment of QoS, bit rate; deferring Background Data Transfers) in reaction to a change in the energy supply mix of the network, or in order to ration energy use, subject to operator policy, regulatory requirements and user consent.

    - The means to target per-UE energy saving actions based on subscription policies, including blocking media delivery, subject to operator policy, regulatory requirements and user consent.

    - The means to assist a Media Application Provider in identifying UEs to target with the aforementioned performance degradation and energy saving actions.

  - Energy-related information exposure to UEs and authorised third parties over specific time periods as required by clause 6.15a.5, subject to operator policy, regulatory requirements and user consent, including energy consumption by the serving network – and the equivalent carbon (CO2e) emissions – for the overall use of the media delivery service as well as for individual media application data flows.

- Update or develop existing potential solutions and proposed normative work associated with the Key Issue on energy-related information exposure, with new elements added in 3GPP specifications such as TS 23.501, TS 23.502 or TS 23.503 including the Energy Information Function (EIF), efficient energy use and energy saving items.

- Propose new potential solutions to phase 1 Key Issues on energy-related monitoring and measurement, and evaluation frameworks for which no normative work has been identified.

- Describe new Key Issues identified during phase 1 about:

  - Energy-related configuration by the Application Service Provider.

  - AS energy management.

  - Client energy management.

and propose potential solutions and normative work related to them.

Normative work already identified in phase 1 of the study and listed in the conclusions of the TR 26.942 v.2.0.0 will be conducted after the conclusion of this phase 2. This includes:

- Creating a new stage-2 TS defining an energy architecture with the definition of two new entities, the Energy Information AF and the Energy Information Collector, using energy information available on Energy Information Function (EIF) allowing end-to-end energy-related information to be exposed to UE applications.

- Extending TS 26.501 and TS 26.506 to instantiate the above architecture in the media context.

- Addressing the relevant stage-3 aspects based on stage-2 work.

Phase 2 will allow the provision of more details and extension of this work.

It is proposed to host this study item in 3GPP SA4 MBS SWG, like the phase 1 of the study.

# 5.	Potential planning

The goal is to complete this 5GA study and normative work in release 20. It seems relevant to start phase 2 of the feasibility study after SA#108 (Jun 25). SA1 work on new use cases will be completed and this would allow to complete potential normative work stage 2 and stage 3 in release 20. Unfortunately, SA2 work will be conducted in parallel. But their plan is to complete their feasibility study phase 2 for SA#110 (Dec 25) and this will probably be complemented with normative work completed at SA#113 (Sep 26). Starting SA4 work once SA2 work is completed would not allow SA4 to complete normative work in release 20.