3GPP TSG-SA4 Meeting #132	S4- 250831

# Fukuoka JP, 19th - 23th May 2025

Source:	Apple Inc.

Title:	[VOPS] On Conformance bitstreams

Agenda item:	9.5

Document for:	Agreement

# 1 Overview

Recently, the scope of SA4 VOPS work has been extended to add work on conformance. The purpose of this document is to gather some of the previously provided source content, test bitstreams, and reference software.

# 2 HEVC Conformance Material

## 2.1 Source content

2.2. Polytech Nantes database

Uncompressed stereoscopic 3D video content dataset which can be used to, for example, generate MV-HEVC conformance streams by Polytech Nantes was pointed to in SA4 contribution [1]. At the time of writing this paper, all the links provided were down. This has already been observed in the past. The content itself, composed of 31 sequences, 1920 x 1080, 10-bit 4:2:2 YUV at 25 fps, was made freely available via the Creative Commons Attribution-ShareAlike License, and hence suitable for 3GPP SA4 purposes.

At the time being the proposal is:

- Rechecking if this resource gets back online

- Checking with the source

- Check if this can be moved to a more stable home.

## 2.2 Compressed Bitstreams

2.2.1 JVET1

A set of conformance bitstreams (elementary streams) for testing Multiview Extended 10 and Multiview Main 10 profile are provided by JVET [2]. The streams themselves are available for download at [3]. The various parameters of the bitstream including the profile to tested, resolution of content, frame rate and number of frames are tabulated below.

|  |  |  |  |  |  | Base layer profile |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Categories | Subcategory | Conformance Profile | Bitstream | File name | Resolution | Main | Main 10 | Level | Frame rate (Frames/sec) |  | # of frames |
| Prediction Structure | Inter-view prediction | Multiview Extended 10 profile | MVHEVCS-J | MVHEVCS_J_APPLE_2 | 1024x768 |  | X | 4 | 30 |  | 300 |
| (2-view) | Inter-view prediction | Multiview Extended 10 profile | MVHEVCS-K | MVHEVCS_K_APPLE_2 | 1024x768 |  | X | 4 | 30 |  | 300 |
|  | Inter-view prediction | Multiview Extended profile | MVHEVCS-L | MVHEVCS_L_Bytedance_2 | 1024x768 | X |  | 4 | 30 |  | 300 |
|  | Inter-view prediction | Multiview Extended profile | MVHEVCS-M | MVHEVCS_M_Bytedance_2 | 1024x768 | X |  | 4 | 30 |  | 300 |
|  | Inter-view prediction | Multiview Main 10 profile | MVHEVCS-N | MVHEVCS_N_APPLE_1 | 1024x768 |  | X | 4 | 30 |  | 300 |
|  | Inter-view prediction | Multiview Main 10 profile | MVHEVCS-O | MVHEVCS_O_APPLE_1 | 1920x1088 |  | X | 4 | 30 |  | 300 |

2.2.2 MV-HEVC stream

An MV-HEVC UHD streaming example is available at [4]. This stream playable only on specific devices listed on the page.

## 2.3 Reference Software

- Reference software for non-multiview content is HM reference software for HEVC [5]

- For multi-view content the reference software is: HEVC multi-view (MV-HEVC) and 3D video coding (3D-HEVC) extensions HTM reference software [6]

# 3 Proposal

It is proposed to review the provided content and add to the conformance part for TS 26.265. Identification of further steps is needed to gather more suitable resources.

# References

[Online] resources checked May 2025

[1] S4-231297 [FS_HEVC_Profiles] Discussion source MV-HEVC content, Goteburg, August 2023

[2] Conformance testing for HEVC multiview extended and monochrome profiles, JVET-AM1008

[3] [Online]: https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/HEVCMultiview/under_test/

[4] Streaming Examples https://developer.apple.com/streaming/examples/

[5] HEVC HM reference software [Online]: https://vcgit.hhi.fraunhofer.de/jvet/HM

[6] HEVC multi-view (MV-HEVC) and 3D video coding (3D-HEVC) extensions HTM reference software  [Online]: https://vcgit.hhi.fraunhofer.de/jvet/HTM