3GPP TSG-SA WG4 Meeting #132						S4-250850

Japan, Fukuoka, 19 – 23 May 2025

Source:	Dolby Laboratories Inc., Novamint

Title:	[FS_ULBC] On test methodologies for ULBC performance evaluation

Agenda item:	7.9

Document for:	DISCUSSION and AGREEMENT

# Background

3GPP SA4 has a long history of standardizing speech codecs that have been successfully deployed in 3GPP networks. Besides the most recently standardized IVAS codec, these are the AMR, AMR-WB and EVS codecs. SA4 adopted generally recognized ITU-T test methodologies for the quality evaluation of these codecs for the purpose of codec selection and characterization.

Similarly, one objective of the new FS_ULBC study item is to identify suitable test methodologies for ultra-low bit rate speech codecs. The fact that the target bit rate is envisioned to be below 3 kbps along with recent technology advances in the area of deep-learning based speech coding means that test methodologies that were used for AMR, AMR-WB or EVS codec evaluations need to be revisited.

This contribution discusses the challenges of ultra-low bit rate codec quality evaluations in the light of potential properties of such codecs and proposes applicable test methodologies.

# Discussion

## Typical quality impairments of ultra-low bit rate speech coding

Speech codecs operating at rates below 3 kbps typically impact speech quality in at least the following categories

- Loss of general audio quality

- Audio bandwidth loss

- Impaired intelligibility

- Impaired speaker identifiability

- Prosodic impairments

- Hallucination, i.e. word and phone confusions

A related quality impairment category is sensitivity to non-speech input. Non-speech input in this context may mean any non-clean speech input such as background noise, music, but also noisy speech, interfering talker speech, reverberant speech.

## Challenges of quality assessment of ultra-low bit rate speech codecs

AMR, AMR-WB and EVS codecs were evaluated using P.800 ACR and (modified) DCR test methodologies. ACR was generally used for clean speech tests while modified DCR was used for speech + background noise quality evaluations. Other categories were not tested since it could be assumed that impairment categories such as impaired intelligibility, impaired speaker identifiability and prosodic impairments played a minor role. Hallucination is a category that plays only a role in ML-based coding systems but not for signal-processing based codecs, which AMR, AMR-WB and EVS are.

Given the quality impairment categories of ultra-low bit rate speech coding other than impact of basic quality and audio bandwidth loss, ACR is generally problematic as it does not provide the deviation with respect to the uncoded (direct) reference.

In contrast, DCR methodology may cover most of the impairments. The most problematic property of DCR methodology is however that any deviation from the uncoded (direct) reference is rated as a degradation. This may be undesirable if the coding system incorporates a speech enhancer, which may enhance non-clean speech input.

Besides the usage of DCR and (potentially) ACR test methods, further subjective or objective test methods may be considered that were specifically developed for the evaluation of ultra-low-rate codecs with the above-listed expected impairments. A non-exhaustive list of such methods is:

- Diagnostic Rhyme Tests (DRT)

- Modified Rhyme Tests (MRT)

- MOS testing for speaker similarity

- Speaker verification/identification tests

- Prosodic naturalness MOS tests

- Intonation recognition tests

- Transcription tests involving testing for word and semantic equivalence

- Phoneme recognition tests

- Automatic speech recognition tests

Notably, these methods may require substantial effort, and it may be difficult to prepare corresponding tests in the short timeframe of the expected ULBC standardization timeframe. It is also expected that subjective test methods that rate the degradation of the coded speech signal in comparison with an uncoded (direct) reference would give sufficient answers to impairments the relevant quality degradation categories. The only expected downside is that degradation category ratings are an overall quality metric, and answers could not be expected to which of the degradation dimensions a degradation score could specifically be attributed.

# Proposal

Given the challenges of quality assessment of ultra-low bit rate speech codecs and the ability of DCR test methodology to cover most degradation categories, it is proposed to apply DCR generally for clean-speech quality evaluations. DCR tests may be realized as P.800 tests or in smaller scale as ITU-R BS.1534 MUSHRA tests.

For complementing non-clean speech tests where the coding system should be allowed to have an enhancement effect, suitable test methodologies may be P.800 CCR or ACR. The specific choice of either CCR or ACR test may still be left for discussion based on the actual test content (material). Especially ACR tests should however only be used as a complement of the main DCR tests with clean speech input to avoid that the impairment categories not covered by ACR test methodology are not left undetected. We further propose to document the test methodology mentioned above through a pCR to 3GPP TR 26.940 as shown below.

# pCR to 26.940

* * * First Change * * * *

# 9	Test methodologies

Editor’s Note:

5b. Identify appropriate test methodologies, regarding speech quality, intelligibility, conversational quality, in particular taking into account:

-	Clean speech and noisy speech

-	Tandeming with existing IMS voice codecs

-	Clean channel and GEO channel conditions

### 9.1	General

Given the challenges of quality assessment of ultra-low bit rate speech codecs and the ability of DCR test methodology to cover most degradation categories, DCR methodology is generally found suitable for clean-speech quality evaluations of ULBC systems.

Depending on the scale of the required evaluations in terms of tested conditions and test corpus, DCR tests will be realized as ITU-T P.800 tests or, in smaller scale, as ITU-R BS.1534 MUSHRA tests.

It is further assumed that the ULBC systems under test may under certain conditions have an enhancement effect. This may be the case if the input signal to the codec is degraded speech, e.g., noisy speech, interfering talker speech, reverberant speech. ULBC quality evaluations for this kind of input signals like also for non-speech content (noise, music) will be done as P.800 CCR or P.800 ACR tests, where the specific choice of either CCR or ACR test may still be left for discussion based on the actual test content (material). Especially ACR tests should however only be used as a complement of the main DCR tests with clean speech input to avoid that the impairment categories not covered by ACR test methodology are not left undetected.

* * * End of Changes * * * *