Source:	Philips International B.V., Qualcomm Incorporated

Title:	IVAS Characterization - Room Acoustics Test Plan

Document for:	Agreement

Agenda Item:	7.5

# Introduction

The IVAS-8b test plan for the characterization phase [1] still lacks details regarding room acoustics tests. The sources have started discussions on room acoustics testing in contributions [3][4] and continued in [5].

The latest contribution [5] was presented during the SA4#131 meeting and received valuable feedback. Concerns have been raised about the proposed methodology, as it may produce results that are easily misinterpreted. More specifically, the results on a MuSCR scale could resemble those on a MUSHRA scale and therefore the results presented in [5] might be mistakenly seen as indicating inferior rendered audio quality. Other points addressed the audio materials used. It was also noted that some rooms have a more reverberant sound than one might expect based on their visual setup.

This contribution addresses all concerns that were brought forward and proposes updates to IVAS-8b including the room acoustics test methodology, test equipment requirements, allocation of room acoustics experiments, as well as annexes containing listening test instructions and description of IVAS room acoustics synthesis related experiments.

# Proposed methodology

Based on the feedback received and subsequent discussions, the sources appreciate that the primary goal of the room acoustics testing is to prove that the IVAS decoder/renderer can produce audio with realistic room effect. Therefore, the sources have considered two alternative options for refining the test methodology for room acoustics testing.

Option A – preference test using anechoic rendering as a reference

With this option, for each trial, an anechoic binauralized output (BINAURAL rendering mode) is presented as a reference followed by the other CuTs in a random order (i.e., as REF, A, B, C). A user must indicate a preference for rendering modes as related to the anechoic reference.

The results directly indicate preference for different room acoustics rendering modes.

Option B – preference test with user-selectable reference

With this option, for each trial a user is presented with all CuTs in randomized order (i.e., as A, B, C, and D). A user must select the best sounding CuT as a reference by assigning it the maximum score. Eventually, the remaining CuTs are assigned scores relative to the selected reference.

Both options are based on a unipolar preference scale, as shown in Figure 1.

Figure 1. Unipolar preference scale

The corresponding test instructions are revised accordingly, also regarding congruence between room acoustics impressions and visual cues, which are provided mainly to indicate the room type. This aspect requires further explanation.

As mentioned in earlier discussions, the sources utilized the processing chain as illustrated in Figure 2. Impulse responses are recorded in real rooms by playing a logarithmic sweep signal through a loudspeaker and capturing it using a spherical microphone array. The sweep signal is then deconvolved from the recording. The recorded impulse response signals are converted to Ambisonics A-format for further processing. Eventually, BRIRs are generated using the Binaural SDM method based on these impulse responses and the IVAS default HRTF set. All these processes, although implemented carefully, may introduce some processing inaccuracies. The sources recall that the aim of the room acoustics rendering testing is to evaluate IVAS rendering with room acoustics. The objective is not to evaluate the processing chain used to generate BRIRs and room acoustics parameters. Therefore, the sources suggest focusing on the approximate congruence of the duration and tonality of the reverberation in relation to the presented virtual environment during the room acoustics synthesis test.

Figure 2. Rendering related processing chain

# Listening test tools

As discussed in the previous contributions, a set of tools has been prepared to enable room acoustics testing, including:

- An evaluation software package based on the Unity gaming engine based on concepts of the Audio Evaluation Platform (AEP) [6] as used for MPEG-I quality testing,

- A lightweight Python reference renderer allowing for real-time frame-based audio processing, and using TCP socket communication for pose and audio communication as proposed in [2],

- An updated version of the BinauralSDM tool [7] for generating BRIR data from HRIR and spatially recorded room impulse responses,

- An updated version of MATLAB scripts for room acoustics parameters computation based spatial impulse response recordings.

The evaluation software package has been already introduced in [4]. It supports 3DoF real-time head tracking, communication with audio renderers using TCP sockets, and audio streaming using NAudio library. The tool has been further updated to facilitate output level normalization, control means for lightweight Python renderer, etc.

The Python reference renderer as developed during the IVAS codec public collaboration does not support frame-based processing as required for real-time rendering. Therefore, a lightweight Python reference renderer has been developed featuring frame-based processing and a socket interface, identical to the one used in IVAS decoder/renderer. This renderer runs as a server, responding to packets containing the head rotation provided by the Unity evaluation software, generating subsequent frames of audio samples, and sending them back to Unity application.

As there are no ‘golden’ reference signals available, the lightweight Python reference rendering is used as a quasi-reference. To reduce the impact of core-coding on quality rating, an IVAS encoded and pass-through decoded audio is used as input to the Python renderer. Both the Python reference renderer and the IVAS decoder in BINAURAL_ROOM_IR mode use the same BRIRs computed using BinauralSDM from the default IVAS HRIR dataset and recorded room impulse responses. The IVAS decoder in BINAURAL_ROOM_REVERB mode uses reverb parameters computed from recorded room impulse responses. Furthermore, binaural rendering with default IVAS HRIR is used as an anchor test condition.

The processing paths for the room acoustic synthesis tests are shown in Figure 1.

Figure 3. Processing paths for room acoustics synthesis testing

Two versions of the listening test tool were provided supporting options A and B, as discussed in section 2. The screenshots of these tools are provided in Figure 4.

Figure 4. Listening tool UI: option A (left), option B (right)

Preliminary tests were conducted to evaluate the proposed methodology and tools using a 3DoF VR environment. These tests were using single talker and single instrument recordings in multi-channel (5.1) input format. The signals appear in the center channel, while the other channels remain silent. Six expert listeners participated in each test. The result summary for option A is shown in Figure 5, and for option B in Figure 6.

Figure 5. Room acoustics preference test results as compared to anechoic rendering (option A)

Figure 6. Preference test results: anechoic and room acoustics rendering (option B)

The tests results with both options are very consistent. The listeners indicate a clear preference for stimuli rendered with synthesized room acoustics for anechoic recordings presented in a VR environment. The differences between the IVAS room output modes and the reference Python renderer are not statistically significant. The results obtained with option B tend to lead to slightly higher preference scores, as the listeners must score one of the stimuli per trial with the highest score. Another observation is a high variance in scores assigned to the anechoic stimulus. The results of this test might require additional normalization per trial with respect to the scores of the anechoic HRTF rendered signal prior to evaluating the preference for synthesized room acoustics options.

# Proposal

The sources propose choosing option A test methodology (preference test using anechoic rendering as a reference) for room acoustics synthesis characterization testing. The primary reason for this choice is that the results for binaural rendering without room acoustics, as provided in option B tests, might give a false impression of inferior rendered audio quality for the anechoic reference. The results for option A reinforce the consistent performance of IVAS in rendering modes that incorporate room acoustics.

Further, the sources propose the following updates to the relevant sections of IVAS-8b document. The formatting applied is intended to facilitate editing of the permanent document.

The test methodology is discussed in section 4.2 of IVAS-8b. It is proposed to modify the opening sentence as follows:

The following primary test methodologies shall be used in the IVAS Characterization test: P.800 [4] and BS.1534 [13]. Additionally, preference testing methodology shall be used for testing room acoustics synthesis. High-level configuration of experiments for envisaged methodologies is outlined below.

It is proposed to add clause 4.2.3 on room acoustics testing methodology as follows:

4.2.3 Preference testing for room acoustics synthesis

For room acoustics testing preference test methodology shall be used. The listeners are requested to provide preference scoring based on quality rating.

  - 4 different rooms

  - 4 audio items per room

  - 4 conditions per test: IVAS rendering with HRTF (reference), IVAS rendering with BRIR, IVAS rendering with synthetic reverb, rendering with BRIR (Python implementation)

  - Total: 64 trials.

Each testing experiment shall be preceded with a training phase in which the listeners should familiarize themselves with the environment and methodology. Unipolar preference scale should be used for rating.

For clause 4.5 Listening Systems and Listening Environments, it is proposed to add the following bullet points:

  - VR setup for 3DoF binaural room acoustics synthesis listening comprising of:

    - A gaming-category PC running Unity game engine, capable to also run 3 IVAS decoder instances and a lightweight Python renderer in parallel,

    - A VR headset, e.g., Meta Quest or HTC Vive Pro,

    - Headphones requirements for binaural tests apply.

In clause 5.4 Allocation of experiments, it is proposed to update Table 9 with four room acoustics synthesis experiments as follows:

Table 9: Allocation of experiments using different methodologies

| Exp | Objective | Input format | Source material | Listening environment | Bitrates [kb/s] | LL |
| --- | --- | --- | --- | --- | --- | --- |
| ACR-1 | 16, 32, 48 kHz |  |  | Headphones |  | Nokia |
| ROOM-1 | Room acoustics | FOA | Generic audio | Headphones | 80 | Philips |
| ROOM-2 | Room acoustics | FOA | Generic audio | Headphones | 96 | Philips |
| ROOM-3 | Room acoustics | MC – 5.1 | Generic audio | Headphones | 96 | Philips |
| ROOM-4 | Room acoustics | MC – 5.1.2 | Generic audio | Headphones | 128 | Philips |

The proposed allocation triggers all applicable rendering modes of the IVAS decoder, as shown below. It also balances the impact of the core coding by using similar bitrates for all input formats.

| Exp | Input format | Bitrate | Rendering mode |
| --- | --- | --- | --- |
| ROOM-1 | FOA | 80 | Parametric binaural renderer |
| ROOM-2 | FOA | 96 | FastConv binaural renderer |
| ROOM-3 | MC – 5.1 | 96 | Time-domain object renderer / Crend binaural renderer |
| ROOM-4 | MC – 5.1.2 | 128 | Crend binaural renderer |

Annex A: Sample Instructions to Subjects and Data Collection currently only contains instructions for P.800 DCR tests. It is proposed to provide separate sections containing instructions for P.800, BS.1534, and MuSCR tests. The following section on the latter should be considered.

Room acoustics test instructions

INSTRUCTIONS TO THE LISTENERS FOR THE IVAS ROOM ACOUSTICS SYNTHESIS EVALUATION TEST In this experiment you will be evaluating systems that might be used for future immersive telecommunication services using spatial audio. Spatial audio is an audio experience where you can perceive sound as if coming from the space around you, outside and around your head. Spatial audio can be enriched by synthetic reverberation and acoustic reflections that simulate the acoustics of a room, creating a more natural and immersive experience. For this test you will be wearing a VR headset and headphones. The VR scene used in the test allows for three degrees of freedom (3DoF). That means that you can change your position only by rotation, and not by moving around. Please familiarize yourself with the equipment during the training session and adjust it appropriately. It is highly recommended to perform the tests while sitting on a swivel chair. It is advised not to take this test while standing. Please take time to observe the scene around you and listen to all the stimuli representing different CuT (conditions under test). Note that it will take a couple of seconds for the playback to start. The CuT provided have been processed with different reverb pathways from the IVAS decoder. A stimulus rendered without room acoustics is provided as a reference. Please rank your CuT preference regarding room acoustic synthesis quality and matching to the visual environment shown, as compared to the reference without room acoustics. Please consider factors such as timbral quality, spatial depth, and spatial envelopment and how do these relate to the image presented. The following quality scale is used in the tests.

Annex F and Annex G of IVAS-8b contains the description of the P800 and BS.1534 experiments. It is proposed to add Annex H containing conditions of room acoustics testing as shown below.

Annex H: Room acoustics testing experiments

H.1 Experiment ROOM-1: Parametric binaural renderer

Table H.1.1 Conditions

| Main Codec Conditions |  |
| --- | --- |
| Codec under test (CuT) | IVAS FX, IVAS FL |
| Input format | FOA |
| Bitrate | 80 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | 20KBP |
| Noise | No noise |
| Error Conditions | 0% |
| References |  |
| Reference (anechoic) | IVAS decoding with anechoic HRIR rendering |
| Other references |  |
| Reference (Python) | Lightweight Python renderer with BRIR support |
| Common Conditions |  |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Kind of samples | Clean speech recordings: - Single talker - Multiple talkers Music recordings: - Single instrument - Multiple instruments All recordings should be made in a non-reverberant environment. |
| Number of categories | 4 different room environments |
| Number of samples | 3 + 1 samples per category |
| Listening Level | Adjusted by listener |
| Listeners | Experienced listeners |
| Randomizations | Individual per listener |
| Rating Scale | Unipolar preference scale following clause 4.2.3 |
| Listening System | Headphones, VR setup in accordance with clause 4.5 |
| Listening Environment | No room noise |

Table H.1.2 Test conditions for Experiment ROOM-1

| Label | Condition – output format | Bitrate [kbps] |
| --- | --- | --- |
| c01 | Reference (anechoic) | 80 |
| c02 | Reference (Python) | 80 |
| c03 | IVAS – BINAURAL_ROOM_IR | 80 |
| c04 | IVAS – BINAURAL_ROOM_REVERB | 80 |

H.2 Experiment ROOM-1: FastConv Binaural Renderer

Table H.2.1 Conditions

| Main Codec Conditions |  |
| --- | --- |
| Codec under test (CuT) | IVAS FX, IVAS FL |
| Input format | FOA |
| Bitrate | 96 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | 20KBP |
| Noise | No noise |
| Error Conditions | 0% |
| References |  |
| Reference (anechoic) | IVAS decoding with anechoic HRIR rendering |
| Other references |  |
| Reference (Python) | Lightweight Python renderer with BRIR support |
| Common Conditions |  |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Kind of samples | Clean speech recordings: - Single talker - Multiple talkers Music recordings: - Single instrument - Multiple instruments All recordings should be made in a non-reverberant environment. |
| Number of categories | 4 different room environments |
| Number of samples | 3 + 1 samples per category |
| Listening Level | Adjusted by listener |
| Listeners | Experienced listeners |
| Randomizations | Individual per listener |
| Rating Scale | Unipolar preference scale following clause 4.2.3 |
| Listening System | Headphones, VR setup in accordance with clause 4.5 |
| Listening Environment | No room noise |

Table H.2.2 Test conditions for Experiment ROOM-2

| Label | Condition – output format | Bitrate [kbps] |
| --- | --- | --- |
| c01 | Reference (anechoic) | 96 |
| c02 | Reference (Python) | 96 |
| c03 | IVAS – BINAURAL_ROOM_IR | 96 |
| c04 | IVAS – BINAURAL_ROOM_REVERB | 96 |

H.3 Experiment ROOM-3: Time-domain object renderer / Crend binaural renderer

Table H.3.1 Conditions

| Main Codec Conditions |  |
| --- | --- |
| Codec under test (CuT) | IVAS FX, IVAS FL |
| Input format | Multichannel 5.1 |
| Bitrate | 96 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | 20KBP |
| Noise | No noise |
| Error Conditions | 0% |
| References |  |
| Reference (anechoic) | IVAS decoding with anechoic HRIR rendering |
| Other references |  |
| Reference (Python) | Lightweight Python renderer with BRIR support |
| Common Conditions |  |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Kind of samples | Clean speech recordings: - Single talker - Multiple talkers Music recordings: - Single instrument - Multiple instruments All recordings should be made in a non-reverberant environment. |
| Number of categories | 4 different room environments |
| Number of samples | 3 + 1 samples per category |
| Listening Level | Adjusted by listener |
| Listeners | Experienced listeners |
| Randomizations | Individual per listener |
| Rating Scale | Unipolar preference scale following clause 4.2.3 |
| Listening System | Headphones, VR setup in accordance with clause 4.5 |
| Listening Environment | No room noise |

Table H.3.2 Test conditions for Experiment ROOM-3

| Label | Condition – output format | Bitrate [kbps] |
| --- | --- | --- |
| c01 | Reference (anechoic) | 96 |
| c02 | Reference (Python) | 96 |
| c03 | IVAS – BINAURAL_ROOM_IR | 96 |
| c04 | IVAS – BINAURAL_ROOM_REVERB | 96 |

H.4 Experiment ROOM-4: Crend binaural renderer

Table H.4.1 Conditions

| Main Codec Conditions |  |
| --- | --- |
| Codec under test (CuT) | IVAS FX, IVAS FL |
| Input format | Multichannel 5.1.2 |
| Bitrate | 128 kbps |
| DTX | DTX off |
| Input level | -26 LKFS |
| Input frequency mask | 20KBP |
| Noise | No noise |
| Error Conditions | 0% |
| References |  |
| Reference (anechoic) | IVAS decoding with anechoic HRIR rendering |
| Other references |  |
| Reference (Python) | Lightweight Python renderer with BRIR support |
| References |  |
| Audio sampling frequency/bandwidth | 48 kHz/FB |
| Kind of samples | Clean speech recordings: - Single talker - Multiple talkers Music recordings: - Single instrument - Multiple instruments All recordings should be made in a non-reverberant environment. |
| Number of categories | 4 different room environments |
| Number of samples | 3 + 1 samples per category |
| Listening Level | Adjusted by listener |
| Listeners | Experienced listeners |
| Randomizations | Individual per listener |
| Rating Scale | Unipolar preference scale following clause 4.2.3 |
| Listening System | Headphones, VR setup in accordance with clause 4.5 |
| Listening Environment | No room noise |

Table H.4.2 Test conditions for Experiment ROOM-4

| Label | Condition – output format | Bitrate [kbps] |
| --- | --- | --- |
| c01 | Reference (anechoic) | 128 |
| c02 | Reference (Python) | 128 |
| c03 | IVAS – BINAURAL_ROOM_IR | 128 |
| c04 | IVAS – BINAURAL_ROOM_REVERB | 128 |

# Conclusions

In this contribution the sources propose updates to the IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase filling the gaps regarding IVAS room acoustics synthesis testing.

In response to the feedback provided during discussions on previous contributions, the sources considered two methodology options, which were evaluated by performing listening tests. Based on the results, option A (preference test using anechoic rendering as a reference) was proposed as the test methodology for the room acoustics characterization testing. The detailed proposal includes test methodology description, test equipment requirements, allocation of room acoustics experiments, and related annexes.

# References

- S4-241195 – IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase v.0.4.0

- S4-240257 – IVAS Characterization Testing – Binaural Rendering, Philips International B.V., Qualcomm Incorporated

- S4-240261 – IVAS Room Acoustics Testing Methodologies, Philips International B.V., Qualcomm Incorporated

- S4-242001 – IVAS Characterization - Room Acoustics Testing, Philips International B.V., Qualcomm incorporated

- S4-250245 – IVAS Characterization - Room Acoustics Test Plan, Philips International B.V., Qualcomm Incorporated

- Thomas Sporer, Sam Jelfs, Jurgen Herre, “Quality testing for AR and VR in MPEG-I immersive audio”, DAGA 2023.
https://pub.dega-akustik.de/DAGA_2023/data/articles/000401.pdf

- MPEG-I Immersive Audio Documentation for the Audio Evaluation Platform (AEP), Version 2 (2021) https://www.mpeg.org/wp-content/uploads/mpeg_meetings/136_OnLine/w20920.zip

- Facebook Research, BinauralSDM GitHub repository: https://github.com/facebookresearch/BinauralSDM

Rendering mode also depends on output format. Time-domain object renderer is used in case of BINARUAL_ROOM_REVERB format, Crend renderer in case of BINAURAL_ROOM_IR format.