3GPP TSG-SA4 Meeting #132	S4-250893

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.567 | CR |  | rev |  | Current version: | 1.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial updates to TS 26.567 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SR_IMS |  |  |  |  |  | Date: |  |  | 2025-05-13 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Provide editorial modifications to the TS 26.567 and to improve the quality of the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates are added to: - 1 using the correct word “adaptation” - 3.3 with new abbreviations - 4, 5, 6, 7, Annex A.1 to remove editor’s note - Correction in Figure 4.2.1 to align with TS 26.264, also adding the clarifications of reference points in 4.3 accordingly. - Annex A.2.3 with crrection to the table header and style. - Other minor editorial imrovements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Abbreviations, Figure 4.2.1 may cause misleading. TS is not aligned with draft rules. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 3.3, 4, 5, 6, 7, A.1, A.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

# 1	Scope

The present document specifies functional entities, reference points and protocols for IMS-based split rendering. It also provides codecs for delivery of split-render media and metadata of split rendered content for both uplink and downlink. The present document also contains procedures of split rendering session establishment, session management, adaptation, as well as other procedures to support split rendering process based on network support functions.

Key use cases enabled by this specification include XR services incorporating real-time and non-real-time media in industrial (e.g., for monitoring, maintenance, collaboration, and tele-operation), enterprise and educational environments; entertainment use-cases, including cloud-gaming, and shared and collaborative entertainment and productivity XR services.

# * * * * 2nd change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

A2P	Application to Person

API	Application Programming Interface

AR	Augmented Reality

AS	Application Server

DC	Data Channel

DC AS	Data Channel Application Server

DCMTSI	Data Channel Multimedia Telephony Service for IMS

DCSF	Data Channel Signalling Function

IMS	IP Multimedia Core Network Subsystem

MF	Media Function

MTSI	Multimedia Telephony Service for IMS

NEF	Network Exposure Function

NRF	Network Repository Function

P2A	Person to Application

P2P	Person to Person

QMC	QoE Measurement Collection

QoE	Quality of Experience

SR	Split Rendering

UE	User Equipment

# * * * * 3rd change * * * *

# 4	System description

## 4.1	Overview

The IMS stage-2 service description is specified in TS 23.228 [2]. The IMS architecture enhancements to support data channel services are specified in TS 23.228 Annex AC [2], which supports separation of signalling function and media function supporting data channel services.

Figure 4.1.1: Architecture of IMS supporting DC usage with MF

Based on the architecture in Figure 4.1.1, TS 26.264 [3] provides the IMS-based conversational AR (Augmented reality) services, including the generalized end-to-end architecture to support AR communication over IMS DC, as shown in Figure 4.1.2.

Figure 4.1.2: Generalized IMS DC Architecture to support AR communication (TS 26.264)

Accordingly, AR Application Server (AR AS) is responsible for AR service control related to AR communication, including AR session media control and AR media capability negotiation with the UE.

The DCSF receives event reports from the IMS AS and decides whether AR communication service is allowed to be provided during the IMS session. Additionally, the DCSF interacts with the AR AS for DC resource control.

MF supports AR conversational service by providing transcoding for terminals with limited capabilities. Additionally, the MF may collect spatial and media descriptions from UEs and create scene descriptions for symmetrical AR call experiences. MF also provide remote rendering for AR-MTSI clients in terminals with limited capabilities based on rendering negotiation. For remote rendering the AR-MTSI client provides AR metadata.

The IMS AS receives the media control instructions from the DCSF and accordingly interacts with the UE for connecting the UE's audio/video media termination to the MF and interacts with MF for data channel media resource management for AR media processing.

According to the architectures above, the present document introduces a mapping to the IMS architecture for IMS-based split rendering.

## 4.2	Reference Architecture

The generalized IMS DC architecture to support split rendering is shown in Figure 4.2.1.

Figure 4.2.1: Generalized IMS DC Architecture to support split rendering

Split-Rendering DCMTSI Client (SR-DCMTSI Client): A DCMTSI Client, that is responsible for acquiring the UE media capabilities and interacting with the MF during the split-rendering process.

Media Function (MF): A Media Function, that is responsible for interacting with SR-DCMTSI Client during split-rendering process, monitoring resource usage, and managing/running the split rendering process, etc.

Data Channel Application Server (DC AS): A DC Application Server, that is responsible for service control related to split-rendering, including session media control and media capability negotiation with the UE via MF, etc.

## 4.3	Reference Points

As shown in Figure 4.2.1, the following reference points are used to enable split rendering over IMS:

Mb: Reference point to enable split rendering between UE and MF, as specified in TS 23.228 [2].

Gm: Reference point to support communication between UE and IMS, as specified in TS 23.228 [2].

MDC1: Reference point for transport of data channel media between data channel media function and DCSF. MF terminates the bootstrap data channel from the UE and forward HTTP traffic between UE and DCSF via MDC1, as specified in TS 23.228 [2].

MDC2: Reference point for transport of data channel media between data channel media function and DC Application Server, for split rendering functions between application server to MF. MF relay traffic on A2P/P2A application data channels between the UE and the DC Application Server via MDC2, as specified in TS 23.228 [2].

The following reference points are also used to support split rendering related procedures:

DC1: Reference point between the DCSF and the IMS AS, as specified in TS 23.228 [2].

DC2: Reference point between the IMS AS and MF, for split rendering related data channel media resource management, as specified in TS 23.228 [2].

DC3: Reference point between the DCSF and NEF, as specified in TS 23.228 [2].

DC4: Reference point between the DCSF and DC Application Server, as specified in TS 23.228 [2].

N33: Reference point between NEF and DC Application server, network exposure to enable split rendering related applications, as specified in TS 23.501 [4].

NOTE: DC5, ISC, N5, and the reference point between IMS-aware application and SR-DCMTSI are out of the scope of this specification.

## 4.4	Split Rendering DCMTSI Client (SR-DCMTSI)

An SR-DCMTSI is a DCMTSI client in terminal defined as per clause 3.1 of TS 26.114 which supports split rendering. It is responsible for acquiring media capabilities and interacting with IMS functions during a split rendering session. It is responsible for the signalling with DCSF via IMS AS to set up a split rendering session and negotiating with the MF and DCAS about the parameters of the split rendering session, for example, the scene to be rendered and the split of rendering operations.

The SR-DCMTSI Client performs the following functions:

- Initiate a request of split rendering session establishment,

- Establish split rendering session, including signalling with DCSF via IMS AS to set up the data channel, and MF and DC AS to set up the split rendering,

- Operate the rendering loop on the UE,

- Update the rendering loop according to the network parameters and media capabilities with IMS functionalities,

- Stop the split rendering session or switch from a split rendering session to a local rendering session.

## 4.5	Media Function (MF)

MF is a Media Function which supports split rendering. It is responsible for interacting with SR-DCMTSI client and the DC AS during split-rendering process, monitoring resource usage, managing and running the split rendering process, etc.

The MF performs the following functions:

- Negotiate and interact with SR-DCMTSI Client (via IMS AS) and DC AS to establish the split rendering session,

- Operate the rendering loop, including, receiving meta-data from the SR-DCMTSI client, rendering frames, and forwarding requests to DC AS based on the split rendering logic,

- Expose its capabilities, capacity and media processing and rendering capabilities to the DC AS,

- Update the rendering loop according to the split rendering logic and requests from UE. If supported, the update can be based on the network parameters and interactions with other IMS functionalities,

- Stop/Pause/Resume the split rendering session according to the requests from UE.

### 4.5.1	Media Function (MF) Capabilities

An MF providing split rendering services specified in this document shall comply with NRF registration and discovery procedures specified in AC.7.4.2 of TS 23.228 [2].

The capabilities required for an MF to support split rendering are specified as SR service profiles each identified by a unique URN. To support split rendering specified in this specification, an MF shall support the SR service profile Basic as defined in clause 4.5.1.1 and may support other SR service profiles for minimum interoperability. Each SR service profile shall be uniquely identified by a urn. An MF shall list the urn of each SR service profile it supports as a MediaCapability in an MfInfo object as respectively defined in clauses 6.1.6.3.2 and 6.1.6.2.119 of TS 29.510 [15] in its NF profile when registering to an NRF.

#### 4.5.1.1	MF Service Profiles for SR

##### 4.5.1.1.1	Profile Basic

MF shall support:

- Scene description processing capabilities as specified in SD-Rendering-glTF-Core in TS 26.119 [6].

- Video encoding capabilities required to encode video complying with the capabilities specified in Annex Y.3 in TS 26.114 [7].

- Audio and speech encoding capabilities required to encode audio and speech complying with the capabilities specified in clause Y.4 of TS 26.114 [7].

- The type urn:3gpp:split-rendering:mf:profile:Basic shall be included in the Split Rendering Configuration defined in clause A.1.3 when the MF signals the SR-DCMTSI client in terminal.

##### 4.5.1.1.2	Profile Advanced

MF shall support the basic profile and the following media processing capabilities:

- Scene description processing capabilities SD-Rendering-gltf-ext1, SD-Rendering-gltf-ext2 and SD-Rendering-gltf-interactive as specified in clause 10.4.5 of TS 26.119 [6].

- Video encoding capabilities required to encode video complying with the capabilities specified in clause 10.4.3 of TS 26.119 [6].

- Audio and speech encoding capabilities required to encode audio and speech complying with the capabilities specified in clause 10.4.4 of TS 26.119 [6].

- The type urn:3gpp:split-rendering:mf:profile:Advanced shall be included in the Split Rendering Configuration defined in clause A.1.3 when the MF signals the SR-DCMTSI client in terminal.

#### 4.5.1.2	MF API

[The MF shall provide a Restful API to the DC AS over MDC2 once an application DC for SR is established. The API shall be:

Option 1:

| Operation name | Allowed HTTP method(s) | Description |
| --- | --- | --- |
| Get MF Services | GET | Get the DC Media services offered by the MF, for example, “DC”, “AR”, “SR” |
| Get MF Service Profiles (MF Service) | GET | A set of operating points of the capabilities for the DC media service(s) |
| Get Capabilities (MF service profile) | GET | Enumerate the XR/AR/SR capabilities, e.g., the supported XR/AR/SR, runtimes, engines supported, Scene graph/Scene description capabilities etc. This may also comprise information about supported input and output media formats. With the output media formats being non-DC media formats. |
| Set MF Profile (MF service profile) | POST/PUT | Set for this session the MF profile. |

Option 2:

| Operation name | Allowed HTTP method(s) | Description |
| --- | --- | --- |
| Get SR Profiles | GET | List of SR service profiles supported by the MF based on its current operating conditions. |
| Get Graphics Capabilities(SR Profile) | GET | Enumerate the graphics capabilities of the MF. This information may detail the rendering capacity of the MF. For example, GPU type, driver version/type, graphics runtimes and engines, VRAM etc, Scene description processing capabilities etc. |
| Set SR Profile(SR Profile) | SET | Set an SR profile |

]

## 4.6	DC Application Server (DC AS)

DC Application Server (DC AS) is responsible for service control related to split-rendering, including session media control and, session setup and media capability negotiation with the SR-DCMTSI client via MF and DCSF where applicable. The DC AS may provide split rendering application source data to the MF. The DC AS may be in the media path via MDC2, for example in Person to Application (P2A), Application to Person (A2P) or P2A2P scenarios as defined in clause AC.7.2.2 and AC.7.2.3 of TS 23.228 [2]. The DC AS may perform split rendering when it is in the media path.

The DC AS performs the following functions:

- Provide the DC application service, including the IMS DC application, to the DCSF.

- Negotiate and interact with SR-DCMTSI Client via MF and DCSF to establish the split rendering session,

- Query MF capabilities and capacity.

- Provide application source data for split rendering to the MF and to the SR-DCMTSI client

- If the DC AS is performing split rendering,

- Operate the rendering loop, including receiving meta-data from the SR-DCMTSI client via the MF, rendering frames based on the split rendering logic.

- Update the rendering loop according to the requests from UE via the MF

- Stop/Pause/Resume the split rendering session according to the requests from the SR-DCMTSI client.

# * * * * 4th change * * * *

# 5	Media codecs, configuration, and data transport

## 5.1	General

The SR-DCMTSI Client shall support the user plane protocol stack defined in Figure 4.3 of TS 26.114 [7]. All media components except data media components are transported over RTP with each respective payload format mapped onto RTP (RFC3550 [11]) streams. The data media components are transported over the data channels using SCTP (RFC4960 [12]) over DTLS (RFC8261 [13]), as specified for WebRTC data channels (RFC8831 [14]).

## 5.2	Media codecs

An SR-DCMTSI client that supports audio shall support the codec requirements for MTSI clients as specified in clause 5.2.1 of TS 26.114 [7].

An SR-DCMTSI client that supports video shall support the codecs requirements for MTSI clients as specified in clause 5.2.2 of TS 26.114 [7].

An SR-DCMTSI client that supports real-time text shall support the codec requirements for MTSI clients as specified in clause 5.2.3 of TS 26.114 [7].

An SR-DCMTSI client that supports still images shall support the codec requirements for MTSI clients as specified in clause 5.2.4 of TS 26.114 [7].

SR DCMTSI clients conforming to device types defined in clause 10 of TS 26.119 [6] should conform to media capabilities recommended for the respective device type in clause 10 of TS 26.119 [6].

## 5.3	Media configuration

SR-DCMTSI clients shall support media configuration requirements specified in clause 6 of TS 26.114 [7].

## 5.4	Data transport

### 5.4.1	General

An SR-DCMTSI client shall support data channel media and support procedures in clause 6.2.10 of TS 26.114 [7].

An SR-DCMTSI client shall support the data transport requirements specified in clause 7 of TS 26.114 [7].

Application data channels over which meta-data is transported shall support the IMS DC requirements in TS 23.228 [2], and requirements specified in clause 5.4.2.

### 5.4.2	Metadata Formats

#### 5.4.2.1	General

SR-DCMTSI client and Media Function shall support the usage of the IMS data channel for the exchange of split rendering metadata with the MF. The data channel shall declare “3gpp-sr” as the data channel sub-protocol. The message content format depends on the type of the message. The data channel sub-protocol is defined in clause 8.3.3 of TS 26.565 [5].

#### 5.4.2.2	Pose Format

For XR services, the pose information format that is used for IMS-based split rendering shall comply with the format defined in clause 12.2 of TS 26.119 [6]. The pose information shall be carried as part of the data channel messaging mechanism. The metadata data channel message format is as defined in clause A.1.2. The message type shall be “urn:3gpp:split-rendering:v1:pose”.

#### 5.4.2.3	Action Format

The action information format that is used for IMS-based split rendering shall comply with the format defined in clause 12.3 of TS 26.119 [6]. The action information shall be carried as part of the data channel messaging mechanism. The metadata data channel message format is as defined in clause A.1.2. The message type shall be “urn:3gpp:split-rendering:v1:action”.

#### 5.4.2.4	Split Rendering Configuration Format

The SR-DCMTSI client and Media Function shall support the split rendering session configuration defined in clause Annex A.1.3.

If DC AS is in the media path performing split rendering, it shall support the split rendering session configuration defined in clause Annex A.1.3.

The split rendering configuration message shall be identified as “urn:3gpp:split-rendering:v2:sr-configuration”.

### 5.4.3	Metadata Data Channel Message Format

For the carriage of metadata defined in clause 5.4.2, such as pose and action information, the SRDCMTSI client and MF shall use an IMS application data channel. The data channel sub-protocol shall be identified as “3gpp-sr”, which shall be included in the dcmap attribute of the SDP.

The transmission order for the data channel shall be set to in-order and the transmission reliability shall be set to reliable.

The split rendering metadata message format shall be set to text-based and the messages shall be UTF-8 encoded JSON messages.

A data channel message may carry one or more split rendering messages as defined in Clause 8.3.3 of TS 26.565 [5] and reproduced in Table A.1.2-1.

Each split rendering message shall follow the format specified in Clause 8.3.3 of TS 26.565 [5] and reproduced in Table A.1.2-2.

# * * * * 5th change * * * *

# 7	Procedures

## 7.1	Procedures for session establishment

### 7.1.1	General procedures

The figure 7.1.1-1 indicates the general procedures of split rendering session establishment including session setup and negotiation.

Figure7.1.1-1: General procedures for split rendering session establishment

The steps are as follows:

1:	The UE1 initiates an IMS session including bootstrap data channel establishment with the originating MF.

2:	UE1 sends a request to IMS AS to establish an application data channel for a split rendering session:

The UE1 can initiate a request of split rendering session establishment if its media capabilities cannot meet the related media rendering requirements. Then the UE1 calculates which objects can be rendered by itself based on its status and decides which part of the objects to be rendered in the UE1 and the others to be rendered in the IMS network.

3:	The IMS AS sends DCSF the event notifications including split rendering related information.

4:	The DCSF receives event notifications from the IMS AS and  processes the session establishment request based on the information in the notification (i.e. associated split rendering related information). The DCSF manages (if applicable) application data channel resources at the MF which meet the split rendering request, to instruct IMS AS to terminate the media flow of the UE1 to the MF.

5 and 6:	The IMS AS receives the data channel control instructions from the DCSF and accordingly interacts with the MF via DC2.

7:	The IMS AS sends a split rendering session establishment request to the DC AS via the DCSF, , the request may include the information of the objects to be rendered in IMS network.

8:	The DC AS sends a description of the split rendering output to the IMS AS via the DCSF.

9:	The IMS AS sends the media resource allocation request to the MF, to reserve XR media rendering resource for the UE1.

10:	When the resources are allocated successfully, the MF returns a successful response to the IMS AS.

11:	The IMS AS returns a successful response to the UE1.

12:	Successful split rendering session is established between UE1 and MF through the application data channel.

### 7.1.2	Procedures for P2P session establishment

The figure 7.1.2-1 indicates the procedures of split rendering session establishment for Person to Person (P2P) scenarios.

Figure7.1.2-1: Procedures for P2P split rendering session establishment

The steps are as follows:

1: 	IMS session establishment between UE1 and UE2, including bootstrap session establishment by UE1 and UE2 with originating MF.

2-12: Split rendering session establishment between UE1 and originating MF according to the steps 2-12 of clause 7.1.1.

13: Split rendering session established for UE2 between the originating MF and UE2. Similarly, seps 2 to 12 are followed for UE2. Step 2 may be a reply to the application data channel establishment (re)INVITE from UE1.

14: Successful split rendering session is established between UE1 and UE2 through the application data channel anchored by the MF.

### 7.1.3	Procedures for P2A(/2P) session establishment

Figure7.1.3-1: Procedures for P2A(/2P) split rendering session establishment

The steps are as follows:

1:	IMS session establishment between UE1 and MF, including bootstrap session establishment by UE1 and UE2 with originating MF.

2-12:	Split rendering session establishment between UE1 and originating MF, Same as in steps 2-12 of clause 7.1.2.

13:	The MF exposes its current capabilities and resources to the DC AS.

The information exposed may be the hardware and software stack and the resources currently available at the MF.

14:	DC AS selects resources at the MF and asks MF to reserve these resources.

15:	DC AS transfers application source data, for example scripts and scene graph, graphical assets and split logic to the MF.

16:	UE relevant application source data like scene graph, application and split logic is sent to the UE.

## 7.2	Procedures for session modification

### 7.2.1	General procedures

A UE may trigger split rendering session modification during an established split rendering session e.g., if the current session no longer meets the media rendering requirements. If the existing MF does not meet the new requirements, the session is migrated to a new MF. The session modification procedure in such a case is shown in Figure 7.2.1-1.

Figure7.2.1-1: General procedures for session modification

The steps are as follows:

1:	IMS session establishment including bootstrap data channel.

2:	Split rendering session establishment between UE1 and the MF1 (details see clause 7.1.1)

3:	Split rendering session is running.

4:		During a running split rendering session, when the UE1 discovers that its media capabilities cannot meet the related media rendering requirements, the UE1; can send a request to modify the split rendering session to the IMS AS.

5:	The IMS AS sends updated event notifications to DCSF to modify the current split rendering session.

6:	The DCSF receives event notification from the IMS AS and  process the session modification request based on the information in the received notification (i.e. associated split rendering related information). The DCSF manages (if applicable) application data channel resources at the MF which meet the new split rendering request; to instruct IMS AS to terminate the media flow of the UE1 to the new MF(s).

7:	The IMS AS receives the updated data channel control instructions from the DCSF.

NOTE:	Here it is assumed that there are more than one MF in the network. It is also assumed that the serving MF does not satisfy the new request (due to unavailability).

8:	The (new) MF2 is discovered (if the existing serving MF1 does not satisfy the UE1 request for session modification).

9:	UE1 sends application request to the newly discovered MF2 to request an application list.

10:	Accordingly, the MF2 responds with application list including an identifier that if the application is capable to be split rendered. DCSF receives the list and forward to the UE1.

11:	The IMS AS sends a split rendering session modification request to the DC AS via DCSF through the established application data channel, the request may include the information of the objects to be rendered in IMS network.

12:	The DC AS sends a description of the split rendering output to the IMS AS via DCSF.

13:	The IMS AS sends the media resource allocation request to the MF2, to reserve media rendering resource for the UE1.

14:	When the resources are allocated successfully, the MF2 returns a successful response to the IMS AS.

15:	The IMS AS returns a successful response to the UE1.

16:	The modified split rendering session is established between UE1 and MF2 through the data channel.

NOTE: 	MF1 serves the split rendering session until MF2 takes over.

### 7.2.2	Procedures for P2P session modification

The figure 7.2.2-1 indicates the procedures of split rendering session modification for P2P scenarios.

Figure 7.2.2-1: Procedures for P2P split rendering session modification

The steps are as follows:

1: 	IMS session establishment between UE1 and UE2, including bootstrap session establishment by UE1 and UE2 with originating MF.

2 to 4: Split rendering session establishment between UE1 and UE2 anchored by MF1 (details see clause 7.1.2).

5 to 17: The split rendering session is modified; the new session is established between UE1 and MF2 through the data channel.(details see clause 7.2.1).

18:	The new split rendering session is established between UE1 and UE2 anchored by MF2.

## 7.3	Network support procedures

### 7.3.1	General procedures

An SR-DCMTSI client or an MF may trigger further procedures during a split rendering, one such procedure may be to adapt the split of rendering operations between the SR-DCMTSI client and the MF during a split rendering session. This split may be due to change in operating conditions of the split rendering session, for example operating conditions of the UE, the MF or changes in the application or scene being rendered, for example changes in the scene description. Split adaptation may include data exchange, for example, exchange of adaptation messages, application state information and assets needed for the split rendering of an DC application. The following generic procedure shall apply, while the exact details may depend on the DC-application being rendered.

Figure7.3.1-1: General procedures for adaptation of split rendering

The steps are as follows:

1: 	The IMS session is established between the SR-DCMTSI client in terminal and a terminating SR-DCMTSI Client which may be in a terminal. For P2P calls, procedures in clause 7.1 are followed.

2: 	A split rendering session is set up between the SR-DCMTSI client and a serving MF.

3: 	Assets related to the application being split rendered may be delivered to participants of the split rendering session. The asset delivery may include javascript assets, scene descriptions and graphical objects needed for the session.

Rendering Loop:

The rendering loop is executed continuously during the duration of the split rendering session, for each frame.

4: 	The SR-DCMTSI client in terminal sends metadata required for rendering to the MF. The metadata may include pose, pose predictions, user inputs etc.

5 and 6:	The SR-DCMTSI client in terminal and the MF render the frame.

7: 	The frame rendered by the MF is transmitted to the SR-DCMTSI client in terminal as well as possible metadata.

8: 	The SR-DCMTSI client in terminal composes a display frame from the received rendered media and media rendered locally.

NOTE: 	Steps 5,6,7 although ordered above, may occur in any order. Step 8 may include pose-correction. Step 8 and step 6 may be executed as a single step.

Further Procedures:

Split Adaptation:

NOTE 1: 	Split Adaptation refers to adaptation of split rendering operations in an ongoing split rendering session between the SR-DCMTSI client and MF, without impacting the MF resources provisioned by IMS AS in step 9 of clause 7.1.1.

NOTE 2: 	Adaptation of split rendering may be used for interactive objects that react to user actions, pose, eye gaze, eye status, etc. The metadata (e.g. level of interaction, and actions as well as status of the user) may change during the lifetime of the session, the rendering is appropriately adapted according to the updated metadata. An SR-DCMTSI client that supports the adaptive split rendering based on eye status information supports the message format defined in Annex A.2.6.

9: 	A trigger to adapt the split occurs at the SR-DCMTSI client in terminal; the trigger may be, for example, a change in available UE resources (e.g. battery, compute) or updated metadata, changes in QoE of the split rendering session, changes in the scene/application being rendered.

10:	The SR-DCMTSI client in terminal decides if a new split of the rendering operations is needed and determines the new split.

11:	The SR-DCMTSI client in terminal sends a request to the MF to adapt the split to the new split.

12:	The MF actuates the new split of the rendering operations.

13:	The MF sends an acknowledgment of the new split to the SR-DCMTSI client in terminal.

14:	The MF and UE may exchange messages and data to support the new split of operations. This may include exchange of messages, for example, for synchronization of the state of the scene being split rendered or exchange of assets, for example, those in Step 3.

The meta-data messages exchanged shall follow the formats specified in clause 5.4.

15:	The rendering loop (steps 4 through 8) continues.

NOTE:  	Split adaptation is shown to be initiated by the SR-DCMTSI client in terminal for clarity, the procedure may be triggered by the MF. Further, other procedures to actuate the new split may be executed during the split rendering session.

# * * * * 6th change * * * *

Annex A (normative):
Metadata Formats and Message Types

# A.1	General

## A.1.1	Overview of Metadata Formats and Message Types

This annex defines the metadata and metadata message types supported by this specification. Metadata formats and meta data channel message types and formats supported in this specification re-use or modify formats and message types defined in other 3GPP specifications as defined in Table A.1.1-1.

Table A.1.1-1 Formats and relationship with 3GPP specifications

| Format | Source specification | Clause in source specification | Modified for this specification | Clause in this specification | urn |
| --- | --- | --- | --- | --- | --- |
| Metadata Data Channel Message | TS 26.565[5] | 8.3.3 | No | 5.4.3, A.1.2 | N/A |
| Split Rendering Configuration | TS 26.565[5] | 8.4.2 | Yes | A.1.3 | urn:3gpp:split-rendering:v2:sr-configuration |
| Pose | TS 26.119[6] | 12.2 | No | 5.4.2.2 | urn:3gpp:split-rendering:v1:pose |
| Action | TS 26.119[6] | 12.3 | No | 5.4.2.3 | urn:3gpp:split-rendering:v1:action |
| Split Adaptation Configuration | TS 26.565[5] | C.2.3.1 | Yes | A.2.3.1 | N/A |
| Split Adaptation Message | TS 26.565[5] | C.2.3.2 | Yes | A.2.3.2 | urn:3gpp:split-rendering:v2:asrp:sr-split |
| State Synchronization Message | TS 26.565[5] | C.2.3.3 | Yes | A.2.3.3 | urn:3gpp:split-rendering:v2:sr-state |
| Seamless Adaptive Split | TS 26.565[5] | C.2.3.2 | Yes | A.2.4 | urn:3gpp:split-rendering:v1:asrp:sr-split-seamless |
| Processing Delay Adaptation Configuration | N/A | N/A | N/A | A.2.5 | urn:3gpp:split-rendering:v1:daqoe:configuration |
| Processing Delay Adaptation Information | N/A | N/A | N/A | A.2.5 | urn:3gpp:split-rendering:v1:daqoe:information |
| Adaptive split rendering with eye status information | N/A | N/A | N/A | A.2.6 | urn:3gpp:split-rendering:v1:sr-split-eyeinfo |
| Asset Request | N/A | N/A | N/A | A.2.7 | urn:3gpp:split-rendering:v1:asrp:sr-asset |

NOTE: The metadata formats specified in Table A.1.1-1, if modified from the source specification for this specification may not be interoperable with the source specification.

# * * * * 7th change * * * *

## A.2.3	Split Adaptation

### A.2.3.1	Configuration format

The configuration format defined in Annex A.1.3 with the additional fields defined below in Table A.2.3.1-A shall be used for split rendering configuration exchange for adaptive split rendering.

Table A.2.3.1-1 Adaptive Split Rendering Configuration Format

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| renderingSplit | Object | 1..1 | An object identifying objects to be rendered and where they are to be rendered (MF or UE). The message shall be a dictionary object. with keys “MF” and “UE”, and values corresponding to a key shall be a list of named nodes from the scene description being rendered in the SR session. The keys shall indicate where the objects named in the corresponding value list are rendered. |
| synchronizedStatesInit | Object | 1..1 | An object identifying states to be synchronized between the MF and UE and their initial state |
| states | Object | 1..1 | A list of state identifiers, their current values |
| state | String/number | 1..n | Identifier of a state |
| initVal | String | 1..n | Initial value of the state |
| stateVals | Array | 1..1 | An array of values possible for the state |

The synchronizedStatesInit object is used during split rendering session establishment to configure which states are to be synchronized between the MF and the UE. In a scene being split rendered, there may be multiple state machines and for adaptive split rendering only a subset of the states may be affected by the split operations.