3GPP TSG-WG SA4 Meeting #132	S4-250895

# Fukuoka, Japan, 19th – 23rd May, 2025		(revision of S4-250xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | 0090 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Improved QoS for media streaming services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMD_PRO-MED |  |  |  |  |  | Date: |  |  | 2025-05-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In FS_AMD, stage 3 work on integrating ECN marking for L4S, PDU Set handling and QoS monitoring is needed. Furthermore, the media access fucntion also needs to behave, e.g. activation of L4S marking. Therefore, this paper intends to update the media plane aspects. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of Improved QoS for media streaming services |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support of improved QoS for media streaming services and work item AMD_PRO-MED cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.2.1, 13.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

13.2.1	Overview

In the following, it is assumed that the Media Player (in this case a DASH client) adheres to a basic set of functionalities as shown in figure 13.2-1. The DASH client downloads, processes and presents a DASH Media Presentation under the control of a 5GMSd-Aware Application via reference point M7d or of the Media Session Handler via reference point M11d.

The 5GMSd-Aware Application may, in addition, configure the presentation of the media, receive notifications on events, or query the internal status of the DASH Player, also supported through reference point M7d. Different functions of the DASH Access Client that are typically necessary to process a DASH Media Presentation, are shown in figure 13.2-1. Additional functions may be available as well.

Figure 13.2.1-1: Architecture of DASH-based 5GMSd Client

The key functionalities of each of the functions as shown in figure 13.2-1 are summarized in the following:

-	5GMSd-Aware Application: Application that makes use of the DASH-based Media Player to play back a DASH Media Presentation using the APIs defined in this clause.

-	Media Player: A complete player for the playback of a Media Presentation, including the Media Playback and Content Decryption Platform as defined in TS 26.511 [35].

-	Access Client: A part of the DASH Player that accesses and downloads of the resources and provides the downloaded resources to the Media Playback Platform and Content Decryption for the playback of DASH content.

-	Management: Controls all internal processes and the communication with the 5GMSd-aware application. In particular this includes the handling of service descriptions and operation points.

-	MPD Processing: parses and processes the MPD and extracts the relevant information.

-	Adaptation Set Selection: selects the Adaptation Set based on user, application and/or device capability information. Information provided through M7d may be used.

-	ABR Controller and Dynamic Switching: runs adaptive bit rate logic and triggers adaptive switching of Representations. Information provided to the DASH client through M7d may be used.

-	Throughput Estimation: estimates the network status, e.g. throughput, congestion level for the transmission link to the 5GMSd Application Server. Information provided to the DASH client through M11d may be used, i.e. QoS monitoring results. The ECN marking for L4S may be activated as notified by the Media Session Handler at interface M11d and the congestion information may be retrieved based on the ECN marking in the DL packets.

-	Metrics Logging: logs relevant low-level metrics and provides those to the metrics aggregation and reporting functions in the Media Session Handler.

-	Media Playback Management and Protection Controller: manages the media playback by moving downloaded information into media playback platform and also addresses handling of protection and DRM related information.

-	Media Playback and Content Decryption Platform: plays back CMAF-based media content according to the playback requirements in TS 26.511 [35]. It also provides status information as well as events that maybe be provided through M7d.

-	Event Processing: Processes DASH events and provides information to the 5GMSd-Aware Application as defined in TS 26.247 [4].

This clause focuses on interactions with the Media Player through reference point M7d. In particular, the following aspects of the API are defined:

1)	Methods to interact with the Media Player at this reference point are defined in clause 13.2.3.

2)	Notification and Error Events raised by the Media Player at this reference point are defined in clause 13.2.4.

3)	Configuration and Settings of the Media Player at this reference point are defined in clause 13.2.5.

4)	Status Information exposed by the Media Player at this reference point is defined in clause 13.2.6.

Communication between the Access Client and the media playback platform of the Media Player is defined in TS 26.511 [35].

A 5GMSd Client for DASH distribution shall support the APIs defined in this clause 13.

NOTE:	The initial APIs have largely been designed based on the dash.js APIs documented here: http://cdn.dashjs.org/latest/jsdoc.

# * * * * Second change * * * *

13.2.5	Notifications and error events

Table 13.2.5-1 provides a list of notification events that are provided by the Media Player to 5GMSd-Aware Applications at reference point M7d and to the Media Session Handler at reference point M11d. Every notification and error event is disambiguated by a media delivery session identifier.

Table 13.2.5-1: Media Player Notification events

| Status | Definition | Payload |
| --- | --- | --- |
| AST_IN_FUTURE | Triggered when playback will not start yet as the MPD's availabilityStartTime is in the future. | Media delivery session identifier, Time before playback will start. |
| AVAILABLE_MEDIA_CHANGED | The list of available media has changed. | Media delivery session identifier, Media type: - video - audio - subtitle - all |
| BUFFER_EMPTY | Triggered when the media playback platform's buffer state changes to stalled. | Media delivery session identifier, Media Type |
| BUFFER_LOADED | Triggered when the media playback platform's buffer state changes to loaded. | Media delivery session identifier, Media Type |
| CAN_PLAY | Sent when enough data is available that the media can be played. | Media delivery session identifier |
| MANIFEST_LOADED | Triggered when the manifest load is complete | Media delivery session identifier |
| METRIC_ADDED | Triggered every time a new metric is added. | Media delivery session identifier |
| METRIC_CHANGED | Triggered every time a metric value changes. | Media delivery session identifier, Metric identifier |
| METRIC_UPDATED | Triggered when the configuration of a metric is updated. | Media delivery session identifier, Metric identifier |
| METRICS_CHANGED | Triggered whenever there is a change to the overall metrics. | Media delivery session identifier |
| OPERATION_POINT_CHANGED | Triggered whenever there is a change of a Service Operation Point parameter. | Media delivery session identifier, External reference identifier of currently selected Service Operation Point. |
| PLAYBACK_ENDED | Sent when media playback completes normally. | Media delivery session identifier |
| PLAYBACK_ERROR | Sent when an error occurs during media playback. The element's error attribute contains more information. | Media delivery session identifier, Error reason (see table 13.2.5-2). |
| PLAYBACK_PAUSED | Sent when media playback is paused. | Media delivery session identifier |
| PLAYBACK_PLAYING | Sent when the media begins to play (either for the first time, after having been paused, or after ending and then restarting). | Media delivery session identifier |
| PLAYBACK_SEEKED | Sent when a media playback seek operation completes. | Media delivery session identifier |
| PLAYBACK_SEEKING | Sent when a media playback seek operation begins. | Media delivery session identifier |
| PLAYBACK_STALLED | Sent when the media playback platform reports stalled. | Media delivery session identifier |
| PLAYBACK_STARTED | Sent when playback of the media starts after having been paused; that is, when playback is resumed after a prior pause event. | Media delivery session identifier |
| PLAYBACK_WAITING | Sent when the media playback has stopped because of a temporary lack of data. | Media delivery session identifier |
| SERVICE_DESCRIPTION_SELECTED | sent when the DASH client has selected a service description. | Media delivery session identifier |
| SERVICE_DESCRIPTION_CHANGED | Sent when the DASH client has changed a service description. | Media delivery session identifier |
| SERVICE_DESCRIPTION_VIOLATED | Provides notification that the service description parameters are currently not met. | Media delivery session identifier, Parameters of service description that are not met |
| SOURCE_INITIALIZED | Triggered when the source is set up and ready. | Media delivery session identifier |
| DOWNLOAD_STARTED | Sent when a non-real-time content download begins. | Media delivery session identifier |
| DOWNLOAD_COMPLETED | Sent when a non-real-time content download is complete. | Media delivery session identifier |
| DOWNLOAD_ERROR | Send when an error occurs during non-real-time content download | Media delivery session identifier, Error reason (see table 13.2.5-2). |
| L4S_ACTIVATED | Sent when a notification of “L4S_REQUIRED” is received from the Media Session Handler and the Media Player supports ECN marking for L4S. | Media delivery session identifier |

Table 13.2.5-2 provides a list of error reasons that are indicated for notifications of type PLAYBACK_ERROR and DOWNLOAD_ERROR.

Table 13.2.5-2: Media Player Error reasons

| Error reason | Definition |
| --- | --- |
| ERROR_MEDIA_ENTRY_NOT_FOUND | The Media Entry Point resource requested by the Media Player could not be located. |
| ERROR_CONTENT_NOT_FOUND | Other content requested by the Media Player could not be located. |
| ERROR_MEDIA_PLAYBACK | There is an error from the media playback platform buffer. |
| ERROR_INVALID_MEDIA_ENTRY | The Media Entry Point resource supplied is not syntactically valid. |
| ERROR_INACCESSIBLE_MEDIA_TIME | The media time requested in a seek operation is not accessible in the current media presentation. |
| ERROR_UNSUPPORTED_PROFILE | The profile of the media presentation described by the Media Entry Point resource is not supported by the media playback platform. |
| ERROR_DOWNLOAD_DEADLINE_MISSED | The download of content did not complete before the requested deadline and the incomplete download has been discarded. |
| ERROR_L4S_ACTIVATED_FAILURE | The ECN marking for L4S could not be activated due to the missing support of ECN marking for L4S. |

# * * * * End of changes * * * *