3GPP TSG-SA4 Meeting #132	S4-250898

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025						   revision of S4-250725

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.113 | CR | 0010 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5G_RTP_Ph2] Enhancements to Dynamic Policy API for SDES RTP HE for MID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Communications, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The conclusion of KI#9 and Ki#14 (traffic detection of multiplexed media flows) from TR 26.822 are as below The following aspects are concluded as principles for normative work: - Based on response from SA2, normative work on multiplexed RTP streams may be needed. Furthermore, it is recommended to add guidelines to TS 26.522 [2] for RTP senders that use multiplexing. There may be potential normative aspects to be added to TS 26.510 [50]. When multiple RTP media streams are multiplexed in an RTP session, each media stream can be identified using the identification-tag (the values of "mid" attribute) in the SDP information. The RTP SDES header extension for MID make it possible for a 5G System or an RTP receiver to associate each PDU or PDU Set to a media stream when the the PDUs in a PDU Set carry the RTP SDES header extension for MID. To enable the traffic detection in 5G System, the mediaTransportParameters paremetr in the Application‌Flow‌Description object shall be updated to include the information of the multiplexed media identification information which includes ssrc id, MID, RTP SDES header extension for MID, Payload type and RTCP packet type fields. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the Dynamic Policy API to include the details of multiplexed media identification information in protocol description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Recommendations from work item description are not met, key 5GA features are not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Provided a NOTE on usage of the IP Filter Set with (S)RTP Multiplexed Media Identification Information for traffic detection by the 5G system. |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.506: "5G Real-time Media Communication Architecture (Stage 2)".

[3]	3GPP TS 26.510: "Media delivery; interactions and APIs for provisioning and media session handling".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	IETF RFC 9110 (2022): "HTTP Semantics".

[6]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[7]	IETF RFC 8834 (2021): "Media Transport and Use of RTP in WebRTC".

[8]	IETF RFC 8835 (2021): "Transports for WebRTC".

[9]	3GPP TS 23.003: "Numbering, addressing and identification".

[10]	IETF RFC 8829 (2021): "JavaScript Session Establishment Protocol (JSEP)".

[11]	IETF RFC 7807 (2016): "Problem Details for HTTP APIs".

[12]	IETF RFC 8825 (2021): "Overview: Real-Time Protocols for Browser-Based Applications".

[13]	IETF RFC 5124 (2008): "Extended Secure RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/SAVPF)".

[14]	IETF RFC 7007 (2013): "Update to Remove DVI4 from the Recommended Codecs for the RTP Profile for Audio and Video Conferences with Minimal Control (RTP/AVP)".

[15]	IETF RFC 3551 (2003): "RTP Profile for Audio and Video Conferences with Minimal Control".

[16]	IETF RFC 4585 (2006): "Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF)".

[17]	IETF RFC 3711 (2004): "The Secure Real-time Transport Protocol (SRTP)".

[18]	IETF RFC 5104 (2008): "Codec Control Messages in the RTP Audio-Visual Profile with Feedback (AVPF)".

[19]	IETF RFC 4588 (2006): "RTP Retransmission Payload Format".

[20]	3GPP TS 26.114: " IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[21]	IETF RFC 9112 (2022): "HTTP/1.1".

[22]	IETF RFC 7478 (2015): "Web Real-Time Communication Use Cases and Requirements".

[23]	3GPP TS 26.119: "Media Capabilities for Augmented Reality"

[24]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[25]	Apple: "Getting Raw Accelerometer Events", https://developer.apple.com/documentation/coremotion/getting_raw_accelerometer_events.

[26]	Google: "Sensor Coordinate System", https://developer.android.com/develop/sensors-and-location/sensors/sensors_overview.

[27]	ITU-R Recommendation BT.601-7 (03/2011): "Studio encoding parameters of digital television for standard 4:3 and wide screen 16:9 aspect ratios".

[28]	Microsoft: "Microphone Array Geometry Descriptor Format", https://learn.microsoft.com/en-us/windows-hardware/drivers/audio/microphone-array-geometry-descriptor-format.

[29]	IETF RFC 8831 (2021): "WebRTC Data Channels".

[30]	IETF RFC 8261 (2017): "Datagram Transport Layer Security (DTLS) Encapsulation of SCTP Packets".

[31]	W3C Recommendation: WebRTC: Real-Time Communication in Browsers, March 2023. https://www.w3.org/TR/webrtc/

[32]	IETF RFC 7874 (2016): "WebRTC Audio Codec and Processing Requirements"

[33]	IETF RFC 7742 (2016): "WebRTC Video Processing and Codec Requirements"

[34]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Services (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[35]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[36]	3GPP TS 26.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[37]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[38]	3GPP TS 29.514: "5G System; Policy Authorization Service".

[39]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes".

second change

## 10.3	Dynamic Policy API

The Dynamic Policy API allows the RTC Media Session Handler of the RTC Client or the ICE Function of the RTC AS or the WebRTC Signalling Function of the RTC AS to request a specific QoS and/or charging policy to be applied to the application flows of an RTC session. The Dynamic Policy API is invoked as a result of SDP negotiation during the WebRTC signalling phase of the RTC session.

The relevant procedures are specified in clause 5.3.3 of TS 26.510 [3].

The resource structure and the data model are specified in clause 9.3 of TS 26.510 [3].

If specific QoS with PDU Set parameters is desired, and PDU Set marking is not enabled for the selected Policy Template as specified in clause 5.3.3.2 of TS 26.510 [3], the Media Session Handler shall additionally populate the mediaTransportParameters property of the Application‌Flow‌Description object (see clause 5.5.4.13 of TS 29.571 [36]) as follows when creating or updating a Dynamic Policy Instance based on that Policy Template:

-	The transportProto property shall be set to the value SRTP.

-	The rtpHeaderExtInfo object (see clause 5.5.4.14 of TS 29.571 [36]) shall be omitted.

-	The rtpPayloadInfoList property shall contain a single member populated as follows:

-	rtpPayloadTypeList shall be set to the RTP Payload Type value(s) to be used by the RTC endpoint (e.g., the RTC Access Function of an RTC Client) for the negotiated SRTP session(s) to be carried by the application flow in question.

-	rtpPayloadFormat shall be populated as appropriate in the absence of RTP header extensions.

If PDU Set marking is enabled for the selected Policy Template as specified in clause 5.3.3.2 of TS 26.510 [3], the Media Session Handler shall additionally populate the mediaTransportParameters property of the Application‌Flow‌Description object (see clause 5.5.4.13 of TS 29.571 [36]) as follows when creating or updating a Dynamic Policy Instance based on that Policy Template:

-	The transportProto property shall be set to the value SRTP.

-	The properties of the rtpHeaderExtInfo object (see clause 5.5.4.14 of TS 29.571 [36]) shall be populated as follows:

-	rtpHeaderExtType shall be set to PDU_SET_MARKING.

-	rtpHeaderExtId shall be set to the value of the ID field to be used by the RTC endpoint (e.g., the RTC Access Function of an RTC Client) in the RTP Header Extension for PDU Set Marking on the application flow in question, as specified in clause 4.2 of TS 26.522 [37]. The value of this parameter is negotiated via the SDP offer/answer procedure during the WebRTC signalling phase of the RTC session.

-	longFormat shall be set according to the use of the one- or two-byte RTP Header Extension for PDU Set Marking, as specified in clause 4.2.1 of TS 26.522 [37]. The value of this parameter is negotiated via the SDP offer/answer procedure during the WebRTC signalling phase of the RTC session.

-	pduSetSizeActive shall be set to reflect the presence of the PDU Set Size field in the RTP Header Extension for PDU Set Marking, as specified in clause 4.2.4 of TS 26.522 [37]. The value of this parameter is negotiated via the SDP offer/answer procedure during the WebRTC signalling phase of the RTC session.

NOTE:	The intention of the RTC Access Function of the RTC Client to include the optional NPDS (Number of PDUs in the PDU Set) field in the RTP Header Extension for PDU Set Marking is not yet signalled in advance to the 5G Core by means of a Boolean flag in the RtpHeaderExtInfo specified in clause 5.5.4.14 of TS 29.571 [36].

-	The rtpPayloadInfoList property shall contain a single member populated as follows:

-	rtpPayloadTypeList shall be set to the RTP Payload Type value(s) to be used by the RTC endpoint (e.g., the RTC Access Function of an RTC Client) for the negotiated SRTP session(s) to be carried by the application flow in question.

-	rtpPayloadFormat shall be omitted because RTP header extensions are present.

In all PDUs it contributes at reference point RTC-4m or RTC-12 that fall within the scope of the application flow description, the RTC Access Function (Media Access Function) shall use the protocol indicated in transportProto; it shall set the SRTP header fields in accordance with rtpPayloadInfoList; and it shall include a one- or two- byte (consistent with the signalled length) RTP Header Extension for PDU Set Marking in the SRTP header with fields set according to the values declared in the rtpHeaderExtInfo property per above.

If an RTP Session requires multiple media flows to be multiplexed into a single RTP Session, the Media Session Handler or the media AS shall additionally populate the mpxMediaInfoList property in the Protocol‌Description object of the Application‌Flow‌Description object (see clause 5.5.4.13 of TS 29.571 [36]) as follows when creating or updating a Dynamic Policy Instance:

-	The transportProto property shall be set to the value SRTP.

-	The properties of the RtpHeaderExtInfo type (see clause 5.5.4.14 of TS 29.571 [36]) shall be set as described above when PDU Set marking is enabled otherwise it shall be omitted.

-	The rtpPayloadInfoList property shall contain a single member populated as follows:

-	rtpPayloadTypeList shall be set to the RTP Payload Type value(s) to be used by the RTC endpoint (e.g., the RTC Access Function of an RTC Client) for the negotiated SRTP session(s) to be carried by the application flow in question.

-	rtpPayloadFormat shall be omitted because RTP header extensions are present.

-	When the RTC endpoints supports the BUNDLE mechanism for multiplexing RTP streams as specified in clause 4.6 of TS 26.522 [37], the mpxMediaInfoList property shall contain at least one MpxMediaInfo object for each media stream. The properties of the MpxMediaInfo object are negotiated by the RTC Access Function of the RTC Client or the RTC AS via the SDP offer/answer procedure during the WebRTC signalling phase of the RTC session using the SDP attribute a= BUNDLE. The properties of each MpxMediaInfo object (see clause 5.6.2.61 in TS 29.514 [38]) shall be populated as follows:

-	ssrcId shall be set to the synchronization source identifier value in the RTP Header to be used by the RTC endpoint (e.g., the RTC Access Function of an RTC Client or the RTC AS) for a media stream of the application flow in question.

-	payloadType shall be set to the RTP Payload Type value to be used by the RTC endpoint (e.g., the RTC Access Function of an RTC Client or the RTC AS) for a media stream of the application flow in question. The value of this parameter is negotiated via the SDP offer/answer procedure during the WebRTC signalling phase of the media session. When this property is present the RTP Payload Type value(s) present in the rtpPayloadTypeList shall be ignored.

-	rsiMid shall be set to the value of the identification-tag or media description identifier (MID) to be used by the RTC endpoint (e.g., the RTC Access Function of an RTC Client or the RTC AS) for a media stream in the SDES RTP Header Extension for MID or the RTCP MID SDES Item for MID on the application flow in question, as described in clause 4.6 of TS 26.522 [37]. The value of this parameter is negotiated via the SDP offer/answer procedure during the WebRTC signalling phase of the media session.

-	rsheMid shall be set to the value of the local identifier or ID field to be used by the RTC endpoint (e.g., the RTC Access Function of an RTC Client or the RTC AS) in the SDES RTP Header Extension for MID on the application flow in question, as specified in clause 4.6 of TS 26.522 [37]. The value of this parameter is negotiated via the SDP offer/answer procedure during the WebRTC signalling phase of the media session.

-	longFormat shall be set according to the use of the one- or two-byte RTP SDES Header Extension for MID, as specified in clauses C.2.2 and C.2.3 of TS 26.522 [37].

-	rtcpPt shall be set to the RTCP Packet Type (PT) value to be used by the RTC endpoint (e.g., the RTC Access Function of an RTC Client or the RTC AS) for a media stream of the negotiated SRTP session(s) to be carried by the application flow in question.

In all PDUs it contributes at reference point RTC-4m or RTC-12 that fall within the scope of the application flow description, a sender as the RTC Access Function (Media Access Function) or the Media Function of the RTC AS shall use the protocol indicated in transportProto; the sender shall set the SRTP header fields in accordance with mpxMediaInfoList; and it shall include a one- or two- byte RTP SDES Header Extension for MID in the SRTP header with fields set according to the values declared in the mpxMediaInfoList property per above to indicate the multiplexed media identification information.

NOTE:	When multiplexed media identification marking is used in this way, multiplexed media traffic identification can be performed by the 5G System for differentiated QoS treatment using the IP Filter Set with (S)RTP Multiplexed Media Identification Information feature defined in clause 8.2.5 of TS 29.244 [39], which inspects certain SRTP header fields, specifically the Synchronization Source identifier and the Payload Type(s) in combination with the media description identifier information present in the MpxMediaInfo object of mpxMediaInfoList property.

End of changes