3GPP TSG-SA WG4 Meeting #132			                                                          S4-250901

Fukuoka, Japan, 19-23 May 2025	(revision of S4-250591, S4aA250024)

Source:	vivo, Fraunhofer IIS, Qualcomm Incorporated, China Mobile Com. Corporation， [Huawei Technologies Co., Ltd.], Viasat, Inmarsat, EchoStar,[Orange], Spreadtrum

Title:	Application scenario for IMS voice call using GEO satellite

Spec:	3GPP TR 26.940 v0.0.1

Agenda item:	7.9

Document for:	Agreement

1. Introduction

At the recent SA#107 meeting, the "Study on Ultra Low Bitrate Speech Codec" has been approved. According to the WID description [1], the primary focus of this study is to develop design constraints and performance requirements for a codec supporting use cases like IMS Voice Call over GEO and the resulting transmission parameters.

The use case of IMS voice all using GEO satellite access is specified in clause 5.1 of TR 22.887 [2], and corresponding requirements and performance requirements are outlined in in clause 6.46.11and in Table 7.4.2-1 of TS 22.261 [3].

#### Current commercialized satellite voices using GEO satellite

There are two main types of commercial satellite phone services available on the market. One type utilizes a directional antenna, exemplified by the Inmarsat phone (iSatPhone2), which operates based on the GMR-2 specification [4], with performance details provided in [5]. The other type uses an omnidirectional antenna, such as vivo and Huawei satellite phones, which rely on proprietary solutions, with performance examples available in [6] and [7].

#### Reference architecture in 3GPP

Regarding satellite radio access types (RATs), from the core network's perspective, there are a total of 12 RAT types, generally categorized into IoT NTN [8a] and NR NTN [8b]. Among these, only four types specifically focus on GEO satellites.

Fig. 1: reference architectures defined in 3GPP

- IoT NTN: NB-IoT (GEO) satellite access, LTE-M (GEO) satellite access, WB-E-UTRA (GEO) satellite access, with referring architecture as Fig.1 (left) shows:

- NR NTN: NR (GEO) satellite access, with referring architecture as Fig.1 (right) shows.

SA2 SID (SP-250400) agrees to study the objective of IMS voice call over GEO via NB-IoT NTN connecting to EPC in Rel-20.

Observation #1: NB-IoT (GEO) satellite access connecting to the EPC is considered a reference architecture for the Rel-20 3GPP specification.

#### Conversation communication between two-parties

As stated in Clause 5 of TR22.887, the use case focuses on conversational communication between two parties:

- Case I: Mobile-originated satellite-to-terrestrial call and Mobile-terminated satellite-from-terrestrial call

- Case II: Satellite-to-satellite call

The IMS platform is utilized due to its ability to ensure interoperability with other operators (e.g., VoLTE/VoNR) and call systems (e.g., fixed-line networks). This means that on the terrestrial side, all originating/termination IMS-supported phone calls with satellite terminal are allowed, leading to interoperability requirements for ULBC to transcode with other operator/system-supported codecs such as AMR and EVS.

Observation#2: If the other party in the conversation is on a terrestrial network, transcoding between ULBC and other IMS-supported codecs available in SDP negotiation should be supported. However, if both parties are using satellite phones that support the same ULBC codec, transcoding may be avoided.

Terminal and satellite requirements

The satellite terminal is specified as handheld, with 3GPP specifications indicating support for 23dBm (pre-R19), and 26dBm, 29dBm (Rel-19). Additionally, the handheld terminal typically uses an omnidirectional antenna and is expected to function similarly to a regular mobile phone. In general, the mobile phone can be enhanced with a 0dBi antenna gain.

The operating GEO satellites available, as specified in 3GPP (pre-Rel-20), include GEO Set 1 and GEO Set 2, as shown in the table below, which is sourced from List of calibration study cases as specified in Table 6.1.1.1-9 in TR 38.821 [8].

#### Transmission data rate

SA1 has agreed upon a transmission data rate for UL/DL on the terminal side of approximately [1-3] kbps.

Observation#3: The estimated data rate provided by SA1 requires further validation by downstream groups, such as RAN1, for validation.

#### Propagation delay

In 3GPP, propagation delay is addressed in at least two specifications. One is in the SA1 specification, TS 22.261 [3], "A 5G system providing service with satellite access shall be able to support GEO based satellite access with up to 285 ms end-to-end latency. NOTE 1: 5 ms network latency is assumed and added to satellite one-way delay."

The other is in the RAN study report, TR 38.821, where Table 7.1-1 provides a delay range of 477.48 ms to 541.46 ms (round trip delay).

Based on the reference architecture and use cases discussed, the delay would be as follows:

| MO satellite call to terrestrial |  |
| --- | --- |
| MT satellite call from terrestrial |  |
| MO to MT satellite call | NOTE: Even though the Mobile-Originated terminal and Mobile-Terminated terminal are under the same GEO satellite coverage, the propagation delay should be calculated twice. This is because the satellite uses a transparent payload, which cannot locally switch the phone calls. |

2. Reason for Change

The present document provides the applied scenarios based on the use case captured in TR 22.887 [2] and requirements captured in TS 22.261 [3]. This revised version also merged S4aA250025’s contents.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.940 [9]

* * * First Change * * * *

# 4	Application scenarios

## 4.1	Introduction

This clause introduces application scenarios that may be relevant for an Ultra-Low Bitrate speech Codec (ULBC). For each scenario, high-level prerequisites on the ULBC codec or the service operation are derived.

## 4.2	Scenario 1: IMS Voice Call over GEO

### 4.2.0	General

This is the primary application scenario in the context of voice communication via Geostationary Earth Orbit (GEO) satellites.

### 4.2.1 	Background

Satellite communication plays an important role in extending terrestrial network coverage, ensuring seamless connectivity for users This technology unlocks new opportunities across various sectors, including smartphones and IoT.

Conversational two-party communication is foreseen as the most common and essential use case for IMS voice over GEO satellite access. This use case is documented in clause 5.1 of TR 22.887 [2]. Due to the limitations of low transmission data rates (in the range of [1-3] kbps) as estimated in Table 7.4.2-1 in TS 22.261 [3], an ultra-low bit rate codec is required. Users may rely on this service when they are beyond terrestrial network coverage but within satellite range, ensuring essential connectivity.

NOTE:	The bit rate range of [1-3] kbps is a working assumption according to TS 22.261 and might be adjusted after coordination with RAN groups.

### 4.2.2 	Scenario Description

#### 4.2.2.1 	General

The typical UE is a handheld device supporting GEO satellite access with built-in microphones and loudspeakers or a monaural hands-free set, as outlined in Table 7.4.2-1 in TS 22.261 [3]. This means there is no need for the user to extend the antenna or carry any extra devices to access the IMS voice call service.

The typical IMS voice service includes both regular call and emergency call as outlined in clause 6.46.11 in TS 22.261 [3].  Such service can be provided as:

-	Supplementary regular IMS voice service provided by the terrestrial operators, especially in areas without terrestrial coverage.

-	Main regular IMS voice service provided by the satellite operators.

-	An IMS voice call service using GEO satellite access in case of emergency situations.

The UEs under consideration is typically commercial smartphones as SA1 defined in Table 7.4.2-1 in TS 22.261 [3].

In the following, different scenarios establishing an end-to-end voice service are considered.

#### 4.2.2.2 	Main Scenario: One UE connects via GEO-satellite access only

In a common scenario, one party in the conversation is assumed to be using a handheld mobile terminal over a GEO satellite network, while the other may be on a terrestrial mobile network (e.g., VoLTE, VoNR), a fixed-line connection, or another IMS-supported platform, as outlined in Figure 4.2.2.2-1, where a sketch of the bi-directional voice data flow for this main scenario is depicted.

Figure 4.2.2.2-1: Bi-directional voice data flow for main scenario

NOTE: 	Core network typically stands for 3GPP core network and IMS core network.

In this scenario, UE1 is a phone supporting IMS voice call service over GEO satellite access. One may distinguish two cases depending on UE2:

- UE2 is a "regular" phone supporting IMS voice call service but not supporting ULBC nor GEO satellite access. In this case, UE2 may not be aware that UE1 is using a GEO satellite link during the communication and transcoding is performed in the core network.

- UE2 is an "upgraded" phone supporting IMS voice call service with ULBC, but using another access than satellite access (e.g., LTE, NR or WLAN). In this case UE2 may be able to communicate with UE1 using  ULBC in a transcoder-free operation.

#### 4.2.2.3 		Sub-Scenario: Both UEs connect via GEO-satellite access

In a less common scenario, both parties in the conversation are connected to a GEO satellite as outlined in Figure 4.2.2.3-1 using IMS-based communication services.

Figure 4.2.2.3-1: Bi-directional voice data flow for sub-scenario

NOTE: 	Core network typically stands for 3GPP core network and IMS core network.

When the GEO satellite operates in a transparent payload, the voice packets are transmitted to the ground before transmitted to the other UE, even if both UEs are connected to the same GEO satellite. In this case, the communication between UE1 and UE2 may use ULBC in a transcoder-free operation.

### 4.2.3	Derived high-level prerequisites

#### 4.2.3.1	General

The following general prerequisites for the ULBC apply based on application scenario 1.

Editor’s note:	Some high-level prerequisites could be added such as:

"To serve application scenario 1, the ULBC codec is expected to meet the following high-level prerequisites:

- Very low bitrate support

- DTX support [to be confirmed]

- Error concealment

- Implementable in real-time (encoding and decoding) on at least a selective set of smartphones

- Good audio quality to ensure a reasonable QoE (Detailed QoE requirements for such services are for study).

Considerations on whether existing audio quality tests (TS 26.131 and TS 26.132) may be considered."

* * * End of Changes * * * *

[1] Tdoc SP-250378, "Study on Ultra Low Bitrate Speech Codec"

[2] TR 22.887, "Study on satellite access - Phase 4"

[3] TS 22.261, "Service requirements for the 5G system"

[4] GMR-2 03.050, "Transmission planning for speech service"

[5] Inmarsat Phone https://www.youtube.com/watch?v=iaI-rVMB6wk

[6] vivo Phone https://youtube.com/watch?v=EskEtJt9aQM&feature=shared

[7] Huawei Phone https://www.youtube.com/watch?v=6xDgaMajWxo

[8a] TR 23.401, "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access"

[8b] TR 23.501, "System architecture for the 5G System (5GS) "

[8] TR 38.821, "Solutions for NR to support non-terrestrial networks (NTN)"

[9] 3GPP TR 26.940, "Study on Ultra Low Bitrate Speech Codecs"