3GPP TSG-SA WG4 Meeting #132			                                                          S4-250902

Fukuoka, Japan, 19-23 May 2025

Source:	vivo, Fraunhofer IIS, Qualcomm Incorporated, China Mobile Com. Corporation， [Huawei Technologies Co., Ltd.], Viasat, Inmarsat, EchoStar, [Orange], Spreadtrum

Title:	Application scenario for IMS voice call using GEO satellite

Spec:	3GPP Pdoc [to be confirmed]

Agenda item:	7.9

Document for:	Agreement

1. Introduction

Application scenario for IMS voice call using GEO satellite is discussed in TR 26.940.

The under-discussion aspects such as assumptions and open issues, interface and interface properties, applicability to IoT devices are proposed to be captured in this Pdoc. All differences are captured with revision mark.

2. Reason for Change

Compared to the proposal to TR 26.940, the present document provides un-resolved issues on application scenario for IMS voice call using GEO satellite access.

3. Proposal

It is proposed to agree the following changes to 3GPP Pdoc [to be confirmed]

* * * First Change * * * *

# 4	Application scenarios

## 4.1	Introduction

This clause introduces application scenarios that may be relevant for an Ultra-Low Bitrate speech Codec (ULBC). For each scenario, high-level prerequisites on the ULBC codec or the service operation are derived.

## 4.2	Scenario 1: IMS Voice Call over GEO

### 4.2.0	General

This is the primary application scenario in the context of voice communication via Geostationary Earth Orbit (GEO) satellites.

### 4.2.1 	Background

Satellite communication plays an important role in extending terrestrial network coverage, ensuring seamless connectivity for users This technology unlocks new opportunities across various sectors, including smartphones and IoT.

Conversational two-party communication is foreseen as the most common and essential use case for IMS voice over GEO satellite access. This use case is documented in clause 5.1 of TR 22.887 [2]. Due to the limitations of low transmission data rates (in the range of [1-3] kbps) as estimated in Table 7.4.2-1 in TS 22.261 [3], an ultra-low bit rate codec is required. Users may rely on this service when they are beyond terrestrial network coverage but within satellite range, ensuring essential connectivity.

NOTE:	The bit rate range of [1-3] kbps is a working assumption according to TS 22.261 and might be adjusted after coordination with RAN groups.

### 4.2.2 	Scenario Description

#### 4.2.2.1 	General

The typical UE is a handheld device supporting GEO satellite access with built-in microphones and loudspeakers or a monaural hands-free set, as outlined in Table 7.4.2-1 in TS 22.261 [3]. This means there is no need for the user to extend the antenna or carry any extra devices to access the IMS voice call service.

The typical IMS voice service includes both regular call and emergency call as outlined in clause 6.46.11 in TS 22.261 [3].  Such service can be provided as:

-	Supplementary regular IMS voice service provided by the terrestrial operators, especially in areas without terrestrial coverage.

-	Main regular IMS voice service provided by the satellite operators.

-	An IMS voice call service using GEO satellite access in case of emergency situations.

The UEs under consideration is typically commercial smartphones as SA1 defined in Table 7.4.2-1 in TS 22.261 [3].

Editor’s Note: whether and how to include IoT devices (such as wearables, automobile) is FFS.

In the following, different scenarios establishing an end-to-end voice service are considered.

#### 4.2.2.2 	Main Scenario: One UE connects via GEO-satellite access only

In a common scenario, one party in the conversation is assumed to be using a handheld mobile terminal over a GEO satellite network, while the other may be on a terrestrial mobile network (e.g., VoLTE, VoNR), a fixed-line connection, or another IMS-supported platform, as outlined in Figure 4.2.2.2-1, where a sketch of the bi-directional voice data flow for this main scenario is depicted.

Figure 4.2.2.2-1: Bi-directional voice data flow for main scenario

NOTE: 	Core network typically stands for 3GPP core network and IMS core network.

Editor’s Note: whether and how to include the interfaces descriptions are FFS.

In this scenario, UE1 is a phone supporting IMS voice call service over GEO satellite access. One may distinguish two cases depending on UE2:

- UE2 is a "regular" phone supporting IMS voice call service but not supporting ULBC nor GEO satellite access. In this case, UE2 may not be aware that UE1 is using a GEO satellite link during the communication and transcoding is performed in the core network.

- UE2 is an "upgraded" phone supporting IMS voice call service with ULBC, but using another access than satellite access (e.g., LTE, NR or WLAN). In this case UE2 may be able to communicate with UE1 using  ULBC in a transcoder-free operation.

#### 4.2.2.3 		Sub-Scenario: Both UEs connect via GEO-satellite access

In a less common scenario, both parties in the conversation are connected to a GEO satellite as outlined in Figure 4.2.2.3-1 using IMS-based communication services.

Figure 4.2.2.3-1: Bi-directional voice data flow for sub-scenario

NOTE: 	Core network typically stands for 3GPP core network and IMS core network.

Editor’s Note: whether and how to include the interfaces descriptions are FFS.

When the GEO satellite operates in a transparent payload, the voice packets are transmitted to the ground before transmitted to the other UE, even if both UEs are connected to the same GEO satellite. In this case, the communication between UE1 and UE2 may use ULBC in a transcoder-free operation.

### 4.2.3	Derived high-level prerequisites

#### 4.2.3.1	General

The following general prerequisites for the ULBC apply based on application scenario 1.

Editor’s note:	Some high-level prerequisites could be added such as:

"To serve application scenario 1, the ULBC codec is expected to meet the following high-level prerequisites:

- Very low bitrate support

- DTX support [to be confirmed]

- Error concealment

- Implementable in real-time (encoding and decoding) on at least a selective set of smartphones

- Good audio quality to ensure a reasonable QoE (Detailed QoE requirements for such services are for study).

Considerations on whether existing audio quality tests (TS 26.131 and TS 26.132) may be considered."

### 4.2.4	Extracted technical assumptions and open issues

#### 4.2.4.1	Assumptions for Main Scenario

The following assumptions apply for Main Scenario described in clause 4.2.2.2.

-	For the connection “UE1 – GEO satellite – Ground station” (UE1 uplink), the transmission data rate is significantly limited ([1-3] kbit/s), requiring an ultra-low bit rate codec fitting the transmission data rate for this link.

-	For the connection “Ground station – GEO satellite – UE1” (UE1 downlink), the transmission data rate is expected to be limited similarly to UE1 uplink.

-	For both uplink and downlink of UE1 it is expected that the link is subject to transmission errors reflecting GEO satellite access

Editor’s Note: "expected" should be replaced by more technical evidence when available (e.g., after coordination with RAN groups).

-	The delay in uplink and downlink of UE1 is expected to be greater than the one of typical terrestrial networks.

Editor’s Note: "expected" should be replaced by more technical evidence when available (e.g., after coordination with RAN groups)

-	For the connection "Core Network – UE2" (UE2 downlink), the transmission data rate of a regular TN network is available. This link could be covered either by an existing IMS codec (transcoding necessary) or by the same ultra-low bit rate codec as used for the satellite link (transcoding-free).

-	To ensure seamless communication across different network types, roaming, etc. transcoding functionality in core network is likely needed.

Editor’s Note: More details may be added.

#### 4.2.4.2	Assumptions for Sub-Scenario

The following assumptions apply for Sub-Scenario described in clause 4.2.2.3.

-	For both connections "UE1 – GEO satellite – Ground station" and "Ground station – GEO satellite – UE2" the transmission data rate is significantly limited ([1-3] kbit/s), requiring an ultra-low bit rate codec fitting this transmission data rate for these links.

-	This scenario may allow both transcoded (ULBC  existing IMS speech codecs ULBC) and transcoding-free operation (ULBC end-to-end)

Editor’s Note: More details may be added.

* * * End of Changes * * * *

[1] Tdoc SP-250378, "Study on Ultra Low Bitrate Speech Codec"

[2] TR 22.887, "Study on satellite access - Phase 4"

[3] TS 22.261, "Service requirements for the 5G system"

[4] GMR-2 03.050, "Transmission planning for speech service"

[5] Inmarsat Phone https://www.youtube.com/watch?v=iaI-rVMB6wk

[6] vivo Phone https://youtube.com/watch?v=EskEtJt9aQM&feature=shared

[7] Huawei Phone https://www.youtube.com/watch?v=6xDgaMajWxo

[8a] TR 23.401, "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access"

[8b] TR 23.501, "System architecture for the 5G System (5GS) "

[8] TR 38.821, "Solutions for NR to support non-terrestrial networks (NTN)"

[9] 3GPP TR 26.940, "Study on Ultra Low Bitrate Speech Codecs"