3GPP TSG-SA WG4 Meeting #132	S4-250907

Fukuoka, Japan, May 19th – 23th, 2025

Title:	Discussion and proposed way forward on MBS over NTN

Release:	Rel-19

Work Item:	TEI19, NR_NTN_Ph3-Core

Source:	SA4

# 1. Overall Description:

During SA4#131-bis-e, 3GPP SA4 received a straightforward LS from RAN2 in S4-250491

RAN2 is working on a solution to provide UEs with geographical information of where within an NTN cell an MBS broadcast service is applicable. This area is called the “intended service area” (ISA) whose geographical definition is provided to the UE via SIB and is associated with an identifier (ISA ID). The attached CR includes the changes introduced by SA2 to adopt the ISA for NTN access.

RAN2 would like to ask whether including the ISA in the Service Announcement would impact in SA4 specifications and would kindly request SA4 to confirm this can be supported within the Rel-19 timeframe.

RAN2 kindly request SA4 to confirm whether the inclusion of the ISA in the Service Announcement can be supported within the Rel-19 timeframe.

# During the SA4-e (AH) MBS SWG post 131-bis-e conference call on 7th of May, two documents have been discussed

- S4aI250077: proposed “Reply on Inclusion of NTN intended service area in the Service Announcement”

- S4aI250080: On NTN intended service area in the Service Announcement.

# There was no agreement to send a Reply LS. Instead, an offline call was proposed and setup with LS contact to clarify the issues. The call took place on Monday, 12th May, 1400h to 1500h.

# 2.	Summary of the NR NTN

This section gives a brief summary of NTN, based on slides by Ignacio Pascual Pelayo (Ericsson, LS contact), discussed during the offline call.

Figure 1: Intended Service Area (ISA) for MBS NTN broadcast services

Figure 2: Today’s assumption ISA in new SIB

Figure 3: Possibility of ISA in service announcement (LS question)

# 3.	On Mapped vs Uu Cell Ids

Format-wise, Mapped Cell Ids are identical to Uu Cell Ids. However, there is a semantic difference between “mapped Cell Ids” and “Uu Cell Ids”.

- A “Mapped Cell Id” is mapped to a geographical area.

- A “Uu Cell Id” is mapped to a satellite and the satellite it not stationary.

- The UE “sees” Uu Cell Ids (i.e. satellites) and is triggering a cell change towards gNB, when the satellite moves on. When the UE is stationary, the UE is still doing frequent cell changes.

- The UE will get Mapped Cell Ids in Service Announcement, while it “sees” Uu Cell Ids at the radio interface.

Figure 4:

# 4.	Summary of findings and discussions

# SA2 has inserted the “MBS broadcast support for NTN” some time ago (see S2-2501328). The NTN insertion seems transparent to SA4 at the first glance, but there are some issues.

- SA2’s stage 2 is only on MB-SMF and MB-UPF. Stage 2 of MBSF and MBSTF is in SA4. CT3 is defining Stage 3 for the APIs exposed by MB-SMF, MBSF and MBSTF.

  - Nmbsmf (Nmb13 and N29mb) is extended with NR NTN support. The NEF / AF may provide the Intended Service Area as additional parameter within the mbsServiceArea when invoking MBS Session Create at Nmbsmf. This Intended Service Area is encoded as a byte sequence and is intended to be consumed by NG-RAN.

    - An external AF (AF outside of the trust domain) should use the geographical area or a civic address as service area description, which is then translated to the CellIds / TAI list and Intended Service Area by the NEF.

  - CT3 is also handling stage 3 for Nmbsmf. Unfortunately, the MbsServiceArea data types (defined in TS 29.571) is also used at Nmbsf (MBSF MBS User Data Ingest Session API, TS 29.581) and also in Service Announcement (TS 26.517).

    - MBS User Data Ingest Session API: The stage 2 for the API is in TS 26.502, stage 3 is in TS 29.581. TS 26.502 does not mention the usage of the Intended Service Area as API parameters.

    - Service Announcement (TS 26.571): The MbsServiceArea data type is also used as common data type in SA4’s Service Announcement. Neiter TS 26.502 nor TS 26.517 mentions the usage of the Intended Service Area as API parameters. The encoding of the Intended Service Area parameter is suitable for NG-RAN, but not suitable for UEs (ASN.1 encoded bitsequence is base64 encoded and embedded as a byte sequence in JSON...).

  - CT3 is also exposing the Nmbsf API via the NEF to external AFs (AT outside of the trust domain). Here, CT3 is using an ExternalMbsServiceArea data type, so that neither Cell Ids nor TAIs are expected by an external AF. Instead, the MBS Service Area is described either as geographical area or as civic addresses and the NEF looks up the matching CellIds or TAIs. TS 26.502 does not describe the mapping by the NEF.

- SA4’s Stage 2 on Service Announcement is no aligned with Stage 3. The Mbs Service Area propert is present in TS 26.517, but not mentioned in TS 26.502.

- Selection of NR NTN as distribution. Selection of NR NTN for Mbs data distribution is considered a local configuration at the NEF. External AFs cannot provide preferences at the API.

# 5. Proposed way forward

On Stage 2 Alignment

  - Agree CRs to add the Mbs Service Area (or better a “target service area”) into TS 26.502, Rel 17 (Cat F), Rel 18 (Cat A), Rel 19 (Cat A)

  - A proposal in S4-250909, S4-250910, S4-250911

On external exposure of “MBS User Data Ingest Session” API, exposed by MBSF

  - Agree CRs (to Rel 17, 18 & 19) to clarify, that external AFs may define the target service area in form of a geographical area or a civic address and the NEF will lookup matching Cell Ids or TAIs from local configuration.

  - A proposal in S4-250909, S4-250910, S4-250911

On the presence of the Intended Service Area within the “MBS User Data Ingest Session” API, exposed by MBSF

  - Agree CR  to Rel 19 to

    - Either: clarify in TS 26.502, that the Intended Service Area parameter may be set by a trusted AF,

    - Or: clarify in TS 26.502, that the Intended Service Area parameter should not be used, and notify CT3 for according Stage 3 clarification.

  - No proposal available

On “Re-used Mbs Service Area Data Type”

  - TS 26.517: Agree a CR to define a separate Target Service Area data type, which is suitable for Service Announcement. The CR can be a correction for Rel 19, since the complexity around Intended Service Areas is not present for earlier releases

  - Proposal in S4-250908

On “ Reply on Inclusion of NTN intended service area in the Service Announcement”

  - Today’s MBS User Service Announcement does not support the description of the mbs service area as a geographical area. Only cell Ids or TAIs are supported by TS 26.502 (Stage 2).

  - It seems straight forward to add a target service area description e.g. in form of a geographical area to service announcement within Rel 19 and add the needed stage 2 definition. Still, inclusion of ISA-Ids seems equally feasible. Additional discussions is needed.

  - It is proposed to inform RAN2 on the current status (i.e. first bullet) and the current agreements (bullet 2)