3GPP TSG-SA4 Meeting #132	S4-250916

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025	revision of S4-250652

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.253 | CR | 0011 | rev | 2 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RTP Header Extensions for IVAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fraunhofer IIS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IVAS_Codec_Ph2 |  |  |  |  |  | Date: |  |  | 2025-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The IVAS RTP payload format has not defined the transport of “Processing Information (PI)” using standard mechanisms such as RTP header extensions (HDREXT). Depending on the application it may be preferable to have those outside the RTP payload. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In addition to transmission of Processing Information (PI) as part of the IVAS RTP payload, all PI frames can be transmitted as RTP header extensions (HDREXT) as described in the CR. The mechanism is based on the general mechanism for RTP header extensions specified in RFC8285. Also additional proposals for a more efficient orientation and position encoding are added, triggered by the observation of large data rates that would require (less-efficient) two-byte header extensions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Only in-payload PI data is allowed. In-payload PI data may have issues like lack of access when SRTP encryption is in use. Inefficient coding of orientation and position. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.5.6.4.5, A.3.5.6.4.6, A.3.5.6.4.7, A.4.1, A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 26.114, CR26253-0009 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR introduces the concept based on the PI data in CR26253-0009 and may not reflect the final set of PI data elements defined for IVAS. The concept is however extensible and should also work for most other potential PI data elements. The overhead of this mechanism and the in-band PI is for the common application of just a few smaller PI frames: 1 PI in-band: 3 bytes RTP: 5 bytes + 1-3 bytes for 32-bit padding 2 PI in-band: 5 bytes RTP: 6 bytes + 1-3 bytes for 32-bit padding 3 PI in-band: 7 bytes RTP: 7 bytes + 1-3 bytes for 32-bit padding 4 Pi in-band: 9 bytes RTP: 8 bytes + 1-3 bytes for 32-bit padding In case of a massive amount of PI frames the RTP header extensions would require the two-byte format. Given the then high meta data bit rate compared to IVAS frames such scenario is likely not the primary one. During the development of the header extensions also an optimization of orientation and position seemed beneficial. The now-proposed smallest-three quaternion component encoding follows industry best practices. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 0: Initial revision provided for information at SA4#131-bis-e Revision 1: Fixed field separators in structure example Revision 2: - Add 32-bit constrained ISM orientation and position variants - Add ivas-mode-switch / mono-init parameter with value -1 - Minor improvements for HDR_EXT |  |  |  |  |  |  |  |  |

CHANGE 1

##### A.3.5.6.4.5	ISM orientation

ISM_ORIENTATION PI data describes the orientation of the audio object(s) in the ISM(s), with respect to the scene orientation, using orientation data structures for a smallest-three quaternion encoding as defined in Figure A.3.5.6.4.5-1, where the present quaternion components are any three of W,X,Y,Z in that order.

0 1 2 3 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦OQI¦ Q_1_enc ¦ Q_2_enc ¦ Q_3_enc ¦ +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure A.3.5.6.4.5-1: ISM orientation data as quaternions.

The components are encoded by first identifying out of the four quaternion components the one with largest abolute value Qi. If Qi is negative, the entire quaternion q shall be negated, i.e. multiplied with -1 first (where q and -q represent the same rotation) to ensure Qi being positive. The other three quaternion components Q1, Q2,  Q3 out of W,X,Y,Z are then each encoded into a 10-bit unsigned integer by first shifting into [0;22], scaling by 1023×22 rounding to nearest integer and clamping to [0…1023], e.g. Q1,enc=Q1+12×1023 × 22. The value for the omitted quaternion component OQI is defined by Table A.3.5.6.4.5-1.

Table A.3.5.6.4.5-1: 2-bit codes for the omitted Quaternion component.

| OQI | Omitted Quaternion Component |
| --- | --- |
| 00 | W |
| 01 | X |
| 10 | Y |
| 11 | Z |

For decoding, Qi can be reconstructed based on the the unit length constraint from the other three components by Qi=1-Q12-Q22-Q32  , where Q1, Q2,  Q3 are dequantized from the 10-bit unsigned integer, e.g. Q1=Q1,enc1023×2-12 .

Following the IVAS coordinate system in 7.4.3.1, a quaternion of (w=0, x=1, y=0, z=0) represents the frontal direction. The positive x-axis points towards the frontal direction, the positive y-axis points towards the left direction and the positive z-axis points towards the up direction.

In the orientation data section, the full orientation representation for each audio object is positioned after one another. For example, the orientation data section begins with the quaternion components (w,x,y,z) for the first object, followed by the components for the second object when the number of ISMs N>1.

##### A.3.5.6.4.6	ISM position (Cartesian)

ISM_POSITION_CARTESIAN PI data indicates the position(s) of audio object(s) in the ISM(s) in cartesian corrdinates. Figure A.3.5.6.4.6-1 shows the position PI data structure as cartesian coordinates (X, Y, Z) for ISM_POSITION PI data frames. The X and Y components are 11 bits two’s-complement signed integer in 0.01m units, the Z component is a 10-bit two’s-complement signed integer inh the same 0.01m units. This gives a range of approximately [-10.24…10.23] meters in X, Y direction and [-5.12…5.11] for Z direction. The carthesian position coordinates follow the representation presented in clause 7.4 (Rendering control), where the x-axis points towards front, the y-axis points towards left and the z-axis points towards up.

0 1 2 3 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ X ¦ Y ¦ Z ¦ +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure A.3.5.6.4.6-1: ISM position PI data frame with cartesian coordinates.

The position PI data structures for the transported ISMs are positioned after one another. For example, the ISM position for the first object is followed by the ISM position for the second object when the number of ISMs N>1.

##### A.3.5.6.4.7	ISM position (Spherical)

ISM_POSITION_SPHERICAL PI data indicates the position(s) of audio object(s) in the ISM(s) in spherical coordinates. Figure A.3.5.6.4.7-1 shows the position PI data structure as spherical coordinates (X, Y, Z) for ISM_POSITION_SPHERICAL PI data frames. The azimuth component is a 12 bits signed integer in units of 0.088°, the elevation component a 10 bits signed integer in units of 0.176°, and the radius component is a 10-bit signed integer with the unit of 0.01 metres. The spherical position coordinates follow the representation presented in clause XX.

0 1 2 3 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ azimuth ¦ elevation ¦ radius ¦ +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure A.3.5.6.4.7-1: ISM position PI data frame with spherical coordinates.

The position PI data structures for the transported ISMs are positioned after one another. For example, the ISM position for the first object is followed by the ISM position for the second object when the number of ISMs N>1.

CHANGE 2

Editor’s Note: The following clause is from CR26253-0009rev1 and would need to be adapted in a revision based top reflect the latest text and full clause.

## A.4.1	IVAS Media Type Registration

[…]

mono-init:	This parameter defines the mode at the start or update of the session for the direction specified by the session directionality attribute or the suffix. Permissible values are “0” and “1” and if the parameter applies to sendonly or recvonly direction also “-1”. If mono-init is 0 or not present, IVAS Immersive mode is used and the coded format is defined by the cf parameter. If mono-init is 1 or -1, the coded format is the EVS-compatible mono mode of IVAS; depending on the setting of evs-mode-switch, EVS Primary or AMR-WB IO mode is used. If the mode is 0 or 1 the mode initially used in the session may later be modified; if the mode is -1 it can only be modified as the result of a session renegotiation.

mono-init-send/mono-init-recv: mono-init parameter in send or receive direction.

[..]

CHANGE 3

# A.5	RTP Header Extensions

## A.5.1	General

In addition to transmission of Processing Information (PI) as part of the IVAS RTP payload, all PI frames can be transmitted as RTP header extensions (HDREXT) as described in the following clauses. The mechanism is based on the general mechanism for RTP header extensions specified in [RFC8285]. The use of one-byte header format is prefered over two-byte header format. The two-byte header format is required when PI frames with a size larger than 16 bytes or when more than 14 header extension types are to be transported.

## A.5.2	Supported RTP Header Extensions for PI

Table A.5.2-1: List of supported RTP header extensions

| PI Frame Type | Clause | RTP HDR_EXT URN | Payload Size (bytes) | Payload Format | Total (incl. 1-byte header) |
| --- | --- | --- | --- | --- | --- |
| SCENE_ORIENTATION | A.3.5.6.1.2 | urn:3gpp:ivas:fsco | 8 | Quaternion: 4 × int16 (Q15: W, X, Y, Z) | 9 |
| DEVICE_ORIENTATION_COMPENSATED | A.3.5.6.1.3 | urn:3gpp:ivas:fdoc | 8 | Quaternion: 4 × int16 (Q15) | 9 |
| DEVICE_ORIENTATION_UNCOMPENSATED | A.3.5.6.1.3 | urn:3gpp:ivas:fdou | 8 | Quaternion: 4 × int16 (Q15) | 9 |
| ACOUSTIC_ENVIRONMENT | A.3.5.6.2 | urn:3gpp:ivas:face | 1, 5, or 8 | AE identifier (1 byte) or compact AE parameters (5 or 8 bytes) | (payload+1) |
| AUDIO_DESCRIPTION | A.3.5.6.3 | urn:3gpp:ivas:faud | 1 to 4 | Audio Identifier(s) (1 byte per identifier) | (payload+1) |
| ISM_NUM | A.3.5.6.4.2 | urn:3gpp:ivas:finm | 1 | ISM count: 2 bits for number (with 6 bits reserved) | 2 ( |
| ISM_ID | A.3.5.6.4.3 | urn:3gpp:ivas:fiid | N (1 byte per ISM) | List of ISM IDs (1 byte each) | 1+N |
| ISM_GAIN | A.3.5.6.4.6 | urn:3gpp:ivas:figa | N (1 byte per ISM) | Gain factors (1 byte per ISM) | 1+N |
| ISM_ORIENTATION | A.3.5.6.4.7 | urn:3gpp:ivas:fiso | 4×N | For each ISM: Quaternion (int32 using smallest-three) | 1+(4×N) |
| ISM_POSITION | A.3.5.6.4.8 | urn:3gpp:ivas:fipo | 4×N | For each ISM: Position vector (int12, int12, int10) | 1+(4×N) |
| ISM_DISTANCE_ATTENUATION | A.3.5.6.4.4 | urn:3gpp:ivas:fida | 3 or 3×N | Attenuation parameters (3 bytes globally or per ISM) | (payload+1) |
| ISM_DIRECTIVITY | A.3.5.6.4.9 | urn:3gpp:ivas:fidr | 2 or 2×N | Directivity parameters (2 bytes globally or per ISM) | (payload+1) |
| DIEGETIC_TYPE | A.3.5.6.5.1 | urn:3gpp:ivas:fdit | 1 | Diegetic type indicator (full byte) | 2 |
| PLAYBACK_DEVICE_ORIENTATION | A.3.5.7.1 | urn:3gpp:ivas:rpdo | 8 | Quaternion: 4 × int16 (Q15) | 9 |
| HEAD_ORIENTATION | A.3.5.7.2 | urn:3gpp:ivas:rhor | 8 | Quaternion: 4 × int16 (Q15) | 9 |
| LISTENER_POSITION | A.3.5.7.3 | urn:3gpp:ivas:rlip | 6 | Position vector: 3 × int16 (Q15) | 7 |
| DYNAMIC_AUDIO_SUPPRESSION | A.3.5.7.4 | urn:3gpp:ivas:rdas | 2 | Suppression parameter (2 bytes) | 3 |
| AUDIO_FOCUS_DIRECTION | A.3.5.7.5 | urn:3gpp:ivas:rafd | 8 | Quaternion: 4 × int16 (Q15) | 9 |
| PI_LATENCY | A.3.5.7.6 | urn:3gpp:ivas:rlat | 4 | 32-bit latency value | 5 |
| NO_PI_DATA | A.3.5.5 | urn:3gpp:ivas:nopi | 0 |  |  |

## A.5.3	SDP Negotiation

SDP negotiation for RTP header extensions shall use the mechanisms described in [RFC8285], i.e. using the extmap attribute. RTP header extensions for PI data may be sendonly, recvonly, or sendrecv; the SDP answer may add sendonly or recvonly in case an RTP header extension is offered as sendrecv.

## A.5.4	Examples

The following example illustrates the SDP part for negotiation of two RTP header extensions for head orientation and listener position as part of the SDP offer of an endpoint that supports sending those data:

a=extmap:3/sendonly urn:3gpp:ivas:rhor

a=extmap:4/sendonly urn:3gpp:ivas:rlip

The resulting complete header extension is illustrated in Figure A.5.4-1 where the generic RTP header extension signaling for one-byte header extensions is followed by the specific header extensions for listener orientation (ID=3) and position (ID=4). No additional padding is required in this example.

0                   1                   2                     3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|         Profile = 0xBEDE      |           Length = 0x0004     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID=3 | len=7 |       W_OR (16 bits)          | X_OR (16 bits)|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      ...      |       Y (16 bits)             | Z_OR (16 bits)|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      ...      |  ID=4 | len=5 |      X_POS (16 bits)          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      Y_POS (16 bits)          |      Z_POS (16 bits)          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure A.5.4-1: Structure of complete RTP header extension for listener orientation and position

END OF CHANGES