3GPP TSG-SA4 Meeting #132	S4-250928

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.253 | CR | 0015 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the IVAS algorithmic description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dolby Laboratories Inc., Ericsson LM, Fraunhofer IIS, Huawei Technologies Co Ltd., Nokia, NTT, Orange, Panasonic Holdings Corporation, Philips International B.V., Qualcomm Incorporated, VoiceAge Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IVAS_Codec |  |  |  |  |  | Date: |  |  | 2025-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some errors were identified in the algorithmic description of the stereo and SBA encoder operations and stereo decoder operations. These should be corrected to avoid confusion and potentially incorrect implementations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarifications and corrections for time-domain stereo encoder (downmix, sub-mode selection for near out-of-phase (NOOP) operation) - Corrections for bit values in PCA encoder usage and clarifications on adaptive PCA mode selection in SBA encoder operation - Removal of duplicated description for DFT-based stereo decoder - Corrections on description of Stereo CNG coherence decoding and Stereo PLC operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguities and errors will remain in the algorithmic description, which can lead to incorrect implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.3.4.1, 5.3.2.3.4.2, 5.3.2.3.4.3, 5.3.2.4.12.2.3, 5.4.6.1, 5.4.6.7, 6.3.2.2.3, 6.3.2.3.10.2, 6.3.5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE 1

###### 5.3.2.3.4.1	NOOP signal detection

The time-domain stereo downmix, described in section 5.3.2.3.3 may not work optimally when the signal in the left channel and the signal in the right channel are nearly opposite in phase. In such cases, the passive mono downmix, created by summing the left channel and the right channel, can lead to canceling of certain harmonic components, making it unsuitable for encoding. To address this the TD stereo coder includes a specialized sub-mode for Near-Out-Of-Phase (NOOP) input signals. The NOOP decision process is based on the parameters extracted during the analysis of the passive mono downmix, described in clause 5.3.2.3.3, the side signal defined in (5.3-59) and some other auxiliary parameters.

CHANGE 2

###### 5.3.2.3.4.2	NOOP sub-mode selection

Following the NOOP signal detection described in clause 5.3.2.3.4.1 the selection of the NOOP sub-mode within the TD stereo coder is made using a series of conditions based on the signal classification of the primary and the secondary channel and the output of the NOOP signal detection block depicted in Fig. 5.3-20. Let the output of the NOOP signal detection block, entitled as “NOOP decision” in Fig. 5.3-20, be represented with a binary flag fNOOP. Let the initial selection of the NOOP sub-mode be defined with the binary parameter NOOPinit and the final selection of the NOOP sub-mode with the binary parameter NOOPfinal. Note, that all binary parameters have either the value of 1 or the value of 0.

The initial selection of the NOOP sub-mode is made as follows

NOOPinit=0,                                                                        if fNOOP=0 AND NOOPfinal[-1]=01,                                                                       if fNOOP=1 AND NOOPfinal[-1]=1 0,    if fNOOP=0 AND NOOPfinal-1=1 AND (rmsL<∆1 AND rmsr<∆1)1,      if fNOOP=0 AND NOOPfinal-1=1 AND  (rmsL≥∆2 OR rmsr≥∆2)1,   if fNOOP=1 AND NOOPfinal[-1]=0 AND  (rmsL<∆1 AND rmsr<∆1)0,      if fNOOP=1 AND NOOPfinal-1=0 AND  (rmsL≥∆2 OR rmsr≥∆2)

NOOPinit=0if fNOOP=0 AND NOOPfinal[-1]=01if fNOOP=1 AND NOOPfinal-1=10if fNOOP=0 AND NOOPfinal[-1]=1 AND rmsL<∆1 AND rmsR<∆11if fNOOP=0 AND NOOPfinal[-1]=1 AND rmsL≥∆2 OR rmsR≥∆21if fNOOP=1 AND NOOPfinal[-1]=0 AND rmsL<∆1 AND rmsR<∆11if fNOOP=1 AND NOOPfinal[-1]=0 AND rmsL≥∆2 OR rmsR≥∆2

where [-1] indicates the value of the binary parameter in the previous frame, rmsL is the root-mean square energy of the left channel and rmsR is the root-mean square energy of the right channeld The value range of thresholds ∆1 is (0.5,1.0),for example, it can be set to 0.5, 0.85, 0.75, 0.65, or 0.81.The value range of thresholds ∆2 is [0.8,1.2], for example, it can be set to 0.8, 0.85, 0.9, 1, or 1.1.

The final selection of the NOOP sub-mode is performed based on the initial selection of the NOOP sub-mode, NOOPinit, and a modification flag, fmod_SM, related to the NOOP-specific mixing ratiofmod_SM,. The modification flag fmod_SM indicates whether the NOOP-specific mixing ratio βSM needs to be modified.  If the modification flag in the previous frame, fmod_SM[-1], indicates that the NOOP-specific mixing ratio, βSM, needs to be modified, then the final selection of the NOOP sub-mode is set to 0, i.e. NOOPfinal=0. If the modification flag of the previous frame, fmod_SM[-1], indicates that the mixing ratio does not need to be modified, then the final selection of the NOOP sub-mode is set based on a series of conditions involving the FEC classification of primary and secondary channel signals, as defined in section 5.1.13.3.1 in [3]. The following conditions are used in the final selection of the NOOP sub-mode:

Condition 1:	the FEC class of the primary channel  in the previous frame is UNVOICED_CLAS and the FEC class of the secondary channel in the previous frame  is VOICED_TRANSITION, or the FEC class of the secondary channel in the previous frame  is UNVOICED_CLAS and the FEC class of the secondary channel from two frames ago  is VOICED_TRANSITION;

Condition 2:	the coder type of both the primary and the secondary channels  in the previous frame is not set  to VOICED;

Condition 3:	the number  of consecutive NOOP frames up to and including the previous frame is greater than 5;

Condition 4:	the FEC class of the primary channel in the previous frame  is UNVOICED_CLAS, or the FEC classof the secondary channel in the previous frame  is UNVOICED_CLAS;

Condition 5:	long-term root mean square energy values of the left channel (rmsL ) and right channel (rmsR), defined in equation (5.3-60) are less than 400;

If Condition 1, Condition 2, and Condition 3 are all satisfied simultaneously, orNOOPfinal=1.  if Condition 2, Condition 3, Condition 4, and Condition 5 are all satisfied simultaneously, the final selection of the NOOP sub-mode is set to 1, i.e., NOOPfinal=1. If neither of these two scenarios occurs, one of the following decision procedures is performed to determine the final selection of the NOOP sub-mode (NOOPfinal):

- When NOOPinit=0 and NOOPfinal[-1]=1, then the final NOOP decision is set to NOOPfinal=1. Note, that the index [-1] refers to the value of the parameter from the previous frame.

- When NOOPinit=1 and NOOPfinal[-1]=0, and the NOOP-specific mixing ratio from the previous frame, βSM-1, is lower than a pre-defined threshold ∆1, then the final NOOP decision is set to NOOPfinal=10. The threshold ∆1 lies within the range (0.5, 1.0).

- When NOOPinit=1 and NOOPfinal[-1]=0 and the NOOP-specific mixing ratio in the previous frame, βSM[-1], is greater than or equal to ∆1, then the final NOOP decision is set to NOOPfinal=1.

- When the current frame is not processed using the NOOP sub-mode and NOOPinit=0, and NOOPfinal[-1]=1, then the final NOOP decision is set to NOOPfinal=0.

- When the current frame is processed using the NOOP sub-mode and NOOPinit=1 and NOOPfinal[-1]=0, and the NOOP-specific mixing ratio from the previous frame,  βSM[-1], is lower than a pre-defined threshold ∆2, then the final NOOP decision is set to NOOPfinal=0. The threshold ∆2 lies within the range (0.8, 1.2).

- When the current frame is processed using the NOOP sub-mode and NOOPinit=1 and NOOPfinal[-1]=0, and the NOOP-specific mixing ratio from the previous frame, βSM[-1], is greater than or equal to ∆2, then then the final NOOP decision is set to NOOPfinal=1.

CHANGE 3

###### 5.3.2.3.4.3	NOOP signal coding

When operating in the NOOP sub-mode, the TD stereo coder modifies the TD stereo downmix described in clause 5.3.2.3.3. In this mode, the left and right channels of the input stereo signal are combined using a NOOP-specific mixing factor, incorporating fade-in and fade-out smoothing techniques.

The primary and secondary channel signals in the current frame are calculated as follows:

if 0≤n<N-delaycom

YnXn=M12*XLnXRn

if N-delaycom≤n<N-delaycom+NOVA_1

YnXn=fade_outn*M12*XLnXRn+fade_inn*M22*XLnXRn

if N-delaycom+NOVA_1 ≤n<N

YnXn=M22*XLnXRn

fade_inn=n-(N-delaycom)NOVA_1

fade_outn=1-n-(N-delaycom)NOVA_1

where n=0,1,…,N-1 is the sample index, fade_inn indicates a fade-in factor and fade_outn indicates a fade-out factor. Furthermore, NOVA_1 indicates a transition processing length which can be set to 0. The signal XLn indicates the left channel signal in the current frame, XRn indicates the right channel signal in the current frame, Y(n) indicates the primary channel signal in the current frame that is obtained through the time-domain processing, and X(n) indicates the secondary channel signal that is in the current frame and that is obtained through the time-domain processing. The parameter delaycom indicates encoding delay compensation which can be set to 0. The matrix M22 is the downmixing matrix corresponding to the mixing ratio for the NOOP signal in the current frame and M12 is the downmixing matrix corresponding to the mixing ratio for the NOOP signal in the previous frame. The mixing ratio for the NOOP signal in the current frame is calculated based on the difference of two correlation measures between the left and right channel and the passive mono downmix. The downmixing matrices are defined as follows

M12=α1_pre-α2_pre-α2_pre-α1_pre, M22=α1-α2-α2-α1

where α1_pre=βSMn-1  and α2_pre=1-βSMn-1. The factor βSMn-1 is the mixing ratio corresponding to the NOOP signal in the previous frame. Furthermore, α1=βSMn and α2=1-βSMn and βSMn is the mixing ratio corresponding to the NOOP signal in the current frame. The calculation of the mixing ratio corresponding top the NOOP signal is described in the next clause.

Due to differences in channel combination schemes between the NOOP sub-mode and the normal/LRTD sub-modes within the TD stereo coder, a segmented time-domain downmix process is applied during transitions to and from the NOOP sub-mode. When switching from the normal/LRTD sub-mode to the NOOP sub-mode, the signals in the primary and secondary channels are divided into three consecutive segments: the start segment, the middle segment, and the end segment.

In the start segment, the signals in both channels are processed using the time-domain downmix based on the mixing ratio of the normal/LRTD sub-mode from the previous frame. Similarly, in the end segment, the signals are processed using the NOOP-specific mixing ratio determined in the current frame. In the middle segment, the signals in both channels are processed using the time-domain downmix based on the mixing ratio of the normal/LRTD sub-mode from the previous frame to obtain a first middle part, and the the signals are processed using the NOOP-specific mixing ratio determined in the current frame to obtain a second middle part, then a weighted sum is applied between the first and the second middle parts.

CHANGE 4

5.3.2.4.12.2.3	Calculation of downmixed signal and residual signal during transitional frame

In stereo coding, obtaining an initial downmixed signal and an initial residual signal of a sub-band corresponding to a preset frequency band in a current frame. If the previous/current frame is a switching frame, the downmixed signal and the residual signal of the sub-band corresponding to the preset frequency band in the current frame are calculated based on a switch fade-in/fade-out factor of a current/previous frame, the initial downmixed signal, and the initial residual signal. The switch fade-in/fade-out factor of the current/previous frame is determined based on a residual signal coding parameter and an inter-frame energy fluctuation parameter. The residual signal coding parameter is used to represent an energy relationship between a downmixed signal and a residual signal of the current/previous frame, and the inter-frame energy fluctuation parameter is used to represent an energy or amplitude relationship between the current/previous frame and a frame previous to the current/previous frame. The switch fade-in/fade-out factor of the current/previous frame is determined according to

when frame_nrg_ratio>3.2 and res_dmx_ratio<0.1, switch_fade_factor=0.75; when frame_nrg_ratio<0.21 and res_dmx_ratio>0.4, switch_fade_factor=0.25;  in another case, switch_fade_factor=0.5; wherein

frame_nrg_ratio represents the inter-frame energy fluctuation parameter of the current /previous frame which is defined as a ratio of total energy of the downmixed signal and the residual signal of the current/previous frame to total energy of a downmixed signal of a frame previous to the current/previous frame and a residual signal of the frame previous to the current/previous frame; res_dmx_ratio represents the residual signal coding parameter of the current/previous frame; switch_fade_factor represents the switch fade-in/fade-out factor of the current/previous frame.

The  downmixed signal and the  residual signal are calculated according to

DMXi,b(k)=DMXi,b(k)+(1-switch_fade_factor)*DMX_compi,b(k)

RESi,b(k)=switch_fade_factor*RESi,b'(k)

wherein DMXi,b(k) represents downmixed signal of a sub-band b in a subframe i in the current frame; DMXi,b(k) represents an initial downmixed signal of the sub-band b in the subframe i in the current frame; DMX_compi,b(k) represents a compensated downmixed signal of the sub-band b in the subframe i in the current frame; RESi,b'(k) represents an initial residual signal of the sub-band b in the subframe i in the current frame; RESi,b(k) represents residual signal of the sub-band b in the subframe i in the current frame; the sub-band b in the subframe i in the current frame is a sub-band in the at least one sub-band corresponding to the preset frequency band; k represents a frequency bin index of the sub-band b in the subframe i in the current frame; and 0≤i≤P-1, wherein P represents a quantity of subframes comprised in the current frame.

when residual coding flag is unequal to residual coding flag value of previous frame, and a modification flag of the residual coding flag of the previous frame is 0 which indicates that the residual coding flag value of the previous frame has not been modified, the residual coding switching flag set to 1 which indicates the frame is a switching frame and the residual coding flag set to 1 indicates the residual signal should be encoded.

CHANGE 5

#### 5.4.6.1	Overview

PCA operates on the generated downmixed signals, siDMXn, 1≤i≤Ndmx; PCA is restricted to the case of FOA input signal and the bit rate of 256 kbit/s where Ndmx=4. The use of PCA is defined by a configuration parameter (Opt_PCA_ON) which may be either 1 (PCA on) or 0 (PCA off).

If PCA is configured to be disabled (i.e., when Opt_PCA_ON=0), 1 bit with value 1 corresponding to the “PCA by-pass indicator” is written in the SBA metadata and and the PCA processing described in the following subclauses 5.4.6.2 to 5.4.6.10 is skipped; in this case the output signals of the PCA processing, siDMXn, 1≤i≤Ndmx, are identical to input signals, i.e. the four channels are left unchanged. The value 1 of the PCA by-pass indicator means that PCA operates in inactive mode (denoted by PCA_MODE_INACTIVE).

CHANGE 6

#### 5.4.6.7	Adaptive PCA mode decision

The decision to activate PCA in the current frame is made based on the following criteria: amount of decorrelation (dist_alt), interframe stability of unit quaternions (min_dot, min_dot2).

Given Λ=diag(λ0,…,λ3), eigenvalues sorted in descending order. The amount of decorrelation is derived from the sorted and normalized eigenvalues λi:

dist_alt=i=03log1λ0+…+λ3Rsmi,i+δ-log1λ0+…+λ3λi++δ

(5.4-108)

with δ=10-8.

The interframe stability of unit quaternions is defined as:

min_dot=q1⋅q1prev

(5.4-109)

min_dot2=q2⋅q2prev

(5.4-110)

where qiprev is the unit quaternion qi in the previous frame and  ⋅ is the vector dot product.

By default, the PCA by-pass indicator is set to active mode (denoted by PCA_MODE_ACTIVE). This decision is changed to inactive mode (denoted by PCA_MODE_INACTIVE) when one of the following conditions is met:

dist_alt < IVAS_PCA_THRES_DIST_ALT

min_dot < IVAS_PCA_THRES_MIN_DOT

min_dot2 < IVAS_PCA_THRES_MIN_DOT2

If the PCA by-pass indicator is set to PCA_MODE_INACTIVE, the matrix V is reset to an identity matrix of dimension 4, the unit quaternions are set to qiprev=(1,0,0,0) and one bit with value 1 corresponding to the “PCA by-pass indicator” is written in the SBA metadata. Otherwise, if the PCA by-pass indicator is set to PCA_MODE_ACTIVE, one bit with value 0 corresponding to the “PCA by-pass indicator” is written in the SBA metadata.

If the PCA by-indicator was set to PCA_MODE_INACTIVE in the previous frame and the PCA by-pass indicator is set to PCA_MODE_INACTIVE in the current frame and , the output signals of the PCA processing, siDMXn, 1≤i≤Ndmx, are identical to input signals, i.e. the four channels are left unchanged.

If the PCA by-indicator was set to PCA_MODE_ACTIVE in the previous frame and the PCA by-indicator is set to PCA_MODE_INACTIVE in the current frame, double quaternion interpolation and PCA matrixing described in clause 5.4.6.9 are applied and the PCA processing in subclause 5.4.6.8 is skipped..

Otherwise, if the PCA by-indicator was set to PCA_MODE_ACTIVE in the previous frame and PCA by-pass indicator is set to PCA_MODE_ACTIVE, the PCA processing described in subclauses 5.4.6.8 to 5.4.6.10 is applied.

CHANGE 7

##### 6.3.2.2.3	Reusing LP filter coefficients of the primary channel and dequantization of LSF parameters for the secondary channel

When the coherence between the coefficients of the LP filter of the primary channel and the coefficients of the LP filter of the secondary channel is high enough, as described in clause 5.3.3.3.4.3, the LP filter coefficients of the primary channel may be reused inside the decoder of the secondary channel. In such case one of the following scenarios occurs:

-	the quantized LSP vector of the primary channel from the previous frame is used to calculate the LP filter coefficients of the secondary channel in the current frame

-	the quantized LSP vector of the primary channel from the previous frame is resampled from the internal sampling frequency of 16 kHz down to 12.8 kHz and then used to calculate the LP filter coefficients of the secondary channel in the current frame

-	in case the coder type of the secondary channel is INACTIVE the LP filter coefficients of the primary channel in the current frame are re-used in the secondary channel

-	same as in the previous case and when the coherence between the LP coefficients of the primary and the secondary channel is low, then a pre-defined default (average) LSP vector is used to calculate the LP filter coefficients of the secondary channel in the current frame

In scenarios involving LSP vectors the conversion of LSP coefficients to LP filter coefficients is done using the procedure described in clause 5.1.9.7 of [3]. The interpolation of LSP coefficients is performed as described in clause 5.3.3.3.4.5 of [3].

In TD stereo decoding the quantized LSF parameters of the primary channel signal in the current frame are obtained from the bitstream. The IVAS decoder obtaining the quantized LSF parameters includes performing spectrum broadening on the quantized LSF parameters of the primary channel signal; obtaining the quantized prediction residual of LSF parameters of a secondary channel signal from the bitstream; determining the quantized LSF parameters of the secondary channel signal by using a sum of the quantized prediction residual of the LSF parameters of the secondary channel signal and the prediction for the secondary channel LSF parameters. Wherein, the prediction for the secondary channel LSF parameters is obtained by performing two-stage prediction on the spectrum-broadened LSF parameters of the primary channel signal. The spectrum-broadened LSF parameters of the primary channel signal are obtained by performing pull-to-average processing on the quantized LSF parameters of the primary channel signal, wherein the pull-to-average processing is performed according to the following formula:

LSFSB(i)=β⋅LSFP(i)+(1-β)⋅LSFS(i)

wherein LSFSB represents a vector of the spectrum-broadened LSF parameters of the primary channel signal, LSFP(i) represents a vector of the quantized LSF parameters of the primary channel signal, i represents a vector index, β represents a broadening factor, 0 < β < 1, for example β = 0.91, LSFS represents a mean vector of an original LSF parameter of the secondary channel signal, 1 ≤ i ≤ M , i is an integer, and M represents a linear prediction parameter. The second stage prediction is the tri-diagonal matrix prediction as in clause 5.3.2.3.9.

CHANGE 8

6.3.2.3.10.2	Side gain recovery after frame loss

During error-free decoding of active frames, the side gain parameter gSb for band b is decoded as described in 6.3.2.3.3. To improve the error recovery after a frame loss, a prediction memory corruption flag gS,CORRUPT is maintained. It is initialized to FALSE, and upon reception of a bad frame it is set to TRUE. Once a non-predictive decoding of the side gain parameter gSb (see g in (6.3-15) and (6.3-16) in clause 6.3.2.3.3) is received, the flag is restored to FALSE. If  gS,CORRUPT=FALSE during error free decoding, a representation of the location of the source can be found by low-pass filtering the mean of gSb according to

gS,LP=0.425gS+0.575gS,LP[-1],   gS≥0.6 gS,LP=0,                                          gS<0.6gS=1Nbandsb=0Nbands-1gSb	(6.3-103)

where Nbands is the number of DFT stereo bands currently in use for gS[b]. The side gain parameter gS[b] largely controls the level difference, and average side gain can be interpreted as a location of the source. If  gS,CORRUPT=TRUE, the value from the previous frame is used.

gS,LP=gS,LP[-1]	(6.3-104)

When a bad frame is indicated, the side gain parameter from the previous frame is used gSb=gS-1b. If a good frame is received, and after decoding of gSb gS,LPit is found that the following two conditions are true

gS,CORRUPT=TRUEgS,LP>0.6 ∨ gS,LP<-0.6	(6.3-105)

where ∨ denotes inclusive OR, the decoded side gain of the previous frame is used, gSb:=gS-1b, replacing the decoded side gain parameters gSb. The condition gS,LP>0.6 indicates that the reconstructed sound source is stable and concentrated to either the left or the right channel in the recently decoded frames. This is a situation which is sensitive to error-propagation and reusing the previously decoded parameters improves the performance in the state of corrupted memory.

CHANGE 9

##### 6.3.5.1.4	Stereo CNG coherence decoding

The intra-frame predictor index q is obtained from the bitstream and the intra-frame predictor p(q)b,i is selected. Based on the available bit budget for the encoded stereo coherence BC for the current frame m, the weighting factor α is obtained according to Table 5.3-23, where the decoded bit now indicates whether to select αlow or αhigh. The coherence for each band b is obtained according to

Cbandm,b=αCintra(q)m,b+1-αCbandm-1,b+Cresm,b	(6.3-175)

where the coherence prediction residual Cresm,b is now obtained from the bitstream and decoded according to Table 5.3-24. The intra-frame prediction Cintra(q)m,b and the inter-frame prediction Cbandm-1,b are obtained in the same way as in 5.3.5.1.5, using the previously reconstructed coherence values. In case the reconstructed values fall outside of the valid range 0,1, the value is clamped to this range according to

Cbandm,b≔0,                     Cbandm,b<01,                     Cbandm,b>1Cbandm,b,otherwise

(6.3-176)

The coherence Cbandm,b is used together with the remaining decoded stereo parameters and the decoded CNG down-mix signal to produce a stereo CNG synthesis as described in clause 6.3.5.1.5.

END OF CHANGES