3GPP TSG-SA4 Meeting #132	 S4-250933

# Fukuoka, 19th May - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.253 | CR | 0009 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Further updates to Annex A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Panasonic Holdings Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IVAS_Codec |  |  |  |  |  | Date: |  |  | 2025-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Practical feature (context adaptation ) is missing for which Annex A does not specify the RTP handling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RTP handling is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of context adaptation may lead to suboptimal immersive quality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3, A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE 1

Annex A (normative):
RTP Payload Format and SDP Parameters

….

### A.3.5.5	Supported PI data types

Supported PI types are listed in tables A.3.5.5-1 and A.3.5.5-2 and described in the following subsections. Table A.3.5.5-1 lists PI types for forward direction signalling. Table A.3.5.5-2 lists additional PI types.

Table A.3.5.5-1 : Supported forward direction PI types in an IVAS session.

| Type bits | Forward direction PI type | Description | SDP indication | Size (bytes) | Described in clause |
| --- | --- | --- | --- | --- | --- |
| 00000 | SCENE_ORIENTATION | Describes the orientation of a spatial audio scene in unit quaternions. | fsco | 8 | A.3.5.6.1.2 |
| 00001 | DEVICE_ORIENTATION_COMPENSATED | Describes the orientation of a device in unit quaternions. The orientation is compensated in the transmitted audio. | fdoc | 8 | A.3.5.6.1.3 |
| 00010 | DEVICE_ORIENTATION_UNCOMPENSATED | Describes the orientation of a device in unit quaternions. The orientation is not compensated in the transmitted audio. | Fdou | 8 | A.3.5.6.1.3 |
| 00011 | ACOUSTIC_ENVIRONMENT | Selects and optionally describes the acoustic environment. | Face | 1,5 or 8 | A.3.5.6.2 |

Table A.3.5.5-1A: Supported reverse direction PI types in an IVAS session.

| Type bits | Reverse direction PI type | Description | SDP indication | Size (bytes) | Described in clause |
| --- | --- | --- | --- | --- | --- |
| 11011 | TERMINAL_STATUS_DESCRIPTION | Describes the change in Terminal Status | rtsd | 1 | A.3.5.7.1 |

Table A.3.5.5-2: Additional PI types in an IVAS session.

| Type bits | PI type | Description | SDP indication | Size (bytes) |
| --- | --- | --- | --- | --- |
|  | Reserved | - | - | - |
| 11111 | NO_PI_DATA | Indicates an empty PI data frame. | nopi | 0 |

NO_PI_DATA PI data type can be used to indicate empty PI data sections. The PM marker bits for a NO_PI_DATA PI data type shall be set as PM=10, see table A.3.5.2-2. For example, if an IVAS RTP payload includes multiple audio frames, and some of the audio frames do not have associated PI data, NO_PI_DATA PI type can be used.

CHANGE 2

### A.3.5.7	Reverse direction PI data types

#### A.3.5.7.1	Terminal Status Description

A receiving device may signal its terminal status to the sender by transmitting Terminal Status Description (TSD) PI data. Based on this information, the sender may respond with a bitstream adapted to the receiver's updated configuration. The TSD PI data conveys the receiver’s terminal status and the size of the TSD PI data is 1 byte, with TSD value accounting for 3 bits and remaining 5 bits are padded to ensure byte alignment. The structure of the TSD PI data frame is illustrated in Figure-A.3.5.7.1, and the TSD bit field definitions are defined in Table-A.3.5.7.1.

0 1 2 3 4 5 6 7
+-+-+-+-+-+-+-+-+
| TSD |0 0 0 0 0|
+-+-+-+-+-+-+-+-+

Figure-A.3.5.7.1: TSD PI data frame

Table-A.3.5.7.1 - TSD Bit Field Definitions

| TSD | Definition |
| --- | --- |
| 000 | Handheld |
| 001 | Handsfree/Handheld Hands-Free |
| 010 | External Headset |
| 011 | Conference (external speakers) |
| 100 to 111 | Reserved for future use |

END OF CHANGES