Title:		           Signaling Terminal Status

Source:	Panasonic Holdings Corporation

Document for:	Discussion & Agreement

Agenda item:	7.5

Introduction

Modern endpoints such as smartphones and communication devices support multiple audio interfaces and exhibit dynamic usage behaviours during voice calls. Users frequently change terminal configurations—switching between handheld, hands-free, or conference modes—and may connect external devices such as headphones or external loudspeakers during ongoing calls. These transitions impact both audio capture and playback pathways.

When combined with the capabilities of the 3GPP IVAS codec, these dynamic changes can significantly enhance the immersive end-to-end communication experience. However, current specifications—particularly TS 26.253 Annex A—do not fully account for real-time terminal status updates. This may lead to incomplete immersive experiences.

Discussion

- Evolution of Mobile Audio Hardware

Unlike traditional mobile phones, modern smartphones and communication devices are equipped with multiple loudspeakers and microphones. These hardware enhancements can support richer spatial audio and immersive communication experiences. However, widely deployed codecs in current communication systems predominantly support mono audio.

The recently standardized Immersive Voice and Audio Services (IVAS) codec supports a wide range of bitrates with support for various immersive coding modes.

Table 1: IVAS coding modes

| Coding Mode | Full Name |
| --- | --- |
| Stereo | Stereo |
| SBA | Scene-based Audio (Ambisonics) |
| MASA | Metadata-assisted Spatial Audio |
| ISM | Objects (Independent Streams with Metadata) |
| MC | Multi-Channel |
| OMASA | Combined Objects and MASA |
| OSBA | Combined Objects and SBA |
| SR | Split Rendering |

- Example Terminal Status Modes

With flexible usage of modern terminals or endpoints, these endpoints can operate in multiple audio configurations, resulting in different terminal status. Example terminal status include:

- Handheld Mode

- Hands-Free/ Handheld-Handsfree Mode

- External Headset

- Conference Mode, etc.,

Each terminal status may account for different audio playout capabilities and capture behaviors. Providing awareness of the receiver's terminal status to the sender can enable context-aware adaptation, allowing the sender to optimize codec selection and coding format—thereby improving or maintaining the overall audio quality of the communication experience.

- Impact of Terminal Status on Audio Behaviour

Example Microphone Capture Configurations:

Handheld Mode: Primary microphone close to the user's mouth, prioritizing mono audio.

Handheld/Hands-Free Mode: Activates multiple microphones for improved far-field voice capture and spatial scene reconstruction.

Example Receiver Configurations:

Handheld Mode: Mono output via the earpiece speaker.

Handheld/Hands-Free Mode: Audio rendered through multiple on device loudspeakers, possibly requiring different coding format/codec comparing to handheld mode

External Headset: Audio rendered through external headset, preferably requiring a binaural playback; ideal for immersive spatial audio.

- Example Use Case

Alice is at home, sitting comfortably on her living room couch. She calls her son Bob, curious to hear what Bob and her grandchildren are up to. Bob is at the racetrack, where his children are racing go-karts. Bob is standing in the middle of a hairpin corner with go-karts driving around him.

Bob answers the call using his smartphone in handheld mode. After exchanging greetings, he wants to share the excitement and ambient experience of the racetrack with Alice. Meanwhile, Alice connects her headphones to her device to better hear the environment and holds the device in a handheld-handsfree position for a more relaxed experience.

Step-by-Step Breakdown

Call Setup

- Alice and Bob are having a call

Terminal Status change at receiver (Alice)

- Alice connects her headphones and adjusts her terminal to a handheld-handsfree position.

- This change in terminal status is communicated from Alice’s device to Bob’s device.

Sender-side Adaptation (Bob)

- Upon receiving the updated terminal status, Bob’s device dynamically switches the audio capture configuration:

- He holds his smartphone in landscape mode, pointing toward the track to maximize spatial audio capture.

- His device updates the IVAS coding format based on Alice’s new terminal configuration and capabilities.

Receiver Experience (Alice)

- Alice now receives the updated immersive audio stream.

- Through her headphones, she can vividly hear the go-karts racing around her and Bob’s enthusiastic cheers, creating a sense of spatial presence and emotional engagement.

This use case illustrates how real-time dynamic terminal status signalling and sender-side adaptation can:

- Enhance the immersive experience,

- Leverage the full potential of the IVAS codec across asymmetric terminal configurations.

Such a context-aware adaptation mechanism enables natural and uninterrupted transitions in communication behaviour, improving Quality of Experience (QoE) for immersive voice communication.

- Context-Aware Codec/Coding format change

Dynamic terminal context at the receiver side—such as switching from handheld to hands-free or to a headset—can significantly influence the perceived benefit of immersive audio features. For example:

- In handheld mode, immersive spatial formats offer minimal perceptual gain due to single-ear listening or close-proximity mono rendering.

- In hands-free, headset, or conference modes, immersive formats (e.g., stereo, binaural, or object-based audio) can greatly enhance the user experience through wider spatial rendering.

To enable real-time optimization based on this context without requiring full session renegotiation, we propose the following mechanism:

Proposal

- New Reverse PI Type: TERMINAL_STATUS_DESCRIPTION

- Introduce a reverse-direction RTP PI type to convey terminal status description from the media receiver to the sender.

- This update is delivered in real time via RTP, with minimal signalling overhead.

Example: Terminal_Status_Description PI Frame Format

0 1 2 3 4 5 6 7
+-+-+-+-+-+-+-+-+
| TSD |0 0 0 0 0|
+-+-+-+-+-+-+-+-+

TERMINAL_STATUS_DESCRIPTION PI Data Frame

- TSD (Terminal Status Description) = 3-bit terminal status code

- Remaining 5 bits are padding for byte alignment

TSD Bit Field Definitions

| TSD | Definition |
| --- | --- |
| 000 | Handheld |
| 001 | Handsfree/Handheld Hands-Free |
| 010 | External Headset |
| 011 | Conference (external speakers) |
| 100 to 111 | Reserved for future use |

This signalling mechanism helps optimize immersive experiences while avoiding unnecessary session renegotiation.

- Interaction with Existing RTP elements

The sender, upon receiving the TERMINAL_STATUS_CHANGE PI, can:

- Use the terminal status to change an appropriate immersive coding mode, considering its own capability and bitrate constraints.

- Trigger codec mode adaptation using:

  - CMR (Codec Mode Request): The sender uses this to signal an updated coding mode back to the receiver.

  - E-byte or a similar mechanism in the payload header to reflect the switch to immersive coding (e.g., from mono to stereo or object-based).

This bi-directional coordination ensures that the sender-side codec dynamically aligns with receiver-side terminal context in real-time.

Conclusion

We propose integrating this terminal-status-based dynamic adaptation mechanism into Annex A of TS 26.253 [2]. It aligns with the IVAS goals of immersive, efficient, and user-centric communication and leverages RTP PI data for scalable, low-latency adaptation across diverse device and usage contexts.

References

- Tdoc S4-250191: On Enhancing Negotiations

- TR 26.253 (v18.4.0) Annex A: RTP Payload Format and SDP Parameters