Source:	Nokia

Title:	Enabling noise reduction and audio focus in IVAS sessions

Document for:	Discussion & Agreement

Agenda Item:	7.5 – IVAS

# 1. Introduction

The IVAS RTP payload format specifies an optional Processing Information (PI) data section that can be used for additional metadata to support rendering of IVAS streams. Recent developments consider extending PI data to sending requests and feedback during an IVAS session.

Extending PI data for requests relating to immersive audio capture for IVAS sessions is generally very well motivated. For example, certain requests relating to IVAS streams are already possible using the codec mode request mechanism. On the other hand, PI data can include optional metadata, where related requests would naturally also become part of PI data. The current input discusses some relevant use cases and provides implementation examples.

The source furthermore proposes efficient ways to enable the relevant signaling using the PI data section of IVAS RTP payload format. Two alternative proposals with different merits are presented for comparison.

# 2. Examples

IVAS immersive features and improvements over existing mono voice services have been publicly demonstrated starting at the 3GPP TSG Plenary meetings in Edinburgh (December 2023). Some prominent examples are the world’s first IVAS real-time immersive call over public 5G network in June 2024 and numerous real-time and playback demonstrations at 3GPP meetings and other industry events.

In the demos, real-time immersive capture, IVAS encoding, and IVAS rendering with head-tracking have been shown for various IVAS encoder formats. For example, transmission over practical networks has been implemented for live calls between two smartphone devices with software implementations of the IVAS codec.

Immersive audio capture can take place in various dynamic environments. Sometimes the backgrounds are of interest and only add to the richness of the experiments, other times they can be disturbing or have some disturbing elements. In practical experiments, it has been observed that it is often useful to allow some control of the presentation of the immersive audio experience for the listener. In practice, the control is often best achieved by remote control, e.g., as made possible by a remote-control noise reduction slider shown in Figure 1.

Figure 1. Example remote control for noise reduction used in IVAS demos

For immersive IVAS sessions, some particularly useful requests from the receiving UE to the capturing UE relate to processing of the captured immersive audio prior to IVAS encoder input. The main examples relate to noise reduction and audio focusing.

Noise reduction or suppression in immersive audio capture refers to various techniques to reduce or altogether remove unwanted sounds (e.g., background noise) while preserving the desired sounds (e.g., talker) as well as the spatial characteristics of the captured audio scene. A simple noise reduction request (e.g., as shown in Figure 1) can control the level of noise reduction or switch the feature on/off. A more advanced noise reduction request can target specific sound types, where, e.g., wanted sound types or contents (e.g., speech, music, ambience) are indicated.

Audio focus in immersive audio capture refers to various techniques to emphasize or completely isolate specific sound sources or directions, while maintaining the spatial characteristics of the captured audio scene. A simple audio focus request can control, e.g., the direction of the focus. It can also include level of the effect (e.g., similarly as shown for noise reduction level in Figure 1).  A more advanced audio focus request can target a specific sound source or sound source type (e.g., speech source) in the scene.

From the point of view of the user (listener), the two related sets of techniques are quite different. Noise reduction aims to clean up the sound by removing of unwanted sounds regardless of their spatial characteristics. Audio focus, on the other hand, aims to enhance the clarity of sound sources based on their direction. User’s preference or intuition to understand how to best control the capture may depend on the specific scene, environment, and use case. For immersive communications, both approaches should maintain the immersion of the audio experience.

# 3. Proposals

## 3.1 Overview

In the following, two alternative proposals are made to enable suitable control or request mechanisms for the techniques discussed above. The source considers it useful to harmonize some of the syntax and usage of similar types of PI data signaling. Commonalities can help to make it easier to  provide correct implementations and use them correctly.

A good example of this targeted harmonization, part of the IVAS RTP payload format, is the harmonized way of providing various orientation information (see highlights in table below):

Table A.3.5.5-1: Supported forward direction PI types in an IVAS session.

| Type bits | Forward direction PI type | Description | SDP indication | Size (bytes) | Described in clause |
| --- | --- | --- | --- | --- | --- |
| 00000 | SCENE_ORIENTATION | Describes the orientation of a spatial audio scene in unit quaternions. | fsco | 8 | A.3.5.6.1.2 |
| 00001 | DEVICE_ORIENTATION_COMPENSATED | Describes the orientation of a device in unit quaternions. The orientation is compensated in the transmitted audio. | fdoc | 8 | A.3.5.6.1.3 |
| 00010 | DEVICE_ORIENTATION_UNCOMPENSATED | Describes the orientation of a device in unit quaternions. The orientation is not compensated in the transmitted audio. | fdou | 8 | A.3.5.6.1.3 |
| 00011 | ACOUSTIC_ENVIRONMENT | Selects and optionally describes the acoustic environment. | face | 1,5 or 8 | A.3.5.6.2 |

The two proposals are here provided as changes against TS 26.253 Annex A in pseudo-CR form. They incorporate also some earlier proposals, e.g., provided in [1].

The first of the two proposals, Proposal A, adds:

- Audio Description forward direction PI type (as seen before, [1] + two new extensions)

- Dynamic Audio Suppression reverse PI type (as seen before, [1])

- Audio Focus Direction reverse PI type (as seen before, [1])

- Audio Focus Level reverse PI type (new PI type)

where harmonized descriptions are used for PI types that share similarities.

The second of the two proposals, Proposal B, adds:

- Audio Description forward direction PI type (as seen before, [1])

- Audio Properties forward direction PI type (new PI type)

- Dynamic Audio Suppression reverse PI type (as seen before, [1])

- Audio Focus Direction reverse PI type (as seen before, [1])

- Audio Focus Level reverse PI type (new PI type)

where harmonized descriptions are used for PI types that share similarities.

The difference between the two proposals is where additional audio descriptions (that are not relating to audio content type) are provided: as part of Audio Description PI type or separately as Audio Properties PI type. Therefore, the reverse PI type proposals are the same between the alternatives.

The advantage of Proposal A is that it is more compact. The advantage of Proposal B is that different types of audio descriptions, those relating to content types and those relating to features, are separated providing better clarity. The merits of the two alternatives are for further consideration by the experts.

## 3.2 Proposal A

CHANGE A-1

Table A.3.5.5-1: Supported forward direction PI types in an IVAS session.

| Type bits | Forward direction PI type | Description | SDP indication | Size (bytes) | Described in clause |
| --- | --- | --- | --- | --- | --- |
| 00000 | SCENE_ORIENTATION | Describes the orientation of a spatial audio scene in unit quaternions. | fsco | 8 | A.3.5.6.1.2 |
| 00001 | DEVICE_ORIENTATION_COMPENSATED | Describes the orientation of a device in unit quaternions. The orientation is compensated in the transmitted audio. | fdoc | 8 | A.3.5.6.1.3 |
| 00010 | DEVICE_ORIENTATION_UNCOMPENSATED | Describes the orientation of a device in unit quaternions. The orientation is not compensated in the transmitted audio. | fdou | 8 | A.3.5.6.1.3 |
| 00011 | ACOUSTIC_ENVIRONMENT | Selects and optionally describes the acoustic environment. | face | 1,5 or 8 | A.3.5.6.2 |
|  | AUDIO_DESCRIPTION | Describes the content in the transmitted audio | faud | 1 to 5 |  |

Table A.3.5.5-1A: Supported reverse direction PI types in an IVAS session.

| Type bits | Reverse direction PI type | Description | SDP indication | Size (bytes) | Described in clause |
| --- | --- | --- | --- | --- | --- |
|  | DYNAMIC_AUDIO_SUPPRESSION | Describes receiver’s preference with respect to audio suppression | rdas | 2 |  |
|  | AUDIO_FOCUS_DIRECTION | Describes a direction of interest for the listener in Quaternions. | rafd | 8 |  |
|  | AUDIO_FOCUS_LEVEL | Describes receiver’s preference with respect to audio focus level | rafl | 1 |  |

CHANGE A-2

A.3.5.6.3	Audio Description PI data (forward direction)

Audio Description (AD) PI data frames can be used to describe the audio content (e.g., speech/music/general audio) transmitted from sender to receiver.

The size of AD PI data varies from 1 to 5 bytes and depends on the IVAS format as described in Table A.3.5.6.3-1. Wherein for OSBA and OMASA formats the number of discrete coded objects are as per Table 5.8-1 and Table 5.9-1 respectively.

Table A.3.5.6.3-1: Audio Description PI data size

| IVAS format | Size (Bytes) |
| --- | --- |
| stereo | 1 |
| SBA | 1 |
| MASA | 1 |
| ISM | Number of Objects |
| MC | 2 (1 for center channel + 1 for all other channels) |
| OMASA | 1 + Number of Discrete coded Objects |
| OSBA | 1 + Number of Discrete coded Objects |

Each Byte in AD PI data payload is an audio identifier (AID) that is defined as follows.

AID: An 8-bit identifier, as described in figure A.3.5.6.3-1, to specify type of audio that is being transmitted. This identifier contains V, M, A, F and S fields, as defined in Table A.3.5.6.3-2, Table A.3.5.6.3-3, Table A.3.5.6.3-4, Table A.3.5.6.3-5 and Table A.3.5.6.3-6, respectively, where V, M, and A fields specify whether audio contains speech, music or ambiance, or a combination of these audio types. The F field specifies whether there is audio-focused audio content, and the S field specifies whether there is noise-suppressed audio content. Value of AID where all V, M, and A fields equal to 0 corresponds to an unspecified audio type which indicates that an audio description is not available for the related audio frames. The reserved bits in AID shall be set to 0 and be ignored by a receiver.

The latest received AD PI data is used until a new AD PI data is received.

0 1 2 3 4 5 6 7 +-+-+-+-+-+-+-+-+ ¦V¦M¦A¦F¦S¦ RES ¦ +-+-+-+-+-+-+-+-+

Figure A.3.5.6.3-1: Audio Identifier.

Table A.3.5.6.3-2: V field in Audio Identifier Byte

| code | value |
| --- | --- |
| 0 | Non-speech |
| 1 | speech |

Table A.3.5.6.3-3: M field in Audio Identifier Byte

| code | value |
| --- | --- |
| 0 | Non-Music |
| 1 | Music |

Table A.3.5.6.3-4: A field in Audio Identifier Byte

| code | value |
| --- | --- |
| 0 | Absence of background Ambiance |
| 1 | Presence of background Ambiance |

Table A.3.5.6.3-5: F field in Audio Identifier Byte

| code | value |
| --- | --- |
| 0 | No audio focus applied |
| 1 | Audio focus applied |

Table A.3.5.6.3-6: S field in Audio Identifier Byte

| code | value |
| --- | --- |
| 0 | No noise suppression applied |
| 1 | Noise suppression applied |

CHANGE A-3 (same as B-3)

### A.3.5.7	Reverse direction PI data types

#### A.3.5.7.x1	Dynamic Audio Suppression

The Dynamic Audio Suppression (DAS) PI data describes receiver’s preference with respect to the type of audio content (e.g., Speech only) that should be enhanced and the amount of suppression to be applied to the background noise, where the background noise is defined as the type of audio content that should be suppressed according to the receiver preference.

The size of DAS PI data is 2 bytes and is described in Figure A.3.5.7.x1-1. The DAS PI data payload contains an Audio Identifier (AID) byte and a Suppression Level Indicator (SLI) byte, as illustrated in Figure A.3.5.7.x1-1. The value of AID field shall be non-zero for the audio identifier bits (V, M, A) and the reserved bits in AID shall be set to 0, unless defined. Likewise, there are 4 useable bits of the SLI while the 4 other (reserved) bits shall be set to 0. The AID byte contains V, M and A field, as defined in Table A.3.5.6.3-2, Table A.3.5.6.3-3 and Table A.3.5.6.3-4 respectively, which specifies whether receiver’s preference is speech, music or ambiance or a combination of these audio types. The Suppression Level Indicator, as defined in Table A.3.5.7.x1-1, allows specifying a desired degree of suppression where audio signal components other than what is specified by the AID field are considered as undesired audio component. The SLI takes values from 0 to 15 wherein the expected amount of audio suppression is proportional (in approximate logarithmic domain) to the indicator value with 0 indicating no audio suppression and 15 indicating maximum audio suppression as offered by the sender side.

The latest received DAS PI data is used until a new DAS PI data is received.

0 1 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦V¦M¦A¦ RES ¦ SLI ¦ RES ¦ +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ + AID + SLI +

Figure A.3.5.7.x1-1: DAS PI data byte

Table A.3.5.7.x1-1: Suppression Level Indicator

| SLI Size bits | Value | Description |
| --- | --- | --- |
| 0000 | 0 | No suppression |
| 0001 | 1 | … |
| … | … | … |
| 1110 | 14 | … |
| 1111 | 15 | Maximum suppression offered by the sender side |

#### A.3.5.7.x2	Audio focus direction

AUDIO_FOCUS_DIRECTION PI frame indicates a direction of interest from the receiver (i.e., the listener) to the sender side to apply audio focus. The focus direction is transmitted similarly as the orientation PI data, see clause A.3.5.6.1.1. The focus direction is relative to the frontal direction (w=0, x=1, y=0, z=0).

The latest received AUDIO_FOCUS_DIRECTION PI data is used until a new AUDIO_FOCUS_DIRECTION PI data is received.

#### A.3.5.7.x3	Audio focus level

The Audio Focus Level PI data describes receiver’s preference with respect to the amount of audio focus to be applied at the capture side, where the audio focus is to be applied in the direction of interest as defined by capture side or by the AUDIO_FOCUS_DIRECTION PI frame, described in clause A.3.4.7.x2.

The size of Audio Focus Level PI data is 1 byte, and it is described in Figure A.3.5.7.x3-1. The Audio Focus Level PI data payload contains Focus Level (FLVL) byte, as illustrated in Figure A.3.5.7.x3-1. The Audio Focus Level indicator, as defined in Table A.3.5.7.x3-1, allows specifying a desired amount of audio focus in the direction of the audio focus. The FLVL takes values from 0 to 15 wherein the expected amount of audio focus is proportional to the indicator value with 0 indicating no audio focus and 15 indicating maximum audio focus as offered by the sender side.

0 1 2 3 4 5 6 7 +-+-+-+-+-+-+-+-+ ¦ FLVL ¦ RES ¦ +-+-+-+-+-+-+-+-+

Figure A.3.5.7.x3-1: Audio Focus Level PI data byte

Table A.3.5.7.x3-1: Audio Focus Level Indicator

| FLVL bits | Value | Description |
| --- | --- | --- |
| 0000 | 0 | No audio focus |
| 0001 | 1 | … |
| … | … | … |
| 1110 | 14 | … |
| 1111 | 15 | Maximum audio focus offered by the sender side |

## 3.3 Proposal B

CHANGE B-1

Table A.3.5.5-1: Supported forward direction PI types in an IVAS session.

| Type bits | Forward direction PI type | Description | SDP indication | Size (bytes) | Described in clause |
| --- | --- | --- | --- | --- | --- |
| 00000 | SCENE_ORIENTATION | Describes the orientation of a spatial audio scene in unit quaternions. | fsco | 8 | A.3.5.6.1.2 |
| 00001 | DEVICE_ORIENTATION_COMPENSATED | Describes the orientation of a device in unit quaternions. The orientation is compensated in the transmitted audio. | fdoc | 8 | A.3.5.6.1.3 |
| 00010 | DEVICE_ORIENTATION_UNCOMPENSATED | Describes the orientation of a device in unit quaternions. The orientation is not compensated in the transmitted audio. | fdou | 8 | A.3.5.6.1.3 |
| 00011 | ACOUSTIC_ENVIRONMENT | Selects and optionally describes the acoustic environment. | face | 1,5 or 8 | A.3.5.6.2 |
|  | AUDIO_DESCRIPTION | Describes the content in the transmitted audio | faud | 1 to 5 |  |
|  | AUDIO_PROPERTIES | Describes the properties of the transmitted audio | faup | 1 to 5 |  |

Table A.3.5.5-1A: Supported reverse direction PI types in an IVAS session.

| Type bits | Reverse direction PI type | Description | SDP indication | Size (bytes) | Described in clause |
| --- | --- | --- | --- | --- | --- |
|  | DYNAMIC_AUDIO_SUPPRESSION | Describes receiver’s preference with respect to audio suppression | rdas | 2 |  |
|  | AUDIO_FOCUS_DIRECTION | Describes a direction of interest for the listener in Quaternions. | rafd | 8 |  |
|  | AUDIO_FOCUS_LEVEL | Describes receiver’s preference with respect to audio focus level | rafl | 1 |  |

CHANGE B-2

A.3.5.6.3	Audio Description PI data (forward direction)

Audio Description (AD) PI data frames can be used to describe the audio content (e.g., speech/music/general audio) transmitted from sender to receiver.

The size of AD PI data varies from 1 to 5 bytes and depends on the IVAS format as described in Table A.3.5.6.3-1. Wherein for OSBA and OMASA formats the number of discrete coded objects are as per Table 5.8-1 and Table 5.9-1 respectively.

Table A.3.5.6.3-1: Audio Description PI data size

| IVAS format | Size (Bytes) |
| --- | --- |
| stereo | 1 |
| SBA | 1 |
| MASA | 1 |
| ISM | Number of Objects |
| MC | 2 (1 for center channel + 1 for all other channels) |
| OMASA | 1 + Number of Discrete coded Objects |
| OSBA | 1 + Number of Discrete coded Objects |

Each Byte in AD PI data payload is an audio identifier (AID) that is defined as follows.

AID: An 8-bit identifier, as described in Figure A.3.5.6.3-1, to specify type of audio content that is being transmitted. This identifier contains V, M, and A fields, as defined in Table A.3.5.6.3-2, Table A.3.5.6.3-3, and Table A.3.5.6.3-4, respectively, specifying whether audio contains speech, music or ambiance, or a combination of these audio types. Value of AID where all V, M, and A fields equal to 0 corresponds to an unspecified audio type which indicates that an audio description is not available for the related audio frames. The reserved bits in AID shall be set to 0 and be ignored by a receiver.

The latest received AD PI data is used until a new AD PI data is received.

0 1 2 3 4 5 6 7 +-+-+-+-+-+-+-+-+ ¦V¦M¦A¦ RES ¦ +-+-+-+-+-+-+-+-+

Figure A.3.5.6.3-1: Audio Identifier for Audio Description.

Table A.3.5.6.3-2: V field in Audio Identifier Byte

| code | value |
| --- | --- |
| 0 | Non-speech |
| 1 | speech |

Table A.3.5.6.3-3: M field in Audio Identifier Byte

| code | value |
| --- | --- |
| 0 | Non-Music |
| 1 | Music |

Table A.3.5.6.3-4: A field in Audio Identifier Byte

| code | value |
| --- | --- |
| 0 | Absence of background Ambiance |
| 1 | Presence of background Ambiance |

A.3.5.6.4	Audio Properties PI data (forward direction)

Audio Properties (AP) PI data frames can be used to describe the audio properties of the audio content (e.g., application of audio focus or noise suppression on the content prior to encoding) transmitted from sender to receiver.

The size of AP PI data varies from 1 to 5 bytes and depends on the IVAS format as described in Table A.3.5.6.4-1. Wherein for OSBA and OMASA formats the number of discrete coded objects are as per Table 5.8-1 and Table 5.9-1 respectively.

Table A.3.5.6.4-1: Audio Properties PI data size

| IVAS format | Size (Bytes) |
| --- | --- |
| stereo | 1 |
| SBA | 1 |
| MASA | 1 |
| ISM | Number of Objects |
| MC | 2 (1 for center channel + 1 for all other channels) |
| OMASA | 1 + Number of Discrete coded Objects |
| OSBA | 1 + Number of Discrete coded Objects |

Each Byte in AP PI data payload is an audio identifier (AID) that is defined as follows.

AID: An 8-bit identifier, as described in Figure A.3.5.6.4-1, to specify audio properties of the audio content that is being transmitted. This identifier contains F and S fields, as defined in Table A.3.5.6.4-2 and Table A.3.5.6.4-3, respectively, specifying whether the audio content has audio focus, noise suppression, or both available, applied, or recommended by the sender. The reserved bits in AID shall be set to 0 and be ignored by a receiver.

The latest received AP PI data is used until a new AP PI data is received.

0 1 2 3 4 5 6 7 +-+-+-+-+-+-+-+-+ ¦ F ¦ S ¦ RES ¦ +-+-+-+-+-+-+-+-+

Figure A.3.5.6.4-1: Audio Identifier for Audio Properties.

Table A.3.5.6.4-2: F field in Audio Identifier Byte

| code | value |
| --- | --- |
| 00 | No audio focus possible |
| 01 | Audio focus available but not applied |
| 10 | Audio focus applied |
| 11 | Audio focus recommended |

Table A.3.5.6.4-3: S field in Audio Identifier Byte

| code | value |
| --- | --- |
| 00 | No noise suppression possible |
| 01 | Noise suppression available but not applied |
| 10 | Noise suppression applied |
| 11 | Noise suppression recommended |

CHANGE B-3 (same as A-3)

### A.3.5.7	Reverse direction PI data types

#### A.3.5.7.x1	Dynamic Audio Suppression

The Dynamic Audio Suppression (DAS) PI data describes receiver’s preference with respect to the type of audio content (e.g., Speech only) that should be enhanced and the amount of suppression to be applied to the background noise, where the background noise is defined as the type of audio content that should be suppressed according to the receiver preference.

The size of DAS PI data is 2 bytes and is described in Figure A.3.5.7.x1-1. The DAS PI data payload contains an Audio Identifier (AID) byte and a Suppression Level Indicator (SLI) byte, as illustrated in Figure A.3.5.7.x1-1. The value of AID field shall be non-zero for the audio identifier bits (V, M, A) and the reserved bits in AID shall be set to 0, unless defined. Likewise, there are 4 useable bits of the SLI while the 4 other (reserved) bits shall be set to 0. The AID byte contains V, M and A field, as defined in Table A.3.5.6.3-2, Table A.3.5.6.3-3 and Table A.3.5.6.3-4 respectively, which specifies whether receiver’s preference is speech, music or ambiance or a combination of these audio types. The Suppression Level Indicator, as defined in Table A.3.5.7.x1-1, allows specifying a desired degree of suppression where audio signal components other than what is specified by the AID field are considered as undesired audio component. The SLI takes values from 0 to 15 wherein the expected amount of audio suppression is proportional (in approximate logarithmic domain) to the indicator value with 0 indicating no audio suppression and 15 indicating maximum audio suppression as offered by the sender side.

The latest received DAS PI data is used until a new DAS PI data is received.

0 1 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦V¦M¦A¦ RES ¦ SLI ¦ RES ¦ +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ + AID + SLI +

Figure A.3.5.7.x1-1: DAS PI data byte

Table A.3.5.7.x1-1: Suppression Level Indicator

| SLI Size bits | Value | Description |
| --- | --- | --- |
| 0000 | 0 | No suppression |
| 0001 | 1 | … |
| … | … | … |
| 1110 | 14 | … |
| 1111 | 15 | Maximum suppression offered by the sender side |

#### A.3.5.7.x2	Audio focus direction

AUDIO_FOCUS_DIRECTION PI frame indicates a direction of interest from the receiver (i.e., the listener) to the sender side to apply audio focus. The focus direction is transmitted similarly as the orientation PI data, see clause A.3.5.6.1.1. The focus direction is relative to the frontal direction (w=0, x=1, y=0, z=0).

The latest received AUDIO_FOCUS_DIRECTION PI data is used until a new AUDIO_FOCUS_DIRECTION PI data is received.

#### A.3.5.7.x3	Audio focus level

The Audio Focus Level PI data describes receiver’s preference with respect to the amount of audio focus to be applied at the capture side, where the audio focus is to be applied in the direction of interest as defined by capture side or by the AUDIO_FOCUS_DIRECTION PI frame, described in clause A.3.4.7.x2.

The size of Audio Focus Level PI data is 1 byte, and it is described in Figure A.3.5.7.x3-1. The Audio Focus Level PI data payload contains Focus Level (FLVL) byte, as illustrated in Figure A.3.5.7.x3-1. The Audio Focus Level indicator, as defined in Table A.3.5.7.x3-1, allows specifying a desired amount of audio focus in the direction of the audio focus. The FLVL takes values from 0 to 15 wherein the expected amount of audio focus is proportional to the indicator value with 0 indicating no audio focus and 15 indicating maximum audio focus as offered by the sender side.

0 1 2 3 4 5 6 7 +-+-+-+-+-+-+-+-+ ¦ FLVL ¦ RES ¦ +-+-+-+-+-+-+-+-+

Figure A.3.5.7.x3-1: Audio Focus Level PI data byte

Table A.3.5.7.x3-1: Audio Focus Level Indicator

| FLVL bits | Value | Description |
| --- | --- | --- |
| 0000 | 0 | No audio focus |
| 0001 | 1 | … |
| … | … | … |
| 1110 | 14 | … |
| 1111 | 15 | Maximum audio focus offered by the sender side |

# 4. Summary & Proposal

There are recent developments that consider extending IVAS PI data to sending requests and feedback during an IVAS session. Audio focus and noise reduction remote control are two examples of features that are currently experimented with for real-time immersive audio sessions using the IVAS encoder input formats and coding. Such features can be enabled by inclusion of certain requests (reverse direction PI data types) in the IVAS RTP payload format. On the other hand, these features can be enabled with further flexibility and power, when suitable forward direction PI data types are also supported.

The present input builds on top of concepts that have been discussed for a quite some time and brought to SA4 for example in [1]. These concepts are here extended. Two alternative proposals to implement the support in IVAS RTP payload format are presented.

The source proposes to discuss the two alternative proposals and agree on one of them.

The first proposal (proposal A) has the advantage of being simpler and more compact. The second proposal (proposal B) has the advantage of better separating two types of audio descriptions, making the RTP payload format easier to understand. This goal is generally supported by the approach to harmonize the descriptions and used syntax as much as possible for PI data types that share similarities, e.g., usage of orientation information, usage of level information, etc.

# References

[1]	S4-250203: “Further corrections to Annex A”, Dolby Laboratories Inc., Ericsson LM, Nokia, Orange