Source:	Nokia

Title:	On compensated microphone pressure frequency response measurement

Document for:	Discussion & Agreement

Agenda Item:	7.6 – DaCAS

# 1. Introduction

In the SA4 131-bis-e meeting initial text for raw and compensated microphone signal requirements was agreed for further offline editing, based on the [1]. In this contribution, further proposal is made on how to perform isotropic measurement of pressure frequency response of a device microphone so that the definition of compensated microphone requirements becomes easier and complex definition of potentially device specific measurement geometry can be avoided. The proposed method can be applied to wide range of devices without need to consider the device shape, e.g., by considering the shape of a folded device during the measurement.

# 2. Compensated microphone frequency response measurements

Compensated frequency response for integrated device microphones requires measurement of isotropic frequency response for each microphone integrated in the UE. This response can be valuable for device implementation when compensated frequency responses can be defined similarly for all microphones without dependency on microphone inlet position on the device surface or requesting similarity between electroacoustic integration of different microphones in the UE. Compensated microphone response characterization should support the use mixture of different transducer types, e.g., SNR, AOP, sensitivity.

Acoustic measurements are typically made in specialized acoustic facilities such as anechoic chambers. This characterization requires typically multiple measurements from different directions. These measurements include the response from device shape, microphone inlet positions as well as integrated microphone frequency response characteristics. Therefore, it becomes a challenge to define a universal measurement setup and geometry that can be repeated in all environments. As an alternative for this type of device measurement we propose a Method for Integrated Microphone Pressure Frequency Response Measurement Using a Probe Microphone (IMPro) [2].

The IMPro method uses a probe microphone to measure the sound pressure at the sound inlet of the integrated microphone. This information is combined with the microphone transducer output signal to calculate the pressure frequency response of the integrated microphone. The method has been validated to be robust for different acoustic environments so that integrated microphone compensation can be done in acoustic laboratories as well as in typical office environments. The proposed method has been validated for typical MEMS microphone integration and corresponding results are provided for Four-Microphone Prototypes Device A and Device B. The proposed method is advantageous for virtual prototyping by combining realistic compensated microphone responses with simulated device geometries. Proposed method is also very cost efficient for commercial deployment, since the measurements can be performed without access to anechoic chamber or other expensive acoustic facilities.

Recent academic publication [2] considered in this contribution is freely downloadable at AES electronic library via https://aes2.org/publications/elibrary-page/?id=22907.

# 3. IMPro measurement results for Device A and Device B

Following IMPro pressure frequency responses measurements have been measured in an anechoic chamber which is the preferred acoustic environment for IMPro method, but corresponding measurement results could be also obtained by repeating the measurements in normal office environment [2].

All acoustic measurement setups have supporting structures holding the device and measurement equipment that are not part of the UE. The proposed method alleviates many of these problems because acoustic pressure is measured very close to microphone sound inlet. However, some averaging is recommended to improve the accuracy of measured pressure frequency response for two reasons. Slight variations in measurement of source direction attenuates reflections from measurement setup and secondly averaging will suppress the effect of background noise if the measurement is done e.g. in typical office environment.

Figure 1 IMPro measurement setup for Device A.

## 3.1 Device A

In device A all microphones can be integrated with very similar mechanical construction, which makes it integrated microphone responses very similar to each other and relevant differences come from small microphone transduce sensitivity variations. The microphone resonances are at high frequencies due to simple microphone integration, which is favorable for wide band audio recordings. The following two figures describe the IMPro measurement results for Device A microphones.

The microphone positions are as follows [3]: Mic#1 – Mic Front Right, Mic#2 – Mic Rear Right, Mic#3 – Mic Front Left,  and Mic#4 – Mic Rear Left.

Figure 2 Device A impulse response measurement for 4 microphones.

Figure 3 Device A microphone frequency responses for microphone compensation.

## 3.2 Device B

The following two figures describe the IMPro measurement results for Device B microphones. Four microphones are integrated into 3D printed device mechanics where microphones Mic#1 Mic#2 and Mic#4 are integrated on curved ends of the device and microphone Mic#3 integrated on flat surface of the camera bump. Flat surface enables typically shorter microphone inlet port which can be seen as lower resonance at high frequencies. In commercial devices mechanical structures are more complex and include ingress protection filter which can typically lead to lower resonance frequency. Lower resonance frequency makes compensation more critical because frequency response distortions can become audible and even disturbing in spatial audio.

Figure 4 Device B impulse response measurement for four-microphones Mic #1 - Mic #4.

Figure 5 Device B microphone frequency responses for four-microphone compensation.

# 3.3 Measurement results

Measurement results for Device A and Devices B are available as impulse responses sampled at 48kHz and stored in Sofa format [4]. Sofa software is available for download for multiple platforms [5]. Sofa format is proposed as a suitable format representing impulse response data with specified measurement geometry. The same information can also be represented in PCM and WAV format without supporting sofa metadata stored in the sofa file header. Measurement results for Device A and Device B are stored in attached Sofa files DaCAS_Device_A_GeneralFIR(1.0).sofa and DaCAS_Device_B_GeneralFIR(1.0).sofa, respectively.

DaCAS database templates would require additional definitions for IMPro measurements. Proposed template changes for IMPro support are provided in the following templates:

- Template for recording setups and scenarios-NOKIA_IMPRO.xlsx

- Template for recordings database-NOKIA_IMPRO.xlsx

# 4. Summary

This contribution introduces a measurement method that is favorable for the measurement of integrated microphone pressure response as a source data for filter design for microphone signal compensation. This method enables isotropic definition of compensated microphone response masks as a compensation targets [6]. Additional benefits include support for realistic representation of virtual prototypes using well known numerical acoustic methods providing device diffraction characteristics that can be combined with accurate integrated microphone characteristics measured with IMPro method [2]. IMPro measurement method supports isotropic characterization of raw microphone signal without an impact of device body diffractions. This is favorable characterization enabling the design of equalization filters for microphone signal compensation.

Use of IMPro measurement method is proposed for agreement.

# References

[1]	S4-250595: “On raw and compensated microphone signal requirements”, Nokia

[2] 	Cozens J., Hämäläinen M, Pekkarinen M, IMPro – Method for Integrated Microphone Pressure Frequency Response Measurement Using a Probe Microphone, AES 158th Convention, Warsaw, Poland 2025 May 22–24. https://aes2.org/publications/elibrary-page/?id=22907

[3]	S4-250567: “Proposed DaCAS target device type“, Nokia

[4]	SOFA (Spatially Oriented Format for Acoustics) – Sofaconventions at https://www.sofaconventions.org/mediawiki/index.php/SOFA_(Spatially_Oriented_Format_for_Acoustics ))

[5]	https://www.sofaconventions.org/mediawiki/index.php/Software_and_APIs

[6]	S4-250948: “On microphone signal requirements”, Nokia