Source:	Nokia

Title:	On microphone signal requirements

Document for:	Discussion & Agreement

Agenda Item:	7.6 – DaCAS

# 1. Introduction

In the SA4 131-bis-e meeting initial text for raw and compensated microphone signal requirements was agreed for further offline editing, based on the [1]. In this contribution, further proposal for the requirements of microphone signals is proposed.

# 2. Requirements for microphone signals

In the previous proposal [1], the requirements for compensated microphone responses were left open. Here, we propose two requirement and recommendation masks for compensated microphone signal frequency responses of the DaCAS target devices: requirement and recommendation masks for the compensated microphone responses, and requirement and recommendation masks for differences of compensated microphone responses.

Table 1 defines the requirement and recommendation masks for the overall frequency response of the compensated microphone signals. The requirement follows the same requirements as defined in 3GPP TS 26.131 [2] for Handset and headset sending sensitivity/frequency requirements. In addition, as slightly stricter mask is proposed for the recommended performance objective.

In Table 2, the requirement and recommendation mask for compensated microphone response differences is proposed. While the overall frequency response mask is defined slightly looser to leave a sufficient room for different device characteristics and to allow developer decisions, the microphone responses should be as similar as possible. Similarity can be guaranteed by defining a requirement for the differences of the compensated microphone responses. The proposed requirement is considered to leave a room for a natural variation of the microphone components obtained due to the manufacturing processes. In addition, recommended target mask is proposed.

Following is proposed for agreement:

[

Raw microphone input for DaCAS solution shall comply with the requirements specified in Table 1.

Table 1 Raw microphone signal requirements

| Feature | Requirement |
| --- | --- |
| Raw frequency response (excluding resonances) | Minimum captured frequency (-3dB point) shall be below 100 Hz Frequency response above the minimum frequency and below the resonances should be as flat as possible. |
| Resonances | Recommended to be above 10 kHz. Should be above 8 kHz. |
| SNR | Should be over 60 dB, recommended to be over 65 dB |
| Sensitivity | Level should be below -25 dBFS (±1 dB) with 1 kHz @ 94 dB SPL |
| Directivity | Recommended to have omnidirectional characteristics. Other directivity patterns are not generally excluded. Microphone directivity characteristics shall be documented clearly. |

Note: It is expected that all the raw integrated microphones comply with the above requirements. In addition, it is favorable to have as similar characteristics as possible for all raw integrated microphones.

Compensated microphone input for DaCAS solution shall comply with the requirements specified in Table 2.

Table 2 Compensated microphone signal requirements

| Feature | Requirement |
| --- | --- |
| Compensated frequency response | Compensated frequency response should be considered in the context of isotropic equalization target. Shall be within mask defined in Table 3 Differences between compensated frequency responses shall be within mask defined in Table 4 |

Table 3: Compensated microphone sensitivity/frequency mask

|  | Required |  | Recommended |  |
| --- | --- | --- | --- | --- |
| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |  |  |
| 100 | 4 |  | 2 |  |
| 200 | 4 | -4 | 2 | -2 |
| 5000 | 4 | -4 | 2 | -2 |
| 12500 | 4 | -6 | 2 | -3 |
| 16000 | 4 |  | 2 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |  |  |

Figure 1 Compensated microphone response masks

Table 4: Compensated of integrated microphone sensitivity/frequency requirement mask

|  | Required |  | Recommended |  |
| --- | --- | --- | --- | --- |
| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) | Upper limit (dB) | Lower limit (dB) |
| 100 | 2 |  | 1 | -1 |
| 200 | 2 | -2 | 1 | -1 |
| 5000 | 2 | -2 | 1 | -1 |
| 12500 | 2 | -4 | 1 | -2 |
| 16000 | 2 |  | 1 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |  |  |

Figure 2 Compensated microphone response difference masks

]

# 3. Obtaining compensated frequency response

In [1] was proposed that the compensated frequency response should be considered in the context of isotropic equalization. To obtain isotropic microphone response of the device, a measurement method described in [3] can be applied. In addition, in [4] we present a further consideration on the measurement method.

# 4. Summary

In this contribution further proposals on the compensated microphone response requirements are presented. Namely, requirements and recommendation masks for

1. compensated microphone responses, and

2. differences of compensated microphone responses

are proposed for agreement.

# References

[1]	S4-250654: “On raw and compensated microphone signal requirements”, Nokia

[2]	3GPP TS 26.131: “Terminal acoustic characteristics for telephony; Requirements” version 18.1.0 Release 18

[3]	Cozens J., Hämäläinen M, Pekkarinen M, IMPro – Method for Integrated Microphone Pressure Frequency Response Measurement Using a Probe Microphone, AES 158th Convention, Warsaw, Poland 2025 May 22–24. https://aes2.org/publications/elibrary-page/?id=22907

[4]	S4-250936: “On compensated microphone pressure frequency response measurement”, Nokia