Source:	Nokia

Title:	Database for Four-microphone prototype device A and B

Document for:	Discussion & Agreement

Agenda Item:	7.6 – DaCAS

# 1. Introduction

In this contribution, recordings for Four-microphone prototype target devices A and B are presented and proposed to be included in the Recordings Database. Proposed recordings comprise recordings according to the recording scenarios 1 and 2. In addition, multiple new recording scenarios for development and evaluation are proposed, including controlled single source recordings, and “ad-hoc” single and multisource scenario recordings, in varying environments.

# 2. Recording database for Four-microphone prototype device A and B

## Recordings for device A and device B were made to collect database for characterization, development and evaluation purposes. Database comprises recordings for characterizing integrated microphone SNR, direction dependent properties, reproducible single talk scenarios in controlled environment, and real talker scenarios in varying environments. Detailed description of the delivered recordings are presented here. In addition, corresponding database entries are attached to the Template for recording setups and scenarios-NOKIA_FUKUOKA.xlsx and Template for recordings database-NOKIA_FUKUOKA.xlsx.

## 2.1 SNR recordings

To characterize Signal-to-noise ratio of integrated microphones of the target devices, the following recordings were made according to the agreed recording scenario 1.

Table 1: SNR recordings for device A and device B

| Target device No | Four-microphone prototype device A | Four-microphone prototype device B |
| --- | --- | --- |
| Recording scenario No | 1 | 1 |
| Filename | deviceA_DaCAS_SNR.wav | deviceB_DaCAS_SNR.wav |
| UE orientation / placement | UE in landscape. First sine towards the front of the device, second sine towards the back of the device | UE in landscape, first sine towards the front of the device, second sine towards the right of device, third sine towards the back of the deivce, last sine towards the left of the device |
| Sample rate | 48 kHz | 48 kHz |
| Bit depth | 24 bit | 24 bit |
| Audio format | .wav | .wav |
| Rec Duration | Total length 60s: - 30 seconds of silence, - then 10 seconds of 1kHz sine from front side of the device (mics 1 and 3 are pointing to source), - then 10 seconds of 1kHz sine from back side of the device (mics 2 and 4 are pointing to source). | Total length 110s: - 30 seconds of silence in the beginning - then 10 seconds of 1kHz sine from front side of the device (mic 3 pointing to source) - then 10 seconds of 1kHz sine from top side of the device (mic 2 is pointing to source), - then 10s of 1kHz from display side, - and finally 1kHz sine from bottom side of the device (mics 1 and 4 are pointing to source). |
| Number of channels and microphone mapping | 4 channels: ch1 = Mic Front Right, ch2 = Mic Rear Right ch3 = Mic Front Left ch4 = Mic Rear Left | 4 channels: ch1 = mic #1 ch2 = mic #2 ch3 = mic #3, ch4 = mic #4 |
| Description/additional info | Sound pressure level 94 dB SPL at the measured mic positions. Distance between UE and sound source was 1.3m. Genelec 8331A was used for the measurement | Sound pressure level 94 dB SPL at the measured mic positions. Distance between UE and sound source was 1.3m. Genelec 8331A was used for the measurement |

For device A, following properties of the integrated microphones were analyzed based on the recordings:

| SNR in dB | Sensitivity with 1kHz @ 94 dB SPL |
| --- | --- |
| SNR ch1 = 71 dB SNR ch2 = 71.3 dB SNR ch3 = 71.5 dB SNR ch4 = 72.3 dB | Sensitivity ch1 = -36.1 dB Sensitivity ch2 = -36.1 dB Sensitivity ch3 = -35.9 dB Sensitivity ch4 = -35.5 dB |

For device B, following properties of the integrated microphones were analyzed based on the recordings:

| SNR in dB | Sensitivity with 1kHz @ 94 dB SPL |
| --- | --- |
| SNR ch1 = 70 dB SNR ch2 = 70.8 dB SNR ch3 = 72.9 dB SNR ch4 = 70.9 dB | Sensitivity ch1 = -35.2 dB Sensitivity ch2 = -35.1 dB Sensitivity ch3 = -34.3 dB Sensitivity ch4 = -35.4 dB |

## 2.2 Rotational sweeps

For obtaining directional properties of the devices, rotational sweep measurements were done according to the recording scenario 2. For device B, the measurements were done in all proposed orientations.

In addition, for recording scenario 2 a minor change to decrease the minimum length of the applied sweep from 5 seconds to 4 seconds is proposed. The presented recordings were done with 4 seconds long sweep, to reduce the total length of the files. Longer sweeps than 4 seconds are also considered acceptable.

Table 2: Rotation recordings for device A and device B

| Target device No | Four-microphone prototype device A | Four-microphone prototype device B |
| --- | --- | --- |
| Recording scenario No | 2 | 2 |
| Filename | deviceA_DaCAS_rotation.wav | deviceB_Landscape_DaCAS_rotation.wav deviceB_Portrait_DaCAS_rotation.wav deviceB_Table_DaCAS_rotation.wav |
| UE orientation / placement | UE in landscape orientation | UE in landscape, portrait and table orientations. |
| Sample rate | 48 kHz | 48 kHz |
| Bit depth | 24 bit | 24 bit |
| Audio format | .wav | .wav |
| Rec Duration | Total length 12min 30s. - Silence before and after the sweeps, turntable rotated between consecutive azimuth angle measurement points. | Total length 12min 30s. - Silence before and after the sweeps, turntable rotated between consecutive azimuth angle measurement points. |
| Number of channels and microphone mapping | 4 channels: ch1 = Mic Front Right, ch2 = Mic Rear Right ch3 = Mic Front Left ch4 = Mic Rear Left | 4 channels: ch1 = mic #1 ch2 = mic #2 ch3 = mic #3, ch4 = mic #4 |
| Description/additional info | Sound pressure 80 dB SPL at the UE position. (Corresponds to the calibration with -3 dBFS pink noise calibrated to 70 dBSPL at the UE position) Sweep number and corresponding angle defined in NOKIA_rotation_angles -sheet For the elevation 0° recordings, Genelec 8331A loudspeaker was used. For other elevations, Genelec 8010A were used. Reference recordings were done with GRAS ½” 46AF free-field microphone. Reference recording names are REF_sweep_<elevation>.wav, where the <elevation> indicates the elevation of measured loudspeaker. Note: 4 seconds long sweep signal was used to reduce the length of the recordings | Sound pressure 80 dB SPL at the UE position. (Corresponds to the calibration with -3 dBFS pink noise calibrated to 70 dBSPL at the UE position) Sweep number and corresponding angle defined in NOKIA_rotation_angles -sheet For the elevation 0° recordings, Genelec 8331A loudspeaker was used. For other elevations, Genelec 8010A were used. Reference recordings were done with GRAS ½” 46AF free-field microphone. Reference recording names are REF_sweep_<elevation>.wav, where the <elevation> indicates the elevatation of measured loudspeaker. Note: 4 seconds long sweep signal was used to reduce the length of the recordings |

## 2.3 Single source in listening room

Recording scenario was constructed and recorded to include single sound source from varying angles in controlled environment. Applied source signal comprises 2 female and 2 male talkers, each talking 4 sentences.

The scenario was recorded in the listening room. Applied source angles were 0°, ±30°, ±60°, ±90°, ±120°, ±150° and 180°. Source distance was set to 1.4 meters from the recording device. The height of the sound source was set to same level (height 1.25m) with the recording device for landscape and portrait orientations. For table orientation, the device height was set to 0.85m, while the loudspeaker was positioned at a height of 1.25m. Loudspeaker was calibrated to produce 67 dB SPL(A) level at the device position.

The listening room where the recordings were done confirms to ISO 3382-2 room echo standard and meets ITU-R BS.1116-3 NR-15 noise floor standard. The same listening room was also used for the recordings described in clause 2.4.

Figure 1 Listening room arrangement

Table 3: Recording scenario for single sound source scenario

| No | N (single source) |
| --- | --- |
| Sound source | High quality loudspeaker |
| Source signal | Speech |
| Signal characteristics | Language: Finnish Gender: Male, Female Length: 48 s Level: -24 dBRMS Sample Rate: 48 kHz Bit depth: 24 bit |
| Sound source calibration | Source signal calibrated to 67 dB SPL(A) at the UE position |
| Acoustic environment | Listening room |
| Detailed Positioning | Source distance: 1.4m Source direction: Loudspeaker diaphragm towards the UE Source angles: 0°, ±30°, ±60°, ±90°, ±120°, ±150°, 180° |
| Reference recording |  |
| Description/additional info | Illustration: Figure 1 |

Table 4: Single source recording database for device A and device B Target device No; Four-microphone prototype device A; Four-microphone prototype device B. Recording scenario No; N (single source); N (single source). Filename; deviceA_ss_lr_landscape_<azimuth>.wav where <azimuth> indicates the sound source direction.; deviceB_ss_lr_landscape_<azimuth>.wav deviceB_ss_lr_portrait_<azimuth>.wav deviceB_ss_lr_table_<azimuth>.wav where <azimuth> indicates the sound source direction.. UE orientation / placement; UE in landscape orientation. UE at the same height as the sound source (1.25m); UE in landscape, portrait and table orientations. UE at the same height as the sound source in landscape and portrait orientations. UE at height of 0.85m and sound source at the height of 1.25m in table orientation. Sample rate; 48 kHz; 48 kHz. Bit depth; 24 bit; 24 bit. Audio format; .wav; .wav. Rec Duration; 48 seconds per file. - first 8 sentences of 2 female talkers - then 8 sentences 2 male talker; 48 seconds per file. - first 8 sentences of 2 female talkers, - then 8 sentences of 2 male talker. Number of channels and microphone mapping; 4 channels: ch1 = Mic Front Right, ch2 = Mic Rear Right ch3 = Mic Front Left ch4 = Mic Rear Left; 4 channels: ch1 = mic #1 ch2 = mic #2 ch3 = mic #3, ch4 = mic #4. Description/additional info; Genelec 8010A was used as a sound source; Genelec 8010A was used as a sound source.

## 2.4 Scenarios with real talkers

5 different recording scenarios were constructed and recorded to include recordings of real single- and multi-talker scenarios for development and evaluation purposes. Recordings were made in 4 different environments:

- Listening room

- Basement (reverberant corridor/hall)

- Arboretum (nature park)

- Traffic (next to crossroads)

Recording scenarios comprises 1 or 2 talkers, and additional loudspeaker playing music. The applied loudspeaker was portable Bluetooth loudspeaker. As the recordings were made in varying environments, the exact distances between the sound sources and the devices couldn’t be guaranteed (e.g., due to the limited amount of space, etc). However, certain limits for the distances are defined in the recording scenarios template. Furthermore, source angles may vary slightly. In addition, it should be noted that as the recordings were made with real talkers, there are some variations present between the recordings of the same scenario.

The height of the device was set approximately at the same height as the talkers, when recorded in landscape and portrait orientations. For table orientations, the recording device was set at 85cm height.

In the following sections, the applied recording scenarios are explained in detail. In addition, recording scenarios and recording database entries are defined.

2.4.1 Recording scenario: real talkers 1

Single source scenario, where the talker is approximately 1.5 meters from the device. Talker starts talking at the front of the device. Talker will start to circulate the device, first towards left of the device, while simultaneously describing where he is. Distance is kept approximately the same throughout the recording.

The same recording scenario is done in all above-described environments, for all defined device orientations.

Figure 2 Illustration of the recording scenario with real talker 1

Table 5: Real talkers 1 recording scenario

| No | N+1 (real talkers 1) |
| --- | --- |
| Sound source | 1 Human |
| Source signal | Speech |
| Signal characteristics | Talker Language: English Gender: Male |
| Sound source calibration | Normal talking voice (relative to the environment) |
| Acoustic environment | Varying environments: - Listening room (indoor) - Basement (indoor) - Arboretum (outdoor) - Traffic (outdoor) |
| Detailed Positioning | Source distance: 1-2m Source direction: Talking towards the devices Source angles: Walking around the device and talking simultaneously |
| Reference recording |  |
| Description/additional info | Illustration: Figure 2 Description of the speech: Talker describing the movement and his positions |

Table 6: Real talkers 1 recording database for device A and device B Target device No; Four-microphone prototype device A; Four-microphone prototype device B. Recording scenario No; N+1 (real talkers 1); N+1 (real talkers 1). Filename; deviceA_rs_1_landscape_<environment>.wav, where the <environment> defines the recording environment. Recorded environments: listening_room, basement, arboretum, traffic; deviceB_rs_1_<orientation>_<environment>.wav where the <environment> defines the recording environment and <orientation> defines the UE orientation. Recorded environments: listening_room, basement, arboretum, traffic Recorded orientations: landscape, portrait, table. UE orientation / placement; UE in landscape orientation. UE at the same height as the talkers; UE in landscape, portrait and table orientations. UE at the same height as the talker in landscape and portrait orientations. UE at height of 0.85m in table orientation. Sample rate; 48 kHz; 48 kHz. Bit depth; 24 bit; 24 bit. Audio format; .wav; .wav. Rec Duration; 30-43 seconds, depends on the recording take; 32-41s seconds, depends on the recording take. Number of channels and microphone mapping; 4 channels: ch1 = Mic Front Right, ch2 = Mic Rear Right ch3 = Mic Front Left ch4 = Mic Rear Left; 4 channels: ch1 = mic #1 ch2 = mic #2 ch3 = mic #3, ch4 = mic #4. Description/additional info; Note: UE was at the same height as the talkers in table orientation in Arboretum environment.

2.4.2 Recording scenario with real talkers 2

Scenario with two talkers, where talker 1 is approximately 1.5 meters from the device at the angle of ~ 45° relative to the device (left of the device). Talker 2 is also approximately 1.5 meters from the device but at the angle of ~ -45° relative to the device (right of the device). Talkers start with English sentences and switch to the Finnish sentences. For the first half of the recordings, the talkers don’t overlap. For the second half of the recording, the talkers overlap.

The same recording scenario is done in all above-described environments, for all defined device orientations, except in basement with device A.

Figure 3 Illustration of the recording scenario 2

Table 7: Real talkers 2 recording scenario

| No | N+2 (real talkers 2) |
| --- | --- |
| Sound source | 2 Human |
| Source signal | Speech |
| Signal characteristics | Talker Language: English and Finnish Gender: Male |
| Sound source calibration | Normal talking voice (relative to the environment) |
| Acoustic environment | Varying environments: - Listening room (indoor) - Basement (indoor) - Arboretum (outdoor) - Traffic (outdoor) |
| Detailed Positioning | Talker 1 Source distance: 1.2-1.8m Source direction: Talking towards the device Source angles: Approximately 45° relative to the device. Talker 2 Source distance: 1.2-1.8m Source direction: Talking towards the device Source angle: Approximately -45° relative to the device. |
| Reference recording |  |
| Description/additional info | Illustration: Figure 3 Description of the speech: Single talk sentences and overlaping talk |

Table 8: Real talkers 2 recording database for device A and device B

| Target device No | Four-microphone prototype device A | Four-microphone prototype device B |
| --- | --- | --- |
| Recording scenario No | N+2 (real talkers 2) | N+2 (real talkers 2) |
| Filename | deviceA_rs_2_landscape_<environment>.wav, where the <environment> defines the recording facilities. Recorded environments: listening_room, basement, arboretum, traffic | deviceB_rs_2_<orientation>_<environment>.wav where the <environment> defines the recording facilities and <orientation> defines the UE orientation. Recorded environments: listening_room, basement, arboretum, traffic Recorded orientations: landscape, portrait, table |
| UE orientation / placement | UE in landscape orientation. UE at the same height as the talkers | UE in landscape, portrait and table orientations. UE at the same height as the talker in landscape and portrait orientations. UE at height of 0.85m in table orientation |
| Sample rate | 48 kHz | 48 kHz |
| Bit depth | 24 bit | 24 bit |
| Audio format | .wav | .wav |
| Rec Duration | 59-75 seconds, depending on the recording take. First part single talk sentences, second part overlapping talkers | 60-75 seconds, depending on the recording take. First part single talk sentences, second part overlapping talkers |
| Number of channels and microphone mapping | 4 channels: ch1 = Mic Front Right, ch2 = Mic Rear Right ch3 = Mic Front Left ch4 = Mic Rear Left | 4 channels: ch1 = mic #1 ch2 = mic #2 ch3 = mic #3, ch4 = mic #4 |
| Description/additional info |  | Note: UE was at the same height as the talkers in table orientation in Arboretum environment |

2.4.3 Recording scenario: real talkers 3

Scenario with two talkers, where the talker 1 is approximately 2 meters from the device at the angle of ~-45° relative to the device (front right of the device). The talker 2 is approximately 1 meter from the device but at the angle of ~-140° relative to the device (back right of the device).

Talkers start with English sentences and switch to the Finnish sentences. For the first half of the recordings, the talkers don’t overlap. For the second half of the recording, the talkers overlap. The talker positions are described in the first sentences of the recordings.

The same recording scenario is done in all above-described environments, for all defined device orientations.

Figure 4 Illustration of the recording scenario 3

Table 9: Real talkers 3 recording scenario

| No | N+3 (real talkers 3) |
| --- | --- |
| Sound source | 2 Males |
| Source signal | Speech |
| Signal characteristics | Talker Language: English and Finnish Gender: Male Loudspeaker Content: Music |
| Sound source calibration | Normal talking voice (relative to the environment) |
| Acoustic environment | Varying environments: - Listening room (indoor) - Basement (indoor) - Arboretum (outdoor) - Traffic (outdoor) |
| Detailed Positioning | Talker 1 Source distance: 1.8-2.2m Source direction: Talking towards the device Source angles: Approximately 45° relative to the device. Talker 2 Source distance: 1-1.2m Source direction: Talking towards the device Source angle: Approximately -140° relative to the device. |
| Reference recording |  |
| Description/additional info | Illustration: figure 4 Description: Single talk sentences and overlapping talk |

Table 10: Real talkers 3 recording database for device A and device B

| Target device No | Four-microphone prototype device A | Four-microphone prototype device B |
| --- | --- | --- |
| Recording scenario No | N+3 (real talkers 3) | N+3 (real talkers 3) |
| Filename | deviceA_rs_3_landscape_<environment>.wav, where the <environment> defines the recording facilities. Recorded environments: listening_room, arboretum, traffic | deviceB_rs_3_<orientation>_<environment>.wav where the <environment> defines the recording facilities and <orientation> defines the UE orientation. Recorded environments: listening_room, basement, arboretum, traffic Recorded orientations: landscape, portrait, table |
| UE orientation / placement | UE in landscape orientation. UE at the same height as the talkers | UE in landscape, portrait and table orientations. UE at the same height as the talker in landscape and portrait orientations. UE at height of 0.85m in table orientation |
| Sample rate | 48 kHz | 48 kHz |
| Bit depth | 24 bit | 24 bit |
| Audio format | .wav | .wav |
| Rec Duration | 59-69 seconds, depending on the recording take. First part single talk sentences, second part overlapping talkers | 60-71 seconds, depending on the recording take. First part single talk sentences, second part overlapping talkers |
| Number of channels and microphone mapping | 4 channels: ch1 = Mic Front Right, ch2 = Mic Rear Right ch3 = Mic Front Left ch4 = Mic Rear Left | 4 channels: ch1 = mic #1 ch2 = mic #2 ch3 = mic #3, ch4 = mic #4 |
| Description/additional info |  | Note: UE was at the same height as the talkers in table orientation in Arboretum environment |

2.4.4 Recording scenario: real talkers 4

Scenario with 2 talkers and a loudspeaker, where talker 1 is approximately 2 meters from the device at the angle of ~90° relative to the device (left of the device). Talker 2 is approximately 1 meter from the device but at the angle of ~180° relative to the device (back of the device). The loudspeaker is positioned at the distance of 2 meters from the device, and with source angle of approximately ~-30° relative to the device (front right of the device).

Talkers start with English sentences and switch to the Finnish sentences. For the first half of the recordings, the talkers don’t overlap. For the second half of the recording, the talkers overlap. In some environments, the talker 1 is closer to the device (approximately 1 meter from device). The talker distances are described in the first sentences of the recordings.

The same recording scenario is done in all above-described environments, for all defined device orientations.

Figure 5 Illustration of the recording scenario 4

Table 11: Real talkers 4 recording scenario

| No | N+4 (real talkers 4) |
| --- | --- |
| Sound source | 2 Males + loudspeaker |
| Source signal | Speech, music |
| Signal characteristics | Talkers Language: English and Finnish Gender: Male Description: Single talk sentences and overlapping talk Loudspeaker Content: Music |
| Sound source calibration | Normal talking voice, loudspeaker adjusted to a slightly lower volume than the speech |
| Acoustic environment | Varying environments: - Listening room (indoor) - Basement (indoor) - Arboretum (outdoor) - Traffic (outdoor) |
| Detailed Positioning | Talker 1 Source distance: 1-2m Source direction: Talking towards the device Source angles: Approximately 90° relative to the device. Talker 2 Source distance: 0.8-1.2m Source direction: Talking towards the device Source angle: Approximately 180° relative to the device. Loudspeaker Source distance: 1.5-2m Source direction: Towards the device Source angle: Approximately -30° relative to the device. |
| Reference recording |  |
| Description/additional info | Illustration: Figure 5 Description of the speech: Single talk sentences and overlapping talk |

Table 12: Real talkers 4 recording database for device A and device B

| Target device No | Four-microphone prototype device A | Four-microphone prototype device B |
| --- | --- | --- |
| Recording scenario No | N+4 (real talkers 4) | N+4 (real talkers 4) |
| Filename | deviceA_rs_4_landscape_<environment>.wav, where the <environment> defines the recording facilities. Recorded environments: listening_room, basement, arboretum, traffic | deviceB_rs_4_<orientation>_<environment>.wav where the <environment> defines the recording facilities and <orientation> defines the UE orientation. Recorded environments: listening_room, basement, arboretum, traffic Recorded orientations: landscape, portrait, table |
| UE orientation / placement | UE in landscape orientation. UE at the same height as the talkers | UE in landscape, portrait and table orientations. UE at the same height as the talker in landscape and portrait orientations. UE at height of 0.85m in table orientation |
| Sample rate | 48 kHz | 48 kHz |
| Bit depth | 24 bit | 24 bit |
| Audio format | .wav | .wav |
| Rec Duration | 74-86 seconds, depending on the recording take. First part single talk sentences, second part overlapping talkers | 70-85 seconds, depending on the recording take. First part single talk sentences, second part overlapping talkers |
| Number of channels and microphone mapping | 4 channels: ch1 = Mic Front Right, ch2 = Mic Rear Right ch3 = Mic Front Left ch4 = Mic Rear Left | 4 channels: ch1 = mic #1 ch2 = mic #2 ch3 = mic #3, ch4 = mic #4 |
| Description/additional info |  | Note: UE was at the same height as the talkers in table orientation in Arboretum environment |

2.4.5 Recording scenario: real talkers 5

Dynamically moving single talk scenario with a loudspeaker playing music simultaneously. In the scenario, the loudspeaker goes above the device from front to back and from left to right, and below the device from back to front and from right to left. In addition, talker and loudspeaker move from front to the left of the device after the first above/below movement. After the second above/below movement, the talker and loudspeaker will go full circle around the device, starting from the left of the device and going counter-clockwise. The talker describes the loudspeaker movement during the recording.

The same recording scenario is done in all above-described environments, for all defined device orientations.

Figure 6 Dynamic movements illustrated in the recording scenario 5. Exact movement is described in the recordings.

Table 13: Real talkers 5 recording scenario

| No | N+5 (real talkers 5) |
| --- | --- |
| Sound source | 1 Male + loudspeaker |
| Source signal | Speech, music |
| Signal characteristics | Talker: Language: English Gender: Males Loudspeaker Content: Music |
| Sound source calibration | Normal talking voice, loudspeaker adjusted to approximately same level as the speech |
| Acoustic environment | Varying environments: - Listening room (indoor) - Basement (indoor) - Arboretum (outdoor) - Traffic (outdoor) |
| Detailed Positioning | Source distance: 0.5-2m Source direction: Talking towards the devices Source angles: Dynamic movement - Loudspeaker going with the talker above from front to back and below the device from back to front - Loudspeaker moving with the talker from center to left - Loudspeaker moving with the talker above device from left to right and below the device from right to left - Loudspeaker and talker circulating the device starting from the left of the device |
| Reference recording |  |
| Description/additional info | Illustration: Figure 6 Description of the speech: Talker describing the dynamic movement |

Table 14: Real talkers 5 recording database for device A and device B

| Target device No | Four-microphone prototype device A | Four-microphone prototype device B |
| --- | --- | --- |
| Recording scenario No | N+5 (real talkers5) | N+5 (real talkers 5) |
| Filename | deviceA_rs_5_landscape_<environment>.wav, where the <environment> defines the recording environment. Recorded environments: listening_room, basement, arboretum, traffic | deviceB_rs_5_<orientation>_<environment>.wav where the <environment> defines the recording environment and <orientation> defines the UE orientation. Recorded environments: listening_room, basement, arboretum, traffic Recorded orientations: landscape, portrait, table |
| UE orientation / placement | UE in landscape orientation. UE at the same height as the talkers | UE in landscape, portrait and table orientations. UE at the same height as the talker in landscape and portrait orientations. UE at height of 0.85m in table orientation |
| Sample rate | 48 kHz | 48 kHz |
| Bit depth | 24 bit | 24 bit |
| Audio format | .wav | .wav |
| Rec Duration | 85-105 seconds, depending on the recording take. | 93-109 seconds depending on the recording take. |
| Number of channels and microphone mapping | 4 channels: ch1 = Mic Front Right, ch2 = Mic Rear Right ch3 = Mic Front Left ch4 = Mic Rear Left | 4 channels: ch1 = mic #1 ch2 = mic #2 ch3 = mic #3, ch4 = mic #4 |
| Description/additional info |  | Note: UE was at the same height as the talkers in table orientation in Arboretum environment |

# 3. Mounting of the devices

For documentation, pictures of the device mountings are included. For device A, 3D printed holder was used to mount the device.

Figure 8 Mounting of the device A

For device B, commercially available phone holders were used. Care was taken to not block any microphone inlets.

Figure 9 Landscape mounting of the device B

Figure 11 Table mounting of the device B

# 4. Conclusion

In the present contribution, a recordings database for Four-microphone prototype device A and device B is provided. Database comprises recordings according to the recording scenario 1 and 2. In addition, new recording scenarios for single source in controlled environment, as well as 5 recording scenarios with real talkers are provided and proposed for agreement.

The recording scenarios are defined in the attached file Template for recording setups and scenarios-NOKIA_FUKUOKA.xlsx, and recordings are defined in attached file Template for recordings database-NOKIA_FUKUOKA.xlsx file.

Example of each recording scenario is provided along the previous version of (S4-250947) this document. Full database will be made available in:

https://deveo.partner.nokia.com/TECH/projects/nokia_dacas_databases/repositories/webdav/4mic-proto-device-A-and-device-B

Access to the database requires an account, which can be requested from the source. One account will be provided per company.