3GPP TSG-SA4 Meeting #132	S4-251141

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.822 | CR | 0003 | rev | 3 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Communication overhead of measurement-based pre-compensation for PDU Set size correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Innovation Center Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The range of how often a message exchange is needed for the measurement-based pre-compensation for PDU Set size correction is too coarse. There is formatting issue in the caption of Figure 6.4.2-4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1st change: added 2 references. 2nd change: corrected missing break line in the caption of Figure 6.4.2-4. 3rd change: added in clause 6.4.2 improved estimates on the need for update (the frequency of which determines the communicaiton overhead) for the measurement-based pre-compensation for PDU Set size correction; added a note on an assmption for the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading informaiton on the communicaiton overhead for the measurement-based pre-compensation for PDU Set size correction. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev3: changed work item code in S4-251134. Rev2: added a note on the requirement for the UE (as receiver) S4-251014. Rev1: showed track changes that were missing in the original S4-250779. |  |  |  |  |  |  |  |  |

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[4]	IETF RFC 8872: "Guidelines for Using the Multiplexing Features of RTP to Support Multiple Media Streams".

[5]	IETF RFC 5761: "Multiplexing RTP Data and Control Packets on a Single Port".

[6]	3GPP TR 23.700-70: "Study on architecture enhancement for Extended Reality and Media service (XRM); Phase 2".

[7]	IETF RFC 8285 (2017): "A General Mechanism for RTP Header Extensions", D. Singer, H. Desineni, R. Even.

[8]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)".

[9]	IETF RFC 9335: "Completely Encrypting RTP Header Extensions and Contributing Sources".

[10]	IETF RFC 6904 (2013): "Encryption of Header Extensions in the Secure Real-time Transport Protocol (SRTP)", J. Lennox.

[11]	IETF RFC 8402 (2018): "Segment Routing Architecture".

[12]	IETF RFC 791 (1981): "Internet Protocol".

[13]	IETF RFC 5109: "RTP Payload Format for Generic Forward Error Correction (ULP FEC): Uneven Level Protection, different redundancies for different packets with different importance".

[14]	IETF RFC 8627: "RTP Payload Format for Flexible Forward Error Correction (Flex FEC): flexible FEC".

[15]	IETF RFC 6681: "Raptor Forward Error Correction (FEC) Schemes for FECFRAME: FEC scheme based on the Raptor".

[16]	IETF RFC 6865: "Simple Reed-Solomon Forward Error Correction (FEC) Scheme for FECFRAME: FEC scheme based on Reed-Solomon".

[17]	IETF RFC 5053: "Raptor Forward Error Correction Scheme for Object Delivery".

[18]	IETF RFC 6330: "RaptorQ Forward Error Correction Scheme for Object Delivery".

[19]	IETF RFC 6363: "Forward Error Correction (FEC) Framework"”.

[20]	IETF RFC 8854: "WebRTC Forward Error Correction Requirements".

[21]	3GPP TR 38.340: "Study on User Equipment (UE) power saving in NR".

[22]	IETF RFC 8298: "Self-Clocked Rate Adaptation for Multimedia".

[23]	Enhancing Video Network Resiliency With LTR and RS Code | At Scale Conferences, available online: https://atscaleconference.com/enhancing-video-network-resiliency-with-ltr-and-rs-code/

[24]	P. Aggarwal et al., [2304.03732] Enabling immersive experiences in challenging network conditions (arxiv.org)

[25]	Nvidia GeForce Now, Video FEC for WebRTC presentation 17 Nov. 2022, available online: https://www.youtube.com/watch?v=igm7QkqxHqk&ab_channel=KrankyGeek

[26]	Holmer S., et al., Handling Packet Loss in WebRTC, 2013 IEEE International Conference on Image Processing, available online: https://static.googleusercontent.com/media/research.google.com/en//pubs/archive/41611.pdf.

[27]	A Google Congestion Control Algorithm for Real-Time Communication, draft-ietf-rmcat-gcc-02, 2016.

[28]	WebRTC source code: https://source.chromium.org/chromium/chromium/src/+/main:third_party/webrtc, retrieved May 1, 2024.

[29]	IETF RFC 8698: "Network-Assisted Dynamic Adaptation: A Unified Congestion Control Scheme for Real-Time Media", 2020.

[30]	Self-Clocked Rate Adaptation for Multimedia, draft-johansson-ccwg-rfc8298bis-screamv2-00, 2024.

[31]	IETF RFC 4588: "RTP Retransmission Payload Format".

[32]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[33]	IETF RFC 4585: " Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF)".

[34]	IETF RFC 8085: "UDP Usage Guidelines".

[35]	IETF RFC 8834: "Media Transport and Use of RTP in WebRTC".

[36]	IETF RFC 2250: "RTP Payload Format for MPEG1/MPEG2 Video".

[37]	IETF RFC 9143: "Negotiating Media Multiplexing Using the Session Description Protocol (SDP)".

[38]	IETF RFC 7941: "RTP Header Extension for the RTP Control Protocol (RTCP) Source Description Items".

[39]	GStreamer: open source multimedia framework:
https://gstreamer.freedesktop.org/.

[40]	Wowza Streaming Engine™:
https://www.wowza.com/.

[41]	Media MTX: Ready-to-use SRT / WebRTC / RTSP / RTMP / LL-HLS media server and media proxy that allows to read, publish, proxy, record and playback video and audio streams:
http://github.com/blueviron/mediamtx.

[42]	Adobe RTMP Specification (veriskope.com):
https://rtmp.veriskope.com/docs/spec/.

[43]	RTP Extensions for Transport-wide Congestion Control, draft-holmer-rmcat-transport-wide-cc-extensions-01
https://datatracker.ietf.org/doc/html/draft-holmer-rmcat-transport-wide-cc-extensions-01

[44]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications".

[45]	IETF RFC 5104: "Codec Control Messages in the RTP Audio-Visual Profile with Feedback (AVPF)".

[46]	IETF RFC 6679: "Explicit Congestion Notification (ECN) for RTP over UDP".

[47]	IETF RFC 8888: "RTP Control Protocol (RTCP) Feedback for Congestion Control".

[48]	IETF RFC 8382: "Shared Bottleneck Detection for Coupled Congestion Control for RTP Media".

[49]	IETF RFC 3611: "RTP Control Protocol Extended Reports (RTCP XR)".

[50]	3GPP TS 26.510: "Media delivery; interactions and APIs for provisioning and media session handling".

[51]	Di Domenico, Andrea, et al. "A network analysis on cloud gaming: Stadia, GeForce Now and PS Now." Network 1.3 (2021): 247-260.

[52]	3GPP TR 26.928: "Extended Reality (XR) in 5G".

[53]		3GPP TS 26.506: "5G Real-time Media Communication Architecture (Stage 2)".

[54] 	3GPP TS 23.228: "IP Multimedia Subsystem (IMS) - Stage 2".

[55]	3GPP TS 29.571 "5G System; common data types for Service based interfaces; stage 3".

[56]	3GPP TS 38.415 "NG-RAN; PDU Session User Plane Protocol".

[57]	Real-Time Transport Protocol (RTP) Parameters (iana.org).

[58]	3GPP TS 26.113: " Real-Time Media Communication; Protocols and APIs".

[59]	3GPP TR 26.922: "Video telephony robustness improvements extensions; Performance evaluation".

[60]	3GPP TR 26.926: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems".

[61]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[62]	IETF RFC 2453: "RIP Version 2."

[63]	IETF RFC 4271: "A Border Gateway Protocol 4 (BGP-4)."

2nd Change

### 6.4.2	Description

As discussed in 5.1.1, there are multiple reasons for causing the PSSize to be inaccurate. Although the impact of those reasons on the size of a single IP packet seems insignificant, a PDU Set may consist of many IP packets and the aggregate impact can still be significant. If the PSSize for which gNB schedules is less than the actual PSSize, when the last packets arrive it may take gNB one or more slots to schedule them, therefore delaying the delivery of the PDU Set, which is detrimental to low-latency applications such as XR applications. The opposite can also happen, which wastes resources.

Therefore, we have the following observation:

Observation 1: it is important to make the PSSize accurate for low-latency applications.

There are efforts to correct the impact of NAT46/64 on the PSSize [11]. However, these efforts are not able to tackle other causes. Moreover, the list of causes in the introduction clause is not complete – even if we list all possible causes today, novel network protocols that change the PSSize are likely to be deployed in the future. We have the following observation:

Observation 2: A generic solution for correcting the PSSize is preferred.

The UPF is the gateway to the 5G core. A UPF may handle a very large amount of traffic. In fact, some network operators have very few UPFs. Therefore, any solution that requires UPF to take action, such as addition, subtraction and multiplication, is undesirable. We have the following observation:

Observation 3: To reduce the UPF complexity, it is preferred not to require UPF to correct the PSSize.

When we don’t know the causes, how do we correct the error? There is a similar problem in physical-layer communication, where the channel seen by a receiver is the result of reflection and refraction of many unknown objects in the radio propagation environment. The solution there is to measure the channel by the sender sending a pilot signal known to the receiver and the receiver comparing the pilot signal and the received signal. We borrow the measurement idea, and the counterpart of the ‘pilot signal’ is the indicated PSSize value in the RTP header extension and the counterpart of the ‘received signal’ is the observed PSSize. If a PDU Set is delivered successfully, the UE will observe the same PSSize (by summing the sizes of all PDUs of a PDU Set) as the gNB does.

Once the UE figures out the difference between the indicated PSSize and the observed PSSize, it can signal to the sender on how to pre-compensate for the PSSize.

Proposal 1: UE computes the difference between the actual PSSize and the indicated PSSize, and signals the difference to the RTP sender for PSSize pre-compensation.

Specifically, the UE calculates a correction ratio - the actual PSSize to the indicated PSSize ratio – and sends the correction ratio to the RTP sender. The RTP sender pre-compensates the PSSize by multiplying the PSSize and this correction ratio.

To validate the effectiveness of the proposed method, we tested the method on video sequences. The results below are for the Racehorse video sequence, and the video encoder is H.264 with a target NAL Unit size of 1400 bytes. The average video frame size is 22.377 kB. The cumulative distribution of the slice size (with each slice encapsulated into an IP packet) is shown in Figure 6.4.2-1. Note that there are a significant number of sizes uniformly distributed between 0 and around 1300 bytes.

NAT46 occurs in the network.

Figure 6.4.2-1: Slice size distribution

Scenario 1 (MTU=576 bytes):

The MTU size is set to 576 bytes (considered as a ‘safe’ MTU, because it is the IP packet size that all IPV4 nodes need to support [12]). This leads to fragmenting a packet size of 1400 bytes into three IP packets. As a result, the error in the PSSize comes from two sources: the presence of NAT46 and IP fragmentation. For the measurement-based correction method, the correction ratio is initialized to 1.

NOTE: The network configuration, e.g., NAT46 and MTU, typically changes much slower than the time scale of the feedback delay which is on the order of RTT. Thus, the feedback mechanism does not cause instability.

NOTE: As needed feedback - The UE can adapt the rate of feedback based on the observed error in the PSSize and a threshold on the tolerance of error.

The correction ratio (blue line) is shown in Figure 6.4.2-2.

Figure 6.4.2-2: Correction for the per-frame feedback (blue line) and one-time feedback (red line).

The various PSSize’s are shown in Figure 6.4.2-3. The errors in the PSSize are shown in Figure 6.4.2-4. Without PSSize correction, the mean absolute error of the PSSize is 1616 bytes; with PSSize correction assuming NAT46 only, the mean absolute error is 1283 bytes; with the proposed measurement based PSSize correction, the mean absolute error is 27.5 bytes.

Figure 6.4.2-3: The actual PSSize (cyan line) vs the PSSize without PSSize correction (blue line), the PSSize with measurement-based correction (red) and the PSSize with the ‘NAT46/64 only correction’ (green).

If the UE sends the feedback once during the transmission of the video sequence, without further correction ratio received, the sender will use a correction ratio of 1 for future frames. The correction ratio is shown by the red curve in Figure 6.4.2-2. The mean absolute error is 23.9 bytes, which is lower than the error when per-frame (or per PDU Set) feedback is used. This is because the first frame is an I-frame with a large size and the estimated ratio based on it tends to be more accurate than the estimates obtained in other smaller sized frames which are P frames.

Observation 4: For measurement-based PSSize correction, one-time feedback can be more accurate than more-frequent feedback.

Figure 6.4.2-4: PSSize error for the cases of without correction (blue), with correction assuming NAT46 only (green), and with the proposed measurement-based correction with per-frame feedback(red) for Scenario 1.

Scenario 2 (MTU=1300 bytes):

This is to show the effect of MTU size. The MTU size is set to 1300 bytes. A typical packet of 1400 bytes is fragmented into 2 packets. The errors in the PSSize are shown in Figure 6.4.2-5. The mean absolute error in the PSSize are 963.3 bytes, 630.0 bytes and 24.7 bytes for the case of no PSSize correction, NAT46 only correction and measurement-based correction,

This is to show the effect of MTU size. The MTU size is set to 1300 bytes. A typical packet of 1400 bytes is fragmented into 2 packets. The errors in the PSSize are shown in Figure 6.4.2-5. The mean absolute error in the PSSize are 963.3 bytes, 630.0 bytes and 24.7 bytes for the case of no PSSize correction, NAT46 only correction and measurement-based correction, respectively.

With one-time feedback, the error is 20.7 bytes, again lower than the error of per-frame feedback 24.7 bytes.

We see that the respective errors decrease compared to smaller MTU size. However, the errors for case of no PSSize correction and the case of NAT46 only correction are still significant.

Figure 6.4.2-5: PSSize error for the cases of without correction (blue), with correction assuming NAT46 only (green), and with the proposed measurement-based correction with per-frame feedback (red) for Scenario 2.

From the results, we observe that:

Observation 5: The proposed pre-compensation based PSSize correction method effectively reduces the PSSize error.

The frequency at which the UE sends the feedback does not exceed the frequency of routing changes. For major routing protocols, the routing changes at a time scale much larger than the video frame period (which, for example, is 16.7ms for 60 frames per second). For example, for Routing Infomration Protocol (RIP), the route is updated every 30 seconds [62]. The minimum amount of time between two route updates is 30 seconds for External BGP (EBGP), and 5 seonds for Internal BGP (IBGP) [63].

Observation 6: The proposed pre-compensation based PSSize correction method incurs a feedback message at a frequency less than once per 5 seconds.

NOTE: This solution adds a requirement for the UE to understand the RTP HE for PDU set marking that is not mandated by TS 26.522 in Rel-18.

For the ‘NAT46/64 only correction’ method to work, the sender needs to be aware of the presence of NAT46/64. The awareness may be obtained by feedback from the receiver on the type of the IP address type seen by the receiver. The feedback may be one time during a session, and can be done by SDP signaling, Simple WebRTC Application Protocol (SWAP) in TS 26.113 [58], or RTCP.

TS 26.522 [2] provides guidelines for preventing IP fragmentation, either through path MTU discovery or by assuming a conservative MTU size at the sender in generating IP packets. Path MTU discovery needs support from the routers on the end-to-end path and incurs communication overhead, and a conservative MTU size may lead to unnecessarily small IP packet sizes which come with a higher packet header cost (i.e., the ratio of the size of packet headers to the size of the media). We consider the guidelines as a solution for IP fragmentation prevention.

We compare the three solutions. The first criteria is whether the solution is generic, i.e., whether the solution can tackle multiple and even unknown causes to the error in the PSSize. The criteria ‘need support from the network?’ means whether the network needs to be configured (e.g., configured to support path MTU discovery) to enable a solution.

Table 6.4.2-1: Pros and Cons of the three solutions

| Solution | Generic | Accuracy | Need support from the network? | Communication overhead? | Need spec change? |
| --- | --- | --- | --- | --- | --- |
| NAT46/64 only correction | No | Low | No | Low | Yes |
| IP fragmentation prevention guidelines | No | Low | Yes if use path MTU discovery; no if use conservative MTU size. | Moderate if use path MTU discovery; none if use conservative MTU size. | No |
| Measurement-based correction | Yes | High | No | From low (one-time feedback) to at most one message per a few seconds | Yes |