3GPP TSG-SA WG4 Meeting #132			                                                          S4-251162

Fukuoka, Japan, 19-23 May 2025

Source:	vivo, Fraunhofer IIS, Qualcomm Incorporated, China Mobile Com. Corporation， Huawei Technologies Co., Ltd., Viasat, Inmarsat, EchoStar, Orange, Spreadtrum

Title:	Application scenario for IMS voice call using GEO satellite

Spec:	3GPP Pdoc [to be confirmed]

Agenda item:	7.9

Document for:	Agreement

1. Introduction

Application scenario for IMS voice call using GEO satellite is discussed in TR 26.940.

The under-discussion aspects extracted technical assumptions and open issues are proposed to be captured in this Pdoc. All differences are captured with revision mark.

2. Reason for Change

Compared to the proposal to TR 26.940, the present document provides un-resolved issues on application scenario for IMS voice call using GEO satellite access.

3. Proposal

It is proposed to agree the following changes to 3GPP Pdoc [to be confirmed]

* * * First Change * * * *

### 4.X.3	Extracted technical assumptions and open issues

#### 4.X.3.1	Assumptions for [Main] Scenario

The following assumptions apply for [Main] Scenario described in clause 4.2.2.2.

-	For the connection “UE1 – GEO satellite – Ground station” (UE1 uplink), the transmission data rate is significantly limited ([1-3] kbit/s), requiring an ultra-low bit rate codec fitting the transmission data rate for this link.

-	For the connection “Ground station – GEO satellite – UE1” (UE1 downlink), the transmission data rate is expected to be limited similarly to UE1 uplink.

-	For both uplink and downlink of UE1 it is expected that the link is subject to transmission errors reflecting GEO satellite access

Editor’s Note: "expected" should be replaced by more technical evidence when available (e.g., after coordination with RAN groups).

-	The delay in uplink and downlink of UE1 is expected to be greater than the one of typical terrestrial networks.

Editor’s Note: "expected" should be replaced by more technical evidence when available (e.g., after coordination with RAN groups)

-	For the connection "Core Network – UE2" (UE2 downlink), the transmission data rate of a regular TN network is available. This link could be covered either by an existing IMS codec (transcoding necessary) or by the same ultra-low bit rate codec as used for the satellite link (transcoding-free).

-	To ensure seamless communication across different network types, roaming, etc. transcoding functionality in core network is likely needed.

Editor’s Note: More details may be added.

#### 4.X.3.2	Assumptions for [Sub-]Scenario

The following assumptions apply for [Sub-]Scenario described in clause 4.2.2.3.

-	For both connections "UE1 – GEO satellite – Ground station" and "Ground station – GEO satellite – UE2" the transmission data rate is significantly limited ([1-3] kbit/s), requiring an ultra-low bit rate codec fitting this transmission data rate for these links.

-	This scenario may allow both transcoded (ULBC  existing IMS speech codecs ULBC) and transcoding-free operation (ULBC end-to-end)

Editor’s Note: More details may be added.

* * * End of Changes * * * *

[1] Tdoc SP-250378, "Study on Ultra Low Bitrate Speech Codec"

[2] TR 22.887, "Study on satellite access - Phase 4"

[3] TS 22.261, "Service requirements for the 5G system"

[4] GMR-2 03.050, "Transmission planning for speech service"

[5] Inmarsat Phone https://www.youtube.com/watch?v=iaI-rVMB6wk

[6] vivo Phone https://youtube.com/watch?v=EskEtJt9aQM&feature=shared

[7] Huawei Phone https://www.youtube.com/watch?v=6xDgaMajWxo

[8a] TR 23.401, "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access"

[8b] TR 23.501, "System architecture for the 5G System (5GS) "

[8] TR 38.821, "Solutions for NR to support non-terrestrial networks (NTN)"

[9] 3GPP TR 26.940, "Study on Ultra Low Bitrate Speech Codecs"