3GPP TSG SA WG 4 Meeting #133-e 	               	TDoc S4-251237

Online, 18th – 25th July 2025

Title:	LS reply on External Resource Handling in IMS Data Channel

Response to:	GSMA UPG14_109r3

Release:	Rel-19

Work Item:

Source:	SA4

To:	GSMA NG UPG

Cc:	SA1, SA2, SA3, SA6, CT

Contact person:	Imed Bouazizi

Bouazizi AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S4-25xxxx

# 1	Overall description

SA4 thanks GSMA NG UPG for their Liaison Statement (UPG14_109r3) highlighting the important issue regarding the ambiguity and potential confusion in handling external resource access in IMS Data Channel applications.

SA4 acknowledges the gaps pointed out by GSMA NG UPG and agrees that the current specification (TS 26.114) does not explicitly address scenarios involving external resource mashups clearly enough. Originally, SA4 assumed that the Data Channel (DC) Application Server (AS) would be the exclusive host for all content, and thus did not anticipate scenarios involving external resources.

To address this unclarity, SA4 plans to implement the following clarifications in TS 26.114:

- Resources specified with a relative URL (path only, without a host component) shall be resolved to the DC AS. This means the IMS client will always fetch these resources internally via the IMS data channel.

- Resources specified with a fully qualified URL (containing a host component) shall be explicitly subjected to Cross-Origin Resource Sharing (CORS) rules. The IMS client will only allow access to external resources when the DC AS explicitly authorizes the external domain through proper CORS headers. Access to domains not explicitly allowed by the DC AS through CSP and CORS shall be denied by the IMS client.

These clarifications will provide clear guidance on handling external URLs and enhance the security posture of IMS data channel applications by clearly defining boundaries for internal and external resource access.

A Change Request (CR) implementing the above clarification is attached to this LS.

# 2	Actions

To GSMA NG UPG

ACTION: 	GSMA NG UPG is kindly asked to review the attached Change Request and provide feedback as necessary.

# 3	Dates of next TSG SA WG 4 meetings

SA4 #134 	17 – 21 November 2025	Dallas, TX, USA

SA4#135	9 – 13 February 2026	India