3GPP TSG-SA WG4 Meeting 133	S4-251244

E-meeting, 18 – 25 July2025

Source:	China Mobile Com. Corporation

Title:	[FS_ULBC] Discussion of FS_ULBC Objective Speech Quality Assessment Method

Agenda item:	7.9 - FS_ULBC (Study on Ultra Low Bitrate Speech Codec)

Document for:	DISCUSSION and AGREEMENT

- Background

In the recent SA4 meeting, challenges of quality assessment of ultra-low bit rate speech codecs was discussed and It was agreed that traditional speech evaluation methodologies—primarily those designed for signal-processing based codecs —may not be sufficient or appropriate for assessing the performance of ultra-low bitrate codecs (ULBC),  especially those AI and non-signal-processing based codecs.A series of evaluation methods that are more suitable for ULBC scenarios are given, such as DCR, DRT, MRT, etc.Mostly subjective methods, and also with some objective ones included.

In order to verify the quality of the encoded speech more conveniently and quickly, some objective speech evaluation methods can be used as a supplement to these subjective test methods.This contribution collects potential objective evaluation speech quality assessment tools that could use in  ULBC evaluation for discussion

- Discussion

  - Overview of Existing Speech Objective Quality Evaluation Method(Tools)

| Method/Tool | Standard | Bitrate Range | Reference Type | Strengths | Limitations |
| --- | --- | --- | --- | --- | --- |
| POLQA | ITU-T P.863 | ≥ 4.75 kbps[1] | Full-reference | - Widely adopted in ITU 3GPP - Supports NB, WB, SWB | - Limited accuracy for bitrates <4 kbps - Not designed for AI codecs |
| PESQ | ITU-T P.862 | ≥ 8 kbps | Full-reference | - Classical standard - Applicable to NB and WB codecs | - Not accurate for ultra-low bitrates - No support for SWB or AI codecs |
| P.563 | ITU-T P.563 | Variable | No-reference | - No need for clean reference - Suitable for real-time | Evaluation of extreme noise or complex distortion may not be as accurate as with full-reference methods |
| ViSQOL | Open Source | Variable[2] | Full-reference | - Performs well for low bitrates& AI codecs | - not standardized - Still under evaluation for 3GPP usage |
| STOI/ESTOI | Open Source | Variable | Full-Reference | -Focus on speech intelligibility, suitable for low bit rate | Not adopted by ITU and 3GPP |

  - Capabilities and Limitations of Existing Speech Objective Quality Evaluation Method for ULBC

POLQA (ITU-T P.863): The most widely adopted ITU-T standard method,  wide range support for narrowband (NB) to super-wideband (SWB), for ultra-low bitrates (<4 kbps)  the results drop off sharply making it difficult to compare or evaluate the performance of ultra-low bitrate codec. and also it is not designed for AI-driven codecs.

PQLQA P.863 V2.4 results in NB mode for long speech files compared to 8s regular speech files[1]

PESQ (ITU-T P.862): The traditional full-reference metric for narrowband (NB) and wideband (WB) speech quality assessment, exhibits significant accuracy degradation below 8 Kbps, now superseded by POLQA in many scenarios due to its limitations in handling super-wideband (SWB) signals and modern codecs.

P.563: A no-reference method useful for real-time applications but less adaptability to non-linear distortions (e.g. neural codecs)

ViSQOL: An open-source, full-reference , for low bitrates speech under 8 kbps, the correlation coefficient between ViSQOL and MOS still maintains good consistency[2],  all though it is not formally standardized by ITU-T or 3GPP.

STOI/ESTOI: Focuses on speech intelligibility assessment. provides a computationally efficient solution with high correlation to subjective tests in noisy conditions [3]. Its extended version (ESTOI) further improves robustness to nonlinear distortions (e.g., neural codecs)  [4].But they’re  not formally standardized by ITU-T or 3GPP.

As analyzed above, there is currently no ideal objective evaluation method that simultaneously meets the requirements of ultra low bitrate, has been officially adopted by ITU or 3GPP, and is suitable for AI-driven codecs. Therefore, after excluding several methods unsuitable for ultra-low bitrate scenarios from the aforementioned objective testing approaches, we recommend considering POLQA, ViSQOL, ESTOI or combining POLQA  with either ViSQOL or ESTOI based on test priorities. as potential objective quality assessment method for ULBC.

- Proposal

We propose to document the consideration of optional objective test method for ultra-low bit rate codecs provided above in a pCR to 3GPP TR 26.940 as shown below.

- pCR to 26.940

* * * First Change * * * *

# 9	Test methodologies

Editor’s Note:

5b. Identify appropriate test methodologies, regarding speech quality, intelligibility, conversational quality, in particular taking into account:

-	Clean speech and noisy speech

-	Tandeming with existing IMS voice codecs

-	Clean channel and GEO channel conditions

## 9.1	General

### 9.1.1	Typical quality impairments of ultra-low bit rate speech coding

Speech codecs operating at ultra-low bit rates may impact aspects of speech communication in at least the following categories

-	Loss of listening-only audio quality

-	Audio bandwidth loss

-	Impaired intelligibility

-	Impaired speaker identifiability

-	Prosodic impairments

-	Hallucination, i.e. word and phone confusions

A related quality impairment category is sensitivity to non-speech input. Non-speech input in this context may mean any non-clean speech input such as background noise, music, but also noisy speech, interfering talker speech, reverberant speech.

Additionally, speech codecs operating at ultra-low bit rates may imply speech enhancement algorithms, such as noise suppression, gain normalization etc.

Editor’s note: Investigate impairment relevance in context of use case, e.g. emergency call.

### 9.1.2	Challenges of quality assessment of ultra-low bit rate speech codecs

Testing of ULBC introduces new challenges in comparison to signal processing-based codecs that may necessitate additional or alternative evaluation methods compared to previous 3GPP SA4 speech coding development activities.

Conversation-opinion tests are intended to reproduce, in a laboratory situation, the actual service conditions experienced by telephone customers. [ITU-T P.800 Section 6]. For practical reasons, and due to the need of strict test control, listening-opinion tests are often employed as an alternative in speech coding development.

According to ITU-T Rec. P.800, the recommended test method for listening-only tests is the "Absolute Category Rating" (ACR) method described in Annex B of ITU-T P.800 (see ITU-T P.800 Clause 6.2). An alternative to the Absolute Category Rating method is the Degradation Category Rating (DCR) method which is described in detail in Annex D of ITU-T P.800. As this is a comparative method it is suitable when the impairment (especially digital impairments) is small.

Based on the principles of ITU-T Rec. P.800, 3GPP established a practice of listening-only evaluations in the context of the standardization of its voice codecs. AMR, AMR-WB and EVS codecs were all evaluated using P.800 ACR and (modified) DCR test methodologies, the latter also applied for tests with larger impairments such as noisy speech over poor channel conditions. ACR was generally used for clean speech tests while P.800 DCR was used for SWB clean speech, mixed-bandwidth,  speech + background noise and mixed/music quality evaluations. Other aspects relevant to speech communication, such as impaired intelligibility, impaired speaker identifiability and prosodic impairments were not the focus of testing in AMR, AMR-WB and EVS codec standardization.

For ULBC, these other aspects may need to be addressed more directly, through dedicated tests. Hallucination, for example, is a category that plays only a role in ML-based coding systems but not for signal-processing based codecs, which AMR, AMR-WB and EVS are.

While P.800 recommends ACR as the listening-only test method, it may not be the optimal test to quantify all potential impairments in speech communication such as hallucination, impaired intelligibility and prosodic impairments. These impairments are possible in ML-based coding systems and could be covered by alternative tests such as automatic speech recognition methods, modified rhyme tests and DCR tests (see below).

In contrast, DCR methodology generally focuses on differences to a reference. If such differences are small and pertain to prosodic differences, these may not directly impact the conversational capability of a communication system but can be relevant for other aspects such as identity recognition.

Besides the usage of ACR and DCR, further subjective or objective test methodologies are available that are specifically designed for the evaluation the above-listed impairment categories of ultra-low-rate speech codecs. A non-exhaustive list of such methods is:

-	Diagnostic Rhyme Tests (DRT)

-	Modified Rhyme Tests (MRT)

-	MOS testing for speaker similarity

-	Speaker verification/identification tests

-	Prosodic naturalness MOS tests

-	Intonation recognition tests

-	Transcription tests involving testing for word and semantic equivalence

-	Phoneme recognition tests

-	Automatic speech recognition tests

For objective speech assessment, POLQA, ViSQOL, STOI/ESTOI or combining POLQA  with either ViSQOL or ESTOI can be considered as a supplement to subjective testing methods.

Speech enhancing tool, which may be part of ultra-low bitrate-codecs, are typically evaluated by multi-dimensional rating scales provided by P.835. Here, the quality of speech and the noise suppression capability are evaluated separately.

Editor’s note: Other options for listening methods are invited for documentation.

* * * End of Changes * * * *

- POLQA 2015 (V2.4) Investigated Technical White Paper

- Hines, A., Skoglund, J., Kokaram, A., & Nair, N. (2015). "ViSQOL: An Objective Speech Quality Model."

- Taal, C. H., Hendriks, R. C., Heusdens, R., & Jensen, J. (2011). "An Algorithm for Intelligibility Prediction of Time–Frequency Weighted Noisy Speech."

- Andersen, A. H., de Haan, J. M., Tan, Z.-H., & Jensen, J. (2018). "Refinement and Validation of the Extended Short-Time Objective Intelligibility Measure." IEEE