| 3GPP TS 26.533 V0.1.1 (2025-05) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Diverse audio Capturing System for UEs (DaCAS) (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	UE device types	8

4.1	General	8

4.2.1	Device type 1: Four-Microphone Prototype device	8

4.2.1.1	Overview	8

4.2.1.2	Four-Microphone Prototype Device A	8

4.2.1.3	Four-Microphone Prototype Device B	10

4.2.1.4	Device electro-acoustical characteristics	11

4.2.2	Device type 2: Three-Microphone Smartphone device	11

4.2.2.1	General	11

4.2.2.2	Detailed parameters	11

4.2.2.3	Device electro-acoustical characteristics	13

4.2.3.1	General	13

4.2.3.2	Detailed parameters	13

4.2.3.3	Device electro-acoustical characteristics	14

4.2.4.1	Overview	14

4.2.4.2	Detailed parameters	14

4.2.4.3	The differences between the Four-Microphone Smartphone device 1 and Four-Microphone Smartphone device 2	15

4.2.4.4	Device electro-acoustical characteristics	15

4.2.5.1	General	15

4.2.5.2	Detailed parameters	15

4.2.5.3	Device electro-acoustical characteristics	17

5	Prerequisites	17

5.1	Raw microphone signals	17

5.2	Compensation on microphone signals	17

5.2.1	General	17

5.2.2	Compensation processing	18

6	High-level technical description	18

6.1	Input interfaces	18

6.2	Processing procedure	18

7	Detailed algorithm description	18

7.1	General	18

7.2	Detailed algorithm of example solution 1	18

7.3	Detailed algorithm of example solution 2	18

8	Adaptability of example solutions	18

8.1	General	18

8.2	 Example solution 1	18

8.2	 Example solution 2	18

Annex <A> (informative): Performance of example solutions	20

A.1	Introduction	20

A.2	Test setup	20

A.3	Example solution 1	20

Annex <A> (informative): Change history	21

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

This clause shall start on a new page.

The present document …

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

It is preferred that the reference to TR 21.905 be the first in the list.

# 3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	UE device types

## 4.1	General

UEs are the target devices for immersive audio capture, proposals highlight the need for devices that enable spatial audio processing while addressing legacy hardware constraints. Key goals include standardizing documentation (e.g., coordinate systems for microphone placement) and accommodating diverse form factors. Two primary device categories emerge: analytical prototypes for free-field modeling and practical designs mimicking mainstream smartphones.

## 4.2	Device Types

### 4.2.1	Device type 1: Four-Microphone Prototype device

#### 4.2.1.1	Overview

Prior to launch of the very first immersive voice services, immersive audio capture devices in the market have generally been designed with other use cases in mind. Prototype device designs can in part help to bridge the gap between commercial example devices and new immersive service opportunities, needs, and expectations.

Device type 1 (Four-Microphone Prototype Devices) comprises two separate hardware configurations, denoted Four-Microphone Prototype Device A (clause 4.2.1.2) and Four-Microphone Prototype Device B (clause 4.2.1.3). Both prototype device configurations utilize four microphones for immersive capture thus implementing four-sided capture geometries.

Device A provides a simplified form factor for a more limited use case, whereas device B corresponds to a practical smartphone design including typical complexities related to more generalized audio design. Both prototype designs aim to optimize certain immersive audio capture principles relevant for their respective form factors, e.g., use of symmetrical microphone array design (Device A) and maximizing the microphone array aperture based on non-planar quadrilateral array shape (Device B). The effects of these choices on performance of the example solutions can provide further guidelines for diverse capture designs relevant for immersive voice services and IVAS encoder format generation.

#### 4.2.1.2	Four-Microphone Prototype Device A

Four-Microphone Prototype Device A, illustrated in Figure 4.2.1.2-1, is a prototype immersive audio capture accessory. This is a cylindrical design that could be incorporated, e.g., in a conference room telco equipment or a webcam device. Such device could, e.g., be placed on a table or on top of a display. The device dimensions and the microphone array design are described in more detail in Table 4.2.1.2-1

Table 4.2.1.2-1: Device and microphone array information

| Property | Description |
| --- | --- |
| Device category | Accessory device |
| Device shape | Cylindrical |
| Dimensions | Length 162 mm Width 12 mm (cylinder diameter) Connector bump (HDMI-D connector): Width 20 mm Height 6 mm Protrudes by 3.95 mm |
| Microphone array design | Symmetrical array design Planar array shape Two-dimensional, four-sided capture geometry |
| Microphone port designations | Channel 1 – Mic Front Right Channel 2 – Mic Rear Right Channel 3 – Mic Front Left Channel 4 – Mic Rear Left |
| Microphone port positions | Front microphone ports centred with a 140-mm spacing Rear microphone ports centred with a 140-mm spacing |

The Device A form factor enables a simple two-dimensional, four-sided capture geometry by symmetrical placement of the four microphone ports towards the ends of the device’s outer shell. There is generally minimal acoustic shadowing of any of the microphone ports, especially in ideal recording scenarios. The prototype provides connectivity for recording using an HDMI-D connector on the bottom side of the device, where a protruding shape is apparent.

Four-Microphone Prototype Device A is used in landscape orientation. When device is, e.g., placed on a display, front side (Mic Front Left and Mic Front Right) faces the main user direction.

Figure 4.2.1.2-1: Schematic diagram of Four-microphone prototype device A

#### 4.2.1.3	Four-Microphone Prototype Device B

Four-Microphone Prototype Device B, illustrated in Figure 4.2.1.3-1, is a prototype immersive audio capture smartphone. It utilizes a typical monoblock shape with a camera bump. The device dimensions and the microphone array design are described in more detail in Table 4.2.1.3-1. Specifically, the Device B overall dimensions are derived based on the average dimensions from data collected for 2020-2024 smartphone devices in TR 26.933.

Table 4.2.1.3-1: Device and microphone array information

| Property | Description |
| --- | --- |
| Device category | Smartphone |
| Device shape | Monoblock |
| Dimensions | Length 160.6 mm Width 74.4 mm Depth 8.6 mm Camera bump (for position, see Figure 4.2.1.3-1): Length 35 mm Width 35 mm Depth 4 mm |
| Microphone array design | Practical array design maximizing microphone array aperture Non-planar quadrilateral array shape Three-dimensional, four-sided capture geometry |
| Microphone port designations | Channel 1 – Mic #1 Channel 2 – Mic #2 Channel 3 – Mic #3 Channel 4 – Mic #4 (Mic #5 and Mic #6 not used in Device B configuration) |
| Capture orientations | Landscape Portrait On table (display side up) |

Targeting high-quality immersive audio capture, this device geometry enables a three-dimensional, four-sided capture geometry that is designed to maximize the aperture of the microphone array. The array has a non-planar quadrilateral shape (or skew quadrilateral).

Four-Microphone Prototype Device B can be used in multiple orientations. Main orientations are portrait orientation (see Figure 4.2.1.3-1), landscape orientation (camera bump in top left corner), and flat on table with display face up. The prototype hardware does not include orientation sensors.

Figure 4.2.1.3-1: Schematic diagram of Four-Microphone Prototype Device B

#### 4.2.1.4	Device electro-acoustical characteristics

TBD

### 4.2.2	Device type 2: Three-Microphone Smartphone device

#### 4.2.2.1	General

This three-microphone smartphone is modelled based on a commercially available device that exemplifies current trends in contemporary smartphone design with three built-in microphones

#### 4.2.2.2	Detailed parameters

The parameters of the devices are in the following tables of table 4.2.2.2-1, table 4.2.2.2-2 and table 4.2.2.2-3.

Table 4.2.2.2-1: Device dimensions (mm)

| Height | 163.6 |
| --- | --- |
| Width | 74 |
| Thickness | 8.26 |

Table 4.2.2.2-2: Camera bump size and location (mm)

| Width | 27 |
| --- | --- |
| Height | 33 |
| Thickness | 1.56 |
| xLocation | 6.5 |
| yLocation | 124.1 |

Table 4.2.2.2-3: Microphone port positions on device surface (mm)

| Ch. | Microphone Name | Defining Face | X-axis | Y-axis |
| --- | --- | --- | --- | --- |
| 1 | Bottom Mic | Bottom Face | 25 | 4.16 |
| 2 | Top Mic | Top Face | 62.2 | 3.6 |
| 3 | Camera Mic | Camera Face | 26.5 | 139.9 |

The schematic diagram of the device is in figure 4.2.2.2-1.

6.5 mm

124.1 mm

1.56 mm

27 mm

33 mm

163.6 mm

74 mm

8.26 mm

M1

M2

M3

M1

163.6 mm

74 mm

8.26 mm

M1

M2

M3

M1

Figure 4.2.2.2-1: Schematic diagram of Three-Microphone Smartphone device

#### 4.2.2.3	Device electro-acoustical characteristics

TBD

### 4.2.3	Device type 3: Four-Microphone Smartphone device 1

#### 4.2.3.1	General

This four-microphone smartphone is modelled based on a commercially available device that exemplifies current trends in contemporary smartphone design with four built-in microphones

#### 4.2.3.2	Detailed parameters

The parameters of the devices refer to the following table 4.2.3.2-1.

Table 4.2.3.2-1: the parameters of the device

| No | Device type 3 |
| --- | --- |
| Type | Four-Microphone Smartphone device |
| Size (mm) | 161.4*75.3*9.2 |
| Shape | Rectangular |
| Microphone type | MEMS |
| Number of microphones | 4 |
| Placement of microphones(mm) | Mic1: (-5.2, 80.7, -10.1) Mic2: (-4.9,80.7,9.2) Mic3: (-1.6, -80.2, 7.4) Mic4: (-15.7, -19.2, 0) |

The schematic diagram of the device is in figure 4.2.3.2-1.

Note: Due to measurement accuracy and rounding, the overall length and width of the device dimensions deviate by 0.1 mm between the overall values in table 4.2.3.2-1 and the detailed measurements in figure 4.2.3.2-1.

Figure 4.2.3.2-1: Schematic diagram of Four-Microphone Smartphone device 1

#### 4.2.3.3	Device electro-acoustical characteristics

TBD

### 4.2.4	Device type 4: Four-Microphone Smartphone device 2

#### 4.2.4.1	Overview

This four-microphone smartphone is similar to the device 1 in clause 4.2.3, however, there are some differences between the two devices. The parameters and differences are shown in the following clauses.

#### 4.2.4.2	Detailed parameters

The brief schematic diagram of the device is in figure 4.2.4.2-1.

Mic 3

71.5 mm

7.85 mm

38.21 mm

36.73 mm

Mic 4

160.0 mm

Mic 1

Mic 2

Display

Back plate

5.38 mm

105 mm

Cam Bump Thickness: 1.5 mm
Corner Radius: 6 mm

7.85 mm

160.0 mm

Figure 4.2.4.2-1: schematic diagram of the device

#### 4.2.4.3	The differences between the Four-Microphone Smartphone device 1 and Four-Microphone Smartphone device 2

TBD

#### 4.2.4.4	Device electro-acoustical characteristics

TBD

### 4.2.5	Device type 5: Three-Microphone XR HMD device

#### 4.2.5.1	General

This XR HMD device is modelled based on a commercially available device that exemplifies current trends in contemporary XR HMD design with three built-in microphones

#### 4.2.5.2	Detailed parameters

The basic information and the microphone positions in table 4.2.5.2-1 are as follows:

-	The shape of the proposed XR HMD target device wear on human head

-	The microphone placement color-coded as

-	Orange: mic 0

-	Green: mic 1

-	Red: mic 2

The illustration of the device is in Figure 4.2.5.2-1.

Figure 4.2.5.2-1. XR HMD device

The right-handed coordinate system is defined for the device, and it is shown in Figure 4.2.5.2-2.

z

x

y

Figure 4.2.5.2-2. Right-handed coordinate system for XR HMD device

Table 4.2.5.2-1: The microphone positions of the XR HMD device

| Mic # | X | Y | Z |
| --- | --- | --- | --- |
| 0 | 166.818 mm | -78.55 mm | 46.124 mm |
| 1 | 163.178 mm | -24.027 mm | 11.712 mm |
| 2 | 166.796 mm | 78.457 mm | 46.133 mm |

#### 4.2.5.3	Device electro-acoustical characteristics

TBD

# 5	Prerequisites

## 5.1	Raw microphone signals

TBD

## 5.2	Compensation on microphone signals

Editor’s note: clarifications on compensation and any potential requirements needed

### 5.2.1	General

TBD

### 5.2.2	Compensation processing

TBD

# 6	High-level technical description

## 6.1	Input interfaces

TBD

## 6.2	Processing procedure

TBD

# 7	Detailed algorithm description

## 7.1	General

TBD

## 7.2	Detailed algorithm of example solution 1

TBD

## 7.3	Detailed algorithm of example solution 2

TBD

# 8	Adaptability of example solutions

## 8.1	General

TBD

## 8.2		Example solution 1

TBD

## 8.2		Example solution 2

TBD

Annex <A> (informative):
Performance of example solutions

Editor’s note: consider test methods and requirements

Editor’s note: How to handle the performance reports will be determined later.

# A.1	Introduction

# A.2	Test setup

# A.3	Example solution 1

Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-05 | SA4#132 | S4-250853 |  |  |  | Initial skeleton | 0.0.1 |
| 2025-05 | SA4#132 | S4-251027 |  |  |  | Merge the contribution S4-250935 and the initial skeleton, implement further online edits based on Audio SWG discussion | 0.1.0 |
| 2025-07 | SA4#133-e | S4-251253 |  |  |  | Implement changes based on S4aA250052 and associated requests for improving level of detail in figures, editorial updates | 0.1.1 |