Agenda item: 	10.8

Source: 	Qualcomm Inc.

Title: 	[AvCall-MED] SDP Negotiation of Avatars

Document for	Discussion and Agreement

# Introduction

In this contribution, we propose text for the negotiation of avatar usage in AR Calls.

# Proposed Changes

## 7.3 	SDP Negotiation and Signaling of Avatars

An AR-MTSI client supporting avatar-based communication shall support negotiation of avatar capabilities and animation parameters through SDP.

If avatars are used in a session, the session setup shall determine:

- Avatar model reference and format

- Supported animation frameworks

- Data channel configuration for animation parameters

- Fallback mechanisms for devices without avatar rendering capabilities

The avatar base model shall be stored in the BAR prior to session establishment.

To indicate support for avatar-based communication, an AR-MTSI client shall include the feature tag "+g.3gpp.3d-avatar-animation" in the Contact header field of the SIP REGISTER request, as specified in TS 24.229 [5].

Additional parameters may be included:

- max-avatar-support: indicates the maximum number of simultaneous avatars the device can handle

- supported-animation-frameworks: comma-separated list of animation tracking framework identifiers

The session setup for avatar-based communication shall establish:

- Reference to the avatar base model to be used

- Animation framework compatibility between endpoints

- Data channel for real-time animation parameter transmission

- Bandwidth requirements for animation data

An AR-MTSI client supporting avatars shall use data channels as specified in [2] clause 6.2.10 for transmission of animation parameters. The sub-protocol for avatar animation data channels shall be "3gpp.avatar-animation".

An AR-MTSI client offering avatar-based communication shall include in the SDP offer:

- An application data channel media description for avatar animation data:

m=application [port] UDP/DTLS/SCTP webrtc-datachannel

- Avatar-specific attributes using the following parameters:

Table X: SDP parameters for avatar negotiation

| Parameter | Usage |
| --- | --- |
| a=avatar-ref | Provides the reference to avatar model (e.g., "bar://secure-temp-id" |
| a=avatar-animation-frameworks | Comma-separated list of supported animation frameworks |
| a=avatar-lod | Supported Levels of Detail (e.g., "high,medium,low") |
| a=dcmap | SHALL be included with label "avatar-anim" and subprotocol "3gpp.avatar-animation" |

Example avatar attributes in SDP offer:

a=avatar-ref:bar://users/alice/a1c2b3f4e5937 a=avatar-frameworks:XR_FB_face_tracking2,XR_FB_body_tracking a=avatar-lod:high=100MB,medium=50MB,low=25MB a=dcmap:1000 label="avatar-anim";subprotocol="3gpp.avatar-animation"

The ABNF for avatar attributes is:

avatar-ref-value = "avatar-ref:" avatar-uri

avatar-uri =  "bar://" base-avatar-token

base-avatar-token = secure-temp-id

secure-temp-id = *(ALPHA / DIGIT / "+" / "/") ["==" / "="]

avatar-frameworks = "a=avatar-frameworks:" framework-list

framework-list = framework-id *("," framework-id)

framework-id = 1*alphanum

avatar-lod-value = "avatar-lod:" lod-list

lod-list = lod-value *("," lod-value)

lod-value = ("high" / "medium" / "low" / token) "=" size
size = *(DIGIT) ["GB"/"MB"/"KB"/"B"]

An AR-MTSI client receiving an SDP offer for avatar-based communication shall:

- Examine the avatar capabilities in the offer

- If avatar rendering is supported:

  - Fetch the avatar model using the provided reference from the BAR

  - Verify compatibility with at least one offered animation framework

  - Accept the data channel for animation parameters

- If avatar rendering is not supported:

  - The session may be rejected, or

  - Network-based rendering may be invoked

An MTSI client accepting avatar-based communication SHALL select at least one animation framework from the offered list that it supports. If no common framework is supported, the avatar media SHALL be rejected.

# Proposal

We propose to agree the content of section 2 and integrate it into the base CR.