Source:	Editor (Ericsson LM)

Title:	IVAS-7b: Processing plan for characterization phase, v.0.6.2

Document for:	Discussion and Agreement

Agenda Item:	7.5

# Introduction

In accordance with the IVAS_Codec_Ph2 project plan (IVAS-2b) [1], the IVAS Characterization Processing Plan (IVAS-7b) [2] is supposed to be finalized at the SA4#133-e meeting. An updated version [3] was agreed by Audio SWG on June 17. This contribution highlights remaining issues and proposes further updates and a way forward for completion of the processing plan in IVAS-7b.

# Proposed changes / remaining issues

- Scripts implementing the processing plan for the characterization are available at 3GPP Forge [4]. There is ongoing work updating the scripts to cover missing functionality and adding test configurations for the experiments. Scripts for 6-DoF processing and the recently agreed ACR and Room Acoustics experiments are among the parts which are not yet fully completed.

- The ACR and Room Acoustics test are added to the list of experiments. A description of specific processing for ACR and Room Acoustics experiments is however missing. Additional file naming conventions might also be missing, if needed. For ACR, the definition of additional low-pass anchor conditions is missing.

- The definition of the static downmix to be used for the stereo downmix experiment is missing. In addition, one of the reference codec conditions was missing, which is addressed by this contribution.

- The versions of EVS and the MASA analyzer tools to be used are to be confirmed. For EVS it is proposed to use the latest available fixed-point code version (TS 26.242, v.18.0.0).

- For ITU-T STL processing tools, it is proposed to use the ITU dev branch, which contains the latest versions of the tools.

- A decision on potential attenuation factor for OMASA, as currently applied for OSBA, is still pending.

- The processing of reference codec conditions ISM+HOA3 is clarified, as requested in the Audio SWG call on June 17.

- The details for 6 DoF processing were missing for IVAS rendering (using IVAS_rend). In addition, it is clarified that listener position metadata is excluded from the head rotation trajectory file for 3 DoF rendering.

- Pre-run numbers were added for the ACR experiment. Such randomization numbers should not be needed for the Room Acoustics experiments.

# Summary

It is suggested to discuss the proposed changes and agree on an updated processing plan.

Hopefully, most of the remaining issues can be resolved during SA4#133-e, but since the work on the implementation of scripts is still ongoing, it may be good to allow for any further clarifications or corrections to be agreed prior to the start of the characterization tests. It is therefore proposed to aim for SA4 agreement of IVAS-7b v.1.0.0 at the current meeting (assuming major remaining issues can be resolved) and ask for SA4 power to agree on further corrections/clarification in a subsequent Audio SWG call.

# References

- Tdoc S4-251074, IVAS-2b: IVAS_Codec_Ph2 Project Plan, v.1.3.0

- Tdoc S4-251100, IVAS-7b: Processing plan for characterization phase, v.0.6.0

- Tdoc S4aA250064, IVAS-7b: Processing plan for characterization phase, v.0.6.1

- https://forge.3gpp.org/rep/ivas-codec-pc/ivas-processing-scripts.git