Agenda item: 	10.7

Source: 	Nokia

Title: 	[AvCall-MED] Security considerations for Base Avatar

Document for	Discussion and Agreement

# Introduction

During SA3#122 meeting (May 2025, Fukuoka), SA3 sent a response LS (S3-252296) to SA4 to address the questions on Avatar Security Aspects from SA4 (S4-250715).

In response LS, SA3 proposed the following suggestions and questions:

[SA3 Observation #I.1] According to SA4 the base avatar is a container which may include multiple digital assets and there may be a need to identify each asset part separately. This is currently not reflected in the architecture in TS 23.228 in which the base avatar representation is identified using a single Avatar ID. Moreover, SA4 assumes that a subset of the base avatar parts can be shared to the other call participants which is also not reflected in the architecture and procedures in TS 23.228. Therefore, the security key issues and solutions in TR 33.790 do not take these aspects into account. Additionally, SA3 would like to have clarifications on what the term "user" represents in this context. Is it a subscriber of the sending UE or receiving UE, or something else?

As SA3 points out, it is currently not reflected in the architecture in TS 23.228 for the possibility that the base avatar is a container which may include multiple digital assets and there may be a need to identify each asset part separately.

Therefore, SA4 should further provide clarifications on e.g., how to classify the multiple digital assets under one container for a base avatar, in order to get further information from SA3.

# Proposed Changes

We propose to add the following statement in the base CR.

1st Change

## 5.6.1	General

The base avatar shall include the following information:

-	A list of the static assets, associated with an Avatar ID. Each asset name may correspond to a visual element such as face, eyes, nose, ear, mouth, etc.

- 	A list of the dynamic assets, associated with an Avatar ID. Each asset name may correspond to a visual element such as glasses, clothes, earrings, masks, hats, as well as other accessories.

2nd Change

## A.2	Avatar Communication Call Flows

### A.2.1		General Avatar Call Flow

Figure 24: IMS Avatar Delivery and Animation Flow

Z. Base Avatar Generation Before Call Setup

The base avatar is generated before step A. Call Setup and Capability Negotiation. It may be uploaded to the BAR by using the Avatar management interface defined in annex B.

NOTE: According to the policies and regulations of the applications using avatar, and user’s selection, network may choose different data protecting and validating mechanisms for different assets of the base avatar. E.g., the static assets of avatar could be identified by the network and then protected by the network using its certificate to sign the static assets of the avatar.

The list of Avatar ID(s) and/or Avatar Representations is downloaded to the UE by following options:

Pre-configured in the UE: The Avatar ID List and/or Avatar Representations is provisioned or downloaded to the UE before a data channel for avatar call is setup.,

Through bootstrap data channel: The Avatar ID List is fetched by the DC AS from the BAR when the associated Avatar communication application is downloaded and transferred from the DC AS to the DCSF and downloaded to UE through bootstrap data channel (see details in Annex AC 11.3.1 in TS 23.228[2].

Through application data channel: The Avatar ID List is fetched by the DC AS from the BAR and downloaded to the UE through application data channel (see details in step A.2).

A. Call Setup and Capability Negotiation

A.1 An audio/video session is established between UE1 and UE2 and parameters of the session are negotiated.

Based on security considerations, the capability of protecting and validating the avatar may be negotiated during established ADC for avatar communication.

Editor's note:	How the network will be protecting and validating the avatar as well as its assets will be decided by SA WG3, and the procedure will be aligned further with SA WG3 decision.

NOTE: details of the avatar selection and negotiation are defined in A.1.6.3.

B. Scene Description Retrieval

The MF and the participating UEs retrieve the scene description. The scene description may be shared by the MF with the UEs, in case of a shared experience, or the UEs may have their own scene descriptions.

C. Scene Description Update

A scene update trigger occurs, e.g., if an object is added to or removed from a scene or if spatial information is updated. The update trigger may originate from the MF itself or the UEs. The UEs may update their scene descriptions independently or the MF may generate an updated scene description and share it with the UEs.

NOTE1:	The step B and C are not needed for 2D avatar.

D.1. Avatar Acquisition

D.1.1: The MF loads the base avatar for UE1 from BAR.

D.2. Avatar Delivery

Alternative #1: Sender-centric

D.2a.1: The MF delivers the base avatar of UE 2 to UE1 through data channel.

Alternative #2: Receiver-centric

D.2b.1: The MF delivers the base avatar of UE1 to UE2 through data channel.

D.3. Animation Data Generation

Based on the capability negotiation result in step A, the UE or network may generate animation data.

Alternative #1: UE centric animation data generation

D.3a.1: The UE1 generates the animation data based on the source data (e.g., audio, video, text) or using an XR runtime. The animation data may be transformed from the source data (e.g., from audio to text), or the same as the source data.

D.3a.2: UE1 delivers the animation data to the entity actuating avatar animation through data channel. The animating entity may be the MF or UE2.

Alternative #2: Network centric animation data generation

D.3b.1: UE1 sends source data for animation data generation to the MF over RTP (audio, video, text) or data channel (text).

D.3b.2: The MF processes the received source data to generate animation data during the session. The animation data may be transformed from the source data (e.g., from audio to text, video to motion data), or the same as the source data.

D.3b.3: The MF delivers animation data over data channel to the UE2 animating the base avatar. If network centric avatar animation is used, this step will be skipped. The animation data may be delivered to UE1 as well.

D.4. Avatar Animation

Based on the capability negotiation result in step A, the UE or network may animate the avatar.

Alternative #1a: Sender-centric avatar animation

[Optional] D.4a.1: UE2 delivers its pose information to UE1 for viewer-dependent avatar animation and rendering.

D.4a.2: UE1 animates and renders the base avatar using animation data. The animation data is generated by UE1 in step D.3a.1.1

D.4a.3: UE1 delivers the animated and rendered avatar to UE2. The animated and rendered avatar may be delivered as a 2D video through RTP.

D.4a.4: UE2 corrects the rendered video (for latency compensation) from UE1 before displaying as rendered avatar.

D.4a.5: UE2 delivers a report of its actual display time to UE1 for the monitoring of the UE1 centric rendering service.

Alternative #1b: Receiver-centric avatar animation

D.4b.1: UE2 animates and renders the base avatar using animation data. The animation data may be generated by the MF, following steps D.3b.1 to D.3b.2 and received by UE2 in step D.3b.3 or it may be generated by UE1 in step D.3a.1 and received by UE2 in step D.3a.2.

Alternative #1c: Network-centric avatar animation

D.4c.1: The MF animates and renders the UE1’s base avatar using animation data. The animation data may be generated by the MF, following step D.3b.1 and D.3b.2 or it may be received from UE1 following steps D.3a.1 and D.3a.2.

D.4c.2: The MF delivers the animated and rendered avatar to the UEs. In the figure, delivery to UE2 is shown as example. The animated and rendered avatar may be delivered as a 2D video through RTP.

NOTE2:	Rendering is not needed for 2D avatar.

End of Change

# Proposals

We propose adding the above content into the base CR on Avatar Communication to TS 26.264.