Source:	Xiaomi Technology

Title:		Update database for DaCAS-1

## Document for:	Discussion & Agreement

## Agenda Item:	7.6

Introduction

This contribution aims to update database for device type 3, Summarize all proposed database and included in DaCAS-1

Content

According to scenarios in [3] a new database recorded on sports field is provided and merged with the database in [1] to proposed as same database for device type 3 in the following table:

Update database for device type 3

| No | 3 |
| --- | --- |
| Target device No | Device type 3 |
| Recording scenario No | 4 |
| File name | e.g. Hall_Original-Horizontal_0deg_M-2 |
| UE orientation / placement | Orientation: horizontal and vertical |
| Sample rate | 48kHz |
| Bit depth | 16bit |
| Audio format | .wav |
| Rec Duration | Overall length 32s, first and last 1s is silence for each file |
| Number of channels and microphone mapping: | 4 channels, ch1=mic1, ch2=mic2, ch3=mic3, ch4=mic4 |
| Description/additional info | Environment: hall(RT60=1.65s,500Hz-1000Hz), Outdoor(Sports field) Gender: male(M) and female (W), Language: Chinese Source distance: d_UE = 3m Speaker: Genelec 8030A Source angles (azi, ele): azi = 0°, ±30°, ±60°, ±90°, ±120°, ±150°, 180° ('+'=right '-'=left ),, ele = 0° |

Figure 1: Placement of UE

Figure 2: Overview of the environment

The database has been updated in the previous link:

Link:

https://kpan.mioffice.cn/webfolder/ext/zh%24DE%24XejVH%24uVm31GQvyw%40%40?n=0.1314974258223881

password: 0x7o

Proposed recording Database excel for DaCAS

Based on the ‘Template for recordings database-NOKIA_FUKUOKA_rev1’from [3] , the proposed recording database are detailed in the attached file, ‘draftDaCAS recordings database.xlsx’ for further work.

A formal version will be generated during the meeting and proposed for inclusion in the DaCAS-1, to replace the previous version (Template for recording database.xlsx ) .

Conclusion

In summary, this contribution proposes changes to the attachment about recording database for the completion of DaCAS-1.

## References

- S4-250643:Adding recording scenarios and database

- S4-250962: Proposal on DaCAS Acoustic Properties Measurements

- S4-251026: Database for Four-microphone prototype device A and B

- S4aA250055: Single source scenario for performance evaluation