Source:	Xiaomi

Title:	On minimum performance requirement/objective criteria for raw microphone signals

Document for:	Discussion and Agreement

Agenda Item:	7.6

- Introduction

As per the time plan of DaCAS [1], there is one objective about the raw microphone signals should be agreed as follows:

- Definition of Minimum performance requirement/objective criteria for raw microphone signals.

The document is to reach the objective based on the contents in [2] and some new findings, the basic principle is to make sure that the current and future smartphone devices have the capability to provide stereo sound and even three-dimensional audio, no matter if the devices are high-end, medium-end or low-end ones, by this manner, the immersive audio services can be deployed among various kinds of end-user devices.

- Discussion

According to clause 7.7 in [2] commercial mobile phone devices with 4 microphones face challenges in achieving uniform frequency response and acoustic performance due to placement, mechanical design, and environmental interactions. Microphone locations (e.g., top, bottom, rear, or near camera modules) expose them to varying acoustic structures, such as ports, dust/water protection membranes, and device geometry. These factors create Helmholtz resonators, leading to frequency-specific resonance peaks (often >10 dB at high frequencies, e.g., >10 kHz). Such resonances distort audio capture, equalization filters to flatten responses across all microphones may be needed, which relies on anechoic chamber measurements or numerical simulations to account for device-specific acoustic diffraction and port impedance.

The differences are mainly summarized as follows:

The first one: non-uniform responses: Microphones at different positions (e.g., top/bottom vs. rear) exhibit mismatches in magnitude, phase, and spatial cues due to structural asymmetry

The second one: resonance management, resonances at >10 kHz may degrade spatial accuracy

The third one: acoustic diffraction: the device’s rigid body scatters sound, altering directional cues

The fourth one: microphone spacing: for stereo or spatial capture, spacing (5–17 cm) impacts interchannel time/level differences (ICTD/ICLD). Foldable devices further complicate this with dynamic spacing (e.g., 2–18 cm in book-style models)

To resolve the difference issues, appropriate resolutions are required to flatten microphone responses, compensating for mismatches in magnitude, phase, and spatial cues. This relies on anechoic chamber measurements or numerical simulations (e.g., finite element modeling) to account for device-specific diffraction patterns and port impedance. There are proposals of compensation methods in [3] and [5]. and the proposed microphone signal requirements in [4].

However, the same compensation solution is used for various types of UEs may be problematic concerning the specific capture configurations, UE form factors, UE categories and etc. Several aspects impact how to choose a compensation method.

- Conclusion

Raw microphone signal compensation is essential to achieve audio signals in the desired format. However, to ensure implementation flexibility, two proposals are as follows::

Proposal#1:The compensation approaches outlined in references [3] and [5] are proposed as optional solutions.

Proposal #2: Example solution developers may selectively adopt these solutions based on specific application requirements.

Proposal #3: Raw microphone signa performance relys on the corresponding example solutions.

References

[1] S4-251169, “ Work Plan for DaCAS-v0.4”

[2] 3GPP TR 26.933 “Study on Diverse Audio Capturing system”

[3] S4-250936, “Compensated microphone pressure frequency response measurement”

[4] S4-250948, “On microphone signal requirements”

[5] S4aA250054, “On raw microphone signal compensation”

,