3GPP TSG-SA WG4 Meeting #133-bis-e	S4-251382

Online, 18 – 25 July 2025

Source:	Fraunhofer IIS

Title:	Revision of GEO voice service delay analysis

Spec:	3GPP TR 26.940

Agenda item:	7.9

Document for:	Agreement

1. Introduction

In Tdoc S4-250819, two issues were raised regarding the estimation of the mouth to ear delay:

- Required processing time of the ULBC

- Air interface delay

Both delay sources are part of the UE delay. It is therefore proposed to revise the UE delay estimation.

2. Reason for Change

The air interface delay is a result of the channel simulation activity. According to S4aA250069, three voice bundling periods are considered for ULBC transmission over GEO, i.e. 80ms, 160ms or 320ms. It is assumed, that the available time budget is entirely used for RUs or repetitions to maximize the spectral efficiency. The delay for the air interface is therefore identical to the voice bundling period.

The voice bundling period also impacts the delay of the entire coding process. Before sending the first packet over the air, all audio samples of the voice bundling period needs to be captured and processed by the encoder. The processing time for encoding – and for decoding – can be estimated in relation on the size of the audio buffer which corresponds to the voice bundling period.

Tdoc S4-250819 concludes that in practical product development, trade-off between computational efficiency and power consumption are always considered which may result in a hardware processing delay close to that of the codec frame size. Therefore, the minimum and maximum processing time of the audio buffer size is assumed to be 80% and 95%.

The following figure outlines the UE delay dependencies assuming a voice bundling of 320ms and ignoring the remaining UE delay sources: JBM, vendor specific delay budget, codec delay.

In this case, the UE delay is between 1152 ms and 1248 ms.

The delay consideration changes if the voice bundling period consists of multiple sub-frames where each subframe is coded separately. In that case, the encoding processing can already start in parallel as soon as the first sub-frame is available.  Similar at the decoder side, the audio playout can start as soon as the decoding process of the first sub-frame is completed.

The following figure outlines UE delay dependencies including sub-frames for the codec. In this case, the UE delay is between 768 ms and 792 ms assuming a codec frame size of 80ms meaning 4 sub-frames.

Consequently, the codec frame size and the voice bundling period need to be considered as independent parameters to estimate the mouth to ear delay. Furthermore, the processing delay is dependent on the frame size of the codec. The UE delay can be described then by:

UE delay =

Voice bundling period *2 	(audio capturing and air interface)

+ 2*codec frame size*x%	(encoder & decoder processing)

+ vendor delay budget + JBM + codec alg. delay

The current version of TR 26.940 lists the following UE delay components:

Table 5.1.2-1-A UE delay components

|  | UE delay in ms (Performance objective) (Note 2) | UE delay in ms (Maximum requirement) (Note 2) |
| --- | --- | --- |
| Frame size (Note 1) | 20 | 20 |
| alg. Codec Delay (Note 1) | 5 | 12 |
| JBM (jitter free) (Note 3) | 40 | 40 |
| Vendor specific budget (Note 4) | 83 | 123 |
| UE delay Ts+Tr | 148 | 195 |
| Note 1: Values reflect the IMS codecs AMR/AMR-WB/EVS Note 2: Requirements and Performance Objectives apply to the UE delay only (sum of send (Ts) and receive (Tr) delays) and only for MTSI-based speech-only with LTE, NR or WLAN access in error and jitter free conditions. Note 3: JBM delay is considered as constant independent of the frame size. Note 4: Vendor specific budget of TS 26.131 may change for GEO satellite connectivity |  |  |

Instead of single codec frame size, it is proposed to estimate the UE delay foreach combination of frame size and voice bundling period and taking the following delay values from TS 26.131 into account. In Table 5.1.2-1, the processing delay is part of the vendor specific delay budget. To exclude this budget for an IMS codec, the 80% to 95% for a 20ms frame need to be subtracted resulting in a vendor specific budget in the range of 67ms to 104ms.

Table 5.1.2-1 UE delay constants from TS 26.131

|  | UE delay in ms (Performance objective) (Note 2) | UE delay in ms (Maximum requirement) (Note 2) |
| --- | --- | --- |
| alg. Codec Delay (Note 1) | 5 | 12 |
| JBM (jitter free) (Note 3) | 40 | 40 |
| Vendor specific budget (Note 4) | 83 | 123 |
| Encoder / Decoder Processing time | codec frame size * 0.8 | codec frame size * 0.95 |
| Note 1: Values reflect the IMS codecs AMR/AMR-WB/EVS Note 2: Requirements and Performance Objectives apply to the UE delay only (sum of send (Ts) and receive (Tr) delays) and only for MTSI-based speech-only with LTE, NR or WLAN access in error and jitter free conditions. Note 3: JBM delay is considered as constant independent of the frame size. Note 4: Vendor specific budget of TS 26.131 may change for GEO satellite connectivity |  |  |

Table 5.1.2-2 UE delay estimation for ULBC

| Voice bundling period | Codec frame size | UE delay (Note 1) |  |
| --- | --- | --- | --- |
|  |  | lower bound | upper bound |
| 80 | 20 | 304 | 354 |
|  | 40 | 336 | 392 |
|  | 80 | 400 | 468 |
| 160 | 20 | 464 | 514 |
|  | 40 | 496 | 552 |
|  | 80 | 560 | 628 |
|  | 160 | 688 | 780 |
| 320 | 20 | 784 | 834 |
|  | 40 | 816 | 872 |
|  | 80 | 880 | 948 |
|  | 160 | 1008 | 1100 |
|  | 320 | 1264 | 1404 |
| Note 1: UE_delay = 2x voice bundling period + 2x encoder/decoder processing delay + vendor delay budget + JBM + codec alg. delay |  |  |  |

Finally, the mouth to ear delay can be estimated dependent on the combination of the two parameters voice bundling period and codec frame size while maintaining the delay assumptions for the other network components.

| Voice bundling period | Codec frame size | Mouth to ear delay main scenario in ms (GEO - TN) (Note 1) |  | Mouth to ear delay sub-scenario in ms (GEO - GEO) (Note 2) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | lower bound | upper bound | lower bound | upper bound |
| 80 | 20 | 584 | 871 | 840 | 1314 |
|  | 40 | 616 | 909 | 872 | 1352 |
|  | 80 | 680 | 985 | 936 | 1428 |
| 160 | 20 | 744 | 1031 | 1000 | 1474 |
|  | 40 | 776 | 1069 | 1032 | 1512 |
|  | 80 | 840 | 1145 | 1096 | 1588 |
|  | 160 | 968 | 1297 | 1224 | 1740 |
| 320 | 20 | 1064 | 1351 | 1320 | 1794 |
|  | 40 | 1096 | 1389 | 1352 | 1832 |
|  | 80 | 1160 | 1465 | 1416 | 1908 |
|  | 160 | 1288 | 1617 | 1544 | 2060 |
|  | 320 | 1544 | 1921 | 1800 | 2364 |
| Note 1: UE delay+GEO transmission+Delay_GSCN+Delay_eNBCN Note 2: UE delay +2x GEO transmission+2x Delay_GSCN |  |  |  |  |  |

3. Conclusion

The present document proposes a correction for the UE delay calculation. The proposal considers the voice bundling period and codec frame size and derives a codec processing delay estimation. It should be noted that the choice of voice bundling period and codec frame size may also depend on other criteria beside the UE delay. The voice bundling period may influence the channel robustness while the codec frame size may influence the coding efficiency.

4. Proposal

The following changes are proposed to TR 26.940 V.2.0:

* * * First Change * * * *

## 5.1	Estimation of mouth to ear delay for GEO scenarios

### 5.1.1	Overview

This clause estimates the mouth to ear (M2E) delay for IMS voice call over GEO satellites based on the application scenario introduced in clause 4.2. Two sub-scenarios are considered:

-	Main Scenario (see clause 4.2.2.2): UE1 is connected via satellite while UE2 is connected via terrestrial network which corresponds to the signal flow UE1 àGEO satellite àGround stationàCore networkà eNodeB àUE2

-	Sub-Scenario 1 (see clause 4.2.2.3): Both UEs are connected to a GEO satellite which corresponds to the signal flow UE1 àGEO satellite àGround stationàCore networkàGround stationàGEO satelliteàUE2

This approach aims to estimate the maximum and minimum delay components in the signal flow and finally to estimate a range of the. mouth-to-ear delay accordingly. The estimation assumes jitter free case and no network congestion.

NOTE: In practical deployments, various jitter and network conditions can arise.

Editor’s note: The scenarios and the terminology of this clause needs to be aligned with clause 4.) “Application Scenario” where a detailed description of the call scenarios is expected.

### 5.1.2	Delay components

#### 5.1.2.1	Overview

In this clause, the individual delay components that contribute to the mouth-to-ear delay are introduced and derived. The derived values are independent of the signal flow direction.

#### 5.1.2.2	UE Delay considering IMS codecs

TS 26.131 [D.1] defines the internal UE delay requirements and objectives depending on the components codec (frame size and algorithmic delay), air interface, jitter buffer depth and vendor specific delay budget. The UE delays in sending (UE1) and receiving directions (UE2) are not separated in TS 26.131, however the sum of the sending and receiving delays can be considered together.

The jitter buffer delay budget contains 40ms if the packet duration is 20ms and it includes the expected jitter profiles for terrestrial network transmission. In case of 40ms packet duration, the budget is doubled to 80ms, which is not further discussed. The value for the air interface in [D.1] just reflects the delay between UE and measurement equipment (2 ms) and needs be replaced by the expected delay for real air interface, i.e. air interface to GEO satellite or terrestrial network.

For MTSI-based speech only services with LTE and NR, the UE delay is outlined in the following table:

Table 5.1.2-1 UE delay components from TS 26.131

|  | UE delay in ms (Performance objective) (Note 2) | UE delay in ms (Maximum requirement) (Note 2) |
| --- | --- | --- |
| Frame size (Note 1) | 20 | 20 |
| alg. Codec Delay (Note 1) | 5 | 12 |
| JBM (jitter free) (Note 3) | 40 | 40 |
| Air interface | 2 | 2 |
| Vendor specific budget (Note 4) | 83 | 123 |
| UE delay Ts+Tr | 150 | 197 |
| Note 1: Values reflect the IMS codecs AMR/AMR-WB/EVS Note 2: Requirements and Performance Objectives apply to the UE delay only (sum of send (Ts) and receive (Tr) delays) and only for MTSI-based speech-only with LTE, NR or WLAN access in error and jitter free conditions. Note 3: JBM delay is considered as constant independent of the frame size. Note 4: Vendor specific budget of TS 26.131 may change for GEO satellite connectivity |  |  |

Editor’s note: This table assumes LTE/NR air interface and needs to be updated for GEO satellite access air interface.

For ULBC, the air interface delay for GEO depends on the selected voice bundling periods for the ULBC transmission where 80ms, 160ms or 320ms are considered. It is assumed, that the available time budget is entirely used for RUs or repetitions to maximize the spectral efficiency. The delay for the air interface is therefore identical to the voice bundling period.

The frame size of the codec in Table 5.1.2-1 represents the audio capturing delay which is for GEO identical to the voice bundling period.

The codec frame size determines the processing delay of the encoder and decoder, which is assumed to be in range of 80% to 95% of the codec frame size. In Table 5.1.2-1, the processing delay is part of the vendor specific delay budget. To exclude this budget for an IMS codec, the 80% to 95% for a 20ms frame need to be subtracted.

With this correction for the vendor specific budget and taking the values for algorithmic delay and JBM, the UE delay for ULBC can be estimated depending on bundling period and codec frame size.

Table 5.1.2-2 UE delay estimation for ULBC

| Voice bundling period | Codec frame size | UE delay (Note 1) |  |
| --- | --- | --- | --- |
|  |  | lower bound | upper bound |
| 80 | 20 | 304 | 354 |
|  | 40 | 336 | 392 |
|  | 80 | 400 | 468 |
| 160 | 20 | 464 | 514 |
|  | 40 | 496 | 552 |
|  | 80 | 560 | 628 |
|  | 160 | 688 | 780 |
| 320 | 20 | 784 | 834 |
|  | 40 | 816 | 872 |
|  | 80 | 880 | 948 |
|  | 160 | 1008 | 1100 |
|  | 320 | 1264 | 1404 |
| Note 1: UE_delay = 2x voice bundling period + 2x encoder/decoder processing delay + vendor delay budget + JBM + codec alg. delay |  |  |  |

#### 5.1.2.3	Core network delay

The delay contribution of the core network consists of the packet transmission delay between two network entities, e.g. ground station to core network or core network to eNodeB. In case of the interop scenario GEO NTN to TN network, an additional delay component for transcoding needs to be considered. Assuming the frame size of both codecs is identical or a multiple of each other, only the algorithmic codec delay contributes to the transcoding delay, i.e. 5ms for AMR/AMR-WB or 12ms for EVS, and an additional delay margin for the processing of the transcoding (2 ms). This means, transcoding with AMR/AMR-WB adds 7ms and with EVS adds 14ms.

Table Y.5.1.2.3-1 Core network delay components

|  | Minimum delay in ms | Maximum delay in ms |
| --- | --- | --- |
| Network delay ground station to core network (Delay_GSCN) | [ 5 Note1-1 , 20 Note1-2] | [200 Note2] |
| Network delay eNodeB to core network (Delay_eNBCN) | 5 | 20 |
| Transcoding | 7 | 14 |
| [Note1-1: In [D.2] 5 ms network latency is assumed] [Note1-2: TS 23.501 assumes a static delay value for the CN PDB of 20ms between a UPF and 5G-AN. ] [Note2: In some NTN deployments, the core network may need to be located far from the ground station due to factors like user distribution, geography, or other practical considerations. As a result, latency can increase, ping statistics between continents, for example, can reach up to 200ms.] |  |  |

#### 5.2.2.4	Transmission delay UE – GEO - Ground station

Clause 7.4.2 of [D.2] defines the KPI requirement for GEO based satellite access, i.e. 280ms. TR 36.763 clause 7.1.1 describes the max. and min. propagation delay contribution which depends on the location of the UE within the beam. As a result, the round-trip-delay can differ by 64ms which corresponds 32ms for one-way transmission. It is proposed to consider the 280ms as the max. transmission delay and consequently 248ms (280ms – 32ms) as the minimal transmission time. This assumes no retransmission over the GEO satellite link.

Table 5.1.2.4-1	Transmission delay GEO satellite

|  | Minimum delay in ms | Maximum delay in ms |
| --- | --- | --- |
| GEO transmission delay | 248 | 280 |
| Note: Transmission delay ground station to core network counted in 5.1.2.3-1. |  |  |

#### 5.2.2.5	ULBC Delay components

Table 5.1.2.2-1 lists the algorithmic delay for the IMS codecs AMR and EVS, i.e. in range of 5ms to 12ms. For ULBC, different delay values may result from codec processing delays as well as algorithmic delays. Exact numbers are for further study.

### 5.1.3	Estimation of Mouth-to-ear delay

Given the values in 5.1.2 the mouth-to-ear delay for scenario can be estimated for the two scenarios outlined in 5.1.1 by summing up the delay components according to the signal flow to derive a lower (minimum values as in tables 5.1.2.2-1, 5.1.2.3-1, 5.1.2.4-1) and an upper bound (maximum values as in tables 5.1.2.2-1, 5.1.2.3-1, 5.1.2.4-1).

As the bitrate for GEO satellite link is very restricted, options for minimizing the protocol overhead need to be considered. One option to reduce the protocol overhead are larger frame sizes or a larger voice bundling period as the protocol stack is transmitted less often. Therefore, the following table outlines the delay values for codec frame sizes of 20ms, 40ms, 80ms, 160ms and 320ms and the voice bundling period 80ms, 160ms and 320ms.

Editor’s Note: Current values assume algorithmic delay of AMR and EVS as given in 5.1.2.2-1. ULBC Delay components documented in 5.1.2 still need to be addressed. For the min. Delay_GSCN, 20ms is assumed.

Table 5.1.3-1	 Mouth-to-ear delay estimation depending on codec frame size

| Voice bundling period | Codec frame size | Mouth to ear delay main scenario in ms (GEO - TN) (Note 1) |  | Mouth to ear delay sub-scenario in ms (GEO - GEO) (Note 2) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | lower bound | upper bound | lower bound | upper bound |
| 80 | 20 | 584 | 871 | 840 | 1314 |
|  | 40 | 616 | 909 | 872 | 1352 |
|  | 80 | 680 | 985 | 936 | 1428 |
| 160 | 20 | 744 | 1031 | 1000 | 1474 |
|  | 40 | 776 | 1069 | 1032 | 1512 |
|  | 80 | 840 | 1145 | 1096 | 1588 |
|  | 160 | 968 | 1297 | 1224 | 1740 |
| 320 | 20 | 1064 | 1351 | 1320 | 1794 |
|  | 40 | 1096 | 1389 | 1352 | 1832 |
|  | 80 | 1160 | 1465 | 1416 | 1908 |
|  | 160 | 1288 | 1617 | 1544 | 2060 |
|  | 320 | 1544 | 1921 | 1800 | 2364 |
| Note 1: UE delay+GEO transmission+Delay_GSCN+Delay_eNBCN Note 2: UE delay +2x GEO transmission+2x Delay_GSCN |  |  |  |  |  |

Editor’s note: The scenarios and the terminology of this clause needs to be aligned with clause 4.) “Application Scenario” where a detailed description of the call scenarios is expected.

* * * End of Changes * * * *