3GPP TSG-SA WG4 Meeting #133-bis-e	S4-251394

Online, 18 – 25 July 2025

Source:	Fraunhofer IIS

Title:	Pseudo-CR on Design Constraints of ULBC: Audio bandwidth

Spec:	3GPP TR 26.940

Agenda item:	7.9

Document for:	Decision

1. Introduction

TR 26.940 V.2.0 (S4-251152) currently only lists a template for the design constraints. This document proposes design constraints which can be considered as independent of other tasks, e.g. channel simulation.

2. Reason for Change

Providing design constraints for ULBC regarding “Sample rate and audio bandwidth”.

3. Proposal

DC: Sample rate and audio bandwidth

Sample rate and audio bandwidth are typically described by the terms

- Narrow band (NB) which means support of 8kHz sampling rate

- Wide band (WB) which means support of 16kHz sampling rate

- Super Wideband (SWB) which means support of 32kHz sampling rate

- Fullband (FB) which means support of 48kHz sampling rate

However, the resulting audio bandwidth may slightly differ depending on the used 3GPP IMS voice codec:

| Codec | Sampling rate | Audio bandwidth |
| --- | --- | --- |
| AMR | 8 kHz | Up to 3.5 kHz |
| AMR-WB | 16 kHz | Up to 7 kHz |
| EVS | 8, 16, 32, 48 kHz | Up to 4, 8, 16, 20 kHz |

It should be noted that only EVS provides an audio bandwidth up to the Nyquist frequency, except for FB. It is proposed to maintain this concept for ULBC.

In order to assess the importance of NB, WB, SWB and FB, the following categories are considered:

Intelligibility

The intelligibility difference of NB and WB is for instance compared in [1] where it is shown that the confusion rate between certain logatomes is significantly reduced when extending the audio bandwidth from NB to WB. According to [2], speech energy above 8 kHz contributes significantly to consonant intelligibility meaning the effect gets even continued for SWB.

Speaker Identification

Higher audio bandwidth is also beneficial for the speaker identification over telecommunication systems. As shown in [3], WB and SWB can improve the speaker identification compared to NB.

Service continuation

Besides emergency calls, ULBC over GEO is also intended as NTN based service extension of TN networks. Nowadays, IMS voice services typically support WB or SWB. This creates a certain user expectation which needs to be continued when moving the voice service to NTN.

Support by technology

Table 7.1.1-1 in TR 26.940 provides an extensive list of modern AI-based coding technologies. The minimum audio bandwidth supported is WB, but even higher bandwidths are supported at low bitrates. As shown for instance in [4] SWB speech may even come with zero overhead regarding the bit rate as blind bandwidth extension while providing a significant quality improvement.

Conclusion

Given the arguments above, it is proposed to focus the support of sampling rate for ULBC to WB at 16 kHz and SWB at 32 kHz. An audio bandwidth up to the Nyquist frequency should be supported.

Limiting the number of supported modes also helps organizing the testing phase.

It is proposed to agree the following changes to 3GPP TR 26.940.

* * * First Change * * * *

Table 6.2-1 List of ULBC design constraint parameter

| Parameter | Design Constraint | Note |
| --- | --- | --- |
| Bit rates |  |  |
| Sample rate and audio bandwidth | The ultra low bitrate codec supports the following sampling rates: 16kHz (WB) and 32kHz (SWB). The supported audio bandwidth for - WB ranges from 50 – 8000Hz - SWB ranges from 50 – 16000Hz |  |
| Frame length |  |  |
| Complexity and memory demands |  |  |
| Algorithmic delay |  | The algorithmic delay is defined as the frame size buffering delay plus any other delays inherent in the codec algorithm (e.g., look-ahead, sample-rate conversion, and decoder post-processing) |
| Packet loss concealment (PLC) |  |  |
| Potential use of noise suppression as part of the codec |  |  |
| Discontinuous transmission including voice activity detection and comfort noise |  |  |
| Robustness to non-speech input |  | Editor’s note: May need to be in performance requirement |

* * * End of Changes * * * *

# References

[1]		Laura Fernandez Gallardo, Sebastian Möller, Phoneme Intelligibility in Narrowband and in Wideband Channels, DAGA 2015, Nürnberg

[2]		R. P. Lippmann, "Accurate consonant perception without mid-frequency speech energy," in IEEE Transactions on Speech and Audio Processing, vol. 4, no. 1, pp. 66-, Jan. 1996

[3]		Gallardo, L. F., & Köster, F. Comparison of Transmission Quality Dimensions of Narrowband, Wideband, and Super-Wideband Speech Channels.

[4]		E. Gómez, M. H. Vali and T. Bäckström, "Low-Complexity Real-Time Neural Network for Blind Bandwidth Extension of Wideband Speech," 2023 31st European Signal Processing Conference (EUSIPCO), Helsinki, Finland