3GPP TSG-SA WG4 Meeting #133-bis-e	S4-251395

Online, 18 – 25 July 2025

Source:	Fraunhofer IIS

Title:	Pseudo-CR on Design Constraints of ULBC: Noise suppression

Spec:	3GPP TR 26.940

Agenda item:	7.9

Document for:	Decision

1. Introduction

TR 26.940 V.2.0 (S4-251152) currently only lists a template for the design constraints. This document proposes design constraints which can be considered as independent of other tasks, e.g. channel simulation.

2. Reason for Change

Providing design constraints for ULBC regarding “Potential use of noise suppression as part of the codec”

3. Proposal

DC: Potential use of noise suppression as part of the codec

TR 26.940 contains a list of ultra-low bitrate codecs, conventional ones as well as AI-based coding systems, see Table 7.1.1-1. Conventional and AI-based codecs, both may include a noise suppression functionality. Especially for the ultra-low bitrate operation, this seems to be an essential tool allowing good audio quality and intelligibility.

In clause 9 (Test methodologies) of TR 26.940, it is also stated that “speech codecs operating at ultra-low bit rates may imply speech enhancement algorithms, such as noise suppression, gain normalization etc”. To prepare the testing methodologies properly, the functionality of noise suppression should be clarified as soon as possible. Therefore, it is proposed that the design constraints should state that noise suppression should be allowed or even expected for ULBC. As TR 26.940 already mentions other speech enhancements, an editor’s note is proposed to capture this.

It is proposed to agree the following changes to 3GPP TR 26.940.

* * * First Change * * * *

Table 6.2-1 List of ULBC design constraint parameter

| Parameter | Design Constraint | Note |
| --- | --- | --- |
| Bit rates |  |  |
| Sample rate and audio bandwidth |  |  |
| Frame length |  |  |
| Complexity and memory demands |  |  |
| Algorithmic delay |  | The algorithmic delay is defined as the frame size buffering delay plus any other delays inherent in the codec algorithm (e.g., look-ahead, sample-rate conversion, and decoder post-processing) |
| Packet loss concealment (PLC) |  |  |
| Potential use of noise suppression as part of the codec | Noise suppression functionality is expected to be integrated in ULBC Editor’s note: Further speech enhancements are TBD |  |
| Discontinuous transmission including voice activity detection and comfort noise |  |  |
| Robustness to non-speech input |  | Editor’s note: May need to be in performance requirement |

* * * End of Changes * * * *