Source:	Fraunhofer IIS

Title:	Combined format items for characterization testing

Document for:	Discussion and Agreement

Agenda Item:	7.5

# Introduction

In the IVAS-8b Test Plan [1], it is expected that preproduced content will be provided by the contributing parties for the combined format experiments: P.800-16, P.800-17, P.800-19, and P.800-20. This content will be used as samples for the respective P.800 tests for characterization. Requirements have been established that describe the scenes of this content per category.

For experiments P.800-17, P.800-19, and P.800-20, it is not explicitly defined whether the objects in the respective categories are speech objects. However, for P.800-16, this is clearly stated.

The requirements for P.800-16 for OSBA with 3-4 objects are as follows:

| Test item generation: pre-processing incl. spatialization | Cat. 1-3: Pre-produced content, 3 ISMs + FOA or HOA3 Cat. 4-6: Pre-produced content, 4 ISMs + FOA or HOA3 |
| --- | --- |

Table F.16.4: Clean and noisy speech categories and scene definitions for OSBA

| Category | Type |
| --- | --- |
| cat 1, 4 | 3-object, 4-objects respectively + ambient background (office, nature, outdoor/indoor environment), where objects are speech (including overlap with max. 2 talkers) |
| cat 2, 5 | 3-object, 4-objects respectively + mixed and music where objects are speech (including overlap with max. 2 talkers) |
| cat 3, 6 | 3-object, 4-objects respectively + General audio background, where objects may be speech, music or/and effects |

Additionally, in clause 4.3.1 of [1], it is stated that the length of the samples for P.800 tests should be between 7-8 seconds. This limitation poses challenges for producing items with 4 objects in categories 4 and 5. Clean complete mono sentences (e.g. Harvard sentences or equivalent phonetically balanced corpora) for objects would be approximately 1.5-3.5 seconds each, along with required pauses between sentences. This necessitates overlap when transitioning from one talker object to another (with a maximum of 2 talkers allowed to overlap) as seen in Figure 1. Given the complexity of the combined formats, having multiple overlapping talkers for a short duration can increase the overall complexity of the test sample.

As it is known, P.800 tests are conducted with naïve listeners, and evaluating complex scenes is already challenging, even for expert listeners. Therefore, special attention should be given to ensuring that the generated preproduced material is as realistic and pleasant to listen to as possible, avoiding any potential frustration. Even more so since the entire experiment would consist of such rich samples.

Figure 1

File length = 8.0 s

Minimum

0.5 s

Minimum

0.5 s

Sentence 1

Sentence 4

Sentence 3

Sentence 2

# Proposal

Given the above discussion, it is proposed that the production of combined formats with 4 objects is kept flexible, without the requirement to have 4 speech objects. The proposed changes to Table F.16.4 are as follows:

| Category | Type |
| --- | --- |
| cat 1, 4 | 3-object, 4-objects respectively + ambient background (office, nature, outdoor/indoor environment), where objects are speech (including overlap with max. 2 talkers) but for 4-objects one object may be music/or effects |
| cat 2, 5 | 3-object, 4-objects respectively + mixed and music where objects are speech (including overlap with max. 2 talkers) but for 4-objects one object may be music/or effects |
| cat 3, 6 | 3-object, 4-objects respectively + General audio background, where objects may be speech, music or/and effects |

This change does not exclude the production of combined format material with 4 speech objects. It will be up to the MC entity to decide whether the specific material is suitable for the testing procedure. However, the source urges the material-contributing parties to produce content that is engaging and does not cause frustration or confusion for either naïve or expert listeners.

# Conclusion

It is proposed to agree on the proposal and update [1] accordingly.

# References

- S4-251138- IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase v.0.7.0