3GPP TSG SA4-133-e	S4-251421

Electronic Meeting, 18th July – 25th July 2025

3GPP TSG-CT WG1 Meeting #155	C1-254120

# Bratislava, Slovakia, 19-23 May 2025

# Title:	Reply LS on Encoding of (S)RTP Multiplexed Media Identification Information

# Response to:	LS (C4-251476) on Encoding of (S)RTP Multiplexed Media Identification Information from CT4

# Release:	Release 19

# Work Item:	XRM_Ph2

Source:	CT1

To:	CT4, SA2

Cc:	SA4, CT3

Contact Person:

Name:	Yumei Song

Tel. Number:

E-mail Address:	yumei.song@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	C1-253663

1. Overall Description:

CT1 thanks CT4 for the LS on Encoding of (S)RTP Multiplexed Media Identification Information.

CT1 has discussed the question raised by CT4 on 1) Alignment of encoding of the MID information; and 2) (S)RTP multiplexed media identification information entry to be used by both (S)RTP and (S)RTCP traffic.

CT1 agreed to align with CT4 encoding of (S)RTP multiplexed media identification information with the deviations:

1. In CT1, the length of the MID identification-tag is not added as a new parameter, instead, the MID identification tag is encoded as length of the MID identification-tag which is the first one octet and followed by value of the MID identification-tag;

2. CT1 refers to RFC 3550  only for SSRC encoding;

3. In CT1, the RTCP packet type field is encoded as a one-octet payload type field, which contains the binary representation of an integer between 200 and 204 (inclusive).

In addition, CT1 would like to point out that, as an implementation option, apart from a single entry of (S)RTP multiplexed media identification information to apply to both (S)RTP and (S)RTCP traffic, a (S)RTP multiplexed media identification information can include one (S)RTP multiplexed media identification information entry for (S)RTP and one (S)RTP multiplexed media identification information entry for (S)RTCP to make the (S)RTP multiplexed media identification information applicable to both (S)RTP and (S)RTCP traffic.

2. Actions:

To CT4 group.

ACTION:

CT1 kindly asks CT4 to take the above into consideration.

To SA2 group.

ACTION:

CT1 kindly asks SA2 to take the above into consideration and consider to update their TS accordingly.

3. Date of Next CT1 Meetings:

CT1#156                           25th August- 29th August 2025.....................Goteborg, Sweden

CT1#157                           13th October- 17th October 2025..................Sophia Antipolis, France

CT1#158                           17th November- 21st November 2025............Dallas, US

CT1#159                           9th February- 13th February 2025..................India