Source:	Cadence Design Systems Inc., Dolby Laboratories Inc., Ericsson LM, Fraunhofer IIS, Huawei Technologies Co Ltd., Nokia, NTT, Orange, Panasonic Holdings Corporation, Philips International B.V., Qualcomm Incorporated, VoiceAge Corporation

Title:	Verification of IVAS BASOP Maintenance

Document for:	Discussion and Agreement

Agenda Item:	7.5

# Introduction

The floating-point code of the IVAS codec was standardized in Rel-18 in TS 26.258 [1]. The task to convert the floating-point implementation to fixed-point (BASOP) code is being done by Ittiam, under a contract issued by ETSI. The verification of the IVAS BASOP Decoder and Renderer can be found in [2], Annex A and the verification of the IVAS BASOP Encoder can be found in Annex B.

Both deliveries were found to be completed to a level of 90%, and the remaining issues were to be addressed in a maintenance phase ending on July 18, 2025. This document covers the verification of the IVAS BASOP maintenance. The focus is on the change in objective quality measures (MLD, PEAQ and SSNR) and the complexity measures (WMOPS and Memory) as described in [2], Annex A and B.

# Change in objective quality measures

## Encoder

| Thu May 8 11:29:37 2025, Commit: 858df806 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operation | MLD < 10 | Max MLD | MIN_ODG > -0.1 | Min MIN_ODG |
| Static | 81% | 82.96 | 10% | -3.69 |
| Flexible | 47% | 115.12 | 0% | -3.75 |
| All | 71% | 115.12 | 7% | -3.75 |

| Fri Jul 18 04:01:04 2025, Commit: b480684c |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operation | MLD < 10 | Max MLD | MIN_ODG > -0.1 | Min MIN_ODG |
| Static | 81% | 99.19 | 11% | -3.83 |
| Flexible | 48% | 106.25 | 0% | -3.73 |
| All | 71% | 106.25 | 8% | -3.83 |

## Decoder/Renderer

| Fri Oct 25 11:09:47 2024, Commit: 8f97e97c |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operation | MLD < 5 | Max MLD | MIN_SSNR > 60 | Min MIN_SSNR |
| Static | 76% | 61.58 | 1% | 21.42 |
| Flexible | 58% | 73.55 | 0% | -9.43 |
| Renderer | 99% | 9.48 | 71% | 10.71 |
| All | 86% | 73.55 | 38% | -9.43 |

| Fri Jul 18 04:01:04 2025, Commit: b480684c |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operation | MLD < 5 | Max MLD | MIN_SSNR > 60 | Min MIN_SSNR |
| Static | 88% | 69.41 | 3% | 20.90 |
| Flexible | 75% | 91.96 | 2% | -12.54 |
| Renderer | 95% | 27.22 | 65% | 13.82 |
| All | 88% | 91.96 | 36% | -12.54 |

# Change in complexity

## WMOPS ratio

The WMOPS ratio between BASOP and float is extracted from the test cases in the daily complexity evaluations on [3]. The target ratio 0.80 is defined by EVS BASOP vs float, as shown in [2], Annex A and Annex B.

Table 1: WMOPS ratios BASOP vs float

|  | Decoder/Renderer Nov 4, 2024 a41b2422 | Encoder May 14, 2025 994b6859 | Decoder/Renderer July 18, 2025 b480684c | Encoder July 18, 2025 b480684c |
| --- | --- | --- | --- | --- |
| WMOPS ratio | 1.95 +/- 0.83 | 1.17 +/- 0.25 | 1.19 +/- 0.22 | 1.12 +/- 0.18 |
| Relative to target | 2.43 | 1.46 | 1.49 | 1.40 |

## Memory

The memory ratios are computed in a similar way as the WMOPS ratios, where the maximum heap and stack usage ratio is computed per test case, and then averaged over all test cases. The ROM is constant for all test cases. Note that the PROM and TROM values are not available for the first delivery of the Decoder/Renderer, and that the target ratios for EVS Decoder is missing.

Table 2: Memory ratios BASOP vs float.

| Category |  | Decoder/Renderer Nov 4, 2024 a41b2422 | Encoder May 14, 2025 994b6859 | Decoder/Renderer July 18, 2025 b480684c | Encoder July 18, 2025 b480684c | EVS Target Decoder | EVS Target Encoder |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RAM | HEAP | 1.56 | 0.89 | 0.91 | 0.89 | - | 0.53 |
|  | STACK | 1.86 | 1.16 | 1.90 | 1.18 | - | 0.75 |
| ROM | PROM | - | 0.92 | 1.02 | 1.14 | - | 0.74 |
|  | TROM | - | 0.96 | 0.82 | 0.84 | - | 0.90 |

# Summary of issues at 3GPP Forge

The issue tracker on [4] contains the observed issues with the current IVAS BASOP code. It contains issues assigned to Ittiam as well as issues to be handled by the group of PC companies. Since the Encoder report on May 16 2025, there are in total 99 issues created and 150 issues closed (July 18, 2025). An illustration of the total number of issues during the IVAS BASOP conversion project is shown in Figure 1. Note that the figure does not reflect if the issues are assigned to Ittiam or the PC group, since the assignee is not tracked over time. An overview of the current open issues is found in Table 1 and Table 2.

Figure 1: Illustration of number of issues during the IVAS BASOP project.

Table 1: Open issues per module

| 16 May 2025 |  |  |  |
| --- | --- | --- | --- |
| Ittiam | 83 | Encoder | 51 |
|  |  | Decoder/Renderer | 31 |
|  |  | Other | 1 |
| PC/ unassigned | 71 | Encoder | 31 |
|  |  | Decoder/Renderer | 25 |
|  |  | Other | 15 |

| 18 July 2025 |  |  |  |
| --- | --- | --- | --- |
| Ittiam | 0 | Encoder | - |
|  |  | Decoder/Renderer | - |
|  |  | Other | - |
| PC/ unassigned | 90 | Encoder | 34 |
|  |  | Decoder/Renderer | 30 |
|  |  | Other | 26 |

Table 2: Open issues with prioritization labels

| 16 May 2025 |  |  |  |
| --- | --- | --- | --- |
| Assignee |  | Priority label |  |
| Ittiam | 83 | Blocker | - |
|  |  | Critical | 9 |
|  |  | Major | 35 |
|  |  | Medium | 29 |
|  |  | Minor | 4 |
|  |  | No priority | 6 |
| PC/ unassigned | 71 | Blocker | - |
|  |  | Critical | 10 |
|  |  | Major | 26 |
|  |  | Medium | 14 |
|  |  | Minor | 7 |
|  |  | No priority | 14 |

| 18 July 2025 |  |  |  |
| --- | --- | --- | --- |
| Assignee |  | Priority label |  |
| Ittiam | 0 | Blocker | - |
|  |  | Critical | - |
|  |  | Major | - |
|  |  | Medium | - |
|  |  | Minor | - |
|  |  | Trivial | - |
|  |  | No priority | - |
| PC/ unassigned | 90 | Blocker | - |
|  |  | Critical | 4 |
|  |  | Major | 22 |
|  |  | Medium | 36 |
|  |  | Minor | 19 |
|  |  | Trivial | 1 |
|  |  | No priority | 8 |

# Summary

The evaluation shows that the most critical issues have been resolved by Ittiam, and that the objective quality scores have improved. The conclusion is that the IVAS fixed-point maintenance has been delivered by Ittiam, and that the maintenance milestone as defined in [4], is reached.

# References

- TR 26.258 Codec for Immersive Voice and Audio Services (IVAS); C code (floating-point)

- IVAS-10 IVAS BASOP Verification (IVAS-10), https://www.3gpp.org/ftp/tsg_sa/WG4_CODEC/TSGS4_132_Fukuoka/Docs/S4-251012.zip

- IVAS BASOP repository on 3GPP Forge, https://forge.3gpp.org/rep/sa4/audio/ivas-basop

- S4-212460, Status of IVAS fixed-point conversion, https://www.3gpp.org/ftp/TSG_SA/WG4_CODEC/TSGS4_130_Orlando/Docs/S4-242160.zip