3GPP TSG SA WG4#133-e	S4-251532

# Online, 17 – 25 July 2025

Source:	China Mobile Com. Corporation

Title:	[FS_Beyond2D] Gaps and Optimization Potential

Agenda item:	9.6

Document for:	Agreement

1. Introduction

This proposal provides a gap analysis and identifies potential optimizations for FS_Beyond2D.

2. Proposal

It is proposed to agree the following changes to the 3GPP draft TR 26.956 V1.0.0

* * * First Change * * *

10	Gaps and Optimization Potential

10.1	Identified Gaps and Deficiencies with Video Capabilities

The Technical Report does not explicitly identify gaps or deficiencies with existing video capabilities in 3GPP standards needing immediate standardization. The focus of this study is primarily on evaluating existing and emerging Beyond 2D video representation formats where the principal findings are:

- 	Stereoscopic Video (see clause 4.3.2): A comprehensive evaluation has been performed revealing that:

-	Stereoscopic Video can be supported by existing content production workflows and captured by emerging mobile devices on the market. It also can be generated from monocular video using AI-based algorithms, decoding and rendering can be performed on the device. It has been observed that the industry is beginning to adopt this format.

-	The AI-based stereoscopic video generation methods often experience artifacts such as edge sharpness mismatch, cardboarding effects and crosstalk. These artifacts are especially noticeable along the contours of foreground objects in a scene. Therefore the objective metric for generated stereoscopic video emphasizing edge region evaluation has been described in clause 7.2.5.2.

-	Both Simulcast HEVC and MV-HEVC codecs have been tested for stereoscopic video.

-	Identified gap: While 3GPP specification TS 26.265 provides support for representation format and codec to support streaming of stereoscopic video content, the extension for stereoscopic representation formats and compression options is worth to be studied in future work.

-	Dense Dynamic Point Clouds (see clause 4.3.3) and Dynamic Meshes (see clause 4.3.5): A comprehensive evaluation has been performed revealing that:

-	Dense dynamic point clouds and dynamic meshes are currently the primary representation formats to produce and deliver camera captured volumetric video. However, due to rather complex production systems (multiple or many calibrated cameras are needed), no widespread adoption of these workflows have yet happened, and the number of produced content items is limited. Work on MPEG Video-based Dynamic Mesh Coding V-DMC [DM-20] for coding dynamic meshes was not finished at the closure date for the first version of the Technical Report and therefore the evaluation has been concentrated on dense dynamic point clouds with MPEG V-PCC [Vol-11] as the codec.

-	Dense dynamic point clouds with around 2 million points per frame encoded with MPEG V-PCC at 20 to 30 Mbit/s using the HEVC video codec provides a satisfying quality for the described scenario. See visual quality examples in clause 7.3.4.5 and videos in clause 9.X.4.1.5.2 subjective evaluation.

-	Rendering of dense dynamic point clouds can produce holes and needs special care. The effect of very simple cube based rendering and simple splat blend-based rendering is investigated, where the latter mitigates the problem. While rendering is considered an implementation aspect and is typically not normatively defined in coding standards, for proper user experience, sufficiently good rendering is essential. See impact of rendering in clause 4.3.3.3. Based on this, if consistent end-to-end quality is of concern for a service provider, potential requirements or recommendations on rendering performance may need to be defined.

-	Real-time decoder implementation of the MPEG V-PCC profile "HEVC Main10 V-PCC Basic Rec0" supporting up to 2-million-point points per frame on off the shelf consumer device has been demonstrated [Vol-8] re-using hardware-implemented HEVC decoders, but performance (CPU, power consumption) of such a hybrid SW/HW solution is for further study.

-	3GPP provided a study on 6G use cases and services requirements in TR 22.870 [Vol-36]. Clause 9.12 of this report describes a use case on personalized interactive immersive guided tour, where assets represented as volumetric video are part of the scene.

-	Identified gap: There is no support in 3GPP specifications for representation formats and codecs to support streaming of professionally produced single asset volumetric video. However, services and applications potentially wanting to use such representation formats may be implemented based on the hybrid HW/SW solution following the demonstration, and the existing HEVC-capabilities in 3GPP specifications would be sufficient to support the use case.

-	Multi-view plus depth (see clause 4.3.4): A comprehensive evaluation has been performed revealing that:

-	Multi-view video enables the viewer to interact with the content by seamlessly moving and reorienting a virtual viewport. This serves two goals from the user perspective: it is possible to look around objects, and it is possible to freely choose a viewpoint. The latter is arguably less immersive, but enables the viewer to observe an action in more detail, i.e. by "being the director".

-	There is at least one codec (MIV) including publicly available encoding, decoding and rendering software that is capable of representing multi-view plus depth content. A real-time decoder and renderer for MIV is publicly available in 5G-MAG. A second codec (MV-HEVC) has been identified but has not been tested in this study.

-	By design multi-view plus depth content is not required to capture a scene from all angles. For instance, all cameras may be on one side of a scene, like in a studio for 2D video. A user has the freedom to move within a constrained volume (the viewing space).

-	There are no commercial end-to-end content production pipelines for this representation format (including tools for camera calibration and depth estimation), which hampers the adoption of the multi-view plus depth representation for practical applications.

-	Coding of realistic scenes needs a high luma sample rate to achieve an acceptable quality, resulting in higher HEVC levels or more HEVC decoders as compared to 2D video. The luma sample rate depends on the number of cameras and their separation, with specific examples provided in this report.

-	Identified gap: There is no support in 3GPP specifications for representation formats and codecs to support streaming of multi-view plus depth content.

10.2	Potential Requirements for New Video Capabilities

From the collecting scenario, future 3GPP standards may need to consider the following new video capability requirements:

-	Extensions for Stereoscopic Video: The collected scenario one indicates a need for enhanced support for stereoscopic video formats to enable more immersive Beyond 2D experiences. This aspect has been addressed in TS 26.265 [26265] and its potential next phase

-	Monitoring Market Adoption of New Beyond 2D Formats: A comprehensive evaluation of emerging Beyond 2D formats, including point clouds, multi-view has been performed. The focus remains on continuous monitoring the market traction of these technologies, especially in content generation. Due to time constraints, Dynamic Mesh was not evaluated in this Technical Report and needs to be addressed in subsequent work. Immediate standardization is not required at this time and MPEG V-PCC and MPEG MIV remain candidate codecs for integration in 3GPP specifications in a future release. A potential future requirement for 3GPP is to define or support representations format(s) and codec(s) for streaming of produced volumetric video with single asset and for streaming of produced multiview plus depth video.

-	Gaussian Splatting: 3DGS gets a lot of attention from academia and industry, but realistic use cases are not yet clear, the format is not yet stabilized and there is no codec from a recognized standards organization. Further study is needed and immediate standardization is not required.

-	AI-Generated Beyond 2D content: The commercialization of AIGC has attracted attention from both academia and industry. This TR introduces AI-generated stereoscopic video, dynamic mesh and 4D content. However, further study is needed to improve quality and efficiency of AI generated content, as well as to develop the quality assessment methodologies.

10.3	Potential Network Optimizations

The network optimizations were not directly addressed in this study and potential needs can be inferred from the introduction of new video capabilities:

-	Transmission Efficiency: More efficient transmission methods (e.g., protocols or distribution strategies) for these more complex video data.

-	Bandwidth Optimization and Network Capabilities: Beyond 2D video technology involves processing, transmitting, and storing massive amounts of data over 3GPP networks, which presents significant challenges to both network bandwidth and user equipment (UE) computational capabilities. Therefore, exploring efficient network solutions and bandwidth optimization is critical for enabling real-time B2D video delivery across a wide range of viewing experiences without sacrificing the sense of immersion.

* * * End of Changes * * *