Agenda item: 	10.8

Source: 	Qualcomm Inc.

Title: 	[AvCall-MED] SDP Negotiation of Avatars

Document for	Discussion and Agreement

# Introduction

In this contribution, we propose text for the negotiation of avatar usage in AR Calls.

# Proposed Changes

## 7.3 	SDP Negotiation and Signaling of Avatars

An AR-MTSI client supporting avatar-based communication shall support negotiation of avatar capabilities and animation parameters through SDP.

If avatars are used in a session, the session setup shall determine:

- Avatar model reference and format

- Supported animation frameworks

- Data channel configuration for animation parameters

- Fallback mechanisms for UEs without avatar rendering capabilities

The avatar base model of the user shall be stored in the BAR prior to session establishment.

To indicate support for avatar-based communication, an AR-MTSI client shall include the feature tag "+sip.3gpp.avatar-support" in the Contact header field of the SIP REGISTER request.

Additional parameters may be included:

- max-avatar-support: indicates the maximum number of simultaneous avatars the device can handle

An AR-MTSI client supporting avatars shall use data channels as specified in [2] clause 6.2.10 for transmission of animation parameters. The sub-protocol for avatar animation data channels shall be "3gpp.avatar-animation".

An AR-MTSI client offering avatar-based communication shall include in the SDP offer:

- An application data channel media description for avatar animation data:

m=application [port] UDP/DTLS/SCTP webrtc-datachannel

- The dcmap attribute for the application specific data channel for avatar data where the subprotocol shall be set to “3gpp.avatar-animation”

- The 3gpp-req-app that signals the avatar application shall be set to "avatar" with the 3gpp-req-app-opt including the following parameters:

Table X: SDP parameters for avatar negotiation

| Parameter | Usage |
| --- | --- |
| avatar-ref | Provides the reference to avatar model (e.g., "bar://secure-temp-id"). This reference may be a temporary identifier of the selected base avatar model. |
| avatar-animation-frameworks | Comma-separated list of supported animation frameworks |

Example avatar attributes in SDP offer:

a=dcmap:1000 subprotocol="3gpp.avatar-animation" a=3gpp-req-app: "avatar";avatar-ref="bar://29324384edf234abc";avatar-animation-frameworks="org.khronos.openxr"

An AR-MTSI client receiving an SDP offer for avatar-based communication shall:

- Examine the avatar capabilities in the offer

- If avatar rendering is supported:

  - Fetch the avatar model using the provided reference from the BAR

  - Verify compatibility with at least one offered animation framework

  - Accept the data channel for animation parameters

- If avatar rendering is not supported:

  - The session may be rejected, or

  - Network-based rendering may be invoked

An MTSI client accepting avatar-based communication SHALL select at least one animation framework from the offered list that it supports. If no common framework is supported, the avatar media SHALL be rejected.

# Proposal

We propose to agree the content of section 2 and integrate it into the base CR.