# 3GPP TSG-SA WG4 Meeting #-(AH) RTC SWG post 133-e	S4-251603

# Paris, FR, 3 – 5 September 2025	rev of S4aR250169

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.113 | CR | 0013 | rev | 5 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5G_RTP_Ph2] PDU handling and marking enhancements to Dynamic Policy API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Nokia, InterDigital Communications, BBC, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 2025-09-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Lack of support in the Real-Time Media Communications Dynamic Policy API for: - PDU Set handling of N6-unmarked PDUs (i.e., PDUs cannot/do not contain PDU Set infromation marking from the source) - Dynamic traffic characteristics markings - Media identification and multiplexing markings N6-unmarked PDUs enhancements PDU Set and End of Data Burst marking only applies to RTP PDUs since marking is done via an RTP header extension. Hence, PDUs belonging to protocols such as RTCP, STUN, etc. cannot be marked i.e., they do not carry the PDU Set Information. In Rel-18, SA2 has agreed that the PSA UPF marks, in the downlink, each N6-unmarked PDU (“lone PDU”) with PDU Set Information into a PDU Set. If the UPF receives a PDU that does not belong to a PDU Set based on Protocol Description for PDU Set identification, the UPF still maps it to a PDU Set and determines the PDU Set Information by implementation-specific means. This means that for N6-unmarked PDUs, PDU Set Information must be determined by the UPF. For some elements of the PDU Set Information, this is straightforward, e.g., PSN=0 since the PDU Set has only one PDU, PSSize is equal to the size of the N6-unmarked PDU (since there is only one PDU in the PDU Set). However, for PSI, the UPF may only assign a preconfigured value (e.g. by the network operator) which may not reflect the application requirements. SA4 concluded in TR 26.822 that it would be beneficial for senders to signal application-defined PDU Set Importance (PSI) value to the 5GC for N6-unmarked PDUs. This signaling requires extensions to the Dynamic Policy API defined in TS 26.113. Support for dynamic traffic characteristics marking Dynamic traffic characteristics (i.e., data burst size, time to next burst, expedited transfer indication) have been defined in Rel-19 of TS 23.501 as downlink enhancements to support XR media services. Furthermore, TS 26.522 defined RTP header extensions to transport in user plane the dynamic traffic characteristics as required by TS 23.501. Yet, TS 26.113 lacks details about how dynamic traffic characteristics are applicable and usable in the context of RTC media delivery system protocols and Dynamic Policy API. Support for media identification and multiplexing markings When multiple RTP media streams are multiplexed in an RTP session, each media stream can be identified using the identification-tag (the values of "mid" attribute) in the SDP information. The RTP SDES header extension for MID make it possible for an RTP receiver to associate each PDU or PDU Set to a media stream when the RTP PDUs carry the RTP SDES header extension for MID. To enable the traffic detection in 5G System, as defined in the IP Packet Filter Set of clause 5.7.3.2, TS 23.501. an Application‌Flow‌Description needs to be updated to include the information of the multiplexed media identification information which includes ssrc id, MID, RTP SDES header extension for MID, Payload type and RTCP packet type fields. |  |  |  |  |  |  |  |  |
| Summary of change: |  | N6-unmarked PDUs enhancements The Media Session Handler in the Media Client includes a list of unmarkedPDUInfo objects in the mediaTransportParameters property of the ApplicationFlowDescription object when it invokes the Dynamic Policy API, if specific QoS with PDU Set parameters is desired for the application flows of an RTC session. If PDU Set marking is enabled, unmarkedPDUInfoList may be used to indicate a single PSI value for all N6-unmarked PDUs, i.e., PDUs of protocols that cannot be marked using the RTP HE for PDU Set marking, such as RTCP or STUN packets. Support for dynamic traffic characteristics marking Complemented RTC Dynamic Policy API and the associated media delivery protocols details with dynamic traffic characteristics (incl. data burst size marking, time to next burst marking and expedited transfer indication marking). Support for media identification and multiplexing markings Updated the Dynamic Policy API to include the details of multiplexed media identification information in Application flow description |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No possibility to indicate and apply sender-defined PSI values to the 5GC for N6-unmarked PDUs Lack of Stage 3 dynamic traffic characteristics and media identification/multiplexing support for RTC media delivery and misalignment with Stage 2 architecture and procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3, 10.3.1 (re-sectioned), 10.3.2 (re-sectioned), 10.3.3 (new), 10.3.4 (new), B.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS26.510 CR0034 TS26.522 CR0012 TS26.506 CR0010 TS29.514 CR0790 TS29.571 CR0665 |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | SA4 has convened that a single PSI value for all N6-unmarked PDUs protocols is sufficient in the Protocol Description in the scope of Rel-19. This resolution has been captured in 10.3.2 and will be communicated subsequently via LS in SA#134 Dallas meeting to CT4 and SA2 groups for specification alignment and TEI19 necessary changes. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev4: - Clarified 10.3.4 scope of endpoints allowed to manage and modify the multiplexed media information properties. - Modified N6-unmarked PDUs in 10.3.2 to a single PSI value for any unmarked protocol Rev3: - Provided multiplexed media identification details for uplink and downlink media flows separately as agreed in CT3#142 meeting. - Aligned naming RTP Header Type for expedited transfer indication to EXPEDITED_TRANSFER_IND to match CT4 agreements against TS 29.571. Resolved outstanding Editor’s Note Rev2 - Editorial enhancements to ease merger - Added Annex B.1 to mark change in OpenAPI specs - Implemented and added OpenAPI code changes - Added EN in 10.3.2 on missing unmarkedPduInfoList from TS 29.571 Protocol Description definition. Rev1 - Merged contributions from CRs: - TS26.113 CR0005rev8 [S4-251078, endorsed at SA4#132] – [5G_RTP_Ph2] Enhancements to RTC Dynamic Policy API for N6-unmarked PDUs. - TS 26.113 CR0008rev3 [S4aR250120, endorsed at SA4-(AH) RTC SWG post 132] - [5G_RTP_Ph2] Enabling Dynamic Policy API with dynamic traffic characteristics markings. - TS 26.113 CR0010rev5 [S4aR250120, endorsed at SA4-(AH) RTC SWG post 132] – [5G_RTP_Ph2] Enhancements to Dynamic Policy API for SDES RTP HE for MID. - Removed square brackets around TTNB given LS out from RAN2 in R2-2504812 - Alignment with Stage-2 CR in TS26.506 CR0010rev2 Rev5: SA4 version of the SWG AH agreement |  |  |  |  |  |  |  |  |

# Code changes

The code changes associated with this Change Request are available for review at the following URL on 3GPP Forge:

https://forge.3gpp.org/rep/sa4/amd-pro-med/-/merge_requests/8

https://forge.3gpp.org/rep/sa4/amd-pro-med/-/merge_requests/8/diffs?commit_id=3dfaf8e64af89b066ca4a77b10f34026bb02235b

The proposed changes are reproduced below for posterity.

## TS26113_Maf_Provisioning.yaml

---a/TS26113_Maf_Provisioning.yaml
+++b/TS26113_Maf_Provisioning.yaml

@@ -1,10 +1,10 @@

1	1		openapi: 3.0.0

2	2		info:

3	3		  title: Maf_Provisioning (Real-Time media Communication)

4		-	  version: 1.0.0

4	+	  version: 1.0.1

5	5		  description: |

6	6		    RTC AF Provisioning API

7		-	    © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

7	+	    © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

8	8		    All rights reserved.

9	9

10	10		tags:

@@ -12,7 +12,7 @@ tags:

12	12		    description: 'Real-Time media Communication: AF Provisioning APIs'

13	13

14	14		externalDocs:

15		-	  description: 'TS 26.113 V18.0.0; Real-Time Media Communication; Protocols and APIs'

15	+	  description: 'TS 26.113 V19.0.0; Real-Time Media Communication; Protocols and APIs'

16	16		  url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/'

17	17

18	18		servers:

## TS26113_Maf_SessionHandling.yaml

---a/TS26113_Maf_SessionHandling.yaml
+++b/TS26113_Maf_SessionHandling.yaml

@@ -1,10 +1,10 @@

1	1		openapi: 3.0.0

2	2		info:

3	3		  title: Maf_SessionHandling (Real-Time media Communication)

4		-	  version: 1.0.0

4	+	  version: 1.0.1

5	5		  description: |

6	6		    RTC AF Provisioning API

7		-	    © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

7	+	    © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

8	8		    All rights reserved.

9	9

10	10		tags:

@@ -12,7 +12,7 @@ tags:

12	12		    description: 'Real-Time media Communication: Media Session Handling APIs'

13	13

14	14		externalDocs:

15		-	  description: 'TS 26.113 V18.0.0; Real-Time Media Communication; Protocols and APIs'

15	+	  description: 'TS 26.113 V19.0.0; Real-Time Media Communication; Protocols and APIs'

16	16		  url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.113/'

17	17

18	18		servers:

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.506: "5G Real-time Media Communication Architecture (Stage 2)".

[3]	3GPP TS 26.510: "Media delivery; interactions and APIs for provisioning and media session handling".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	IETF RFC 9110 (2022): "HTTP Semantics".

[6]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[7]	IETF RFC 8834 (2021): "Media Transport and Use of RTP in WebRTC".

[8]	IETF RFC 8835 (2021): "Transports for WebRTC".

[9]	3GPP TS 23.003: "Numbering, addressing and identification".

[10]	IETF RFC 8829 (2021): "JavaScript Session Establishment Protocol (JSEP)".

[11]	IETF RFC 7807 (2016): "Problem Details for HTTP APIs".

[12]	IETF RFC 8825 (2021): "Overview: Real-Time Protocols for Browser-Based Applications".

[13]	IETF RFC 5124 (2008): "Extended Secure RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/SAVPF)".

[14]	IETF RFC 7007 (2013): "Update to Remove DVI4 from the Recommended Codecs for the RTP Profile for Audio and Video Conferences with Minimal Control (RTP/AVP)".

[15]	IETF RFC 3551 (2003): "RTP Profile for Audio and Video Conferences with Minimal Control".

[16]	IETF RFC 4585 (2006): "Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF)".

[17]	IETF RFC 3711 (2004): "The Secure Real-time Transport Protocol (SRTP)".

[18]	IETF RFC 5104 (2008): "Codec Control Messages in the RTP Audio-Visual Profile with Feedback (AVPF)".

[19]	IETF RFC 4588 (2006): "RTP Retransmission Payload Format".

[20]	3GPP TS 26.114: " IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[21]	IETF RFC 9112 (2022): "HTTP/1.1".

[22]	IETF RFC 7478 (2015): "Web Real-Time Communication Use Cases and Requirements".

[23]	3GPP TS 26.119: "Media Capabilities for Augmented Reality"

[24]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[25]	Apple: "Getting Raw Accelerometer Events", https://developer.apple.com/documentation/coremotion/getting_raw_accelerometer_events.

[26]	Google: "Sensor Coordinate System", https://developer.android.com/develop/sensors-and-location/sensors/sensors_overview.

[27]	ITU-R Recommendation BT.601-7 (03/2011): "Studio encoding parameters of digital television for standard 4:3 and wide screen 16:9 aspect ratios".

[28]	Microsoft: "Microphone Array Geometry Descriptor Format", https://learn.microsoft.com/en-us/windows-hardware/drivers/audio/microphone-array-geometry-descriptor-format.

[29]	IETF RFC 8831 (2021): "WebRTC Data Channels".

[30]	IETF RFC 8261 (2017): "Datagram Transport Layer Security (DTLS) Encapsulation of SCTP Packets".

[31]	W3C Recommendation: WebRTC: Real-Time Communication in Browsers, March 2023. https://www.w3.org/TR/webrtc/

[32]	IETF RFC 7874 (2016): "WebRTC Audio Codec and Processing Requirements"

[33]	IETF RFC 7742 (2016): "WebRTC Video Processing and Codec Requirements"

[34]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Services (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[35]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[36]	3GPP TS 26.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[37]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[38]	3GPP TS 29.514: "5G System; Policy Authorization Service".

[39]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes".

# * * * * Next change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

API	Application Programming Interface

AR	Augmented Reality

DTLS	Datagram Transport Layer Security

FFS	For Further Study

FoV	Field of View

HTTP	Hyper-Text Transfer Protocol

ICE	Interactive Connectivity Establishment

IMU	Inertial Measurement Unit

MNO	Mobile Network Operator

NAT	Network Address Translation

OTT	Over-The-Top

PSI	PDU Set Importance

RGB	Red-Green-Blue colour space

RTC	Real-Time Communication

RTP	Real-time Transport Protocol

RWT	Response Wait Time

SCTP	Stream Control Transmission Protocol

SRTCP	Secure Real-time Transport Control Protocol

SRTP	Secure Real-time Transport Protocol

SSE	Server-Sent Events

STUN	Session Traversal Utilities for NAT

SWAP	Simple WebRTC Application Protocol

TLS	Transport Layer Security

TURN	Traversal Using Relays around NAT

UPF	User Plane Function

WebRTC		Web Real-Time Communication

XR		Extended Reality

# * * * * Next change * * * *

## 10.3	Dynamic Policy API

### 10.3.1	Introduction

The Dynamic Policy API allows a Dynamic Policy invoker (i.e., the RTC Media Session Handler of the RTC Client or the ICE Function of the RTC AS or the WebRTC Signalling Function of the RTC AS) to request a specific QoS and/or charging policy to be applied to the application flows of an RTC session. The Dynamic Policy API is invoked as a result of SDP negotiation during the WebRTC signalling phase of the RTC session.

The relevant procedures are specified in clause 5.3.3 of TS 26.510 [3].

The resource structure and the data model are specified in clause 9.3 of TS 26.510 [3].

### 10.3.2	Enabling PDU Set handling in dynamic policies

If specific QoS with PDU Set parameters is desired, and PDU Set marking is not enabled for the selected Policy Template as specified in clause 5.3.3.2 of TS 26.510 [3], the Dynamic Policy invoker, i.e., the RTC Media Session Handler or the RTC AS, shall additionally populate the mediaTransportParameters property of the Application‌Flow‌Description object (see clause 5.5.4.13 of TS 29.571 [36]) as follows when creating or updating a Dynamic Policy Instance based on that Policy Template:

-	The transportProto property shall be set to the value SRTP.

-	The rtpHeaderExtInfo object (see clause 5.5.4.14 of TS 29.571 [36]) shall be omitted.

-	The rtpPayloadInfoList property shall contain a single member populated as follows:

-	rtpPayloadTypeList shall be set to the RTP Payload Type value(s) to be used by the RTC endpoint (e.g., the RTC Access Function of an RTC Client) for the negotiated SRTP session(s) to be carried by the application flow in question.

-	rtpPayloadFormat shall be populated as appropriate in the absence of RTP header extensions.

-	When the unmarked-pdu-info attribute (as specified in clause 6.1 of TS 26.522 [37]) is present at the session level with the unmarked-proto attribute property set to the value “ANY” in the SDP offer/answer, the unmarkedPduInfoList property shall contain only one unmarkedPduInfo member. The properties of the single unmarkedPduInfo member of the unmarkedPduInfoList are negotiated by the RTC Access Function of the RTC Client via the SDP offer/answer procedure during the WebRTC signalling phase of the RTC session using the SDP attribute a=unmarked-pdu-info. The properties of the unmarkedPduInfo object (see clause 5.5.4.17 in TS 29.571 [36]) shall be populated as follows:

-	unmarkedProtocol shall be set as indicated by the attribute property unmarked-proto in the SDP offer/answer.

NOTE 1:	In this version of the present document, protocol-level differentiation and unmarkedProtocol values other than ANY (e.g., RTP. RTCP, STUN etc.) are not supported for N6-unmarked PDUs.

-	pduSetImportance shall be set to the desired PSI value for N6-unmarked PDUs on the application flow in question. The setting shall follow the semantics defined for PSI in clause 4.2.4 of TS 26.522 [37], with a value in the range of 1 to 15 (inclusive).

NOTE 2:	The use at the UPF of the single PSI value applicable to all protocols, as indicated in the unmarked‌Pdu‌Info‌List property of the downlink Protocol Description, is not specified in the present document.

NOTE 3:	A new unmarkedPduInfoList property corresponding to the presence of unmarked-pdu-info SDP attribute defined in clause 6.1 of TS 26.522[37] is expected to be specified in TS 29.571.

If PDU Set marking is required by the selected Policy Template as specified in clause 5.3.3.2 of TS 26.510 [3], the Dynamic Policy invoker, i.e., the RTC Media Session Handler or the RTC AS, shall additionally populate the mediaTransportParameters property of the Application‌Flow‌Description object (see clause 5.5.4.13 of TS 29.571 [36]) as follows when creating or updating a Dynamic Policy Instance based on that Policy Template:

-	The transportProto property shall be set to the value SRTP.

-	The properties of the rtpHeaderExtInfo object (see clause 5.5.4.14 of TS 29.571 [36]) shall be populated as follows:

-	rtpHeaderExtType shall be set to PDU_SET_MARKING.

-	rtpHeaderExtId shall be set to the value of the ID field to be used by the RTC endpoint (e.g., the RTC Access Function of an RTC Client) in the RTP Header Extension for PDU Set Marking on the application flow in question, as specified in clause 4.2 of TS 26.522 [37]. The value of this parameter is negotiated via the SDP offer/answer procedure during the WebRTC signalling phase of the RTC session.

-	longFormat shall be set according to the use of the one- or two-byte RTP Header Extension for PDU Set Marking, as specified in clause 4.2.1 of TS 26.522 [37]. The value of this parameter is negotiated via the SDP offer/answer procedure during the WebRTC signalling phase of the RTC session.

-	pduSetSizeActive shall be set to reflect the presence of the PDU Set Size field in the RTP Header Extension for PDU Set Marking, as specified in clause 4.2.4 of TS 26.522 [37]. The value of this parameter is negotiated via the SDP offer/answer procedure during the WebRTC signalling phase of the RTC session.

-	pduSetPduCountActive shall be set to reflect the presence of the Number of PDUs in the PDU Set in the RTP Header Extension for PDU Set Marking, as specified in clause 4.2.4 of TS 26.522 [37]. The value of this parameter is negotiated via the SDP offer/answer procedure during the WebRTC signalling phase of the RTC session.

-	The rtpPayloadInfoList property shall contain a single member populated as follows:

-	rtpPayloadTypeList shall be set to the RTP Payload Type value(s) to be used by the RTC endpoint (e.g., the RTC Access Function of an RTC Client) for the negotiated SRTP session(s) to be carried by the application flow in question.

-	rtpPayloadFormat shall be omitted because RTP header extensions are present.

-	When the unmarked-pdu-info attribute (as specified in clause 6.1 of TS 26.522 [37]) is present at the session level with the unmarked-proto attribute property set to the value “ANY” in the SDP offer/answer, the unmarkedPduInfoList property shall contain only one unmarkedPduInfo member. The properties of the unmarkedPduInfo members of the unmarkedPduInfoList are negotiated by the RTC Access Function of the RTC Client via the SDP offer/answer procedure during the WebRTC signalling phase of the RTC session using the SDP attribute a=unmarked-pdu-info. The properties of the unmarkedPduInfo object (see clause 5.5.4.17 in TS 29.571 [36]) shall be populated as follows:

-	unmarkedProtocol shall be set as indicated by the attribute property unmarked-proto in the SDP offer/answer.

NOTE 4:	In this version of the present document, protocol-level differentiation and unmarkedProtocol values other than ANY (e.g., RTP. RTCP, STUN etc.) are not supported for N6-unmarked PDUs..

-	pduSetImportance shall be set to the desired PSI value for N6-unmarked PDUs on the application flow in question. The setting shall follow the semantics defined for PSI in clause 4.2.4 of TS 26.522 [37], with a value in the range of 1 to 15 (inclusive).

NOTE 5:	The use at the UPF of the single PSI value applicable to all protocols, as indicated in the unmarked‌Pdu‌Info‌List property of the downlink Protocol Description, is not specified in the present document.

NOTE 6:	A new unmarkedPduInfoList property corresponding to the presence of unmarked-pdu-info SDP attribute defined in clause 6.1 of TS 26.522[37] is expected to be specified in TS 29.571.

In all PDUs it contributes at reference point RTC-4m or RTC-12 that fall within the scope of the application flow description, an RTC endpoint sender as the RTC Access Function (Media Access Function) or the Media Function of the RTC AS shall use the protocol indicated in transportProto; the sender shall set the SRTP header fields in accordance with rtpPayloadInfoList; and the sender shall include a one- or two- byte (consistent with the signalled length) RTP Header Extension for PDU Set Marking in the SRTP header with fields set according to the values declared in the rtpHeaderExtInfo property per above.

### 10.3.3	Enabling dynamically changing traffic characteristics marking in dynamic policies

#### 10.3.3.1	Dynamically changing traffic characteristics marking for data bursts

If any dynamically changing traffic characteristics marking for data bursts is required by the selected Policy Template, as specified in clause 5.3.3.2 of TS 26.510 [3] (i.e., downlinkData‌Burst‌Size‌Marking‌Required is present and set to true, and/or downlinkTime‌To‌Next‌Burst‌Marking‌Required is present and set to true in the policy binding of the Service Access Information), the Dynamic Policy invoker, i.e., the RTC Media Session Handler or the RTC AS, shall additionally populate the media‌Transport‌Parameters property of the Application‌Flow‌Description object (see clause 5.5.4.13 of TS 29.571 [36]) as follows when creating or updating a Dynamic Policy Instance based on that Policy Template:

-	The transportProto property shall be set to the value SRTP.

-	The properties of the RtpHeaderExtInfo type (see clause 5.5.4.14 of TS 29.571 [36]) as either a rtpHeaderExtInfo object or as an element of the addRtpHeaderExtInfo object (see clause 5.5.4.13 of TS 29.571 [36]) shall be populated as follows:

-	rtpHeaderExtType shall be set to DYN_CHANGING_TRAFFIC_CHAR.

-	rtpHeaderExtId shall be set to the value of the ID field to be used by the Media Function of an RTC AS in the RTP Header Extension for Dynamically Changing Traffic Characteristics Marking on the application flow in question, as specified in clause 4.5 of TS 26.522 [37]. The value of this parameter is negotiated via the SDP offer/answer procedure during the WebRTC signalling phase of the RTC session.

-	longFormat shall be set according to the use of the one- or two-byte RTP Header Extension for Dynamically Changing Traffic Characteristics Marking, as specified in clause 4.5.1 of TS 26.522 [37]. The value of this parameter is negotiated via the SDP offer/answer procedure during the WebRTC signalling phase of the RTC session.

If any dynamically changing traffic characteristics marking for data bursts is required by the selected Policy Template (see clause 5.2.7.1 of TS 26.510 [3]), in all PDUs it contributes for media delivery at reference point RTC-4m that fall within the scope of the application flow description, the Media Function of the RTC AS shall use the protocol indicated in transportProto and in addition shall behave as follows:

-	If data burst size marking is required (i.e., downlink‌Data‌Burst‌Size‌Marking‌Required is present and set to true in the Policy Template instantiated by the Dynamic Policy Instance), the Media Function of the RTC AS shall include in at least one SRTP header of each downlink data burst it transmits a one- or two-byte (consistent with the signalled length) RTP Header Extension for Dynamically Changing Traffic Characteristics Marking with fields set according to the values declared in the rtpHeaderExtInfo property per above and a data burst size indication, BSize, per clause 4.5.4 of TS 26.522 [37].

-	If time to next burst marking is required (i.e., downlink‌Time‌To‌Next‌Burst‌Marking‌Required is present and set to true in the Policy Template instantiated by the Dynamic Policy Instance), the Media Function of the RTC AS shall include in at least one SRTP header of each downlink data burst it transmits a one- or two-byte (consistent with the signalled length) RTP Header Extension for Dynamically Changing Traffic Characteristics Marking with fields set according to the values declared in the rtpHeaderExtInfo property per above and a time to next burst indication, TTNB, per clause 4.5.4 of TS 26.522 [37].

NOTE 1:	The frequency and occurrence of RTP Header Extension for Dynamically Changing Traffic Characteristics relative to associated dynamically changing traffic characteristics is left to sender implementation. For more details, see guidelines provided in clause 4.5 of TS 26.522 [37].

NOTE 2:	Procedures to configure the required RTC AS behaviour via reference point RTC-3 are not defined in this version of the present document

#### 10.3.3.2	Dynamically changing traffic characteristics marking for expedited data transfers

If dynamically changing traffic characteristics marking for expedited data transfers is required by the selected Policy Template, as specified in clause 5.3.3.2 of TS 26.510 [3] (i.e., downlink‌Expedited‌Transfer‌Indication‌Marking‌Required is present set to true in the policy binding of the Service Access Information), the Media Session Handler shall additionally populate the media‌Transport‌Parameters property of the Application‌Flow‌Description object (see clause 5.5.4.13 of TS 29.571 [36]) as follows when creating or updating a Dynamic Policy Instance based on that Policy Template

-	The transportProto property shall be set to the value SRTP.

-	The properties of the RtpHeaderExtInfo type (see clause 5.5.4.14 of TS 29.571 [36]) as either a rtpHeaderExtInfo object or as an element of the addRtpHeaderExtInfo object (see clause 5.5.4.13 of TS 29.571 [36]) shall be populated as follows:

-	rtpHeaderExtType shall be set to EXPEDITED_TRANSFER_IND.

-	rtpHeaderExtId shall be set to the value of the ID field to be used by the Media Function of an RTC AS in the RTP Header Extension for Expedited Transfer Indication Marking on the application flow in question, as specified in clause 4.7 of TS 26.522 [37]. The value of this parameter is negotiated via the SDP offer/answer procedure during the WebRTC signalling phase of the RTC session.

-	longFormat shall be set according to the use of the one- or two-byte RTP Header Extension for Expedited Transfer Indication Marking, as specified in clause 4.7.1 of TS 26.522 [37]. The value of this parameter is negotiated via the SDP offer/answer procedure during the WebRTC signalling phase of the RTC session.

If dynamically changing traffic characteristics marking for expedited data transfers is required by the selected Policy Template (see clause 5.2.7.1 of TS 26.510 [3]), in all PDUs it contributes for media delivery at reference point RTC-4m that fall within the scope of the application flow description, the Media Function of the RTC AS shall use the protocol indicated in transportProto and shall include in all SRTP headers of downlink packets a one- or two-byte (consistent with the signalled length) RTP Header Extension for Dynamically Changing Traffic Characteristics Marking with fields set according to the values declared in the rtpHeaderExtInfo property per above.

NOTE 1:	PDUs contributed within the scope of the application flow description at RTC-4m that cannot be marked (e.g., RTCP, STUN, see clause 4.7.6 of TS 26.522 [37]) are not expedited and can be handled by the 5G System on a default QoS flow depending on the User Plane Function configuration, see TS 29.244 [39].

NOTE 2:	Procedures to configure the required RTC AS behaviour via reference point RTC-3 are not defined in this version of the present document.

### 10.3.4	Enabling multiplexed media flow handling in dynamic policies

If an RTC Session multiplexes multiple media flows into a single RTP Session as described in section 4.4 of RFC 8834 [7] (because the RTC endpoints involved have successfully negotiated media multiplexing as specified in clause 4.6 of TS 26.522 [37]) and differentiated QoS handling is required for the multiplexed media streams, the Dynamic Policy invoker (i.e., the RTC Media Session Handler or the RTC AS) shall additionally populate the downlink‌Multiplexed‌Media‌Infos and/or uplink‌Multiplexed‌Media‌Infos properties of the Application‌Flow‌Description object (see clause 7.3.3.2 of TS 26.510 [3]) as follows when creating or updating a Dynamic Policy Instance:

-	For uplink application flows, including those terminating on the RTC AS, the uplink‌Multiplexed‌Media‌Infos property shall be present and the array shall contain at least one MpxMediaInfo object for each media stream in the multiplexed media application flow.

-	For downlink application flows, including those originating from the RTC AS, the downlink‌Multiplexed‌Media‌Infos property shall be present and the array shall contain at least one MpxMediaInfo object for each media stream in the multiplexed media application flow.

-	For application flows between the RTC Access Function of two Media Clients (i.e., uplink followed by downlink via the UPF at reference point RTC-12 or via the Media Function of the RTC AS at reference point RTC-4m, as applicable to the 5G System deployment configuration), the uplink‌Multiplexed‌Media‌Infos and downlink‌Multiplexed‌Media‌Infos arrays shall be populated with identical Mpx‌Media‌Info objects according to the respective specifications above. It is an error for these to be set differently by the Dynamic Policy invoker in the context of an RTC session.

NOTE 1:	In this case, uplink‌Multiplexed‌Media‌Infos describes the multiplexed media stream transmitted by the sending Media Client in its uplink and downlink‌Multiplexed‌Media‌Infos describes the same multiplexed media stream received by the peer Media Client in its downlink.

-	The properties of the MpxMediaInfo object are in all cases negotiated by the RTC Access Function of the RTC Client or the RTC AS (e.g., using the BUNDLE group attribute) in the SDP offer/answer procedure during the WebRTC signalling phase of the RTC Session. The properties of each MpxMediaInfo object (see clause 5.6.2.61 of TS 29.514 [38]) shall be populated to aid traffic identification of each corresponding media stream in the 5G System, based on the RTP packet header values to be used by the sending RTC endpoint (i.e., the RTC Access Function of an RTC Client or the Media Function of the RTC AS) on the media stream in question, as follows:

-	ssrcId may be set to the synchronization source (SSRC) value (if known) to be used by the sending RTC endpoint.

-	payloadType shall be set to the RTP Payload Type value(s) to be used by the sending RTC endpoint. The value of this parameter is negotiated via the SDP offer/answer procedure during the WebRTC signalling phase of the RTC Session.

-	identificationTag shall be set to the value of the identification tag or media description identifier (MID) to be used by the sending RTC endpoint in the SDES RTP Header Extension for MID or the RTCP MID SDES Item for MID, as specified in clause 4.6 of TS 26.522 [37].

-	rtpSdesHdrExtId shall be present when the sending RTC endpoint includes the SDES RTP Header Extension for MID in SRTP packets, and it shall be set to the value of the local identifier or ID field to be used by the sending RTC endpoint in the SDES RTP Header Extension for MID, as specified in clause 4.6 of TS 26.522 [37].

-	rtcpPacketType may be set to the RTCP Packet Type (PT) value to be used by the sending RTC endpoint.

NOTE 2:	A combination of SSRC, Payload Type and/or MID values is required for multiplexed media identification.

In all PDUs it contributes at reference point RTC-4m or RTC-12 that fall within the scope of the application flow description, the sending RTC endpoint (i.e., the RTC Access Function of an RTC Client or the Media Function of the RTC AS) shall set the SRTP header fields in accordance with the MpxMediaInfo object for the media stream in question; and it shall include a one- or two- byte RTP SDES Header Extension for MID in the SRTP header with MID fields set according to the values declared in the MpxMediaInfo object per above to indicate the multiplexed media identification information.

NOTE 3:	When multiplexed media identification marking is used in this way, multiplexed media traffic identification is performed by the 5G System for differentiated QoS treatment using the IP Filter Set with (S)RTP Multiplexed Media Identification Information feature defined in clause 8.2.5 of TS 29.244 [39], which inspects certain SRTP header fields −specifically the Synchronization Source identifier and the Payload Type(s) – in combination with the media description identifier information present in the relevant MpxMediaInfo object.

Next change

# B.1	General

The normative code specifying the APIs defined in clauses 6 and 10 of the present document, including JSON Schema representations of HTTP message bodies to be used with these APIs, is published on 3GPP Forge according to the OpenAPI 3.0.0 specification [34]. The YAML files corresponding to this version of the present document shall be published to the following location:

https://forge.3gpp.org/rep/all/5G_APIs/-/tags/TSG109-Rel19

Informative copies of these YAML files shall be distributed with the present document for the convenience only. Where any discrepancy exisits, the version on 3GPP Forge shall be considered definitive.

End of changes