3GPP Audio Telco Post TSG SA WG4 #133-e	 S4-251458

Online, July 18-25, 2025

# Title:	Calibration of RAN simulation results for ULBC over NTN

Source:	Qualcomm

Agenda item:	7.9 FS_ULBC

# Document for:	Information

# 1 Introduction

Companies provided RAN simulation results in:

- Xiaomi: S4-251251[1],

- Vivo: S4-251332[2],

- Qualcomm: S4-251397[3] and the Annex in the present paper

according to the parameters agreed in S4aA250069 [4] and for the scenarios with receiver assumptions described in S4-251303 [5].

# 2 BLER vs SNR

All three companies provided BLER vs SNR curves for NPUSCH; two companies (vivo and Qualcomm) provided the curves for NPDSCH.

The scenarios are:

- NPUSCH: Subcarrier Spacing (SCS) 3.75kHz, 1 tone, 2 RU’s (64ms NPUSCH), no repetition, DMRS bundling used across all slots and Rus for channel estimation (ChEst)

  - Scenario 1.1: TBS 114 bits

  - Scenario 1.2: TBS 256 bits

- NPDSCH: SCS 15kHz, 12 tones, 4 subframes and 2 repetitions, hence (8ms NPDSCH), NRS bundling used across all subframes and repetitions for ChEst

  - Scenario 2.1.1: TBS 144 bits, 1 RX antenna

  - Scenario 2.1.2: TBS 144 bits, 2 RX antenna

  - Scenario 2.2.1: TBS 256 bits, 1 RX antenna

  - Scenario 2.2.2: TBS 256 bits, 2 RX antenna

For Scenario 1.1, BLER vs SNR from Xiaomi [1], vivo [2] and from Qualcomm [3] are plotted in Figure 1.

NOTE: DMRS bundling is not used in [2].

Figure 1 BLER vs SNR from Xiaomi [1], vivo [2] and Qualcomm [3].

The required SNRs for different BLERs are shown in Table 1.

Table 1 Required SNR for BLERs

| Required SNR (dB) | 10% | 6% | 2% | 1% |
| --- | --- | --- | --- | --- |
| NPUSCH (TBS 144): Scenario 1.1 |  |  |  |  |
| Vivo | 5.95 | 6.43 | 7.43 | 8.19 |
| Qualcomm | 3.1 | 3.6 | 4.6 | 5.2 |
| Xiaomi | 1.48 | 1.68 | 2.08 | 2.28 |
| NPUSCH (TBS 256): Scenario 1.2 |  |  |  |  |
| Vivo | 9.40 | 10.04 | 11.18 | 11.99 |
| Qualcomm | 6.8 | 7.4 | 8.5 | 9.2 |
| NPDSCH (TBS 144, 1 RX), Scenario 2.1.1 |  |  |  |  |
| Vivo | -3.47 | -2.72 | -1.27 | -0.32 |
| Qualcomm with short NRS for CEST [3] | -3.8 | -3.0 | -1.5 | -0.6 |
| Qualcomm with long NRS for CEST (Annex) | -4.8 | -4.1 | -2.6 | -1.6 |
| NPDSCH (TBS 144, 2 RX), Scenario 2.1.2 |  |  |  |  |
| Qualcomm with long NRS for CEST (Annex) | -8.5 | -8.0 | -7.0 | -6.5 |
| NPDSCH (TBS 256, 1 RX), Scenario 2.2.1 |  |  |  |  |
| Vivo | -1.70 | -0.96 | 0.48 | 1.37 |
| Qualcomm with long NRS for CEST (Annex) | -2.4 | -1.7 | -0.2 | 0.7 |
| NPDSCH (TBS 256, 2 RX), Scenario 2.2.2 |  |  |  |  |
| Qualcomm with long NRS for CEST (Annex) |  |  | -4.8 | -4.3 |

# 3 Channel consistency check

Qualcomm [3] verified that the channel is consistent between Scenario 1.1 and Scenario 1.2.

Xiaomi [1] provided the traces in .txt and .csv files.

# Annex: Additional results from Qualcomm

Qualcomm provided the following simulation results in addition to those in [3].

Figure 2 BLER vs SNR for NPDSCH with TBS 144 bits

Figure 3 BLER vs SNR for NPDSCH with TBS 144 bits and 256 bits and long NRS for CEST

The short NRS for channel estimation and long NRS for channel estimation are shown in Figure 4.

Figure 4 Short NRS and long NRS for channel estimation for NPDSCH

References

[1] S4-251251, “Uplink simulation results for RAN simulation calibration for ULBC over NTN,” Xiaomi, 3GPP TSG SA WG4 #133-e, July 18-25, 2025.

[2] S4-251332, “Updates on end to end simulation for error trace generation,” vivo, 3GPP TSG SA WG4 #133-e, July 18-25, 2025.

[3] S4-251397, “Results for RAN simulation calibration for ULBC over NTN,” Qualcomm, 3GPP TSG SA WG4 #133-e, July 18-25, 2025.

[4] S4aA250069, “On the remaining parameters for the RAN simulation for ULBC over NTN”, Qualcomm, vivo, Huawei Technologies Co. Ltd, 3GPP Audio Telco post TSG SA WG4 #132, June 18, 2025.

[5] S4-251303, “Scenarios for RAN simulation calibration for ULBC over NTN”, Qualcomm, vivo, Xiaomi, Huawei Technologies Co. Ltd, 3GPP TSG SA WG4 #133-e, July 18-25, 2025.