3GPP TSG-S4 Meeting#133-e	S4-251336

# Online, 18th–25th July 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.265 | CR | pseudo | rev | - | Current version: | 1.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [VOPS] On Multiview features |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc., Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | VOPS |  |  |  |  |  | Date: |  |  | 2025-07-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are remaining EN’s on missing features like 3D reference info SEI which needs to be implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add recommendation for 3D reference info SEI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Several key features will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.3.6	3GPP MV-HEVC Stereo

#### 6.3.6.1	Introduction

The MV-HEVC Stereo Operation Point permits consistent distribution of stereoscopic content using MV-HEVC. The remainder of this clause 6.3.6 defines the Bitstream and Receiver requirements for the 3GPP-MV-HEVC-Stereo receiver.

#### 6.3.6.2	Bitstream Requirements

A 3GPP-MV-HEVC-Stereo Bitstream shall conform to the following requirements

-	the Representation Format included in the Bitstream shall conform to the 3GPP Stereoscopic format as defined in clause 4.4.3.4.

-	The bitstream shall conform to the constraints specified in the MV-HEVC-Dual-layers-UHD420-Dec decoding capabilities as defined in clause 5.3.2.

We can do it in textual form like this.

-	The bitstream shall contain a first output layer set containing the layer (nuh_layer_id = 0) which follows the constraints specified in the MV-HEVC-Dual-layers-UHD420-Dec decoding capabilities as defined in clause 5.3.2.

-	The bitstream shall contain a second output layer set containing the layer (nuh_layer_id = 0) as output layer and a second layer as output layer which follows the constraints specified in the MV-HEVC-Dual-layers-UHD420-Dec decoding capabilities as defined in clause 5.3.2. This second layer corresponds to a scalability dimension of type Multiview.

NOTE:	Although the operating point allows for layers in the bitstream that are not output layers, the added storage and/or transport capacity needed for such layers should be taken into account when provisioning a service.

Or, we can do it by syntax constraints like this.

-	In the VPS,

-	The value vps_num_layer_sets_minus1 shall be equal to or greater than 1.

-	The value layer_id_included_flag[ 1 ][ 0 ] shall be equal to 1 and there shall be a value of j with j different from 0 for which layer_id_included_flag[ 1 ][ j ] is equal to 1.

-	The value of scalability_mask_flag[ 1 ] shall be equal to 1.

-	The value of ScalabilityId[ 1 ][ 1 ] shall be equal to 1.

-	The value of default_output_layer_idc shall be equal to 0.

End of if.NOTE:	Although the operating point allows for layers in the bitstream that are not output layers, the added storage and/or transport capacity needed for such layers should be taken into account when provisioning a service.

-	The chroma sub-sampling shall be 4:2:0 and the value of chroma_format_idc shall be set to 1.

-	The vps_num_direct_ref_layers[1] may be present, and if present,

-	it shall be set to 1.

-	the vps_direct_ref_layer_id[1][0] shall be set to 0.

NOTE:	This implies, that layer-dependency is possible, but not needed. The two layers may be independent, or the second layer depend on the base layer.

-	The aspect_ratio_idc value shall be set to 1, indicating a square pixel format.

-	In the VUI,

-		The aspect_ratio_idc value shall be set to 1, indicating a square pixel format.

-	Either,

-		the values of colour_primaries, transfer_characteristics and matrix_coeffs each shall be set to 1.

-	The value of chroma_sample_loc_type_top_field shall be set to 0.

-	or

-	the values of colour_primaries and matrix_coeffs each shall be set to 9, and the value of transfer_characteristics shall be set to one of the following values: 14 (for SDR with WCG), 16 	(for PQ) and 18 (for HLG).

-	The value of the chroma_sample_loc_type_top_field shall be set to 2.

The timing information may be present.

-	If the timing information is present, i.e. the value of vui_timing_info_present_flag is set to 1, then the values of vui_num_units_in_tick and vui_time_scale shall be set according to the frame rates allowed for each operation point. The timing information present in the video Bitstream should be consistent with the timing information signalled at the system level.

-	The frame rate shall not change between two RAPs. fixed_pic_rate_general_flag value, if present, shall be set to 1.

Bitstreams not required to be associated with frame packing information for all coded video sequences. It is also possible that such information, when present, may differ from one coded video sequence to another.

The Bitstream shall include the three_dimensional_reference_displays_info SEI message as specified in Recommendation ITU-T H.265 / ISO/IEC 23008-2 [h265] and at least one reference display shall be specified.

Editor’s Note: More details of the message need to be present. The included parameters allow a decoder or rendering engine to reconstruct the spatial layout of the display(s) used during content creation, which is crucial for accurate 3D rendering, especially in multi-display or immersive environments. Should be added to the representation signal.

- num_displays_minus1: Indicates how many reference displays are described (actual number is this value + 1).

  - Shall be at least 1 (one for each eye)

- display_id: Identifier for each display.

  - Anything in 3GPP we want to do

- display_center_*: 3D coordinates of the center of the display in the reference coordinate system.

  - X, Y, Z parameter

- display_rotation_*: Rotation angles (or vectors) describing the orientation of the display.

  - X, Y, Z parameter

- display_width_*, display_height_*: Vectors defining the physical dimensions of the display surface.

  - X, Y, Z

- display_normal_*: Normal vector to the display surface.

  - X, Y, Z

- display_orientation_flag: Indicates whether the display orientation is defined explicitly or implicitly.

three_dimensional_reference_displays_info( )

{

num_displays_minus1 = 1  // Two displays: left and right

// Display 0: Left Eye

display_id = 0

display_center_x = -0.032  // 32 mm to the left

display_center_y = 0.0

display_center_z = 0.6     // 60 cm from viewer

display_rotation_x = 0.0

display_rotation_y = 0.0

display_rotation_z = 0.0

display_width_x = 0.057

display_width_y = 0.0

display_width_z = 0.0

display_height_x = 0.0

display_height_y = 0.032

display_height_z = 0.0

display_normal_x = 0.0

display_normal_y = 0.0

display_normal_z = -1.0

display_orientation_flag = 1

// Display 1: Right Eye

display_id = 1

display_center_x = 0.032   // 32 mm to the right

display_center_y = 0.0

display_center_z = 0.6

display_rotation_x = 0.0

display_rotation_y = 0.0

display_rotation_z = 0.0

display_width_x = 0.057

display_width_y = 0.0

display_width_z = 0.0

display_height_x = 0.0

display_height_y = 0.032

display_height_z = 0.0

display_normal_x = 0.0

display_normal_y = 0.0

display_normal_z = -1.0

display_orientation_flag = 1

}

#### 6.3.6.3	Receiver Requirements

Receivers conforming to this Operation Point 3GPP-MV-HEVC-Stereo shall support decoding and rendering Bitstreams with the restrictions defined in clause 6.3.6.2, including the necessary processing of three_dimensional_reference_displays_info SEI message as specified in Recommendation ITU-T H.265 / ISO/IEC 23008-2 [h265].

Editor’s Note: More details of the requirements for rendering needs to be present.

- Examples Parse the SEI Message:

  - The decoder extracts the SEI message from the bitstream.

  - It reads the number of displays and their associated parameters (position, orientation, size, etc.).

- Reconstruct Display Geometry:

  - Using vectors like display_center, display_width, display_height, and display_normal, the receiver reconstructs the 3D plane of each display.

  - This defines the viewing frustum for each eye.

- Map Views to Displays:

  - The receiver maps the left and right eye views to the corresponding display surfaces.

  - This ensures correct parallax and depth perception.

- Adjust for Local Display Setup:

  - If the actual display differs from the reference (e.g., different size or viewer distance), the receiver can transform the scene to preserve the intended 3D effect.

- Render the Scene:

  - The rendering engine uses the reconstructed geometry to project the video frames onto the virtual display planes.

  - This is especially important in multi-view or immersive environments (e.g., VR, AR, CAVE systems).

  - Example: VR Headsets: Adjusting the stereo rendering pipeline to match the original content's intended depth.

NOTE 1:	Rendering includes adherence to the parameters signalled in the bitstream to characterize the distributed Representation format.

There are no requirements on output timing conformance for H.265/HEVC decoding (Annex C of [6]). The Hypothetical Reference Decoder (HRD) parameters, if present, should be ignored by the Receiver.

* * * End of Changes * * * *