3GPP TSG-S4 Meeting#133-e	S4-251340

# Online, 18th–25th July 2025

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.143 | CR | 0004 | rev | 1 | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [VOPS] On video coding capabilities and operating points |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc., Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | VOPS |  |  |  |  |  | Date: |  |  | 2025-07-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 26.265 is the new home for video coding capabilities. TS 26.143 currently refers to TS 26.511 for this purpose, and this needs to be updated. AVC-HD encoding and decoding capabilities are not defined in TS 26.265 and these should be removed along with the EN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated references for video coding capabilities to TS 26.265. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Video coding capabilities in the TS would point to an incorrect reference. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.6.1, 5.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Khronos glTF 2.0, glTF™ 2.0 Specification (khronos.org)

[3] 	ISO/IEC 23090-14 AMD 2, Information technology — Coded representation of immersive media — Part 14: Scene description — Amendment 2: Support for haptics, augmented reality, avatars, Interactivity, MPEG-I audio, and lighting

[4]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, Codecs and Formats".

[5]	3GPP TS 26.117: "5G Media Streaming (5GMS); Speech and audio profiles".

[6]	IETF Draft draft-ietf-mimi-content-01: "More Instant Messaging Interoperability (MIMI) message content", 	Rohan Mahy

[7]	3GPP TS 22.140: "Multimedia Messaging Service (MMS); Stage 1".

[8]	Open Mobile alliance, "MMS Architecture" OMA-AD-MMS-V1_3-20110913-A.

[9]	Open Mobile alliance, "Multimedia Messaging Service Encapsulation Protocol" OMA-TS-MMS_ENC-V1_3-20110913-A.

[10]	GSMA "RCS Universal Profile Service Definition Document", Version 2.6, 19 December 2022.

[11]	GSMA PRD RCC.07 version 13.0 - "Rich Communication Suite - Advanced Communications Services and Client Specification" 19 December 2022.

[12]	IETF RFC 2046, "Multipurpose Internet Mail Extensions (MIME) Part Two: Media Types".

[13]	ISO/IEC 14496-12: "Information technology - Coding of audio-visual objects -Part 12: ISO base media file format".

[14]	ISO/IEC 23000-24:2023 Preliminary Draft of: Information technology — Multimedia application format (MPEG-A) — Part 24: Messaging Media Application Format (MeMAF) ".

NOTE: 	A preliminary draft of this standard is available as MDS23345_W03_N1082 here: https://www.mpeg.org/wp-content/uploads/mpeg_meetings/144_Hannover/w23345.zip

[15]	3GPP 23.140: "Multimedia Messaging Service (MMS); Functional Description; Stage 2".

[16]	ITU-T Recommendation T.81: "Information technology; Digital compression and coding of continuous-tone still images: Requirements and guidelines".

[17]	"JPEG File Interchange Format", Version 1.02, September 1, 1992.

[18]	"Exchangeable image file format for digital still cameras: EXIF 2.2", Specification by the Japan Electronics and Information Technology Industries Association (JEITA), April 2002, URL: http://www.exif.org/

[19]	CompuServe Incorporated: "GIF Graphics Interchange Format: A Standard defining a mechanism for the storage and transmission of raster-based graphics information", Columbus, OH, USA, 1987.

[20]	Compuserve Incorporated, Columbus, Ohio (1990): "Graphics Interchange Format (Version 89a)".

[21]	IETF RFC 2083: "PNG (Portable Networks Graphics) Specification version 1.0 ", T. Boutell, et. al., March 1997.

[22]	ISO/IEC 23000-22:2019 Information technology — Multimedia application format (MPEG-A) — Part 22: Multi-image application format (MIAF)

[23]	IETF RFC 2045, "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies", November 1996

[24]	ISO/IEC 23008-12:2019 Information technology — High efficiency coding and media delivery in heterogeneous environments — Part 12: Image File Format

[25]	ITU-T Recommendation H.265 (02/2018): "High efficiency video coding".

[26]	3GPP TS 26.244: "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)"

[27]	3GPP TS 26.245: "Transparent end-to-end packet switched streaming service (PSS); Timed text format"

[28]	ISO/IEC 14496-30: "Information technology - Coding of audio-visual objects - Part 30: Timed text and other visual overlays in ISO base media file format".

[29]	IETF RFC 2387, "The MIME Multipart/Related Content-type"

[30]	IETF RFC 6381, "The 'Codecs' and 'Profiles' Parameters for "Bucket" Media Types"

[31]	3GPP TS 26.307, "Presentation Layer for 3GPP Services"

[32]	3GPP TS 26.140, "Multimedia Messaging Service (MMS); Media formats and codecs"

[33]	IETF RFC 2077, "The Model Primary Content Type for Multipurpose Internet Mail Extensions"

[34]	3GPP TS 26.119, "Media Capabilities for Augmented Reality"

[XY]	3GPP TS 26.265: "Media Delivery: Video Capabilities and Operation Points"

* * * Next Change * * * *

### 5.6.1	Player and Decoding capabilities

The capability 26143_VIDEO_AVC-HD is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the AVC-HD-Dec decoding capabilities as defined in clause 4.2.1.1 of TS 26.511 [4],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'avc1' as defined in ISO/IEC 14496-15 [15],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_VIDEO_AVC-HD shall be signalled with video/mp4, profile="3gp9" codecs="avc1.640028" or an equivalently compatible media type.

The capability 26143_VIDEO_AVC-FullHD is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the AVC-FullHD-Dec decoding capabilities as defined in clause 5.3.1 of TS 26.265 [XY],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'avc1' as defined in ISO/IEC 14496-15 [15],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_VIDEO_AVC-FullHD shall be signalled with video/mp4, profile="3gp9" codecs="avc1.640029" or an equivalently compatible media type.

The capability 26143_VIDEO_HEVC-HD is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the HEVC-HD-Dec decoding capabilities as defined in clause 5.3.2 of TS 26.265 [XY],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'hvc1' as defined in ISO/IEC 14496-15 [15],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_VIDEO_HEVC-HD shall be signalled with video/mp4, profile="3gp9" codecs="hvc1.1.2.L93.B0" or an equivalently compatible media type.

The capability 26143_VIDEO_HEVC-FullHD is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the HEVC-FullHD-Dec decoding capabilities as defined in clause 5.3.2 of TS 26.265 [XY],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'hvc1' as defined in ISO/IEC 14496-15 [15],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_VIDEO_HEVC-FullHD shall be signalled with video/mp4, profile="3gp9" codecs="hvc1.2.4.L123.B0" or an equivalently compatible media type.

The capability 26143_VIDEO_HEVC-UHD is defined as the capability of playing back (decoding and rendering) a file that

-	is decodable by a decoder capable of the HEVC-UHD-Dec decoding capabilities as defined in clause 5.3.2 of TS 26.265 [XY],

-	is encapsulated in an ISO BMFF Track [14] conforming with the requirements of the sample entry 'hvc1' as defined in ISO/IEC 14496-15 [15],

-	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

In the context of this specification, the media type for files with this capability 26143_VIDEO_HEVC-UHD shall be signalled with video/mp4, profile="3gp9" codecs="hvc1.2.4.L153.B0" or an equivalently compatible media type.

NOTE: 	In the absence of knowledge of detailed capabilities, 16:9 and 9:16 image formats are preferably used.

* * * Next Change * * * *

### 5.6.2	MMBP Content Generator capabilities

The capability 26143_VIDEO_ENC_AVC-HD for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from a video signal in real-time, such that the file can be played back by a player with the capability 26143_VIDEO_AVC-HD,

-	the AVC-HD-Enc encoding capabilities as defined in clause 4.1.2.2 of TS 26.511 [4] to generate a bitstream from the video signal

-	the capability to generate an ISO BMFF track from the bitstream that conforms with the requirements of the sample entry 'avc1' as defined in ISO/IEC 14496-15 [15].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using video/mp4, profile="3gp9" codecs=" avc1.640028" or an equivalently compatible media type.

The capability 26143_VIDEO_ENC_AVC-FullHD for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from a video signal in real-time, such that the file can be played back by a player with the capability 26143_VIDEO_AVC-FullHD,

-	the AVC-FullHD-Enc encoding capabilities as defined in clause 5.4 of TS 26.265 [XY] to generate a bitstream from the video signal

-	the capability to generate an ISO BMFF track from the bitstream that conforms with the requirements of the sample entry 'avc1' as defined in ISO/IEC 14496-15 [15].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using video/mp4, profile="3gp9" codecs=" avc1.640029" or an equivalently compatible media type.

The capability 26143_VIDEO_ENC_HEVC-HD for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from a video signal in real-time, such that the file can be played back by a player with the capability 26143_VIDEO_HEVC-HD,

-	the HEVC-HD-Enc encoding capabilities as defined in clause 5.4 of TS 26.265 [XY] to generate a bitstream from the video signal

-	the capability to generate an ISO BMFF track from the bitstream that conforms with the requirements of the sample entry 'hvc1' as defined in ISO/IEC 14496-15 [15].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using video/mp4, profile="3gp9" codecs="hvc1.1.2.L93.B0" or an equivalently compatible media type.

The capability 26143_VIDEO_ENC_HEVC-FullHD for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from a video signal in real-time, such that the file can be played back by a player with the capability 26143_VIDEO_HEVC-FullHD,

-	the HEVC-FullHD-Enc encoding capabilities as defined in clause 5.4 of TS 26.265 [XY] to generate a bitstream from the video signal

-	the capability to generate an ISO BMFF track from the bitstream that conforms with the requirements of the sample entry 'hvc1' as defined in ISO/IEC 14496-15 [15].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using video/mp4, profile="3gp9" codecs="hvc1.2.4.L123.B0" or an equivalently compatible media type.

The capability 26143_VIDEO_ENC_HEVC-UHD for a content generator is defined as the combination of the following capabilities:

-	the capability to generate a file from a video signal in real-time, such that the file can be played back by a player with the capability 26143_VIDEO_HEVC-UHD,

-	the HEVC-UHD-Enc encoding capabilities as defined in clause 5.4 of TS 26.265 [XY] to generate a bitstream from the video signal

-	the capability to generate an ISO BMFF track from the bitstream that conforms with the requirements of the sample entry 'hvc1' as defined in ISO/IEC 14496-15 [15].

-	the generation of a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability as defined in clause 5.2.

-	the provisioning of media type signalling with the generated file using video/mp4, profile="3gp9" codecs="hvc1.2.4.L153.B0" or an equivalently compatible media type.

* * * End of Changes * * * *