Agenda item: 	9.7

Source: 	Qualcomm Inc.

Title: 	[FS_3DGS_MED] GS generation and rendering

Document for	Discussion and Agreement

# Introduction

In this contribution, we describe state of the art capture and rendering of gaussian splats.

# 3DGS Capture and Rendering

## 2.1		Capture

### 2.1.1	Self-calibration and sparse reconstruction

Given frames Ik with unknown intrinsics/extrinsics {Kk,Rk,tk}:

- Feature detection and matching: detect keypoints (e.g., using SIFT or ORB), build tracks and a view graph, then perform geometric verification.

- Relative pose: estimate the essential matrix E via a 5-point solver within RANSAC; decompose to (R,t) and select the cheirality-consistent solution.

- Incremental SfM + Bundel Adjustments: grow the reconstruction, triangulate {μi}, and refine intrinsics/extrinsics/structure with bundle adjustment (BA) using a robust loss and optional radial distortion κ.

This translates into the following optimization problem, with observations xk,i and projection π:

min⁡{Kk,Rk,tk},{μi}k,i∈Vρ(∥xk,i-π (Kk,Rk,tk;μi,κ)∥22)

This equation minimizes the reprojection error between observed 2D points and projected 3D points across multiple views, where:

- {Kk,Rk,tk}- Camera parameters for each view k:

  - Kk: Intrinsic calibration matrix (focal length, principal point, skew)

  - Rk: Rotation matrix (camera orientation)

  - tk: Translation vector (camera position)

- {μi}- 3D coordinates of the i-th point in the scene (the sparse reconstruction)

- V: Visibility set, pairs (k,i) where point i is visible in camera k

- xk,i: Observed 2D pixel coordinates of point iin image k

- π(Kk,Rk,tk;μi,κ): Projection function that maps 3D point μi to 2D image coordinates using camera parameters and optional distortion κ

- ∥⋅∥22: Squared L2 norm for squared Euclidean distance

- ρ(⋅): Robust loss function (e.g., Huber, Cauchy) to reduce influence of outliers

The goal is to minimize the difference between where points actually appear in images (xk,i) and where they should appear given the estimated 3D structure and camera parameters (π(...)).

### 2.1.2	Gaussian initialization

From the sparse cloud, instantiate an anisotropic Gaussian per point/patch with mean μi, rotation Ri, scales Si=diag(σi,x,σi,y,σi,z), and covariance Σi=RiSi2Ri T.

Each splat has opacity scale τi and view-dependent color encoded by SH coefficients {βi,lm}l≤L, a standard low-order lighting basis.

### 2.1.3	Projective footprint

In camera k:

xcam=Rk(μi-tk),Σcam=RkΣiRk ⊤.

Using perspective Π([X,Y,Z]⊤)=[fxX/Z+cx,  fyY/Z+cy]⊤with Jacobian JΠ at xcam, the screen-space ellipse is

pi=Π (Kkxcam),Σ2D=JΠ Σcam JΠ⊤.

For pixel u,

ωi(u)=e-12(u-pi)⊤Σ2D-1(u-pi)⁡,αi(u)=1-e-τi ωi(u),
ci(v)=l=0Lm=-llβi,lm Ylm(v)

Front-to-back alpha compositing (Porter–Duff) yields [1]:

C(u)=iTi-1(u) αi(u) ci(v),Ti-1(u)=j<i(1-αj(u)).

### 2.1.4	Loss and optimization

For training, we supervise the reconstruction using observed frames {Ik} and corresponding masks {Mk} with the following objective:

L=λRGBL1(Irender,Igt)+λSSIMLSSIM(Irender,Igt)+Lmask

where LSSIM=1-SSIM(Irender,Igt) follows the structural similarity metric of [2].

The 3DGS generation becomes a differentiable optimization problem where the entire rendering pipeline, from 3D Gaussian parameters through projection, alpha blending, to final pixel colors, can be backpropagated through to compute gradients.

This differentiability enables machine learning techniques to automatically refine all parameters simultaneously: 3D positions {μᵢ}, covariances {Σᵢ}, opacities {αᵢ}, colors {cᵢ}, and camera parameters {Kₖ, Rₖ, tₖ}. Machine learning optimizers like Adam then use gradients to iteratively minimize error through gradient descent, just as in neural network training.

## Rendering

### 2.2.1	Spatial Binning to Image Tiles

Each 3D Gaussian projects to a 2D ellipse characterized by its center pi and covariance matrix Σ2D. We compute an axis-aligned bounding box for each ellipse by extracting the kσ contour (typically with k≈3) from Σ2D using either eigen-decomposition or direct bounds computation.

Each Gaussian is then assigned to all overlapping 16×16 pixel tiles in the image. For each tile, we store pairs (i,zi) where i is the Gaussian index and zi is the camera-space depth of the Gaussian's center, or an occlusion-aware depth proxy [1].

### 2.2.2	Front-to-Back Ordering via Radix Sort

To achieve correct alpha blending, Gaussians within each tile must be sorted by depth in front-to-back order. We quantize the depth values to fixed-width integer keys:

di=⌊s⋅zi⌋∈[0,224-1]

where s is a scaling factor that maps the depth range to the integer key space. We then perform a stable least-significant-digit (LSD) radix sort per tile, typically using either three 8-bit passes or two 12-bit passes [3].

### 2.2.3	Per-Pixel Fused Evaluation and Early Termination

For each pixel u within a tile, we iterate through the depth-sorted Gaussian indices. For each Gaussian that passes the ellipse membership test:

(u-pi)⊤Σ2D-1(u-pi)≤k2
we compute the Gaussian weight ωi(u) and opacity αi(u) in a fused manner to minimize memory bandwidth consumption. We then accumulate the color and transmittance using the over operator [4]:

C←C+Tαi(u)ci(v),T←T(1-αi(u))

where Cis the accumulated color, Tis the remaining transmittance, and ci(v)is the view-dependent color of Gaussian i evaluated at viewing direction v. The iteration terminates early when the transmittance falls below a threshold T<ϵ, as subsequent Gaussians contribute negligibly to the final pixel color.

### 2.2.4	Numerical Precision and Robustness

All accumulation operations use float32 (32-bit floating point) precision with alpha values clamped to the range α∈[0,1]. The transmittance T is biased away from zero to avoid denormal floating-point numbers, which can significantly degrade performance. The depth scaling factor s must be chosen to preserve correct near-to-far ordering throughout the depth range, with special consideration for reverse-Z depth buffer configurations if employed. As an optional optimization, per-tile occlusion culling can terminate processing once the cumulative transmittance for all pixels in the tile drops below a threshold, thereby reducing computational work in densely populated regions of the scene.

# Proposal

We propose to agree this contribution and document the content of section 2 in the TR.

# References

[1]	 	Kerbl et al. “3D Gaussian Splatting for Real-Time Radiance Field Rendering”, ACM Transactions on Graphics, volume 42(4), July 2023

[2] 	Wang et al., “Image Quality Assessment: From Error Visibility to Structural Similarity”, IEEE TRANSACTIONS ON IMAGE PROCESSING, VOL. 13, NO. 4, APRIL 2004

[3] 	Satish et al., “Designing Efficient Sorting Algorithms for Manycore GPUs”, Proceedings of IEEE International Symposium on Parallel & Distributed Processing (pp. 1-10).

[4] 	Porter et al., “Compositing Digital Images”, Computer Graphics (SIGGRAPH '84 Proceedings), 18(3), 253-259.